| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0356 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0223 |
| --- | --- |

| Title: | 3D/MV-HEVC HLS: Dependency signaling for extending the supported number of layers |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gerhard Tech Karsten Suehring Robert Skupin Yago Sanchez Thomas Schierl | Email: | gerhard.tech@hhi.fraunhofer.de karsten.suehring@hhi.fraunhofer.de robert.skupin@hhi.fraunhofer.de yago.sanchez@hhi-extern.fraunhofer.de thomas.schierl@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

In this contribution an alternative method for dependency signalling for "Extending the supported number of layers" as presented in JCT3V-E0092 is proposed. It is suggested to signal layer dependencies for the layer id extension mechanism hierarchically. The existing direct_dependency_flag syntax element is used to indicate dependencies between layer subsets, whereby a subset consists of the layers associated with the same particular nuh_layer_id value. Only for dependent subsets, dependencies for the layers within the subsets are signalled. This sparse representation of dependencies allows bit savings in the VPS. Moreover the hierarchical approach enables network elements to determine dependencies based on nuh_layer_id and direct_dependency_flag values, without considering the extension mechanism in the slice header.

# Introduction

HEVC allows 64 values of nuh_layer_id to be used for scalable layers, views and depth. This implies a constraint of max. 64 views or 32 view plus depth combinations. For the most common application this is totally sufficient. On the other hand we see huge camera arrays with more the 100 views showing up for future applications like light field imaging [1], [2] or holographic displays [3].

In contribution JCT3V-E0092 we proposed an extension mechanism for nuh_layer_id that does not hurt coding performance in the common cases (i.e. which does not change the NAL unit header) but allows increasing the number of views.

This contribution proposes an hierarchical signalling of dependencies, which can be used as an alternative to the syntax and semantics proposed in JCT3V-E0092. Hierarchical signalling enables a sparse representation of layer dependencies and allows that extraction of layers can still be carried out based on direct_dependency_flag syntax elements and nuh_layer_id values.

In current MV-HEVC design and in JCT3V-E0092 dependencies between layers are signalled using a binary matrix. For each layer a binary pattern is signalled indicating the dependency of the layer from other layers.

When using the extension mechanism, the dependency signalling has two drawbacks:

- Since the number of bits used for signalling is proportional to the squared number of used layers the number of bits would highly increase.

- Network elements would not be able to derive dependencies based on nuh_layer_id and direct_dependency_flag values.

To overcome these drawbacks a hierarchical sparse dependency signalling is proposed.

# Basic idea

It is proposed to signal dependencies hierarchically on two different levels.

At the first level direct_dependency_flag syntax elements from the current design are utilized. However, instead of using theses flags to indicate that a dependent layer depends on reference layer, they are used to signal that one or more layers of a set of layers associated with a nuh_layer_id (dependent set) depend on one or more layers of a set of layers associated with another nuh_layer_id (reference set).

At the second level for each dependency between sets signalled at the first level, a binary matrix is signalled to indicate the dependencies of the layers in dependent set from layers in the reference set. When it is signalled at the first level that two sets of layers are not dependent, the binary matrix at the second level is not signalled but each element is inferred to be equal to 0.

Figure 1 Hierarchical dependency signaling

Dependencies between layers associated with the different nuh_layer_id

An example of the hierarchically dependency signalling is depicted in Figure 1. It is assumed that each nuh_layer_id value is associated with a layer set containing four layers and that each nuh_layer_id is mapped to a value vpsLIdx, in the range of 0 to vps_max_num_layer_minus1, inclusive, with layer_id_in_nuh[ vpsLIdx ] equal to nuh_layer_id.

The dependency matrix signalled by direct_dependency_flag syntax elements is depicted in the middle of figure. In vertical direction the dependent vpsLIdx values are given. Values ordered in horizontal direction represent the reference vpsLIdx values. When a set of layers depends on another set of layers, this is indicated by a shaded box in this example. For each dependency between two sets of layers a sub-matrix (a,b,c,d in the example) is signalled indicating the dependencies of layers associated with the depended vpsLIdx value of the layers associated with the reference vpsLIdx.

For e.g. (c) layers associated with vpsLIdx equal to 2 depend on the layers associated with vpsLIdx 1, therefore the sub-matrix depicted on the bottom left is signalled. Values in vertical direction next to the sub-matrix denote the layer_id_ext values of the dependent layers associated with vpsLIdx equal to 2. In the horizontal direction the reference layer_id_ext values of layers associated with vpsLIdx 1 are given. Again dependencies between the associated layers are indicated by a shaded box in the sub-matrix.

Dependencies between layers associated with the same nuh_layer_Id

To indicate dependencies between layers associated with the same vpsLIdx value, an additional direct_dependency_flag is signalled for each vpsLIdx value. When this flag is equal to 1, the layer set depends on itself.

As it can be seen from the example that layers associated with vpsLIdx equal to 1 can depend on layers associated with vpsLIdx equal to 1 (b). In such a case the signalling of a triangular matrix is sufficient, since a layer having a particular layer_id_ext value and being associated with a particular vpsLIdx value shall not depend on a layer being associated with the same particular vpsLIdx value and having a layer_id_ext value greater of equal than the particular layer_id_ext value.

layer_id_in_nuh, layer_id_in_nalu and LayerIdxInVps

In JCT3V-E0092 the syntax element layer_id_in_nuh was replaced by the syntax element layer_id_in_nalu to map a VPS layer indices from the extended range from 0 to ( VpsMaxNumLayers− 1 ) to the LayerId values of the extended range. For hierarchical dependency signalling the layer_id_nuh values are still utilized. Therefore layer_id_in_nalu values are not signalled explicitly, but a variable layerIdInNalu is derived from the layer_id_in_nuh values. LayerId values can be mapped to the VPS layer indices, in the range of 0 to ( VpsMaxNumLayers− 1 ), inclusive by using the LayerIdxInVps variable.

# Syntax and semantics

The specifications proposed in JCT3V-E0092 apply, with the following modifications and additions:

Please note that changes from JCT3V-E0092 are marked yellow, whereas changes proposed in this document are marked turquoise.

## VPS extension

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_layer_flag | u(1) |
| splitting_flag | u(1) |
| layer_id_ext_len | u(4) |
| if( layer_id_ext_len > 0 ) |  |
| max_layer_id_ext | u(v) |
| for( i = 0, NumScalabilityTypes = 0; i < 16; i++ ) { |  |
| scalability_mask[ i ] | u(1) |
| NumScalabilityTypes += scalability_mask[ i ] |  |
| } |  |
| for( j = 0; j < ( NumScalabilityTypes − splitting_flag ); j++ ) |  |
| dimension_id_len_minus1[ j ] | u(4) |
| vps_nuh_layer_id_present_flag | u(1) |
| for( i = 1, m = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| for( n = 0; n < MaxNumExtLayers; n++, m++ ) { |  |
| LayerIdInNalu[ m ] = ExtendedLayerId( layer_id_in_nuh[ i ] , n ,layer_id_ext_len ) |  |
| if( !splitting_flag ) |  |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ m ][ j ] | u(v) |
| } |  |
| } |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i + ( layer_id_ext_len > 0 ? 1 : 0); j++ ) { |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| if ( direct_dependency_flag[ i ][ j ] && ( layer_id_ext_len > 0 ) ) |  |
| for( k = ( ( i = = j ) ? 1 : 0 ); k < MaxNumExtLayers; k++ ) |  |
| for( l = 0; l < ( ( i = = j ) ? k : MaxNumExtLayers ); l++ ) |  |
| direct_ext_dependency_flag[ i ][ j ][ k ][ l ] | u(1) |
| } |  |
| for( i = 0; i< VpsMaxNumLayers − 1; i++ ) |  |
| max_tid_il_ref_pics_plus1[ i ] | u(3) |
| vps_number_layer_sets_minus1 | u(10) |
| vps_num_profile_tier_level_minus1 | u(6) |
| for( i = 1; i <= vps_num_profile_tier_level_minus1; i ++ ) { |  |
| vps_profile_present_flag[ i ] | u(1) |
| if( !vps_profile_present_flag[ i ] ) |  |
| profile_ref_minus1[ i ] | u(6) |
| profile_tier_level( vps_profile_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| numOutputLayerSets = vps_number_layer_sets_minus1 + 1 |  |
| more_output_layer_sets_than_default_flag | u(1) |
| if( more_output_layer_sets_than_default_flag ) { |  |
| num_add_output_layer_sets_minus1 | u(10) |
| numOutputLayerSets += num_add_output_layer_sets_minus1 + 1 |  |
| } |  |
| if( numOutputLayerSets > 1 ) |  |
| default_one_target_output_layer_flag | u(1) |
| for( i = 1; i < numOutputLayerSets; i++ ) { |  |
| if( i > vps_number_layer_sets_minus1 ) { |  |
| output_layer_set_idx_minus1[ i ] | u(v) |
| lsIdx = output_layer_set_idx_minus1[ i ] + 1 |  |
| for( j = 0 ; j < NumLayersInIdList[ lsIdx ] − 1; j++) |  |
| output_layer_flag[ i ][ j ] | u(1) |
| } |  |
| profile_level_tier_idx[ i ] | u(v) |
| } |  |
| max_one_active_ref_layer_flag | u(1) |
| direct_dep_type_len_minus2 | ue(v) |
| for( i = 1; i <= VpsMaxNumLayers − 1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| if( DirectDependencyFlag[ i ][ j ] ) |  |
| direct_dependency_type[ i ][ j ] | u(v) |
| vps_shvc_reserved_zero_flag | u(1) |
| } |  |

layer_id_ext_len specifies the length, in bits, of the layer_id_ext and the max_layer_id_ext syntax element.

max_layer_id_ext specifies the maximum value of the layer_id_ext_syntax element. The length, in bits, of max_layer_id_ext syntax element is equal to layer_id_ext_len. When not present max_layer_id_ext is inferred to be equal to 0.

The variable MaxNumExtLayers is set equal to max_layer_id_ext + 1

The variable VpsMaxNumLayers specifying the maximum number of layers in the extended layer range is set equal to  vps_max_layers_minus1 * MaxNumExtLayers + 1.

The function ExtendedLayerId( nuhLayerId, extensionId ,extLen ) is defined as follows:

ExtendedLayerId( nuhLayerId, extId ,extLen ) = ( nuhLayerId <<  extLen ) + extId

layer_id_in_nuh[ i ] specifies the value of the nuh_layer_id syntax element in VCL NAL units of the i-th layer. For i in a range from 0 to vps_max_layers_minus1, inclusive, when not present, the value of layer_id_in_nuh[ i ] is inferred to be equal to i.

When i is greater than 0, layer_id_in_nuh[ i ] shall be greater than layer_id_in_nuh[ i – 1 ].

For i in a range from 0 to VpsMaxNumLayers − 1, inclusive, the variable LayerIdxInVps[ LayerIdInNalu[ i ] ] is set equal to i.

direct_dependency_flag[ i ][ j ] equal to 0 specifies that the layers with index j are not a direct reference layers for the layers with index i. direct_dependency_flag[ i ][ j ] equal to 1 specifies that the layers with index j may be direct reference layers for the layers with index i. When direct_dependency_flag[ i ][ j ] is not present for i and j in the range of 0 to vps_max_layers_minus1, it is inferred to be equal to 0.

NOTE – When layer_id_ext_len is greater than 0, direct_dependency_flag[ i ][ j ] signals that dependencies between layers associated with nuh_layer_id values layer_id_in_nuh[ i ] and layer_id_in_nuh[ j ] may exist. When layer_id_ext_len is equal to 0 only one layer is associated with each nuh_layer_id value. .

direct_ext_dependency_flag[ i ][ j ][ k ][ l ] equal to 0 specifies that the l-th layer associated with layer_id_in_nuh[ j ] is not a direct reference layer for the k-th layer associated with layer_id_in_nuh[ i ]. direct_ext_dependency_flag[ i ][ j ][ k ][ l ] equal to 1 specifies that the l-th layer associated with layer_id_in_nuh[ j ] may be a direct reference layer for the k-th layer associated with layer_id_in_nuh[ i ]. When not present direct_ext_dependency_flag[ i ][ j ][ k ][ l ] is inferred to be equal to ( layer_id_ext_len  >  0 ) ? 0 : direct_dependency_flag[ i ][ j ]. The variable DirectDependencyFlag[ m ][ n ] is derived as specified in the following:

m = LayerIdxInVps[ ExtendedLayerId( layer_id_in_nuh[ i ], k, layer_id_ext_len ) ]  
n = LayerIdxInVps[ ExtendedLayerId( layer_id_in_nuh[ j ], l , layer_id_ext_len ) ] ]
DirectDependencyFlag[ m ][ n ] = direct_ext_dependency_flag[ i ][ j ][ k ][ l ]

direct_dependency_type[ i ][ j] is used ...

for( i = 1; i  <=  VpsMaxNumLayers − 1; i++ ) {
	iNuhLId = LayerIdInNalu[ i ]
	for( j = 0; j < i; j++ )
		if( DirectDependencyFlag[ i ][ j ] ) {
			RefLayerId[ iNuhLId ][ NumDirectRefLayers[ iNuhLId ]++ ] = LayerIdInNalu[ j ]

SamplePredEnabledFlag[ iNuhLId ][ j ] = ( ( direct_dependency_type[ i ][ j ] + 1 ) & 1 )
			NumSamplePredRefLayers[ iNuhLId ] += SamplePredEnabledFlag[ iNuhLId ][ j ]
			MotionPredEnabledFlag[ iNuhLId ][ j ] = ( ( ( direct_dependency_type[ i ][ j ] + 1 ) & 2 ) >> 1 )
			NumMotionPredRefLayers[ iNuhLId ] += MotionPredEnabledFlag[ iNuhLId ][ j ]
		}

}

## Conclusion

A method for hierarchically sparse dependency signalling has been proposed. Benefits of proposed approach are:

- Compatibility with nuh_layer_id value based dependency signalling.

- Reduction of the number of bits used for dependency signalling.

# References

- WILBURN, Bennett, et al. High performance imaging using large camera arrays. ACM Transactions on Graphics, 2005, 24. Jg., Nr. 3, S. 765-776.

- WILBURN, Bennett S., et al. Light field video camera. In: Electronic Imaging 2002. International Society for Optics and Photonics, 2001. S. 29-36.

- HORIMAI, Hideyoshi, et al. Full-color 3D display system with 360 degree horizontal viewing angle. In: Proc. Int. Symposium of 3D and Contents. 2010. S. 7-10.

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e.V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).