| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0357 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0224 |
| --- | --- |

| Title: | 3D/MV-HEVC HLS: Flexible layer clustering for extending the supported number of layers |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gerhard Tech Karsten Suehring Robert Skupin Yago Sanchez Thomas Schierl | Email: | gerhard.tech@hhi.fraunhofer.de karsten.suehring@hhi.fraunhofer.de robert.skupin@hhi.fraunhofer.de yago.sanchez@hhi-extern.fraunhofer.de thomas.schierl@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

In this contribution an alternative derivation process or LayerId values for "Extending the supported number of layers" as presented in JCT3V-E0092 is proposed. The proposal targets a flexible clustering of layers. In JCT3V-E0092 it was suggested that each nuh_layer_id value not equal to 0 can be associated with multiple layers. Here it is proposed that the restriction to have only one associated layer applies for a larger subset of nuh_layer_id values. Only particular nuh_layer_id values are allowed to be associated with multiple layers (in the following called extendable nuh_layer_id values). Extendable and not extendable nuh_layer_id values are then combined to create a variable number of clusters each including the same number of layers. Finally the extended layer id values are derived from cluster and extension indices. The proposed method can be combined with hierarchical dependency signalling as proposed in JCT3V-E0224.

# Introduction

HEVC allows 64 values of nuh_layer_id to be used for scalable layers, views and depth. If no scalability is used, this implies a constraint of max. 64 views or 32 view plus depth combinations. For the most common application this is totally sufficient. On the other hand we see huge camera arrays with more the 100 views showing up for future applications like light field imaging [1], [2] or holographic displays [3].

In contribution JCT3V-E0092 we proposed an extension mechanism for nuh_layer_id that does not hurt coding performance in the common cases (i.e. which does not change the NAL unit header) but allows increasing the number of views.

This contribution proposes an alternative derivation of LayerId values, which can be applied instead of the derivation method in presented in JCT3V-E0092. In JCT3V-E0092 clustering of layers is proposed to overcome the restriction imposed by the 6 bit nuh_layer_id value to have a maximum of 64 layers in a bitstream. Therefore each nuh_layer_id value is allowed to be associated with multiple layers. The effective layer id value (LayerId) is derived from the nuh_layer_id value and an extension value (layer_id_ext) signalled in the slice segment header, parameter sets and SEI messages.

Figure 1: Clustering of layers, each nuh_layer_id value (apart from 0) is associated with a set of layers and represents a cluster.

An example for clustering is depicted in Figure 1. It can be seen that each nuh_layer_id value is associated with a cluster of layers, apart from nuh_layer_id value 0 which cannot be extended since backward compatibility is required. Therefore the number of layers cannot be equal in all clusters.

Dependencies between the variables can be described by the following equations, where nuh_layer_id and layer_id_ext_len are signalled in the VPS and layer_id_ext is signalled in the slice header, when layerIdExtPresentFlag is equal to 1.

clusterIdx	= nuh_layer_id

layerIdExtPresentFlag	= ( nuh_layer_id > 0 )

layerIdExt	= layerIdExtPresentFlag ? layer_id_ext : 0

extensionIdx	= layerIdExt

LayerId	= ( clusterIdx  << ( layer_id_ext_len ) ) + extensionIdx

# Proposal

## Indication of extendibility by LSB of nuh_layer_id ( Special case of the proposal )

The method for flexible layer clustering proposed in this document allows having an equal number of layers per cluster. Therefore also a subset of nuh_layer_id values not equal to 0 is restricted to be associated with only one layer. These non-extendable nuh_layer_id values are then combined with other extendable nuh_layer_id to form clusters with an equal number of layers.

An example for flexible layer clustering is depicted in Figure 2. Here the five most significant bits are used to derive the cluster index (clusterIdx). Hence two nuh_layer_id that differ in the least significant bit values belong to each cluster. When the least significant bit is equal to 0 the nuh_layer_id value is not extendable and the layer associated with the nuh_layer_id is the layer with extentionIdx 0 in the cluster. When the least significant bit of nuh_layer_id is equal to 1, the nuh_layer_id is extendable and associate with multiple layers. In this case the layer_id_ext values for the associated layers are signalled in slice segment headers and the extentionIdx within the cluster is equal to ( 1 + layer_id_ext ).

Figure 2: Flexible layer clustering: Every second nuh_layer_id value is associated with a set of layers. These layers form together with the layer associated with the preceding nuh_layer_id value a cluster.

Following equations summarize the derivation of variables used for indexing:

clusterIdx	= nuh_layer_id  >>  1

layerIdExtPresentFlag	= ( ( nuh_layer_id & 1 ) = = 1 )

layerIdExt	= layerIdExtPresentFlag ? layer_id_ext : 0

extensionIdx	= layerIdExt + ( nuh_layer_id & 1 )

LayerId	= ( clusterIdx  << ( layer_id_ext_len ) ) + extensionIdx

## Proposal: Indication of extendibility by the N LSBs of nuh_layer_id

The idea of using the least significant bit of nuh_layer_id to indicate extendibility can be generalized to use N bits instead. With this method the ( 6 – N ) most significant bits are used to derive the clusterIdx value. Hence ( 1  <<  N ) nuh_layer_id values belong to each cluster. When the N least significant bits represent a value smaller than the maximal value they could represent ( ( 1  <<  N ) – 1 ) the nuh_layer_id value is not extendable and the layers associated with the nuh_layer_id value is the layer with extensionIdx value equal to the value represented by the N least significant bits of nuh_layer_id values. When the N least significant bits of nuh_layer_id are equal to the maximal value they could represent, the nuh_layer_id is extendable and associated with multiple layers. The layer_id_ext values for the associated layers are then signalled in slice headers and the extensionIdx within the cluster is ( 1 << N ) – 1 + layer_id_ext.

It is proposed to signal the value of N ( layer_id_ext_offset_len ) in the bitstream.

Following equations summarize the derivation of variables used for indexing, with layer_id_ext_offset_len signalled in the VPS:

clusterIdx	= nuh_layer_id >> N

maxLayerIdOffset 	= ( 1 << N ) – 1

layerIdExtPresentFlag	= ( nuh_layer_id & maxLayerIdOffset ) = = maxLayerIdOffset

layerIdExt	= layerIdExtPresentFlag ? layer_id_ext : 0

extensionIdx	= layerIdExt + ( nuh_layer_id & maxLayerIdOffset )

LayerId	= ( clusterIdx  << layer_id_ext_len ) + extensionIdx

Further examples for the flexible clustering are given in Figure 3 for N equal to 2 and in Figure 4 for N equal to 6. Please note that for N equal to 6 only nuh_layer_id 63 is extended.

Figure 3: Flexible clustering for N = 2. Every forth nuh_layer_id value is associated with a set of layers. These layers form together with the layers associated with the three preceding nuh_layer_id values a cluster.

Figure 4: Clustering for N = 6. Only nuh_layer_id equal to 63 is associated with a set of layers. These layers form together with the layers associated with the 63 preceding nuh_layer_id values a cluster.

## Combination with Hierarchical dependency signalling

The proposal can also be combined with hierarchical dependency signalling as described in JCT3V-E0223. An example for this is depicted in Figure 5. Here the vertical and horizontal size of the sub matrices can differ depending on number of layers associated with the dependent and reference layer set, respectively. When only one layer is associated with the dependent as well as with the reference layer set, the signalling of the sub-matrix can be omitted (e).

Figure 5 Hierarchical dependency signaling with flexible clustering. In the example only uneven indices are associated with four layers. Even indices are associated with one layer only.

# Syntax and semantics

Syntax and semantics are based on changes proposed in JCT3V-D0092 (yellow) and JCT3V-D0223 (turquoise). Changes proposed in this document are highlighted (green).

## VPS extension

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_layer_flag | u(1) |
| splitting_flag | u(1) |
| layer_id_ext_len | u(3) |
| if( layer_id_ext_len > 0 ) { |  |
| max_layer_id_ext | u(v) |
| layer_id_ext_offset_len | u(3) |
| } |  |
| for( i = 0, NumScalabilityTypes = 0; i < 16; i++ ) { |  |
| scalability_mask[ i ] | u(1) |
| NumScalabilityTypes += scalability_mask[ i ] |  |
| } |  |
| for( j = 0; j < ( NumScalabilityTypes − splitting_flag ); j++ ) |  |
| dimension_id_len_minus1[ j ] | u(3) |
| vps_nuh_layer_id_present_flag | u(1) |
| for( i = 1, m = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| for( n = 0; n < maxNumExtLayers[ i ]; n++, m++ ) { |  |
| LayerIdInNalu[ m ] = ExtendedLayerId( layer_id_in_nuh[ i ], n) |  |
| if( !splitting_flag ) |  |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ m ][ j ] | u(v) |
| } |  |
| } |  |
| VpsMaxNumLayers = m |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i + ( layer_id_ext_len > 0 ? 1 : 0); j++ ) { |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| extPresentFlag[ i ][ j ] = ( LayerIdExtFlag[ i ] ¦ ¦ LayerIdExtFlag[ j ] ) |  |
| if ( direct_dependency_flag[ i ][ j ] && extPresentFlag[ i ][ j ]) |  |
| for( k = ( ( i = = j ) ? 1 : 0 ); k < maxNumExtLayers[ i ] ; k++ ) |  |
| for( l = 0; l < ( ( i = = j ) ? k : maxNumExtLayers[ j ] ); l++ ) |  |
| direct_ext_dependency_flag[ i ][ j ][ k ][ l ] | u(1) |
| } |  |
| for( i = 0; i< VpsMaxNumLayers − 1; i++ ) |  |
| max_tid_il_ref_pics_plus1[ i ] | u(3) |
| vps_number_layer_sets_minus1 | u(10) |
| vps_num_profile_tier_level_minus1 | u(6) |
| for( i = 1; i <= VpsMaxNumLayers − 1; i ++ ) { |  |
| vps_profile_present_flag[ i ] | u(1) |
| if( !vps_profile_present_flag[ i ] ) |  |
| profile_ref_minus1[ i ] | u(6) |
| profile_tier_level( vps_profile_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| numOutputLayerSets = vps_number_layer_sets_minus1 + 1 |  |
| more_output_layer_sets_than_default_flag | u(1) |
| if( more_output_layer_sets_than_default_flag ) { |  |
| num_add_output_layer_sets_minus1 | u(10) |
| numOutputLayerSets += num_add_output_layer_sets_minus1 + 1 |  |
| } |  |
| if( numOutputLayerSets > 1 ) |  |
| default_one_target_output_layer_flag | u(1) |
| for( i = 1; i < numOutputLayerSets; i++ ) { |  |
| if( i > vps_number_layer_sets_minus1 ) { |  |
| output_layer_set_idx_minus1[ i ] | u(v) |
| lsIdx = output_layer_set_idx_minus1[ i ] + 1 |  |
| for( j = 0 ; j < NumLayersInIdList[ lsIdx ] − 1; j++) |  |
| output_layer_flag[ i ][ j ] | u(1) |
| } |  |
| profile_level_tier_idx[ i ] | u(v) |
| } |  |
| max_one_active_ref_layer_flag | u(1) |
| direct_dep_type_len_minus2 | ue(v) |
| for( i = 1; i <= VpsMaxNumLayers − 1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| if( DirectDependencyFlag[ i ][ j ] ) |  |
| direct_dependency_type[ i ][ j ] | u(v) |
| vps_shvc_reserved_zero_flag | u(1) |
| } |  |

layer_id_ext_len specifies the length, in bits, of the layer_id_ext and the max_layer_id_ext syntax element.

max_layer_id_ext specifies the maximum value of the layer_id_ext syntax element. The length, in bits, of max_layer_id_ext syntax element is equal to layer_id_ext_len. When not present ,max_layer_id_ext is inferred to be equal to 0.

The variable MaxNumExtLayers is set equal to max_layer_id_ext + 1.

layer_id_ext_offset_len specifies the number of least significant bits of nuh_layer_id that are interpreted as an binary representation of an offset value for the derivation of value of the LayerId variable. When not present, layer_id_ext_offset_len is inferred to be equal to 0.

NOTE – The number of most significant bits of nuh_layer_id that can be interpreted as binary representation of the index of cluster variable is equal to (6 − layer_id_ext_offset_len).

The variable MaxLayerExtOffset, specifying a maximal offset, is set equal to ( 1  <<  layer_id_ext_offset_len ) − 1.

The function ExtendedLayerId( nuhLayerId, layerIdExt ) is defined as follows:

clusterIdx = ( nuhLayerId  >>  layer_id_ext_offset_len )
extensionIdx = layerIdExt + ( nuhLayerId & MaxLayerExtOffset )
ExtendedLayerId( nuhLayerId, layerIdExt ) =
					( clusterIdx  <<  layer_id_ext_len ) + extensionIdx

layer_id_in_nuh[ i ] specifies the value of the nuh_layer_id syntax element in VCL NAL units of the i-th layer. For i in a range from 0 to vps_max_layers_minus1, inclusive, when not present, the value of layer_id_in_nuh[ i ] is inferred to be equal to i.

When i is greater than 0, layer_id_in_nuh[ i ] shall be greater than layer_id_in_nuh[ i – 1 ].

For i in a range from 0 to vps_max_num_layer_minus1, inclusive, the variable NuhLayerIdToIdxInVps[ layer_id_in_nuh[ i ] ] is set equal to i.

For i in a range from 0 to VpsMaxNumLayers − 1, inclusive, the variable LayerIdxInVps[ LayerIdInNalu[ i ] ] is set equal to i.

The variable LayerIdExtFlag[ i ] is derived as specified in the following:

LayerIdExtFlag[ i ] = ( ( layer_id_in_nuh[ i ] & MaxLayerExtOffset ) = = MaxLayerExtOffset )

The variable maxNumExtLayers[ i ] specifying the number of layers associated with layer_id_in_nuh[ i ] is derived as specified in the following:

maxNumExtLayers[ i ] = layerIdExtFlag[ i ] ? MaxNumExtLayers : 1

direct_dependency_flag[ i ][ j ] equal to 0 specifies that the layers with index j are not a direct reference layers for the layers with index i. direct_dependency_flag[ i ][ j ] equal to 1 specifies that the layers with index j may be direct reference layers for the layers with index i. When direct_dependency_flag[ i ][ j ] is not present for i and j in the range of 0 to vps_max_layers_minus1, it is inferred to be equal to 0.

NOTE – When layer_id_ext_len is greater than 0, direct_dependency_flag[ i ][ j ] signals that dependencies between layers associated with nuh_layer_id values layer_id_in_nuh[ i ] and layer_id_in_nuh[ j ] may exist. When layer_id_ext_len is equal to 0 only one layer is associated with each nuh_layer_id value. .

direct_ext_dependency_flag[ i ][ j ][ k ][ l ] equal to 0 specifies that the l-th layer associated with layer_id_in_nuh[ j ] is not a direct reference layer for the k-th layer associated with layer_id_in_nuh[ i ]. direct_ext_dependency_flag[ i ][ j ][ k ][ l ] equal to 1 specifies that the l-th layer associated with layer_id_in_nuh[ j ] may be a direct reference layer for the k-th layer associated with layer_id_in_nuh[ i ]. When not present direct_ext_dependency_flag[ i ][ j ][ k ][ l ] is inferred to be equal to extPresentFlag[ i ][ j ] ? 0 : direct_dependency_flag[ i ][ j ].

The variable DirectDependencyFlag[ m ][ n ] is derived as specified in the following:

m = LayerIdxInVps[ ExtendedLayerId( layer_id_in_nuh[ i ], k ) ]  
n = LayerIdxInVps[ ExtendedLayerId( layer_id_in_nuh[ j ], l ) ]
DirectDependencyFlag[ m ][ n ] = direct_ext_dependency_flag[ i ][ j ][ k ][ l ]

## Slice segment header

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| … |  |
| if( !dependent_slice_segment_flag ) { |  |
| if ( nuh_layer_id > 0 && LayerIdExtFlag[ NuhLayerIdToIdxInVps[ nuh_layer_id ] ] ) { |  |
| layer_id_ext | u(v) |
| } |  |
| for( i = 0; i < (num_extra_slice_header_bits); i++ ) |  |
| slice_reserved_flag[ i ] | u(1) |
| … |  |
| } |  |

layer_id_ext is used in combination with nuh_layer_id, to identify the layer. The syntax element layer_id_ext is coded using layer_id_ext_len bits. layer_id_ext shall be in the range of 0 to max_layer_id_ext − MaxLayerExtOffset, inclusive. When not present, layer_id_ext is inferred to be equal to 0.

The variable LayerId is set equal to ExtendedLayerId( nuh_layer_id , layer_id_ext )

[ Ed. In parameter sets and SEI messages the value of layer_id_ext is also restricted as above. ]

# Conclusion

A method for flexible layer clustering has been proposed. Benefits of the proposal are:

- A flexible number of nuh_layer_id values that can constraint to be associated with only one layers.

- Possibility for network elements to identify nuh_layer_id values associated with multiple layers.

- Symmetric clustering of layers.

# References

- WILBURN, Bennett, et al. High performance imaging using large camera arrays. ACM Transactions on Graphics, 2005, 24. Jg., Nr. 3, S. 765-776.

- WILBURN, Bennett S., et al. Light field video camera. In: Electronic Imaging 2002. International Society for Optics and Photonics, 2001. S. 29-36.

- HORIMAI, Hideyoshi, et al. Full-color 3D display system with 360 degree horizontal viewing angle. In: Proc. Int. Symposium of 3D and Contents. 2010. S. 7-10.

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e.V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).