| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0358 |
| --- | --- |

| Title: | Cross-check for JCTVC-N0183 Non-RCE2: Enhanced angular intra prediction for screen content coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Matteo Naccari Marta Mark Block D, Centre House 56 Wood Lane, W12 7SB London, UK | Email: | matteo.naccari@bbc.co.uk marta.mrak@bbc.co.uk |
| Source: | BBC Research & Development |  |  |

_____________________________

# Abstract

This document reports the BBC’s cross-check results for Document JCTVC-N0183 proposed by Samsung. All the results released by the proponent are tested and it is reported that all the test points are confirmed. Analysis of the source code reveals perfect matching between the algorithm and the implementation.

# Introduction

Document JCTVC-N0183 proposes to change the bilinear prediction used intra coding to improve compression efficiency for screen content coding. In particular, it is observed that bilinear prediction is not well suited for screen content videos where sharp edges are present. As a solution it is proposed to disable bilinear prediction for some prediction directions and use the nearest reference sample instead. The selection between bilinear and nearest sample prediction is adaptive with respect to the type of content being encoded. More precisely, a criterion based on the absolute difference between the reference samples used for interpolation is devised. If this difference is greater than a given threshold, bilinear interpolation is disabled. The results for the method presented in Document JCTVC-N0183 are presented in the next section.

# Test results

The combination is tested for both lossless and lossy coding using the RCE2 test conditions described in Document JCTVC-M1122. This cross-check document reports the same results in the following tables.

Table 1: Compression efficiency for lossless coding.

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.3 | -0.1% | 31.7 | 31.7 | 0.0% | 49.8 | 49.7 | 0.0% |
| Class B | 2.2 | 2.2 | 0.0% | 2.6 | 2.6 | 0.0% | 2.6 | 2.6 | 0.0% |
| SC RGB 444 | 10.1 | 10.4 | -3.3% | 100.4 | 103.5 | -2.8% | 381.6 | 398.8 | -2.8% |
| SC YUV 444 | 11.4 | 11.4 | -0.3% | 128.9 | 129.1 | 0.0% | 325.6 | 327.8 | -0.3% |
| RangeExt | 2.4 | 2.4 | 0.0% | 2.5 | 2.5 | 0.0% | 2.5 | 2.5 | 0.0% |
| Enc Time[%] | 100% |  |  | 104% |  |  | 103% |  |  |
| Dec Time[%] | 99% |  |  | 108% |  |  | 108% |  |  |

Table 2: BD-rates for lossy coding.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.6% | -0.5% | -0.3% | -0.5% | -0.4% | -0.3% | -0.4% | -0.4% | -0.3% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -4.5% | -4.8% | -4.5% | -4.2% | -4.4% | -4.3% | -4.0% | -4.2% | -4.1% |
| SC YUV 444 | -0.9% | -0.5% | -0.4% | -0.7% | -0.3% | -0.3% | -0.5% | -0.1% | -0.1% |
| RangeExt | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  | 102% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 102% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.4% | -0.3% | -0.2% | -0.3% | -0.2% | -0.2% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -3.9% | -4.2% | -4.0% | -3.8% | -4.2% | -3.9% |  |  |  |
| SC YUV 444 | -0.7% | -0.4% | -0.3% | -0.3% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.3% | -0.2% | 0.2% | -0.2% | -0.2% | -0.2% |  |  |  |
| Class B | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | -0.1% |  |  |  |
| SC RGB 444 | -3.9% | -4.3% | -3.9% | -3.7% | -4.2% | -3.6% |  |  |  |
| SC YUV 444 | -0.3% | 0.1% | 0.2% | -0.2% | 0.1% | 0.2% |  |  |  |
| RangeExt | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% |  |  |  |
| Enc Time[%] | 101% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |  |  |  |

# Source code inspection

The proponent implemented the proposed changes in the HM-10.0-rext-3.0 reference software. Two source code versions have been released: one for lossless and one for lossy coding. The two versions are basically the same beside the different values for the macros which enable lossless coding in HM-10.0-rext-3.0. Therefore the reported inspection will be common for both lossy and lossless coding.

To enable the adaptive selection of bilinear prediction, the following macros have been defined:

RExt__NON_INTERPOLATE_INTRA_PREDICTION

RExt__NON_INTERPOLATE_INTRA_PREDICTION_SAMPLE_ADAPTIVE

The first macro enables the adaptive selection between bilinear and nearest sample prediction while the second modifies the intra prediction in the function “xPredIntraAng” of class TComPrediction. It is reported that the changes implemented in the source code reflect the algorithmic description in Document JCTVC-N0183.