| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0359 |
| --- | --- |

| Title: | Non-RCE1: Cross-check of JCTVC-N0223 In-loop Chroma Enhancement for HEVC Range Extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Liwei Guo 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | + 1-858-847-7532 liweig@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution verified the BD-rate results of JCTVC-N0223 which is proposed by Interdigital.

# Introduction

JCTVC-N0223 describes a method that a reconstructed chroma pixel is enhanced by adding an appropriate offset where the offset is derived by applying NxN filter on the surrounding luma pixels. The filter coefficients are derived by the encoder and transmitted using high level syntax.

In this contribution, we verified the results for the case of 5x5 filter support.

# Experimental results

The JCTVC-N0223 software provided by the proponents has been used to generate the results and compare with results generated with HM-10.1+RExt-3.0 under RCE1 common test conditions [1]. Encoding time and decoding time were obtained using heterogeneous cluster. It was confirmed that for the test case, the BD-rate results exactly match to the results provided by the proponents.

Table 1 Results of 444 coding

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic_2560x1600_30_10bit_444_crop.yuv | 0.0% | -1.7% | -7.6% | 0.0% | -0.9% | -4.5% | 0.0% | -0.5% | -2.1% |
| Kimono1_1920x1080_24_10bit_444.yuv | 0.1% | -2.7% | -7.3% | 0.0% | -0.6% | -3.0% | 0.0% | -0.2% | -1.0% |
| EBULupoCandlelight_1920x1080_50_10bit_444.yuv | 0.1% | -7.7% | -1.9% | 0.0% | -5.1% | -4.0% | 0.0% | -2.0% | -6.4% |
| EBURainFruits_1920x1080_50_10bit_444.yuv | 0.0% | -3.6% | -7.9% | 0.0% | -3.8% | -8.0% | 0.0% | -2.9% | -6.6% |
| VenueVu_1920x1080_30_10bit_444.yuv | 0.1% | -1.6% | -1.0% | 0.0% | -1.0% | -0.8% | 0.0% | -0.5% | -0.5% |
| BirdsInCage_1920x1080_60_10bit_444.yuv | 0.1% | -0.8% | -13.1% | 0.0% | -0.3% | -11.7% | 0.0% | -0.1% | -4.9% |
| CrowdRun_1920x1080_50_10bit_444.yuv | 0.0% | -2.7% | -3.3% | 0.0% | -0.7% | -1.4% | 0.0% | -0.1% | -0.5% |
| Overall | 0.0% | -3.0% | -6.0% | 0.0% | -1.8% | -4.8% | 0.0% | -0.9% | -3.1% |
| Enc Time[%] | #VALUE! |  |  | #VALUE! |  |  | #VALUE! |  |  |
| Dec Time[%] | 155% |  |  | 151% |  |  | 148% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Traffic_2560x1600_30_10bit_444_crop.yuv | -0.1% | -3.5% | -9.4% | -0.2% | -2.0% | -5.6% |  |  |  |
| Kimono1_1920x1080_24_10bit_444.yuv | 0.2% | -5.4% | -9.0% | 0.0% | -1.5% | -6.9% |  |  |  |
| EBULupoCandlelight_1920x1080_50_10bit_444.yuv | 1.2% | -8.3% | -1.4% | 0.5% | -8.3% | -5.2% |  |  |  |
| EBURainFruits_1920x1080_50_10bit_444.yuv | 0.3% | -5.0% | -9.4% | 0.0% | -6.5% | -11.6% |  |  |  |
| VenueVu_1920x1080_30_10bit_444.yuv | 0.2% | -1.8% | -1.2% | 0.1% | -1.5% | -1.1% |  |  |  |
| BirdsInCage_1920x1080_60_10bit_444.yuv | 2.6% | -1.5% | -22.0% | 0.7% | -1.0% | -23.4% |  |  |  |
| CrowdRun_1920x1080_50_10bit_444.yuv | 0.0% | -4.9% | -6.0% | -0.1% | -2.0% | -4.0% |  |  |  |
| Overall | 0.6% | -4.3% | -8.4% | 0.2% | -3.3% | -8.3% |  |  |  |
| Enc Time[%] | #VALUE! |  |  | #VALUE! |  |  |  |  |  |
| Dec Time[%] | 187% |  |  | 177% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Traffic_2560x1600_30_10bit_444_crop.yuv | -0.3% | -4.2% | -8.8% | -0.3% | -2.3% | -4.9% |  |  |  |
| Kimono1_1920x1080_24_10bit_444.yuv | 0.1% | -4.0% | -6.1% | -0.3% | -1.2% | -4.2% |  |  |  |
| EBULupoCandlelight_1920x1080_50_10bit_444.yuv | 1.7% | -3.3% | 1.2% | 0.9% | -2.6% | -0.3% |  |  |  |
| EBURainFruits_1920x1080_50_10bit_444.yuv | 0.1% | -3.6% | -6.3% | -0.3% | -5.0% | -8.8% |  |  |  |
| VenueVu_1920x1080_30_10bit_444.yuv | 0.0% | -0.8% | -0.9% | -0.1% | -0.7% | -0.9% |  |  |  |
| BirdsInCage_1920x1080_60_10bit_444.yuv | 3.9% | -0.8% | -23.0% | 0.8% | -0.8% | -22.9% |  |  |  |
| CrowdRun_1920x1080_50_10bit_444.yuv | -0.3% | -4.3% | -5.8% | -0.4% | -1.6% | -3.5% |  |  |  |
| Overall | 0.7% | -3.0% | -7.1% | 0.0% | -2.0% | -6.5% |  |  |  |
| Enc Time[%] | #VALUE! |  |  | #VALUE! |  |  |  |  |  |
| Dec Time[%] | 197% |  |  | 176% |  |  |  |  |  |

Table 2 Results of 422 coding

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | 0.0% | -3.3% | -9.9% | 0.0% | -2.1% | -6.8% | 0.0% | -1.1% | -3.8% |
| Kimono1_1920x1080_24_10bit_422.yuv | 0.1% | -6.8% | -12.2% | 0.0% | -1.8% | -7.1% | 0.0% | -0.6% | -2.9% |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.0% | -3.5% | -2.7% | 0.0% | -1.2% | -2.0% | 0.0% | -0.4% | -0.9% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | 0.0% | -0.3% | -0.5% | 0.0% | -0.2% | -0.3% | 0.0% | -0.1% | -0.1% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.0% | -3.3% | -10.0% | 0.0% | -1.0% | -4.1% | 0.0% | -0.3% | -1.2% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.0% | -12.8% | -7.8% | 0.0% | -5.0% | -2.9% | 0.0% | -1.4% | -0.9% |
| Seeking_1920x1080_50_10bit_422.yuv | 0.0% | -6.4% | -5.6% | 0.0% | -3.1% | -2.8% | 0.0% | -1.2% | -1.0% |
| Overall | 0.0% | -5.2% | -7.0% | 0.0% | -2.1% | -3.7% | 0.0% | -0.7% | -1.5% |
| Enc Time[%] | #VALUE! |  |  | #VALUE! |  |  | #VALUE! |  |  |
| Dec Time[%] | 147% |  |  | 143% |  |  | 141% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Traffic_2560x1600_30_10bit_422_crop.yuv | 0.0% | -6.0% | -13.1% | -0.1% | -4.4% | -9.7% |  |  |  |
| Kimono1_1920x1080_24_10bit_422.yuv | 0.3% | -10.0% | -13.8% | 0.0% | -3.8% | -11.7% |  |  |  |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.3% | -5.7% | -3.5% | 0.1% | -1.7% | -3.0% |  |  |  |
| EBUGraphics_1920x1080_50_10bit_422.yuv | 0.2% | -0.1% | -0.3% | 0.1% | 0.0% | -0.1% |  |  |  |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.1% | -10.0% | -15.3% | 0.0% | -4.2% | -9.9% |  |  |  |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.2% | -22.2% | -12.0% | 0.0% | -14.1% | -7.0% |  |  |  |
| Seeking_1920x1080_50_10bit_422.yuv | 0.1% | -10.5% | -7.3% | -0.1% | -6.2% | -6.0% |  |  |  |
| Overall | 0.2% | -9.2% | -9.3% | 0.0% | -4.9% | -6.8% |  |  |  |
| Enc Time[%] | #VALUE! |  |  | #VALUE! |  |  |  |  |  |
| Dec Time[%] | 157% |  |  | 148% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Traffic_2560x1600_30_10bit_422_crop.yuv | 0.0% | -6.0% | -11.9% | -0.2% | -4.4% | -8.7% |  |  |  |
| Kimono1_1920x1080_24_10bit_422.yuv | 0.1% | -6.7% | -9.2% | -0.3% | -3.2% | -8.2% |  |  |  |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.7% | -3.0% | 0.1% | 0.3% | 0.0% | 2.0% |  |  |  |
| EBUGraphics_1920x1080_50_10bit_422.yuv | 0.2% | 0.2% | 0.5% | 0.1% | 0.3% | 0.5% |  |  |  |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.2% | -2.4% | -2.1% | 0.1% | -1.6% | -3.4% |  |  |  |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.3% | -20.2% | -11.9% | 0.0% | -12.5% | -6.2% |  |  |  |
| Seeking_1920x1080_50_10bit_422.yuv | 0.0% | -9.1% | -6.4% | -0.2% | -5.4% | -5.2% |  |  |  |
| Overall | 0.2% | -6.7% | -5.8% | 0.0% | -3.8% | -4.2% |  |  |  |
| Enc Time[%] | #VALUE! |  |  | #VALUE! |  |  |  |  |  |
| Dec Time[%] | 175% |  |  | 170% |  |  |  |  |  |

# Conclusions

This contribution reports the cross-verification results for JCTVC-N0223. It was confirmed that the test results matched those provided by the proponent.

# References

- D. Flynn, K. Sharman, “Common test conditions and software reference configurations for HEVC range extensions,” JCTVC-L1006, Geneva, Switzerland, Jan. 2013.

# Patent rights declaration(s)