| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0099 |
| --- | --- |

| Title: | RCE2: Cross-verification report for sample adaptive intra prediction for oblique modes in lossy coding (JCTVC-N0177) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rajan Joshi 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-658-4511 rajanj@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution presents the results of a cross-verification performed by Qualcomm of sample adaptive intra prediction for oblique modes in lossy coding (JCTVC-N0177). This report provides cross-verification for using sample adaptive intra prediction for diagonal modes (modes 2, 18 and 34) in addition to horizontal and vertical intra prediction modes for lossy coding. The BD-rate performance results match those provided by the proponent. The implemented algorithm agrees with the Samsung’s proposal.

# Introduction

JCTVC-N0177 proposes use of sample adaptive intra prediction for oblique modes in lossy configuration. This report provides cross-verification for using sample adaptive intra prediction for diagonal modes (modes 2, 18 and 34) in addition to horizontal and vertical intra prediction modes for lossy coding. JCTVC-N0177 asserts that the decoder can be fully parallelized for these diagonal modes. This has been verified.

The method was implemented on top of the lossy intra RDPCM for horizontal and vertical intra prediction modes as described in JCTVC-N0052. The software base is HM10.1-RExt-3.0. The implemented algorithm agrees with the description in JCTVC-N0177.

# Results

The proposed method was tested for lossy configuration under the RCE2 common test conditions Error: Reference source not found. The simulation platform is a homogenous LINUX cluster consisting of Intel(R) XEON CPUs.The performance is compared to the anchor in terms of BD-rates.

Table 1: BD-rate performance of sample adaptive prediction for horizontal, vertical and diagonal modes (lossy configuration)

# Conclusions

The results of JCTVC-N0177 on sample adaptive intra prediction for diagonal modes have been verified. The implemented algorithm agrees with the description in JCTVC-N0177. The BD-rates match exactly with those provided by the proponents. The encoding and decoding times also match closely..

# References

- R. Joshi, P. Amon, R. Cohen, S. Lee, M. Naccari, “HEVC Range Extensions Core Experiment 2 (RCE2): Prediction and coding techniques for transform-skip and transform-bypass blocks,” JCTVC-M1122, Incheon, KR, April 2013.