| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0365 |
| --- | --- |

| Title: | Cross-check for JCTVC-N0207: RCE2: Combination of Mirroring of Coefficients (Test B.3), Significance map context (Test B.1) and RDPCM (Test D.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | K. Sharman, N. Saunders, J. Gamei Sony BPRL, Jays Close, Viables, Basingstoke, RG22 4SB, UK | Tel: Email: | +44 1256 355 011 karl.sharman@eu.sony.com nick.saunders@eu.sony.com |
| Source: | Sony Europe Ltd |  |  |

_____________________________

# Abstract

This contribution provides a cross-check of JCTVC-N0207, “RCE2: Combination of Mirroring of Coefficients (Test B.3), Significance map context (Test B.1) and RDPCM (Test D.1)”, which is an RCE2 related contribution, whereby a specific combination of existing proposals is tested. The source code and simulation results were provided by the proponents and are verified in this contribution. Draft text changes were not included; however, these changes would be specified by the contributors of the individual tools.

The source code for the mirroring of coefficients and significance map context implement the algorithm as described in the document; the source code for RDPCM, especially inter has yet to be fully analysed due to the number of changes.

The intra simulation result matches the result provided by the proponents; the inter simulation results are still processing.

# Introduction

Proposal N0207 combines three other tools that are being examined in RCE2, namely:

Test B.3 – 	the mirroring of coefficients in transform skip mode (inter or intra),
				proposed in JCTVC-N0175 and cross-checked in JCTVC-N0099.

Test B.3 applies a simple mapping for the coefficients in transform skip mode. It is applicable in intra mode, but is also now adaptive in inter mode, due to a prediction direction being used in RDPCM. The reason for the remapping is because there are smaller residuals nearer the source reference samples.

Test B.1 – 	significance map context,
				proposed in JCTVC-N0044 and cross-checked in JCTVC-N0097.

Test B.1 applies a single (new) significance map context for all entries in the transform skipped block. The reason for this is that in transform skipped blocks, the lack of the transform does not distribute the coefficients in the usual low-high frequency manner, and therefore all samples are as likely as each other as being significant. In addition, this prevents the transform skip blocks from polluting the adaptation of transformed blocks.

Test D.1 – 	RDPCM,
				proposed in JCTVC-N0083 and cross-checked in JCTVC-N0136.

Test D.1 applies a differential residual coding along the intra direction. In order to implement differential residual coding for inter, a horizontal direction/vertical direction/do not use evaluation is made.

# Source Code Analysis

Changes regarding B.3 are limited changes and are easy to follow.

Changes regarding B.1 are limited changes and are easy to follow.

Changes regarding D.1 are reasonably complicated, especially regarding inter. Further study for the inter source code is required, although this should also have been covered by N0136.

The controls of the tool are only via macros, so the system can only be made compatible with HM 4:2:0 by disabling the tools at compile time. It is expected that these would be controlled by the profile or chroma format.

# Draft Text Analysis

The draft text was not provided by the proponents. This is because the draft text would be included in the individual tool proposals.

# Results

Results are presented in Error: Reference source not found below. The simulation was performed using the lossy conditions of screen content coding as defined in Section 5.3 of JCTVC-M1123. Refer to the accompanying spreadsheet for results with greater precision.

The intra results match those supplied by the proponents. The inter results will be included in the cross-check once they are available.

Table 1 - BD-rate changes for the tested tool at intra lossy screen content coding conditions.

# Conclusion

The source code for two of the tools was studied and found to match the description of the algorithm. The source code for the third tool is still being examined, due to the number of changes involved.

The intra simulation result matched the result reported by the proponents.