| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0366 |
| --- | --- |

| Title: | Cross-check for JCTVC-N0266: Non-RCE1: Inter Color Component Residual Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | K. Sharman, N. Saunders, J. Gamei Sony BPRL, Jays Close, Viables, Basingstoke, RG22 4SB, UK | Tel: Email: | +44 1256 355 011 karl.sharman@eu.sony.com nick.saunders@eu.sony.com |
| Source: | Sony Europe Ltd |  |  |

_____________________________

# Abstract

This contribution provides a cross-check of JCTVC-N0266, “Non-RCE1: Inter Color Component Residual Prediction”, which is an RCE1 related contribution, whereby the inter colour component residual prediction method, previously proposed in JCTVC-M0230, has been extended to include negative scale factors.

The source code and simulation results were provided by the proponents and are verified in this contribution. Draft text changes have not yet been provided.

The source code has been examined and adds the described functionality.

The intra simulation results for YCbCr with the “NOQPPLUS” macro set to 1 match the results provided by the proponents; results for “NOQPPLUS” set to 0 have not been supplied yet. Other results do not match, but this is due to different test conditions being used in RCE1. Results in this cross-check will be updated once they become available.

# Introduction

Previously, in JCTVC-M0230, a tool called “inter colour component residual prediction” was proposed. This tool allows a scaled amount of the luma residual to be subtracted from the chroma residual. The scaling was by simple bit shifts, and was sent in the bit-stream to the decoder.

Since the resulting chroma residual could technically be one additional bit in size, the internal/input bit depth of chroma is inflated after loading the images.

Proposal JCTVC-N0266 extends this tool by allowing an inversely-proportional setting, i.e. providing the option for the scaled amount of the luma residual to be added to the chroma residual.

# Source code analysis

The source code changes include those of JCTVC-M0230 and JCTVC-N0266. The changes for JCTVC-N0266 add the required functionality.

The tool has two additional options, controlled by the macro “NOQPPLUS”, which controls the adjustment in JCTVC-M0230 of the internal bit depth of chroma. When set to 0, this macro is effectively disabled, and JCTVC-M0230 adjusts the internal bit depth of chroma. When set to 1, this adjustment is disabled.

Both options are exercised in the cross-check.

# Draft Text Analysis

The draft text has not yet been provided by the proponents.

# Results

Results are presented in the tables below. The simulation was performed using the standard Range Extension lossy conditions as defined in L1006, and the lossy conditions of screen content coding as defined in Section 5.3 of JCTVC-M1123. In addition both sets of conditions were tested with NOQPPLUS set to 0 and 1. Refer to the accompanying spreadsheet for results with greater precision.

The intra YCbCr Range Extension results match those supplied by the proponents. Others do not match, but this is because:

- the RGB were not run with a chroma QP offset in the cross check – the proponents used a chroma QP offset of 6, in accordance with the modified test conditions of RCE1.

- The screen content coding sequences in RCE1 are not exactly the same.

The different test conditions will be used to produce further results which will be included with the inter results once they are available.

Table 1 – The BD-rate changes due to the contribution relative to base RExt3.0 test conditions, with NOQPPLUS set to 0.

Table 2 - The BD-rate changes due to the contribution relative to base RExt3.0 test conditions, with NOQPPLUS set to 1.

Table 3 - The BD-rate changes due to the contribution relative to base Screen Content test conditions, with NOQPPLUS set to 0.

Table 4 - The BD-rate changes due to the contribution relative to base Screen Content test conditions, with NOQPPLUS set to 1.

# Conclusion

The source code was studied and found to match the description of the algorithm.

The intra simulation results for YCbCr with the “NOQPPLUS” macro set to 1 match the results provided by the proponents. Results for “NOQPPLUS” set to 0 have not been supplied yet. Other results do not match, but this is due to different test conditions being used in RCE1. Results in this cross-check will be updated once they become available.

The draft text has not been supplied yet.