| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0368_r1 |
| --- | --- |

| Title: | Non-RCE1: Chroma intra prediction with mode-dependent reduced reference |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kei Kawamura Tomonobu Yoshino Sei Naito 2-1-15, Ohara, Fujimino-shi, Saitama, JAPAN | Tel: Email: | +81 49 278 7411 ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI R&D Laboratories, Inc.) |  |  |

_____________________________

# Abstract

This contribution proposes the two chroma prediction method with reduced references which predicts chroma samples by using linear combination of luma samples for non 4:2:0 format. When a block size is large, a load of the parameter derivation process for each transform unit is reduced by using limited reference samples. One method utilizes fixed reduction pattern, while the other utilizes luma-intra mode-dependent reduced pattern. The Y BD-rate gains of HE Main / High / Super-High tiers are 1.8% / 1.5% / 1.0% for all intra conditions of YUV4:4:4. Compared with the top of TU-based chroma prediction in JCTVC-N0227, the Y BD-rate loss is less than 0.1% for whole AI conditions while the number of reference samples are limited.

# Introduction

HEVC range extensions study the chroma intra prediction based on the linear combination of luma samples which was called LM chroma in short. Due to the hardware complexity issue, the LM mode was dropped at the profile discussion of HEVC version 1.

Non-4:2:0 chroma format consists of larger amount of chroma pixels than that of 4:2:0 chroma format. Therefore, a contribution by the LM mode to BD-rate becomes large.

This contribution proposes a further simplification to the top of LM chroma mode proposed in the JCTVC-N0227. A key approach is a reduction of the number of reference samples when a block size is large. One method fixes a reduced reference sample position while another method varies a reference sample position depending on the luma intra prediction.

# Proposed method and test set

## Description

A tool of a luma-chroma intra prediction (LM mode) can reduce an inter component redundancy. The LM mode being applied to each TU block or each sub-TU block, a prediction coefficient derivation occurs for each TU block or sub TU block. This approach requires the heavy load for a large block because the all neighboring samples are utilized for parameter derivation.

The proposed method reduces the reference samples in the parameter derivation by using sub-sampling. In the viewpoint of the worst case, the sub-sampling is utilized when the block size is larger than 4x4 or 8x8 for 4:2:2 and 4:4:4 chroma format. Actual sub-sampling methods are shown in Fig. 1 and Fig. 2 for 4:2:2 and 4:4:4 chroma format. When the chroma format is 4:2:2, a chroma intra prediction is applied by each sub-TU block. The first sub TU block utilizes circle positions, while the second sub-TU utilizes triangle positions. Therefore, the number of reference points keeps a power-of-two.

Figure 1 Reference sample positions for 4:2:2 chroma format.

Figure 2 Reference sample positions for 4:2:2 chroma format.

## Method 1 (LM with reduced reference)

A method 1 uses a reduced reference which is not depends on a luma intra prediction mode. When the chroma TU size is larger than 8, reference pixels are reduced half. Otherwise, full reference samples are used for parameter derivation.

## Method 2 (LM with mode-dependent reduced refenrece)

A method 2 uses a reduce reference which depends on a luma intra prediction mode. When the chroma TU size is larger than 4, reference pixels are reduced half based on the luma intra prediction mode. When a direction of the luma prediction is horizontal or near horizontal, the middle pattern in Figure 3 is utilized. When the direction is vertical or near vertical, the right pattern in Figure 3 is utilized. Otherwise, the left pattern in Figure 3 is utilized.

Figure 3 Mode-dependent reduced reference for 4:2:2 chroma format.

# Experimental results

The proposed methods are implemented to HM10.1-RExt3.0. A test condition follows RCE1 test condition [2] and the common test condition for HEVC range extensions [3]. Following tables show the summary of BD-rate gain and codec runtime. A test of RGB sequences requires a result of a YUV domain. Unfortunately, some anchor results of such sequences by our experiments are not matched with provided results by RCE1. In this contribution, only YUV results are reported.

Table 1 Results by the normal reduced reference method in the all intra conditions for YUV4:4:4

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | -0.8% | -2.7% | -2.8% | -0.6% | -1.3% | -2.4% | -0.4% | -0.7% | -1.8% |
| Kimono1_1920x1080_24_10bit_422.yuv | -1.7% | -5.4% | -3.0% | -1.5% | -1.9% | -2.4% | -0.8% | -1.0% | -1.3% |
| EBUHorse_1920x1080_50_10bit_422.yuv | -1.2% | -2.3% | -4.9% | -0.7% | -1.1% | -2.1% | -0.4% | -0.6% | -0.8% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | -3.2% | -6.7% | -6.1% | -2.4% | -4.5% | -3.9% | -1.9% | -2.9% | -2.5% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | -0.6% | -2.0% | -2.0% | -0.5% | -1.3% | -1.3% | -0.3% | -0.9% | -0.7% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.0% | -1.5% | -1.6% | 0.0% | -0.6% | -0.7% | 0.0% | -0.3% | -0.4% |
| Seeking_1920x1080_50_10bit_422.yuv | -4.7% | -18.2% | -9.9% | -4.4% | -9.4% | -6.4% | -3.3% | -4.7% | -3.8% |
| Overall | -1.8% | -5.6% | -4.3% | -1.5% | -2.9% | -2.7% | -1.0% | -1.6% | -1.6% |
| Enc Time[%] | 108% |  |  | 108% |  |  | 108% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  | 101% |  |  |

Table 2 Results by the normal reduced reference method in the all intra conditions for YUV4:2:2

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | -0.8% | -2.7% | -2.8% | -0.6% | -1.3% | -2.4% | -0.4% | -0.7% | -1.8% |
| Kimono1_1920x1080_24_10bit_422.yuv | -1.7% | -5.4% | -3.0% | -1.5% | -1.9% | -2.4% | -0.8% | -1.0% | -1.3% |
| EBUHorse_1920x1080_50_10bit_422.yuv | -1.2% | -2.3% | -4.9% | -0.7% | -1.1% | -2.1% | -0.4% | -0.6% | -0.8% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | -3.2% | -6.7% | -6.1% | -2.4% | -4.5% | -3.9% | -1.9% | -2.9% | -2.5% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | -0.6% | -2.0% | -2.0% | -0.5% | -1.3% | -1.3% | -0.3% | -0.9% | -0.7% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.0% | -1.5% | -1.6% | 0.0% | -0.6% | -0.7% | 0.0% | -0.3% | -0.4% |
| Seeking_1920x1080_50_10bit_422.yuv | -4.7% | -18.2% | -9.9% | -4.4% | -9.4% | -6.4% | -3.3% | -4.7% | -3.8% |
| Overall | -1.8% | -5.6% | -4.3% | -1.5% | -2.9% | -2.7% | -1.0% | -1.6% | -1.6% |
| Enc Time[%] | 108% |  |  | 108% |  |  | 108% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  | 101% |  |  |

Table 3 Results by the normal reduced reference method compared with normal LM method (JCTVC-N0227 Exp1) in the all intra conditions for YUV4:4:4

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | 0.01% | 0.02% | 0.02% | 0.01% | 0.00% | 0.01% | 0.01% | 0.00% | 0.00% |
| Kimono1_1920x1080_24_10bit_422.yuv | 0.02% | 0.07% | 0.04% | 0.03% | 0.07% | 0.06% | 0.03% | 0.08% | 0.05% |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.01% | 0.01% | 0.02% | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | 0.00% | 0.04% | 0.01% | 0.00% | 0.02% | 0.00% | 0.00% | 0.02% | 0.00% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.00% | 0.01% | 0.02% | 0.00% | 0.01% | 0.01% | 0.00% | 0.01% | 0.01% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.00% | -0.02% | 0.00% | 0.00% | 0.00% | 0.02% | 0.00% | 0.01% | 0.01% |
| Seeking_1920x1080_50_10bit_422.yuv | 0.02% | 0.06% | 0.05% | 0.02% | 0.06% | 0.06% | 0.01% | 0.05% | 0.04% |
| Overall | 0.01% | 0.03% | 0.02% | 0.01% | 0.03% | 0.02% | 0.01% | 0.02% | 0.02% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 102% |  |  |

Table 4 Results by the normal reduced reference method compared with normal LM method (JCTVC-N0227 Exp1) in the all intra conditions for YUV4:2:2

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | 0.01% | 0.02% | 0.02% | 0.01% | 0.00% | 0.01% | 0.01% | 0.00% | 0.00% |
| Kimono1_1920x1080_24_10bit_422.yuv | 0.02% | 0.07% | 0.04% | 0.03% | 0.07% | 0.06% | 0.03% | 0.08% | 0.05% |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.01% | 0.01% | 0.02% | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | 0.00% | 0.04% | 0.01% | 0.00% | 0.02% | 0.00% | 0.00% | 0.02% | 0.00% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.00% | 0.01% | 0.02% | 0.00% | 0.01% | 0.01% | 0.00% | 0.01% | 0.01% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.00% | -0.02% | 0.00% | 0.00% | 0.00% | 0.02% | 0.00% | 0.01% | 0.01% |
| Seeking_1920x1080_50_10bit_422.yuv | 0.02% | 0.06% | 0.05% | 0.02% | 0.06% | 0.06% | 0.01% | 0.05% | 0.04% |
| Overall | 0.01% | 0.03% | 0.02% | 0.01% | 0.03% | 0.02% | 0.01% | 0.02% | 0.02% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 102% |  |  |

Table 5 Results by the mode-dependent reduced reference method in the all intra conditions for YUV4:4:4

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic_2560x1600_30_10bit_444_crop.yuv | -0.8% | -2.0% | -3.1% | -0.6% | -1.1% | -2.4% | -0.5% | -0.8% | -1.6% |
| Kimono1_1920x1080_24_10bit_444.yuv | -2.1% | -2.9% | -3.1% | -1.6% | -1.3% | -1.8% | -0.8% | -0.8% | -0.8% |
| EBULupoCandlelight_1920x1080_50_10bit_444.yuv | -1.8% | -19.3% | -6.8% | -2.3% | -9.4% | -6.4% | -1.6% | -3.0% | -4.5% |
| EBURainFruits_1920x1080_50_10bit_444.yuv | -2.2% | -10.2% | -15.6% | -3.2% | -7.8% | -15.2% | -3.8% | -5.9% | -13.2% |
| VenueVu_1920x1080_30_10bit_444.yuv | -3.0% | -8.7% | -6.2% | -3.3% | -7.9% | -5.9% | -3.5% | -7.0% | -5.2% |
| BirdsInCage_1920x1080_60_10bit_444.yuv | -1.9% | -2.5% | -8.3% | -2.2% | -2.1% | -8.7% | -2.9% | -2.5% | -4.5% |
| CrowdRun_1920x1080_50_10bit_444.yuv | -1.8% | -5.6% | -4.5% | -2.4% | -3.4% | -3.4% | -2.2% | -2.3% | -2.3% |
| Overall | -1.9% | -7.3% | -6.8% | -2.2% | -4.7% | -6.2% | -2.2% | -3.2% | -4.6% |
| Enc Time[%] | 108% |  |  | 108% |  |  | 108% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  | 102% |  |  |

Table 6 Results by the mode-dependent reduced reference method in the all intra conditions for YUV4:2:2

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | -0.7% | -2.6% | -2.6% | -0.6% | -1.3% | -2.3% | -0.4% | -0.7% | -1.7% |
| Kimono1_1920x1080_24_10bit_422.yuv | -1.6% | -5.3% | -3.0% | -1.5% | -1.9% | -2.4% | -0.8% | -1.0% | -1.3% |
| EBUHorse_1920x1080_50_10bit_422.yuv | -1.1% | -2.2% | -4.9% | -0.7% | -1.1% | -2.1% | -0.4% | -0.6% | -0.8% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | -3.2% | -6.7% | -6.0% | -2.4% | -4.4% | -3.8% | -1.8% | -2.9% | -2.5% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | -0.6% | -2.0% | -2.0% | -0.5% | -1.3% | -1.3% | -0.3% | -0.9% | -0.7% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.0% | -1.4% | -1.5% | 0.0% | -0.6% | -0.7% | 0.0% | -0.3% | -0.4% |
| Seeking_1920x1080_50_10bit_422.yuv | -4.6% | -17.9% | -9.8% | -4.3% | -9.3% | -6.4% | -3.3% | -4.6% | -3.7% |
| Overall | -1.7% | -5.5% | -4.3% | -1.4% | -2.8% | -2.7% | -1.0% | -1.6% | -1.6% |
| Enc Time[%] | 108% |  |  | 108% |  |  | 108% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  | 99% |  |  |

Table 7 Results by the mode-dependent reduced reference method compared with the normal LM method (JCTVC-N0227 Exp1) in the all intra conditions for YUV4:4:4

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic_2560x1600_30_10bit_444_crop.yuv | 0.07% | 0.06% | 0.13% | 0.04% | 0.03% | 0.08% | 0.02% | 0.01% | 0.04% |
| Kimono1_1920x1080_24_10bit_444.yuv | 0.15% | 0.11% | 0.14% | 0.10% | 0.06% | 0.07% | 0.06% | 0.03% | 0.02% |
| EBULupoCandlelight_1920x1080_50_10bit_444.yuv | 0.17% | 0.47% | 0.26% | 0.16% | 0.20% | 0.07% | 0.10% | 0.06% | 0.04% |
| EBURainFruits_1920x1080_50_10bit_444.yuv | 0.16% | 0.30% | 0.35% | 0.15% | 0.17% | 0.31% | 0.12% | 0.11% | 0.24% |
| VenueVu_1920x1080_30_10bit_444.yuv | 0.16% | 0.24% | 0.19% | 0.13% | 0.14% | 0.13% | 0.10% | 0.09% | 0.09% |
| BirdsInCage_1920x1080_60_10bit_444.yuv | 0.14% | 0.08% | 0.21% | 0.13% | 0.02% | 0.06% | 0.13% | -0.02% | 0.00% |
| CrowdRun_1920x1080_50_10bit_444.yuv | 0.11% | 0.14% | 0.09% | 0.10% | 0.06% | 0.05% | 0.09% | 0.04% | 0.04% |
| Overall | 0.14% | 0.20% | 0.20% | 0.12% | 0.10% | 0.11% | 0.09% | 0.05% | 0.07% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  | 100% |  |  |

Table 8 Results by the mode-dependent reduced reference method compared with the normal LM method (JCTVC-N0227 Exp1) in the all intra conditions for YUV4:2:2

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | 0.06% | 0.15% | 0.20% | 0.04% | 0.07% | 0.13% | 0.03% | 0.04% | 0.08% |
| Kimono1_1920x1080_24_10bit_422.yuv | 0.15% | 0.23% | 0.10% | 0.11% | 0.10% | 0.08% | 0.07% | 0.07% | 0.05% |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.04% | 0.05% | 0.02% | 0.02% | 0.02% | 0.01% | 0.01% | 0.02% | 0.00% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | 0.05% | 0.10% | 0.09% | 0.03% | 0.05% | 0.03% | 0.02% | 0.02% | 0.01% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.05% | 0.06% | 0.04% | 0.02% | 0.04% | 0.03% | 0.01% | 0.03% | 0.02% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.00% | 0.09% | 0.09% | 0.00% | 0.02% | 0.02% | 0.00% | 0.01% | 0.01% |
| Seeking_1920x1080_50_10bit_422.yuv | 0.16% | 0.42% | 0.18% | 0.10% | 0.18% | 0.12% | 0.06% | 0.10% | 0.09% |
| Overall | 0.07% | 0.16% | 0.10% | 0.05% | 0.07% | 0.06% | 0.03% | 0.04% | 0.04% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |

# Conclusion

This contribution proposes the chroma intra prediction with reduced references. The load of the parameter derivation process is reduced at the worst case while the BD-rate loss is less than 0.1%. Method 1 provide better results than method 2 due to the TU size setting. We recommend adopting LM mode with reduced reference (method 1) in HEVC range extensions.

# References

- J. Kim, “RCE1: EXP1/4 The performance of extended chroma mode for non 4:2:0 format”, JCTVC-N0227, Vienna, AT, 25 July – 2 Aug. 2013

- T. Nguyen, et al., “HEVC Range Extensions Core Experiment 1 (RCE1): Inter-Component Decorrelation Methods”, JCTVC-M1121, Incheon, KR, 18–26 Apr. 2013.

- D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-L1121, Geneva, CH, 14–23 January, 2013.

# Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).