| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0369 |
| --- | --- |

| Title: | AHG 5 and 18: Cross-check of Internal Precision for High Bit Depths (JCTVC-N0188) by Sony |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | C. Rosewarne 1 Thomas Holt Drive, North Ryde, NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
|  | M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: | +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia, Canon Inc. |  |  |

_____________________________

# Abstract

This contribution provides a cross-check of proposal JCTVC-N0188 by Sony.  For this cross-check, a subset of the results presented in the proposal was reproduced.  The simulation result matches the result reported by Sony.  The source code was studied and found to match the description in the proposal.

# Introduction

In JCTVC-N0188 [1], a study of the internal accuracy of HEVC is presented.  Bit-depths from 8 to 16 are tested, along with a set of new transforms, designed for higher precision.

This contribution provides a cross-check of a subset of the results presented in [1].

# Source code analysis

The provided source code was analysed and the following observations were made:

- Support for specifying TRANSFORM_SHIFT_MATRIX, ENTROPY_CODING_DYNAMIC_RANGE, MAX_TR_DYNAMIC_RANGE, QUANT_SHIFT and IQUANT_SHIFT via the command line is introduced.

- Support for various TRANSFORM_SHIFT_MATRIX values is introduced:

  - DCT and DST matrices are defined for TRANSFORM_MATRIX_SHIFT values of 6-bits (HM) up to 14-bits, however these are disabled (i.e. MATRIX_MULT = 0).

  - The partial butterfly implementations are also modified to support the same precisions.

  - New DCTs for TRANSFORM_MATRIX_SHIFT values from 7 to 14 were found to match those produced by the DCT formula with appropriate gain and rounding applied.

  - Shift operations in the forward and inverse transform compensate for the gain inherent for a given TRANSFORM_MATRIX_SHIFT value.

- Support for ENTROPY_CODING_DYNAMIC_RANGE is introduced:

  - The quantiser output is clipped according to this parameter.

  - The dequantiser input is clipped according to this parameter.

  - Sign bit hiding extreme magnitude limits now accord to this parameter.

- Support for various MAX_TRANSFORM_DYNAMIC_RANGE values is already present in the code.

- In the experiments performed by Sony, QUANT_SHIFT and IQUANT_SHIFT are set to 14 and 6 respectively.  The cross-checker reports that these values are correct to compensate for the gains introduced by values in the ‘g_quantScales’ table and the ‘g_invQuantScales’ table, respectively.

These observations match the behaviour described in [1].

# Simulation conditions

To reproduce the results reported by Sony, test cases are defined that realise all combinations of parameters, as described below:

## Configurations

A set of new configurations are defined, that are based upon the ‘Intra’ configuration of the HEVC Range extensions common conditions [2].  Table 1 lists the new configurations and the differences in each configuration.

Table 1. Configurations for Internal Accuracy test.

| Name | Internal bit-depth | Transform Skip | Minimum TU size | RDOQ |
| --- | --- | --- | --- | --- |
| ai_bd8_ts0_tu4x4 | 8 | Off | 4x4 | Off |
| ai_bd8_ts1_tu4x4 | 8 | On | 4x4 | Off |
| ai_bd10_ts0_tu4x4 | 10 | Off | 4x4 | Off |
| ai_bd10_ts1_tu4x4 | 10 | On | 4x4 | Off |
| ai_bd12_ts0_tu4x4 | 12 | Off | 4x4 | Off |
| ai_bd12_ts1_tu4x4 | 12 | On | 4x4 | Off |
| ai_bd14_ts0_tu4x4 | 14 | Off | 4x4 | Off |
| ai_bd14_ts1_tu4x4 | 14 | On | 4x4 | Off |
| ai_bd16_ts0_tu4x4 | 16 | Off | 4x4 | Off |
| ai_bd16_ts1_tu4x4 | 16 | On | 4x4 | Off |

Note that in [1], results for minimum TU sizes of 8x8, 16x16 and 32x32, each with internal bit-depths of 8, 10, 12, 14 and 16 were also presented.  To reduce simulation run-time, these results are not verified in this cross-check.

## Test sequences

The following RGB 4:4:4 sequences were used:

- OldTownCross

- CrowdRun

- DucksTakeOff

- InToTree

- ParkJoy

- Traffic

Except for ‘Traffic’, these are the ‘1080p’ sequences from the SVT Multiformat set.

Additionally, filtered versions of the following sequences were tested (filtering is included as an optional pre-processing step in the software provided by Sony):

- DucksTakeOff  ([1, 2, 1] filter applied horizontally and vertically)

- InToTree ([1, 4, 6, 4, 1] filter applied horizontally and vertically)

For each sequence only the first 10 frames is encoded.

## QPs

A separate set of QPs (or ‘tier’) is defined for each internal bit-depth in Table 2.

Table 2. QP sets for the Internal Accuracy test.

| Internal bit-depth | QPs |
| --- | --- |
| 8 | 0, 2, 4, 6, 8, 10, 12, 14, 16, 18, 20, 22 |
| 10 | -12, -10, -8, -6, -4, -2, 0, 2, 4, 6, 8, 10 |
| 12 | -24, -22, -20, -18, -16, -14, -12, -10, -8, -6, -4, -2 |
| 14 | -36, -34, -32, -30, -28, -26, -24, -22, -20, -18, -16, -14 |
| 16 | -48, -46, -44, -42, -40, -38, -36, -34, -32, -30, -28, -26 |

## Internal accuracy test parameters

MAX_TR_DYNAMIC_RANGE is varied from 15 to 22.

TRANSFORM_MATRIX_PRECISION is set to 6, 8, 10, 12 and 14.

ENTROPY_CODING_DYNAMIC_RANGE is fixed at 22.

# Experiment Results

The spreadsheet provided by Sony in [1] is used as a template for presentation of results in this cross-check. Execution took place on a Linux cluster using GCC 4.1.2.  The simulation result of this cross-check exactly matches the result reported by Sony.

To provide a summary of the simulation result, several graphs presented in [1] are reproduced (see , Figure 2 and Figure 3 below).

Figure 1. Results for OldTownCross, Green channel, MAX_TR_DYNAMIC_RANGE = 22, TRANSFORM_MATRIX_PRECISION = 14, Transform Skip disabled, QPs from -48 to 22, various bit-depths.

Figure 2. Results for InToTree, with [1, 4, 6, 4, 1] smoothing, Green channel, various MAX_TR_DYNAMIC_RANGE values, TRANSFORM_MATRIX_PRECISION = 12, Bit-depth = 12, Transform Skip disabled, QPs from -36 to -14.

Figure 3. Results for InToTree, with [1, 4, 6, 4, 1] smoothing, Green channel, MAX_TR_DYNAMIC_RANGE = 20, various TRANSFORM_MATRIX_PRECISION values, Bit-depth = 14, Transform Skip disabled, QPs from -36 to -14.

# Conclusion

The source code was studied and found to match the description of the algorithm.  Also, the simulation result matched the result reported by the proponents.

# References

- K. Sharman, N. Saunders, J. Gamei “AHG 5 and 18: Internal Precision for High Bit Depths”, Joint Collaborative Team on Video Coding, JCTVC-N0188, Vienna, AT, July 2013.

- D. Flynn, C. Rosewarne “Common test conditions and software reference configurations for HEVC range extensions”, Joint Collaborative Team on Video Coding, JCTVC-L1005, Geneva, CH, January 2013.