| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Vienna, 25 July – 2 August 2013 | Document: JCTVC-N0372 |
| --- | --- |

| Title: | HM Software modifications for interlaced coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zineb AGYO (z.agyo@ateme.com), Jérôme VIERON (j.vieron@ateme.com), Jean Marc THIESSE (jm.thiesse@ateme.com) |  |  |
| Source: | ATEME |  |  |

_____________________________

# Abstract

This contribution is further to the contributions M27901 [1] and M29197 [2], where the HM software was modified in order to handle field based coding.

An HM 11.0 based patch was made for the reference software in order to introduce the field coding features previously presented in those contributions. Furthermore, SEI messages are now handled in this patch version so the decoder can correctly display interlaced content.

# Additions made to the software patch

## Previous modifications

The previous modifications added to the software are detailed in [1]. These modifications include field separation at the input of the encoder and re-interlacing at the output, PSNR computation and the handling of a GOP structure that is more suitable to interlaced content.

These modifications have been brought to the version 11.0 of the HM.

## SEI messages handling

The main addition in the new version of the patch is that the decoder handles deinterlacing the output video. It is however necessary to signal in the bitstream that the content is interlaced and the field order using the Picture Timing SEI messages. In particular, the field in this SEI message which concern our application is “pic_struct”, thus it is set to the right value while encoding and written in the bitstream.

In addition to the encoding parameters presented in [1] as well in order to set the encoder to field coding mode, it is needed to signal that SEI messages will be written using the existing parameters below:

VuiParametersPresent		  : 1

SEIPictureTiming		  : 1

FrameFieldInfoPresentFlag	  : 1

This way, in case of field coding, the modified HM decoder can detect that the picture is actually a field plus its field parity, and re-interlace the decoded field to reconstruct the interlaced content.

# Conclusion

This new version of the patch applied to the reference software provides a clean encoder/decoder solution for an efficient HEVC interlace coding. The tests previously made to assess HEVC interlaced coding using this modified software presented in [3] show a considerable performance gain.

Note that separating the fields at the input of the encoder as it is done in the modified software causes the chroma displacement problems described in [4]. The solution proposed in [4] in order to overcome this problem is not implemented in this version of the patch.

# References

[1] 	Zineb Agyo, Jean-Marc Thiesse, Jérôme Viéron, “HEVC Software modifications for field based coding”, M27901, Geneva, Switzerland

[2] 	Zineb Agyo, Jean-Marc Thiesse, Jérôme Viéron, “HEVC Software modifications for interlaced coding”, M29197, Incheon, Rep. of Korea

[3] 	Zineb Agyo, Jean-Marc Thiesse, Jérôme Viéron, “Ad hoc on Study of interlace coding in MPEG HEVC: Results of Random Access High Efficiency coding tests”, M27904, Geneva, CH, Jan 2013.

[4]	Jérôme Viéron, Jean-Marc Thiesse, Pierre Larbier, Antoine Depreux, “HEVC interlaced coding assessment and Chroma issue consideration”, JCTVC- K0216, Shanghai, China.

# Patent rights declaration(s)

ATEME does not have any current or pending patent rights relating to the technology described in this contribution.