| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0375 |
| --- | --- |

| Title: | BoG Report on arbitrary scalability ratios support in SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | BoG report |  |  |
| Author(s) or Contact(s): | Edouard François | Email: | edouard.francois@crf.canon.fr |
| Source: | BoG chair |  |  |

_____________________________

# Abstract

The BoG on arbitrary scalability ratios support addresses the following topics.

- Definition of test conditions (test material, downsampler) for the upcoming investigation,

- Selection of an initial set of upsampling filter coefficients as a comparison point.

The BoG met first on 27 July. The status of the current discussions is as follows:

- It is preferred to use for test condidions one (or a limited set of) spatial scalability ratio(s) enabling to test as many different phases as possible. Further analyzis is being made to identify such ratio(s).

- It is preferred to use the JSVM downsampler with phase offset set to 0. Further analyzis of the JSVM downsampler is being made to evaluate the amount of changes required in the software.

- It is recommended to select a set of upsampling filter coefficients which ensures the SHM behavior unchanged for common test conditions.

The BoG plans to meet again on July 29.

# Introduction

The support of arbitrary scalability ratios is considered for SHVC. Currently only spatial scalability ratios of 1, 1.5 and 2 are supported in the SHVC working draft (JCTVC-M1008). In addition it is possible to specify the cropping window in the reference layer used to predict the enhancement layer using 4 parameters signaled in the SPS extension.

The goals of the BoG are the following:

- to define test conditions (test material, downsampler) for the upcoming investigation,

- select an initial set of filter coefficients as a comparison point.

# Test conditions for arbitrary scalability ratios support

## First discussed issue : what scalability ratios should be considered ?

2 options are considered:

- Test a large nb of spatial ratios – it is probably unpractical due to the very large number of tests it would require – this option is therefore not considered

- Select one (or a limited number of) particular spatial ratio(s) (chosen in order to have as many different phases as possible) and test the different candidate filters

  - 1 suggestion to use ratio close to 1.5

  - 1 suggestion to select spatial ratio within ‘reasonable range’ – recommended to be in between 1.5 – 2

  - One proposed approach is to consider a ratio N/M, and to identify M such that a maximum number of different phases are used in the upsampling process

  - One expert suggests to select N/M ratio that involves picking a large number of suboptimal phases given the 16 phases available and that also would have the largest possible (or just large) distance from the optimal phase.

Homework for next BoG meeting – find the right scalability ratio(s)

- Based on the 'N/M’ proposed approach

- Check the ‘compatibility’ with actual resolutions used in practical applications

## Second discussed issue : Downsampler

- A current version, not completely aligned with current downsampler (with 2x and 1.5x capabilities), can be made available by InterDigital by next meeting

- One suggestion is to use JSVM downsampler, with phase offset forced to 0 – most of the experts consider it is probably the most feasible approach

  - It was commented that the JSVM downsampling filters have a quite different cutoff frequency than the downsampling filter used so far for the SHVC work. So, in order to have reasonable comparisons, it will needed to also test 1.5x and 2x using the same JSVM downsampler.

Homework for next BoG meeting – JSVM downsampler evaluation

- Evaluate among of work needed in JSVM downsampler before next BoG meeting – is it do-able in 2 weeks or not ?

- If not, a call for downsampler to be proposed at next meeting would be needed

# Upsampling filters for arbitrary scalability ratios support

Third discussed issue: selection of upsampling filters for arbitrary scalability ratios support

Current SHVC working draft considers 16-phases 8-tap luma filters and 4-tap chroma filters. Currently only phases 0, 5, 8 and 11 (over 16 possible values) are defined to support spatial scalability ratios of 1.5 and 2.

2 proposals are currently on the table:

JCTVC-N0219 Non-SCE1: On arbitrary spatial ratio scalability in SHVC [J.Chen, X.Li, M.Karczewicz (Qualcomm), E.Alshina, A.Alshin (Samsung), K.Ugur (Nokia)]

JCTVC-N0273 On the selection of fixed filters for upsampling [K. Minoo, D. Baylon (??)]

Comments

- Once a starting set of filters will be selected, we need to define proper tests to select the right coefficients

- 1 suggestion to keep the behavior unchanged for common test conditions – this means not changing the coefficients for phases 0, 5, 8 and 11 – agreement from several experts on this point

- 1 expert considering existing filters could be changed, but agrees that the starting set should not modify coefficients for these 4 phases.

Homework for next BoG meeting – off-line check the 2 proposed sets of filters

Next BoG meeting planed for Monday (July 29) afternoon