| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0379 |
| --- | --- |

| Title: | Cross-check for JCTVC-N0275: AHG18: Modified scaling factor for transform-skip blocks to support higher bit depths greater than or equal to 14 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | K. Sharman, N. Saunders, J. Gamei Sony BPRL, Jays Close, Viables, Basingstoke, RG22 4SB, UK | Tel: Email: | +44 1256 355 011 karl.sharman@eu.sony.com nick.saunders@eu.sony.com |
| Source: | Sony Europe Ltd |  |  |

_____________________________

# Abstract

This contribution provides a cross-check of JCTVC-N0275, “AHG18: Modified scaling factor for transform-skip blocks to support higher bit depths greater than or equal to 14”, which proposes limiting the shift applied during the transform-skip process so that no bits are discarded when coding high-bit-depth source video. The source code, working draft text and simulation results were provided by the proponents and are verified in this contribution.

The 14-bit simulation result matches the result provided by the proponents.

# Introduction

In HEVC, the quantiser derives a value transformShift, which is the sum of all left-shifts applied (or effectively applied via matrix multiplication) during the transform process. The value of transformShift is then taken into account when determining the shift required for the quantisation process. For this reason, a shift is also applied during the transform-skip process so that the coefficients entering the quantiser are of the same precision regardless of whether they were transformed or not.

For high-bit-depth source video the required left-shift in the transform-skip can be negative (i.e. a right-shift) and therefore some accuracy in the least-significant bits can be lost. This negative shift occurs for the current system of 4x4 transform-skip blocks for 14-bit or more; if transform-skip were to ever be considered for bigger blocks, this negative shift would occur at lower bit depths.

JCTVC-N0275 proposes to limit the shift in the transform-skip path (and calculate the same limited shift for transform-skipped TUs entering the quantiser) so that no right-shift is possible and all the bits are preserved.

# Source Code Analysis

The changes required to implement the proposal are limited and easy to follow.

# Draft Text Analysis

The changes required to implement the proposal are limited and easy to follow.

# Results

Graphical results are presented in the figure below; the numerical results are available in the accompanying spreadsheet. The simulation was performed using high-bit-depth source material provided by SVT at 14-bit precision (truncated from 16-bit source) at QPs -36, -32, -28 and -24. Only the intra_rextdev configuration was tested. Four of the five sequences matched those supplied by the proponents. The fifth differed slightly, in anchor and result, but appears to be because the source sequence file was not the same. Although there is a small difference, this is not a concern.

Figure 1 - PSNR/bitrate curve for one of the sequences tested.

# Discussion

The system has been tested for 14-bit. However, a further modification will be required for function at 16-bit operation – the entropy coder is currently limited to processing 15 bit magnitude values, but the proposed change could create 16-bit values (or 17 bit if the QP is less than -44). It is recommended that the authors consider the implications of clipping in 16-bit processing. One possibility is to prevent input QPs less than 4-(bitDepth-8)*6 for transformSkip blocks from increasing the magnitude of the coefficients, thereby the system would only produce up to 16-bit magnitude values for 16-bit data.

# Conclusion

The source code for the proposed system was studied and found to match the description of the algorithm.

The 14-bit simulation result matches the result reported by the proponents.