| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0380 |
| --- | --- |

| Title: | Cross-check for JCTVC-N0189: AHG 5 and 18: Entropy Coding Compression Efficiency for High Bit Depths |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Matteo Naccari Marta Mark Andrea Gabriellini Block D, Centre House 56 Wood Lane, W12 7SB London, UK | Email: | matteo.naccari@bbc.co.uk marta.mrak@bbc.co.uk andrea.gabriellini@bbc.co.uk |
| Source: | BBC Research & Development |  |  |

_____________________________

# Abstract

This document reports the BBC’s cross-check results for Document JCTVC-N0189 proposed by Sony. The document reports several methods and in this document only the “Golomb-Rice Parameter Modification” method is tested for lossless coding. All the results released by the proponent are tested and it is reported that all the test points are confirmed. Analysis of the source code reveals perfect matching between the algorithm and the implementation.

# Introduction

Document JCTVC-N0189 relates to AHG 18, i.e. “high bit-rate and bit-depth operating points” and proposes new entropy coding methods for escape-coded coefficients. In particular, it is observed that at very high bitrates most of the coefficients are coded as escape-codes and the current design is not particularly efficient for this kind of entropy encoding. Therefore it is proposed to encode directly these coefficients by dividing them into the MSB and LSB. The split is done using a variable denoted as BF in the contribution and signalled in the slice header. The split operated with the variable BF is combined with the Rice parameter used to code escape-code. In particular the variable BF is used as offset and added to the “cRiceParam” parameter. The offset is also made adaptive with respect to the coefficient statistics. In the next section the cross-checked experimental results are presented.

# Test results

The proposal is tested for lossless coding using the RCE2 test conditions described in Document JCTVC-M1122. This cross-check document reports the same results in the following tables.

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.3 | -1.6% | 31.7 | 32.2 | -0.8% | 49.8 | 65.2 | -33.9% |
| Class B | 2.2 | 2.2 | 0.0% | 2.6 | 0.0 | -200.0% | 2.6 | 0.0 | -200.0% |
| SC RGB 444 | 10.1 | 12.6 | -16.4% | 100.4 | 126.9 | -13.9% | 381.6 | 495.3 | -13.4% |
| SC YUV 444 | 11.4 | 12.9 | -8.3% | 128.9 | 171.7 | -25.5% | 325.6 | 456.7 | -26.2% |
| RangeExt | 2.4 | 2.4 | -2.4% | 2.5 | 0.0 | -400.0% | 2.5 | 0.0 | -400.0% |
| Enc Time[%] | 484% |  |  | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | 459% |  |  | 271% |  |  | 216% |  |  |

Note that the binaries used for the verification experiments was erroneously built without the usual optimisations, hence the unusually high reported run times.

# Source code inspection

The proponent implemented the proposed changes in the HM-10.0-rext-3.0 reference software in the version released for high bit depth coding. For the method tested in this report, the following macro has been defined:

RExt__VARIABLE_GOLOMB_RICE_PARAMETER_RANGE

This macro enables the reading from command line of the Rice parameter offset (20 tested in this report) and the update of this offset according to the coefficient statistics. The main changes related by the Rice parameter offset are located in the functions “codeCoeffNxN” (encoder) and “parseCoeffNxN” (decoder) where the offset added to the Rice parameter is updated. The source code inspection done by the cross-checker reveals that the proponent implemented the changes in a simple and effective way. Moreover the implementation matches the algorithmic description provided in JCTVC-N0189.