| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0384 |
| --- | --- |

| Title: | BoG report on non-RCE2 contributions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rajan Joshi Robert Cohen | Email: | rajanj@qti.qualcomm.com cohen@merl.com |
| Source: | BoG chairs |  |  |

_____________________________

# Abstract

This report summarizes activities of the BoG on non-RCE2 contributions. The BoG met on July 30th, July 31st and August 1st to review the contributions classified as non-RCE2. The contributions discussed were from 5 different categories: Rice parameter initialization and update, related to subtest C of Range Extensions core experiment 2, transform skip, sample adaptive prediction and intra filtering and prediction changes.

# Introduction

This report summarizes activities of the BoG on non-RCE2 contributions. The BoG met on July 30th, July 31st and August 1st to review the contributions classified as non-RCE2. The contributions discussed were from 5 different categories: Rice parameter initialization and update, related to subtest C of Range Extensions core experiment 2, transform-skip, sample adaptive prediction and intra filtering and prediction changes.

# Summary of results and discussions

## Subtest A related (Rice parameter initialization and update)

JCTVC-N0137 Non-RCE2: Golomb-Rice parameter initialization for transform-skip and transquant-bypass modes [V. Kolesnikov, C. Rosewarne, M. Maeda (Canon)] (crosscheck: JCTVC-N0350 MERL)

This method is applied only to transform-skip and transform-bypass blocks. In HEVC, Rice parameter is initialized to 0 for each 4×4 subblock. In this contribution, if the current subset has 8 or more greater_than_one flags (same criteria as used in high throughput binarization). Some loss (0.2% for lossy YUV444) was observed. Proposal was perceived to be orthogonal to A.3. The combination with A.3 has not been tested. Higher gains were observed for RangeExt sequences in lossless configuration (1.7 to 2.3%).

Recommended action: Further study (possibly in a CE)

Table 1: Summary of BD-rate results for lossless configuration

|  |  | AI | RA | LB |
| --- | --- | --- | --- | --- |
| JCTVC-N0137 | Class F | -0.6% | -0.2% | -0.1% |
|  | Class B | 0.0% | 0.1% | 0.1% |
|  | SC RGB 444 | -1.2% | -0.8% | -0.8% |
|  | SC YUV 444 | -0.9% | -0.3% | -0.6% |
|  | RangeExt | -2.3% | -1.7% | -1.7% |

Table 2: Summary of BD-rate results for lossy configuration

|  |  | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| JCTVC-N0137 | Class F | -0.1% | -0.2% | -0.3% | 0.0% | -0.1% | -0.2% | -0.1% |
|  | Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC RGB 444 | -0.3% | -0.6% | -1.0% | -0.3% | -0.6% | 0.1% | -0.5% |
|  | SC YUV 444 | 0.0% | -0.2% | -0.4% | 0.2% | 0.1% | 0.1% | 0.1% |
|  | RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

JCTVC-N0181 Non-RCE2: Rice Parameter Initialization [M. Karczewicz, J. Sole, R. Joshi (Qualcomm)

This contribution proposed modified Rice parameter initialization to be a value from 0 to 4 (or 5, both methods tested), depending upon four 8-bit counters, one for each 4×4 subblock type. The sub-block type is computed as 2*isLuma + (iSubSet>0), and the statistics are updated based upon the first coeff_abs_level_remaining of the 4x4 sub-block. The updated statistics are used to derive the initialization value for the next sub-block (so the statistics don’t affect the current sub-block). “HEVC update” only adds this initialization method. “Fast update” adds this initialization method on top of the Rice parameter update method proposed in A.3. The fast update method was the preferred choice of the proponents. Some participants asked how the gains were affected by A.3. Gains were not additive, e.g. 8.5% for proposed method, 4.2% for A.3, and 8.6% for the combination (lossless conditions on SC YUV444). The gains of A.3 on top of the proposed method were approx. 0.3% for lossless and approx. 0.1-0.2% for lossy (0.4% for Class F). A participant suggested given that the gains from A.3 on top of this method were relatively small.

Table 3: Summary of BD-rate results for lossless configuration

|  | AI | RA | LDB |
| --- | --- | --- | --- |
| Class F | -1.9% | -1.0% | -0.6% |
| Class B | -0.1% | 0.0% | 0.0% |
| SC RGB 444 | -10.4% | -8.9% | -8.1% |
| SC YUV 444 | -7.7% | -7.1% | -7.3% |
| RangeExt | -3.1% | -2.4% | -2.4% |

Table 4: Summary of BD-rate results for lossy configuration

|  | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LDB-MT | LDB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class F | -0.6% | -1.0% | -1.4% | -0.4% | -0.7% | -0.3% | -0.5% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -3.5% | -5.7% | -8.2% | -3.0% | -5.0% | -2.0% | -4.6% |
| SC YUV 444 | -0.7% | -1.5% | -2.9% | -0.4% | -1.1% | -0.2% | -0.9% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

Recommended action: Further study in a CE.

JCTVC-N0232 Non-RCE2: Rice parameter update method [J. Min, S. Lee, C. Kim (Samsung)] (crosscheck: JCTVC-N0325 Qualcomm)

This method proposes a Rice parameter update technique similar to A.3. There are minor differences in the update formula (one extra addition). Effective for lossless, but losses observed for yuv444 lossy (up to 0.7% for method 1, up to 0.2% for method 2). Gains of 0.8% were observed for YUV444 AI-SHT configuration. Proponent commented that the focus was improving lossless and near-lossless performance. Results were provided for applying it to all the blocks and only transform-skip blocks. The above figures are for the transform-skip block application.

JCTVC-N0281 Non-RCE2 Rice parameter extension for transform-skip blocks [S. H. Kim, K. Misra, A. Segall (Sharp)] (crosscheck: JCTVC-N0333 Samsung)

This contribution presents a modification of A.1. Max Rice parameter is extended to 6. A Rice parameter from the set {0, 2, 4, 6} is explicitly signalled for each 4×4 sub-block that contains coeff_abs_level_remaining syntax element. This method is applied only for transform-skip and transform-bypass blocks. For the lossless configuration, gains of approximately 7.5% were observed across all configurations. More than 2% gains were observed for RangeExt sequences.

For lossy coding, a slice level flag was proposed to switch this method off. This was done on the encoder side when the QP was 32 or higher to mitigate losses for some class of sequences. Shows 2.0% gain for AI-SHT. Loss (0.1%) was observed for some classes of sequences and configurations.

One participant commented that additional encoder complexity (substantial for lossless) is needed to determine the parameter for each 4x4 subblock. Rice parameter update is eliminated, but only on transform-skip blocks. One participant commented that this may not be useful since for non-transform-skip blocks, the update needs to be performed.

## Subtest C related

JCTVC-N0042 Non-RCE2: Restriction on the Residual DPCM block size [J. Sole, R. Joshi, M. Karczewicz (Qualcomm)] (crosscheck: JCTVC-N0279 Canon)

Currently RDPCM is applied to all block sizes at the TU level, from 4x4 up to 32x32. This proposal restricts RDPCM application up to a maximum block size, to reduce the operation count. For maximum block size of 16×16, virtually no difference in performance was observed; for maximum RDPCM block size of 8×8 gain/loss of about 0.1% or 0.2% was observed; for maximum RDPCM block size of 4×4, losses up to 0.8% were observed. Finally, for no RDPCM, the losses were up to 15.3%. There was a discussion regarding decoder implementation and complexity, and whether the conclusions would be the same if one chose not to code 4x4 blocks. The proponent recommends restricting RDPCM to 4×4 and 8×8 blocks. Some participants commented that the savings in an actual decoder implementation may not be significant if the logic is shared. The proponent asserted that the savings were worthwhile. A cross-checker commented that the proposed limit can reduce operations and logic in the decoder, with negligible penalty in performance. A participant commented that the proposal may unnecessarily limit the encoder flexibility while not reducing complexity in a practical decoder implementation.

One participant commented that if inter RDPCM is adopted and the block size restriction is applied to inter, there may be higher losses. In that case, further study would be necessary (maybe in a CE?).

Recommendation: Discuss in the main track

JCTVC-N0072 RCE2-related: Variants of simplified sample-based weighted prediction [P. Amon, A. Hutter (Siemens), E. Wige, A. Kaup (Universität Erlangen-Nürnberg)] (crosscheck: JCTVC-N0364 Qualcomm)

This contribution proposed variations of the sample-based weighted prediction (C.2). This is applicable onlt to lossless configuration. Two variants were presented:

Variant 1: Instead of weighted prediction of 4 samples in the neighborhood, only the pixel with the least SAD between templates is used. Some improvement in performance with respect to C.3 was observed for screen content, but all the gains for class B and RangeExt sequences were lost. A participant noted that the minimum SAD calculation is still needed at the decoder.

Variant 2: In this variant, the C.3 method is used for PLANAR mode and variant 1 is used as a replacement for the DC mode. This variant showed gains over the C.3 method for most categories (from losses of up to 0.2% to gains of up to 0.7%).

One participant commented that if this is not applied for lossy transform-skip blocks, there may be different decoding processes for the same mode. It was suggested that this method could be tested for transform-skip blocks in lossy configuration.

Recommendation: Further study

JCTVC-N0075 Non-RCE2: Complexity reduction for inter residual DPCM in lossless coding [M. Naccari, M. Mrak (BBC)] (crosscheck: JCTVC-N0293 Microsoft)

This contribution proposes applying inter RDPCM process on chunks of samples. It resets the residual prediction every 8 samples in the DPCM direction. For lossless TU-based inter RDPCM, the losses are up to 0.8%. One participant commented that this was unnecessary (see comments for N0042) and that this proposal also had losses. Proponent commented that the losses were higher than this if inter RDPCM was restricted up to 8×8 block size.

Recommendation: Could be in a CE if there is one.

JCTVC-N0258 Non-RCE2: Extension of TU-Based Inter RDPCM [C. Pang, J. Sole, R. Joshi, M. Karczewicz (Qualcomm)] (crosscheck: JCTVC-N0342 Huawei)

This contribution proposes extending inter RDPCM to the first row/column of a TU, when the neighboring sample being used for prediction is in the same CU as the TU. For lossy configuration, the range of BD-rates observed was up to 0.1% loss to 0.2% gain. For lossless configuration, range of BD-rates observed was up to 0.1% loss to 0.7% gain. The complexity increase is one extra addition per first row/column, which is negligible according to the proponent and a participant. A participant commented that there is an increase in dependency on the neighboring TU.

Recommendation: No action

JCTVC-N0100 Non-RCE2: Unified lossless residual coding [Y. H. Tan, C. Yeo (I2R)]

This method is applicable only to lossless coding. It is proposed that the RDPCM direction is explicitly signalled for intra as well as inter. For inter coded blocks, this method is similar to JCTVC-N0074, the only difference is that in this contribution, for coding the RDPCM direction, no context is used. For intra-coded blocks, the prediction direction and the RDPCM direction are decoupled. For inter-coded blocks, the gains were similar to C.1. For intra-coded blocks, BD-rate gains of approximately 1.0% were observed for class B and RangeExt sequences. Loss of 0.5% was observed for YUV444 screen content sequences. There was a variation in performance for YUV444 sequences (up to 2% losses and up to 2% gains). For HEVC sequences, average gains of 1.4% were observed for non-class F sequences. One participant commented that the loss could only come from signaling and seemed a bit too high for the lossless configuration. The proponent commented that the use of context to signal the RDPCM direction could reduce the losses. This proposal introduces new syntax for intra (no longer mode dependent). One participant commented that there would be more complexity on the encoder side, and it would be desirable to be able to disable the signalling for intra-coded blocks. One participant commented that this should be tested for lossy conditions as well if C.1 and C.2 are adopted.

Recommendation:Further study (possibly in a CE) to test with lossy.

## Combinations

JCTVC-N0207 RCE2: Combination of Mirroring of Coefficients (Test B.3), Significance map context (Test B.1) and RDPCM (Test D.1) [R.Weerakkody, M.Mrak, M.Naccari (BBC)]

JCTVC-N0222 Non-RCE2: Results for combination of methods [J. Sole, R. Joshi, L. Guo, M. Karczewicz (Qualcomm)] (crosscheck: JCTVC-N0353 BBC)

JCTVC-N0354 Non-RCE2: Results for combination of Mirroring of non-transformed residual blocks (Test B.3) with other methods

Recommendation: Further study. These combinations provide additional information about whether the gains from different tools are additive.

## Transform Skip

JCTVC-N0288 Non-RCE2: Transform skip on large TUs [X. Peng, J. Xu (Microsoft), L. Guo, J. Sole, M. Karczewicz (Qualcomm)] (crosscheck: JCTVC-N0335 late)

Currently, transform-skip is applied only to 4×4 blocks. By extending to 8×8, additional gains were observed for lossy conditions on screen-content sequences. The BD-rate gains were in the range 0.7% -.0.8% for Class F, 1.0% - 1.3% for on YUV444). Extending transform-skip to 16x16 TUs yields gains of 0.6% - 0.9% for Class F, 1.3% - 1.6% for YUV444. Extending transform-skip to 32×32 yields 0.6% - 0.9% for Class F, 1.2% - 1.8% for YUV444. Encoder run time for all Intra conditions increases as follows: Extending to 8×8 115%, extending to 16×16 122%, extending to 32×32 131%. Another experiment used RExt without transform-skip as an anchor, and applied TS on 4×4 only, 8×8 through 32×32 only, 16×16 to 32×32 only, and 32×32 only. This shows gains were available even when 4×4 transform-skip was disabled. A participant commented that it would also be interesting to see results when 4×4 blocks were not used at all by the encoder. For signalling transform-skip, one context was used for all block sizes. Proponents recommend one of two options: extending transform-skip to 8×8, or extending transform-skip to all TU sizes. The cross-checker confirmed the results and supports either option in principal, and given the complexity/performance trade-off, prefers extending it to 8×8. Another participant commented that if this is adopted, maximum encoder flexibility should be allowed by extending to transform-skip to all the block sizes. Additionally, allowing transform-skip for all block sizes vs. restricting it makes little difference with respect to the decoder complexity. However, another participant commented that at bit-depths of 14 and higher, allowing transform-skip at 32×32 and may be at 16×16 may make the decoder different at different block sizes, due to the right shift needed to equalize norms when using transform-skip. Depending upon the bit-depth and block size, this normalization would become a left-shift. A participant commented that using HM 11.0 with other test sequences (not common-conditions test sequences), if RQT is disabled, there aren’t many 32×32 or 16×16 TUs used; thus extending transform-skip to 16×16 and 32×32 yields little benefit. Other participants asked for additional details regarding the previous statement. The proponent commented that extending transform-skip to all block sizes aligns transform-skip with transquantbypass and provides more encoder flexibility. A participant was concerned about the encoder run-time increase when extending transform-skip to all block sizes.

Recommendation: Discuss in the main track or plenary.

JCTVC-N0167 Transform skip based on minimum TU size [Kwanghyun Won, Seungha Yang, Byeungwoo Jeon (SKKU)] [late] (crosscheck: JCTVC-N0289 Microsoft)

This contribution proposes enabling transform skip when the current TU size is equal to the minimum TU size. For example, if one decides never to encode 4×4 TUs, the minimum TU size would be 8×8. This proposal would then allow transform-skip to be applied to 8×8 blocks. Given an anchor of HM 11.0 with minimum TU size of 4×4 with transform-skip enabled, the loss for Class F sequences using minimum TU size of 8×8 with transform-skip disabled is 37.5%. With anchor of HM 11.0 without transform-skip and minimum TU size of 8×8, gains of enabling transform-skip on 8×8 for this configuration is 15.1% for Class F, with encoder run time ratio of 120%. For another test, with an anchor of HM 11.0 with transform-skip enabled for 4×4 and minimum TU size of 4×4 and a tested configuration of HM 11.0 with minimum TU size of 8×8 and transform-skip enabled for 8×8, the BD-Rate loss is 15.3%, and the encoder run time ratio is 65%. It was noted that this contribution used HM instead of HM RExt, so the 444 screen-content sequences were not part of the experiments. A participant asked whether the complexity increase of enabling transform-skip on 8×8 in this case is more or less than just enabling 4×4 TUs and using transform-skip on 4×4 (when in both cases the LCU size was 16×16). It was noted that this proposal and JCTVC-N0288 may affect each other. The decoder-complexity comments made by participants for JCTVC-N0288 were also made for this proposal, however, in this proposal, transform-skip is only applied to one TU size, so the issues related to extending to larger block sizes don’t apply.

Recommendation: Further study to address the above comments; perhaps in a CE.

## Sample based intra prediction

JCTVC-N0079 Non-RCE2: Simplified sample based intra prediction for lossless coding [J.Zhu, W. Zheng, K. Kazui (Fujitsu)] (crosscheck: JCTVC-N0367 KDDI)

The purpose of this contribution is to reduce the encoding and decoding complexity of sample based intra prediction, for lossless coding. 13 out of 35 possible intra prediction modes are used: 11 angular modes, DC and planar. DC and planar modes are unchanged. The remaining angular modes use the nearest reference sample’s value in a specific direction as a prediction value rather than using a filter. The sample-based prediction is done without interpolation.

For angular modes, additional checks are required for the current diagonal modes, which is a change in how intra mode coding is done. For lossless test conditions, the average BD-Rate performance on Class F sequences is -0.7%, and for YUV444 -2.8%. There was some loss for class B (0.2%). The encoding time is reduced to 86%.

There was an architecture-related question regarding whether inverse RDPCM can be done after inverse quantization, or does it need to be done in the intra prediction block. It was noted that this proposal cases the intra prediction process to differ between lossy and lossless conditions.

There was a question as to what the impact would be from keeping strictly diagonal modes the same as HEVC. One participant commented that it may be 0.1-0.2%.

For diagonal modes, the inverse DPCM may need to be done in the intra prediction part (raster scan may not be possible).

If the HEVC intra mode coding process was used, how much gain would remain? (proponent commented that the reduction in gain may be less than 1.0%).

One participant commented that we are adding 11 modes to the current design.

A cross-checker reported that their anchor had some mismatches with the anchor distributed in RCE2. The cross-checked results for the proposed method, however, matched.

Recommendation: Further study, perhaps in a CE.

JCTVC-N0176 Non-RCE 2: On sample adaptive intra prediction for oblique modes in lossless coding [H. Chen, A. Saxena, F. Fernandes (Samsung)] (crosscheck: JCTVC-N0319 TI)

This contribution applies only for lossless coding. SAP is extended to 3 strictly diagonal (multiples of 45 degrees) and 4 pseudo-diagonal modes (for example, move throught the block two positions in the x direction and one position in the y direction). Performance improvement is 1.8% for YUV444 and 0.8% for class F. It was asserted to be fully parallel at the encoder and decoder. The average number of operations is less than RDPCM: for 4x4, 0.875 adds/sample, and 0.375 compared with 1.5. For 8x8, 2.1875 adds/sample, 1.3125 compared with 3.5.

The performance is summarized as follows:

- For 3 diagonal modes, all block sizes: -1.5% (AI), -1.0% (RA), -1.2% (LB)

  - For 4x4 only: -1.1%, -0.6%, -0.8%

  - For 4x4 + 8x8 only: -1.4%, -1.0%, -1.2%

- For 3 diagonal + 4 pseudo-diagonal, all block sizes: -1.8%, -1.2%, -1.3%

  - For 4x4 only: -1.2%, -0.6%, -0.9%

  - For 4x4 and 8x8 only: -1.6%, -1.1%, -1.2%

Some participants commented that different scan lines for a particular diagonal direction have different number of residuals that need to be added. More logic or switching would be needed. Some additional logic for pseudo diagonal mode parallel operation would be needed as well.

One participant asked what the additional gains for pseudo-diagonal modes were?: 0.3%, 0.2%, 0.1% for YUV444, for all block sizes.

JCTVC-N0177 Non-RCE 2: On sample adaptive intra prediction for oblique modes in lossy coding [A. Saxena, H. Chen, F. Fernandes (Samsung)] (crosscheck: JCTVC-N0363 Qualcomm)

This contribution is an extension of JCTVC-N0176 to lossy conditions (applied only to 4x4 and 3 diagonal modes).

Reported gains are: 0.6%-0.7% (AI), 0.5% (RA) and 0.4% (LB) for YUV444 over RCE2 Test C.2. For class F, the improvements over Test C.2 were approximately 0.1%-0.2%.

The BBC cross-checked the results after a bugfix.

It was commented that the decoder is fully parallel.

A cross-checker commented that from a compression efficiency point of view, these gains justify possible adoption. This cross-checker had no comment from complexity point of view. It was commented that if lossless is adopted, it makes sense to apply it for lossy coding as well, for a consistent design. These comments are applicable to both N0176 and N0177.

Generally it was felt that extending this to pseudo-diagonal directions was not worth the additional complexity.

Recommendation: Need additional opinions on whether the gains were worth the additional complexity/logic. It was felt that if the lossless version is adopted, for a consistent design it makes sense to include lossy.

## Misc

JCTVC-N0080 Non-RCE2: Skip of neighbouring samples filtering in intra prediction for lossless coding [J. Zhu, K. Kazui (Fujitsu)]

This contribution affects only lossless coding. It proposes disabling filtering of samples before prediction is applied. The reported gain is concentrated on one sequence, pcb_layout. The overall gains are -2.1% for YUV444 and -1.2% for RGB444. For all other sequences, there is negligible change in coding efficiency performance. The largest reported loss is 0.05%.

One participant commented that change is not justified based on gains for just one sequence. It was also noted that the decision is different for lossy and lossless.

No crosscheck was performed.

Recommendation: Discuss in main track or plenary.

JCTVC-N0113 Cross Residual DPCM for HEVC lossless coding [Y.-L. Lee, S.-W. Hong (Sejong University)]

This proposal applies to intra lossless conditions only. It applies horizontal RDPCM after vertical RDPCM and vice versa. A syntax element signals whether the 2nd stage of RDPCM is used. The flag is encoded in bypass mode at a TU level. The results on screen content conditions were incomplete, as only Class F and Class B sequences were tested. For Class F, some inconsistency in results was observed for the proposed method applied to HM9.0 vs. HM10.1-RExt-2.0.

It was asked whether the increase in number of operations (addtions) and new syntax element is justified by the gains.

One participant commented that this seems similar to a lossless transform.

Coding results for HVCE test sequences (AI lossless configuration) were better than JPEG2000 (CR avg. 2.78 vs 2.70). No crosscheck was performed.

Recommendation: Further study.

JCTVC-N0183 Non-RCE2: Enhanced angular intra prediction for screen content coding [H. Chen, A. Saxena, F. Fernandes (Samsung)]

This proposal uses nearest neighbor interpolation instead of bilinear interpolation for intra prediction. There is one additional condition check per pixel (the difference between pixels). Then switching between bilinear and nearest neighbor is done, as proposed in revision. Lossless YUV444 gains are 0.3% (AI), 0.0% (RA) and 0.3%(LB). For class F, gains are 0.1%, 0%, 0%. There is loss in performance for the proposed method in lossless coding. For lossy coding, YUV444 gains are 0.9% (AI MT), 0.7% (AI HT), 0.5% (AI SHT), 0.7% (RA MT), 0.3% (RA HT) and 0.3% (LD MT), 0.2% (LD HT). There are performance losses of up to 0.2% for the RExt sequences.

This contribution also proposed a variance-based criteria. The interpolation method for the whole block is decided based on variance.

It was commented that it would be interesting to explore turning the filtering off based on the same criteria, using SAD.

The encoder may have to perform the computations twice for the selection criteria: once with original samples and again with reconstructed samples. This was more of a concern for a variance based approach.

Recommendation: Further study in CE if there is a CE related to this topic