| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0386 |
| --- | --- |

| Title: | Cross-check for JCTVC-N0177: Non-RCE2: On sample adaptive intra prediction for oblique modes in lossy coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Matteo Naccari Marta Mark Rajitha Weerakkody Block D, Centre House 56 Wood Lane, W12 7SB London, UK | Email: | matteo.naccari@bbc.co.uk marta.mrak@bbc.co.uk rajitha.weerakkody@bbc.co.uk |
| Source: | BBC R&D |  |  |

_____________________________

# Abstract

This document reports the BBC’s cross-check results for Document JCTVC-N0177 proposed by Samsung. All the results released by the proponent are tested and it is reported that all the test points are confirmed. Analysis of the source code reveals perfect matching between the algorithm and the implementation.

# Introduction

Document JCTVC-N0177 proposes to extend the RDPCM to lossy coding and for more directions than the horizontal and vertical one. This contribution can be therefore seen as an extension of JCTVC-N0052 which refers to the extension of RDPCM to lossy coding. In the next section the cross-checked experimental results are presented.

# Test results

The proposal is tested for lossy coding using the RCE2 test conditions described in Document JCTVC-M1122. This cross-check document reports the same results in the following tables. Notice that the BD-rates for RGB content presented in this document are ordered differently from the ones released by the proponent. In particular the cross checker generated the anchors with the encoder parameter --SNRInternalColourSpace set to zero while the proponent set it to one. The overall effect of this parameter is a different ordering of the PSNR values according to the used input colour space conversion.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -2.5% | -3.0% | -3.1% | -2.6% | -3.2% | -3.2% | -2.6% | -3.2% | -3.2% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -6.2% | -6.4% | -6.2% | -6.3% | -6.6% | -6.5% | -5.6% | -6.1% | -5.8% |
| SC YUV 444 | -4.9% | -5.4% | -5.3% | -5.2% | -5.9% | -6.0% | -5.2% | -5.8% | -5.7% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 99% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -1.4% | -1.7% | -2.0% | -1.3% | -1.6% | -1.8% |  |  |  |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -3.8% | -3.8% | -3.8% | -3.8% | -3.9% | -3.9% |  |  |  |
| SC YUV 444 | -3.2% | -3.6% | -3.6% | -3.3% | -4.0% | -3.9% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.7% | -0.8% | -1.2% | -0.5% | -0.8% | -1.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -2.5% | -2.6% | -2.5% | -2.6% | -2.8% | -2.6% |  |  |  |
| SC YUV 444 | -1.7% | -1.7% | -1.8% | -1.8% | -1.9% | -2.0% |  |  |  |
| RangeExt | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

# Test results

The proponent implemented the proposed changes in the HM-10.0-rext-3.0 reference software. To implement the proposed changes the following macros have been defined:

RDPCM_DIAG

RDPCM_VER

RDPCM_HOR

RDPCM_DDR

RDPCM_DDL

RDPCM_DUR

These macros enable to test different sets of intra prediction directions for the proposed lossy RDPCM. Source inspection reveals perfect matching between the algorithmic description and the submitted implementation.

# Text changes

The proposed text changes match the description provided in the main proposal’s document.