| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0387 |
| --- | --- |

| Title: | BoG report on color gamut scalability |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | BoG report |  |  |
| Author(s) or Contact(s): | Andrew Segall | Email: | asegall@sharplabs.com |
| Source: | BoG chair |  |  |

_____________________________

# Abstract

This is the report on the BoG on color gamut scalability.  The BoG met on July 27, 2013, and August 1, 2013.  Additionally, the BoG conducted a viewing session on July 31, 2013.

# Meeting notes

[July 27, 2013]

The BoG reviewed proposals related to wide color gamut.  Notes of the presentation and discussion follow:

JCTVC-N0163 AHG14: Wide Color Gamut Test Material Creation [Pierre Andrivon, Philippe Bordes (Technicolor)]

This contribution reports the way Wide Color Gamut content has been created in order to provide JCT-VC AhG14 on Color Gamut Scalability with test material. Content was acquired with Sony F65 cinema camera then color graded in post-production facilities in order to provide AhG14 with two sets of the same sequences but color graded differently. Four sequences were graded for Rec.709/HDTV reference monitors and also for DCI-P3 gamut monitors. It is stated that provided sequences are 8 and 10-bit, Y’CbCr 4:2:0 with 1920x1080, 1280x720 (1.5x down-sampled) and 960x540 (2x down-sampled) resolutions. License agreement is provided in attachment.

The proponent reported that the colorist took care of giving a natural looking style to graded content as well as creating out-of-gamut Rec.709 colors for the P3 graded contents.

It was reported that each sequence length is 500 frames.

It was also reported that the same gamma calculation applied for both BT.709 and P3 data.

One participant suggested arranging viewing of the data during the week.

The proponent reported that out of gamut (with respect to BT.709) colors were observed.

During the presentation, there was further discussion about test conditions.  This discussion included:

It was requested that the enhancement layer contain 4kx2k data

The proponent noted that the graded content was provided in 1080p.

Some concern was expressed that the 540p result (derived from the 1080p) may have aliasing due to filter design and that it should be checked.

There was a suggestion to consider 2x scalability with 4k enhancement layer.

There was discussion that representing the P3 gamut in a BT.2020 container may result in some quantization effects.

There was a suggestion to analyze the quantization error that would occur when expressing P3 data in BT.2020.

JCTVC-N0168 AHG14: Color Gamut Scalable Video Coding using 3D LUT: New Results [Philippe Bordes, Pierre Andrivon, Franck Hiron (Technicolor)]

This contribution reports new results in the area of color gamut scalable video coding. In the last meeting, JCTVC-M0197 proposed an inter-layer prediction model for color gamut scalable video coding based on 3D color Look-Up Tables (LUT). It was asserted the application requirements in term of color gamut scalable video coding is not limited to simple transformation between Base layer color space (ex: Rec.709) and Enhancement layer color space (ex: Rec.2020), but also the cases the Base layer and the Enhancement layer has been color graded differently.

It is asserted the size of the 3D LUT can be chosen to meet both the application complexity requirements and the BD-rate distortion trade-off: small size for describing simple color mapping transformation between Base layer and Enhancement layer, and larger size for representing more complex color differences in-between the layers.

Results are reported in 8 bits with the 4 Color graded sequences proposed in AHG-14, between Base layer using Rec.709 color space and Enhancement layer using DCI-P3 color space. It is reported the BD-rate gains of SHM2.0-CGS compared to the simulcast scenario (SHM2.0) is between 4.8% to 32% for luma in average, the BD-rate gains of SHVC (SHM2.0-CGS vs SHM2.0) is between 5.5% to 6.8% for luma in average (between 6.2% to 16.7% for EL).

It was reported that the SHM needs to be modified to support an 8-bit baselayer and 10-bit enhancement layer.

The proponent requested there be further study in a core experiment.

The proponent also reported that the LUTs (look up tables) are content adaptive and signaled in the bit-stream at the sequence level.  The LUTs were designed using analysis of the first frame from every sequence.

One participant asked to clarify if a change in spatial resolutions between layers was considered.  The proponent responded that a change in spatial resolutions was considered.  Furthermore, the proponent reported that the down-sampler used to generate the test sequences was the same as used in SHVC development.

A participant suggested that the group should consider and address the following questions:

- The need for supporting color gamut scalability with different bit-depths.

- The test conditions that should be used for evaluation.

- The appropriateness of the test sequences as well as the desirability of using BT.2020 as a container.

JCTVC-N0274 AHG14: Cross-checking of JCTVC-N0168 from Technicolor on color gamut scalable video coding using 3D LUT [C. Auyeung (Sony)] [miss]

JCTVC-N0278 AHG14: cross-check of JCTVC-N0168 on Color Gamut Scalable Video Coding using 3D LUT: New Results [P. Onno (Canon)] [late]

JCTVC-N0271 AHG14: Color gamut scalable video coding with piecewise linear predictions [C. Auyeung (Sony)]

It is reported that JCTVC-L0334 provided results for color gamut scalability with SMuC-0.1.1 where the input of the base layer is in BT.709 with 8 bits and the input of the enhancement layer is in BT.2020 with 10 bits. This contribution compares the color space linear predictor called Gain-Offset Sequence Adaptive (GO-SA) in JCTVC-L0334 with the proposed piece-wise linear predictor for color gamut scalable video coding. The overall luma BDRs for AI-2x, RA-2x, and SNR scalability are -0.8%, -0.7%, and -0.9%, respectively. It is reported that there were some losses in BDR for SNR scalability at 0.5% and 1.1% for Cb and Cr. It is suggested that the coding loss in Cb and Cr in piece-wise linear prediction is likely to be caused by lack of QP and configuration adaptation implemented in the Gain-Offset prediction.  This contribution also introduces shift-offset model for color gamut prediction, which is simple and yet can provide better coding efficiency than bit-increment model as proposed by JCTVC-L0334.

The proponent reported some losses in chroma, but it was asserted to be due to the design of the encoder.

There was a suggestion to consider for study the proposed method in any on-going work on color gamut scalability

[July 31, 2013]

The BoG conducted an informal viewing session to review the sequences provided in JCTVC-N0163.  Approximately 20 experts participated in the viewing.

[ August 1, 2013]

The BoG met again on August 1, 2013, to discuss the available test content and direction forward.  Notes from the discussion follow:

A straw man for common test conditions was discussed, and a strawman for the common conditions was suggested as follows:

Goal: Target UHD enhancement layer and HD baselayer

Test 1: Enhancement layer: 4K resolution, 10-bit, BT.2020 gamut / Baselayer layer: 1080p, BT.709, 8-bit

Test 2: Enhancement layer: 4K resolution, 10-bit, BT.2020 gamut / Baselayer layer: 1080p, BT.709, 10-bit

There was a suggestion to reduce the number of frames in the test sequences to 300 frames.  Agreed.

There was a suggestion to crop sequences to size as Class A.

Multiple participants expressed concern about cropping due to changing the spectral characteristics of the sequences.

There was a comment that the content we currently have is graded in the P3 gamut.  It is noted that we can express the same gamut in BT.2020 with the understanding that the content will not fill the entire BT.2020 gamut.

It was commented that support of BT.2020 is intended by multiple manufactures.

It was suggested that we convert the enhancement layer versions of test sequences to BT.2020.  This was agreed, and Technicolor volunteered to do the conversion.

There was a comment that it may be acceptable to apply current color grading to the UHD (3840x2160p) footage.  (The current color grading was performed at a 1080p resolution.)  It was agreed to apply the 1080p current grading to the UHD resolution version of the content.  Technicolor volunteered to do the grading.

It was commented that the current test material is sufficient to go forward with the study of color gamut scalability. Agreed.

The group discussed that additional content is always welcome.

Comment that while the group agreed that reducing the number of frames would be desirable, it would be best to keep all of the frames of the “flash” sequence to maximize the number of colors used by the flash.

One participant suggested to keep all the frames from the “flash” sequence by dividing the sequence into two sequences.  Agreed.

Recommendation to create sequences that are 5 seconds in duration.  For 50Hz content, this would be 250 frames.  For 60Hz content, this would be 300 frames.  Agreed.

It was suggested that the beginning of the party scene may have more variety in colors.

Technicolor reported that an additional 100 frames of the “flash” sequence could be made available in order to create two versions each with 300 frames.

It was suggested that the “car” sequence was similar in terms of characteristics for the entire sequence.  So, it is possible to choose any 300 frame subset without concern.

Recommendation to form an AhG including the mandate “to create and distribute test content”. Agreed.

Recommendation to form an AhG including the mandate “develop and study software for testing color gamut scalability”.  Agreed.

It was suggested to use only four QP points from the SHVC common test conditions.

Recommendation to form an AhG including the mandate “study and define test conditions for color gamut scalability”.  Agreed.

Question: Should we also form a CE to study techniques?

Suggestion to form a CE to study proposals in the area of color gamut scalability (including bit-depth scalability when the enhancement and baselayer have different bit-depths and different gamuts). Agreed.

Recommendation to form an AhG including the mandate “provide common test conditions and test content within 6 weeks following the meeting”.  Agreed.

Agreed on strawman test conditions above.

Discussion about needed software modification in the AhG.  Comment that bit-depths different than 8-bits are not supported SHM2.0, but there is a contribution (JCTVC-N0214) that appears to fix the problem. Suggest that the software used for experiments include the bugfix. Agreed.

Comment that JCTVC-N0146 and JCTVC-N0218 are related to bit-depth scalability and relevant to this study.

Suggested plan going forward:

- Continue AhG on color gamut scalability with additional mandates above.

- Create CE for organized testing of proposals for color gamut scalability.  To include JCTVC-N0146, JCTVC-N0168, JCTVC-N0218, JCTVC-N0271, and JCTVC-M0214.