| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 |  |  | Document: JCTVC-N1101r1 |  |
| --- | --- | --- | --- | --- |
| Title: | Description of HEVC Scalable Extension Core Experiment SCE1: Arbitrary scalability ratio support |  |  |  |
| Status: | Output Document of JCTVC |  |  |  |
| Purpose: | Core Experiment Description |  |  |  |
| Author(s) or Contact(s): | Edouard François (primary) Elena Alshina Jianle Chen | Tel: Email: |  | edouard.francois@crf.canon.fr elena_a.alshina@samsung.com cjianle@qti.qualcomm.com |
| Source: | CE coordinators |  |  |  |

_____________________________

# Abstract

This document provides the description of HEVC Scalable Extension Core Experiment SCE1. The topic of the Core Experiment is testing re-sampling filters with fixed coefficients variants for arbitrary scalability ratio support.

# Introduction

During the 14th JCT-VC meeting in Vienna, two proposals of fixed re-sampling filters sets needed for arbitrary scalability ratio support in SHVC design were proposed. It was then decided to coordinate the further study and testing of these filters in this Core Experiment (CE).

# Participants List

| Nr. | Name | Company | Email | Remarks |
| --- | --- | --- | --- | --- |
| 1 | Jianle Chen Xiang Li | Qualcomm | cjianle@qti.qualcomm.com lxiang@qti.qualcomm.com | P, C |
| 2 | Elena Alshina Alexander Alshin | Samsung | elena.alshina@gmail.com elena_a.alshina@samsung.com alexander_b.alshin@samsung.com | P, C |
| 3 | Edouard François | Canon | edouard.francois@crf.canon.fr | C |
| 4 | Kemal Ugur | Nokia | kemal.ugur@nokia.com | P, C |
| 5 | Jie Dong Yuwen He Yan Ye | Interdigital | jie.dong@interdigital.com yuwen.he@interdigital.com yan.ye@interdigital.com | C |
| 6 | Koohyar Minoo David Baylon | Arris Group, Inc. | koohyar.minoo@arrisi.com david.baylon@arrisi.com | P, C |

# Tools to test

The tools in this CE are as follows:

- SCE-1.1 Non-SCE1: On arbitrary spatial ratio scalability in SHVC (JCTVC-N0219)

- SCE-1.2. On the selection of fixed filters for upsampling (JCTVC-N0273)

Summary of the testing tools is provided in the following table.

| Test | Technique | Proponent | Crosschecker |
| --- | --- | --- | --- |
| 1.1 | Non-SCE1: On arbitrary spatial ratio scalability in SHVC | Jianle Chen (cjianle@qti.qualcomm.com) Elena Alshina (elena_a.alshina@samsung.com) | Canon |
| 1.2 | On the selection of fixed filters for upsampling | Koohyar Minoo (koohyar.minoo@arrisi.com) | Samsung, Qualcomm |

## Non-SCE1: On arbitrary spatial ratio scalability in SHVC (JCTVC-N0219)

In suggested test reference, position and phase derivation process is identical to SHM3.0 and recent SHVC specification (1/16 accuracy both for Luma and Chroma). Interpolation filter set is identical to JCTVC-N0375, except Luma filter for 6/16 position which is

{6,-1,4,-11,50,28,-8,2,0}/64.

This re-sampling filter modification doesn’t affect performance in spatial scalability tests ×1,5 and ×2. Only arbitrary scalability test is required.

## On the selection of fixed filters for upsampling (JCTVC-N0273)

In this test the suggested filters in N0273 would replace the reference filters in N0375 with the exception of filters which are identical in N0273 and N0375.

In this test 1.5x and 2x and the arbitrary scaling ratios elaborated in N0375 will be tested using the new base layer sequences generated by modified JSVM downsampler software.

Specific to this test, as some of the filters in N0273 will replace the corresponding filters for 1.5x, the CTC condition for spatial scalability needs to be tested with the suggested filters in N0273.

# Experimental Conditions

  - Test Sequences, Bit Rates and Coding Conditions

Tools under study affect only spatial scalability tests. The LD-P test configuration is also mandatory to be tested. For enhancement layer video sequences defined in JCTVC-N1009 will be used in all tests in order to use single layer coder as comparison point. Various down-sampler techniques and various resolutions for down-sampled sequences will be studied.

For this SCE, 3 groups of tests will be performed.

Group 1: Common test conditions and sequences

The SHVC common test conditions and sequences defined in JCTVC-N1009 shall be used for this SCE (no change of the BL sequences). Test 1.1 doesn’t change performance of SHM-3.0 for this group of tests, so only 1.2 will be tested.

Group 2: Test sequences down-sampled using JSVM down-sampler.

The JSVM down-sampler will be modified to use phase offset set to 0 for the purpose of this test. Base layer sequences will be generated specifically for this SCE, keeping the same resolutions as CTC:

- Class A ×2 – horizontal: 2560 / 1280 – vertical: 1600 / 800

- Class B ×2 – horizontal: 1080 / 540 – vertical: 1920 / 960

- Class B ×1,5 – horizontal: 1080 / 720 – vertical: 1920 / 1280

SHM3.0 s/w will be tested with modified base layer (using CTC conditions and tests set) in order to check performance with alternative down-sampler.

Test 1.1 doesn’t change performance of SHM-3.0 for this group of tests, so only 1.2 will be tested.

Group 3: Arbitrary scalability ratio.

The JSVM down-sampler will be modified to use phase offset set to 0 for the purpose of this test. Base layer sequences will be re-generated using scalability ratio (~1,75) which allows to exersice re-sampling filters for all fractional positions:

- Class A ×1,75 – horizontal: 2560 / 1456 – vertical: 1600 / 912

- Class B ×1,75 – horizontal: 1080 / 616 – vertical: 1920 / 1096

SHM3.0 with filter set mentioned in JCTVC-N0375r2 will be used as an anchor in this test.

Both filters sets (Test 1.1 and Test 1.2) will be tested for this content.

Software

Reference software SHM-3.0 (with filter set mentioned in JCTVC-N0375r2 integrated as agreed by JCTVC group) will be used as the anchor and base software in this CE.

  - Evaluation of CE Results

Results of the SCE will be evaluated on the basis of subjective quality, coding efficiency and complexity. Since base layer is different for 3 groups of tests the comparison with single layer HEVC will be used.

### Subjective quality assessment

Visual quality evaluation for base layers created using different techniques: JSVM down-sampler and “CTC”-downsampler is scheduled to be done at the next meeting.

Both anchor, test 1.1 and test 1.2 will be evaluated visually during next meeting. Exact test point for visual quality evaluation will be defined before the start of JCTVC 15th meeting.

### Coding efficiency assessment

Coding efficiency will be measured based BD-rate saving based on enhancement layer PSNR and total bit rate of base layer and enhancement layer.

### Complexity assessment

Complexity assessment is performed on the basis of encoding abd decoding complexity measurement. To measure software run time, the software of the proposed method and the anchor software shall be tested on homogenous cluster or a single PC.

In the worst case memory access and computation complexity will be the same for both anchor, test 1.1 and test 1.2. Average memory access and computations will be evaluated using methodologies defined in JCTVC-M0455.

# Time-line and Responsibilities

2013-Aug-16:	SCE-1 description finalized and uploaded. Release of the Excel template for Complexity evaluation

2013-Aug-16:	Release of reference software SHM3.0 from AHG ‘SHVC software’

2013-Aug-23:	Release of reference software SHM3.0 with complexity assessment modules

2013-Aug-16:	Release of downsampler software (by Qualcomm)

2013-Aug-23:	Alternative down-sampled sequences are up-loaded (by Samsung)

2013-Aug-30:	Release of anchors test results SHM3.0 for the modified JSVM downsample BL with 1.5x, 2x and the alternative scaling ratio in N0375

2013-Sep-13:	Cross-verification begins, proponents provide software, draft of contribution text and results to cross-verifiers(s) and CE coordinators

2013-Sep-20:	Proponents provide complexity analysis results to cross-verifiers(s) and CE coordinators

2013-Oct-04:	Cross-verifiers report results to CE participants

2013-Oct -09:	Contribution documents uploaded, the document number and title are reported to coordinators.

2013-Oct -23:	Summary report uploaded

# References

- E. François, “BoG Report on arbitrary scalability ratios support in SHVC”, JCTVC-N0375, Vienna, Austria, 25 July-2 August. 2013

- E. Alshina, E. François, “BoG Report on SHVC complexity assessment”, JCTVC-M0455, Incheon, South Korea, 18–26 Apr. 2013

- X. Li, J. Boyce, P. Onno, Y. Ye, “Common SHM test conditions and software reference”, JCTVC-N1009, Vienna, Austria, 18–26 Apr. 2013