| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N1102 |
| --- | --- |

| Title: | HEVC Scalable Extension Core Experiment SCE2: Key pictures and single-loop decoding |  |  |
| --- | --- | --- | --- |
| Status: | Output Document by the JCT-VC |  |  |
| Purpose: | CE-Description |  |  |
| Author(s) or Contact(s): | Mathias Wien Krishna Rapaka Xiaoyu Xiu | Tel: Email: | wien@ient.rwth-aachen.de krapaka@qti.qualcomm.com Xiaoyu.Xiu@InterDigital.com |
| Source: | CE Coordinators |  |  |

_____________________________

# Abstract

This document contains the description of SHVC core experiment SCE2 on key-pictures and single-loop decoding. The core experiment is based on the input documents JCTVC-N0161, JCTVC-N0186, and JCTVC-N0202 to the Vienna JCT-VC meeting, July 2013.

# Participants

| Participants | Affiliation | Proponent | Crosscheck |
| --- | --- | --- | --- |
| Krishna Rapaka (krapaka@qti.qualcomm.com), Jianle Chen (cjianle@qti.qualcomm.com) | Qualcomm | X |  |
| Christian Feldmann (feldmann@ient.rwth-aachen.de), Mathias Wien (wien@ient.rwth-aachen.de) | RWTH Aachen University | X |  |
| Xiaoyu Xiu (Xiaoyu.Xiu@InterDigital.com), Yan Ye (Yan.Ye@InterDigital.com), Yuwen He (Yuwen.He@InterDigital.com) | InterDigital | X |  |
| Kemal Ugur (Kemal.Ugur@nokia.com) | Nokia |  |  |
| Elena Alshina (elena_a.alshina@samsung.com) Ankur Saxena (asaxena@sta.samsung.com ) | Samsung |  |  |
| Jill Boyce (jill@vidyo.com) | Vidyo |  |  |
| Yu-Wen Huang (yuwen.huang@mediatek.com), Peter Chuang (peter.chuang@mediatek.com), Shan Liu (shan.liu@mediatek.com) | Mediatek |  |  |
| Tomoyuki Yamamoto (yamamoto.tomoyuki@sharp.co.jp), Takeshi Tsukuba (tsukuba.takeshi@sharp.co.jp) | Sharp Corporation |  |  |
| Hahyun Lee (hanilee@etri.re.kr) | ETRI |  |  |

# Technology under Evaluation

Document JCTVC-N0161 proposes to apply the key-picture concept of SVC in the multi-loop approach of SHVC. Thereby, reconstructed enhancement layer (EL) pictures are used for inter prediction in base layer (BL) pictures. The scheme improves the compression efficiency in the EL. Further, the scheme also enables single-loop decoding to avoid the need of motion compensation in the reference layers.

Document JCTVC-N0186 proposes a single-loop decoding approach for SHVC. The BL motion information is applied to the EL reference pictures for inter prediction and the BL residual is added in addition to the EL residual. Constrained intra prediction is applied in the base layer. The scheme enables single-loop decoding and does not induce drift into the reconstructed BL pictures.

Document JCTVC-N0202 proposes a single-loop decoding approach for SHVC using the key-picture concept as applied in SVC. The reconstructed EL pictures are used for inter prediction in BL non-key-pictures.  BL motion vector coding is applied without reference to EL syntax elements. The proposed scheme induces drift to the reconstructed BL pictures if only the BL is reconstructed. The drift is controlled by periodic insertion of key-pictures (at tid=0). The contribution presents results with multi-loop decoding as well as with single-loop decoding. In the latter case, constrained intra prediction is applied in the base layer.

## JCTVC-N0161: Using decoded pictures from higher layer as references in SHVC

This contribution proposes a design to enable using the decoded pictures from higher quality layer as reference for lower layer pictures for SNR scalability in SHVC. The document uses the concepts of key pictures to reduce error drift that is induced in the base layer.

Further, this contribution proposes few encoder only constraints to allow single-loop decoding design for SHVC SNR scalability. Based on the key picture framework, separate encoder only constraints for key and non-key pictures are proposed to avoid the need of motion compensation in the reference layer.

The method is to be tested alone and together with other methods in this CE.

## JCTVC-N0186: Single-loop decoding of SNR scalability for refIdx based SHVC

This contribution describes a single-loop decoding scheme for SNR scalability of reference index based SHVC. In the current reference index based SHVC, the reconstructed base layer (BL) picture (after up-sampling if necessary) is used for the inter-layer prediction (ILP) of enhancement layer (EL) coding. This implies that multiple motion compensation (MC) operations have to be performed for all dependent layers. In this contribution, a single-loop decoding scheme is achieved by introducing an alternative ILP picture by applying BL motion information on EL temporal reference pictures. In addition, BL residue is added to the alternative ILP picture for quality improvement. As both BL motion information and BL residue could be obtained without the full reconstruction of BL picture, the single-loop decoding requirement is fulfilled by replacing the conventional ILP picture with the alternative ILP picture for EL prediction. Compared to the existing single-loop decoding approaches studied in AHG16, the proposed scheme does not require any low-level changes to single-layer HEVC.

## JCTVC-N0202: Key picture concept and single loop decoding

In this document a performance analysis of the key picture concept in SHVC is presented. The scheme has similarity with MGS as used in SVC. Key-pictures are introduced, that only utilize other key-pictures as reference. For non-key-pictures a prediction from a higher layer reconstruction is performed. The proposal increases the overall performance but also introduces drift when decoding only the base layer.

As a modification of the scheme, a variant is proposed that adds further restrictions to the encoder. Namely these restrictions are the use of constrained intra prediction in the base layer and utilization of the unfiltered BL reconstruction for inter layer prediction. These restrictions enable single loop decoding for the enhancement layer.

# Testing conditions

## Definition of test anchors

Testing configurations according to the Random Access and Low-Delay configurations in the CTC shall be used in this CE.

### Multi-loop anchor

The following anchors shall be used for evaluation of the proposals in the CE.

- Anchor1: SHM3.0 according to SHM CTC JCTVC-N1009
Note: Prediction structure includes reference across hierarchy layers and does not allow for temporal scalability

- Anchor2: SHM3.0 CTC with modified reference picture lists; configuration enabling temporal scalability (according to JCTVC-L0322)

  - Key-pictures period of 8 pictures for random access

  - Key-picture period of 4 pictures for low-delay

The proponents are obliged to provide the comparisons of the proposed methods with both of two anchors.

### Single-loop anchor

As suggested in section 6.2.8 of the Vienna meeting report, an additional reference point shall be used for assessment of the proposed methods, making use of IRAP-only single-loop scalable coding based on max_tid_il_ref_pics_plus1.

# Evaluation methodology

## Rate-distortion assessment

Two XLS with results comparing the proposals to Anchor1 and Anchor2 of section 3.1.1 are to be provided by CE proponents. Comparison to the SL anchor defined in section 3.1.2 is suggested.

## Subjective assessment

Informal subjective assessment of visual quality of BL-only reconstruction to be conducted if different from the single loop BL reconstruction, reporting to be included in the CE responses. The method of subjective evaluation shall be reported by participants.

## Complexity assessment

### Run time

To measure software run time, software with the anchor configuration and the proposal implemented on the software shall be used. To measure decoding time, the decoding shall be executed on the same machine without output of decoded YUV data. The computational time must be measured for each test sequence and test case for both anchor and proposal.

### Memory bandwidth

Use memory bandwidth measurement tool maintained by AHG17 for motion compensation. Further, inter-layer residual data access and inter-layer (inter and intra) sample data access will be measured.

The data will be reported in a XLS sheet based on the reporting sheet of AHG17. [Template prepared by Qualcomm]

### Pixel-level processing

(The number of pixels processed by motion compensation is represented by the memory access analysis)

The number of pixels processed by the deblocking filter and by SAO shall be reported. [Template prepared by RWTH]

# Timeline

2013-08-16		SHM 3.0 available

2013-08-23		SCE x description finalized

2013-08-30		SHM3.0 CE-Anchor  available

2013-09-15		Exchange of software for crosscheck

2013-09-22		Complexity results released

2013-10-14		Upload of input contributions

# References

- X. Li, J. Boyce, P. Onno, Y. Ye, “Common SHM test conditions and software reference configurations”, JCTVC-M1009

- E. Alshina, E. François, “BoG Report on SHVC complexity assessment”, JCTVC-M0455