| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 |  |  | Document: JCTVC-N1103_v2 |  |
| --- | --- | --- | --- | --- |
| Title: | Description of HEVC Scalable Extension Core Experiment SCE3: Inter-layer filtering |  |  |  |
| Status: | Output Document of JCTVC |  |  |  |
| Purpose: | Core Experiment Description |  |  |  |
| Author(s) or Contact(s): | Jianle Chen (primary) Elena Alshina Jie Dong Maxim Sychev | Email: |  | cjianle@qti.qualcomm.com elena_a.alshina@samsung.com jie.dong@interdigital.com Sychev.Maxim@huawei.com |
| Source: | CE coordinators |  |  |  |

_____________________________

# Abstract

This document provides the description of HEVC Scalable Extension Core Experiment SCE - 3. The topic of the Core Experiment is testing Inter-layer filters for SHVC.

# Introduction

During the 14th JCT-VC meeting in Vienna, multiple proposals for fixed, adaptive and additional inter-layer filters for the SHVC design were proposed. Two proposals (JCTVC-N0229 and JCTVC-N0070) shows significant gain. It was then decided to coordinate for further study and testing of the two filters in this Core Experiment (CE).

# Participants List

| Nr. | Name | Company | Email | Remarks |
| --- | --- | --- | --- | --- |
| 1 | Jianle Chen Xiang Li Wei Pu Marta Karczewicz | Qualcomm | cjianle@qti.qualcomm.com lxiang@qti.qualcomm.com wpu@qti.qualcomm.com martak@qti.qualcomm.com | P, C |
| 2 | Elena Alshina Alexander Alshin Ankur Saxena | Samsung | elena.alshina@gmail.com elena_a.alshina@samsung.com alexander_b.alshin@samsung.com asaxena@sta.samsung.com | P, C |
| 3 | Jie Dong Yuwen He Yan Ye | Interdigital | jie.dong@interdigital.com yuwen.he@interdigital.com yan.ye@interdigital.com | P, C |
| 4 | Haitao Yang Maxim Sychev Valery Anisimovskiy Ikonin Sergey | Huawei | haitao.yang@huawei.com Sychev.Maxim@huawei.com Anisimovskiy.Valery@huawei.com Sergey.Ikonin@huawei.com | P, C |
| 5 | Shang Liu Yuwen Huang Peter Chuang Polin Lai | MediaTek | shan.liu@mediatek.com yuwen.huang@mediatek.com peter.chuang@mediatek.com polin.lai@mediatek.com |  |
|  | Hahyun Lee Jung Won Kang Jinho Lee, | ETRI | hanilee@etri.re.kr jungwon@etri.re.kr jinosoul@etri.re.kr |  |

# Tools to test

The tools in this CE are as follows:

- SCE-3.1 Region based Inter-layer Cross-Color Filtering (JCTVC-N0229)

- SCE-3.2 Inter-layer prediction based on base layer sharpness filter (JCTVC-N0070)

Summary of the testing tools is provided in the following table.

| Section | Technique | Proponent | Crosschecker |
| --- | --- | --- | --- |
| 3.1 | Region based Inter-layer Cross-Color Filtering | Xiang Li (lxiang@qti.qualcomm.com) Elena Alshina (elena_a.alshina@samsung.com) Jie Dong (jie.dong@interdigital.com) | Maxim Sychev (Sychev.Maxim@huawei.com) |
| 3.2 | Inter-layer prediction based on base layer sharpness filter | Maxim Sychev (Sychev.Maxim@huawei.com) | Elena Alshina (elena_a.alshina@samsung.com) |

## Region based Inter-layer Cross-Color Filtering (JCTVC-N0229)

In JCTVC-N0229, a region based inter-layer cross-color filtering was proposed.

This inter-layer processing tool is the Chroma samples refinement based of filtereing surrounding Luma samples. 8-taps cross-shape filter shown on Figure below is used. Variable filter coefficents are limited in range -8…7 and signalled in a bit-stream.

To obtain good adaptation to video content, an enhancement picture is split into equal-size regions. With picture split on uniform partitioning. Up to 64 regions are supported in the current implementation. Maximum number of regions is signalled in SPS. For each chroma component of an enhancement picture, number of regions  is indicated by partition depth  where . For example,  and  indicate that Cb component is split into 4 region and Cr component is split into  regions.

For each region in each Chroma component, a set of inter-layer cross-color filter (on/off flag, 7 from 8 coefficients and scaling multiplier limited in range -1024…1024) parameters are signaled in bitstream. The scan order of regions is the same as the z-order in CU.

In the current implementation, the partitioning depth of each chroma component is adaptively determined by the encoder before coding an enhancement picture.

To demonstrate the performance of the proposed method, supplementary results of fixed number of regions will be also provided.

## Inter-layer prediction based on base layer sharpness filter (JCTVC-N0070)

JCTVC-0070 presents an approach based on interlayer intra sharpening technique.

Upscaled image sharpening is performed using a blurred edge map and warping pixels to edge boundaries. Edge map obtained by applying a Prewitt filter  and  to luma component and calculating truncated simplified magnitude . Then, blurring is performed for this edge map. For each CU, which is coded with refidx == SharpenedFrameIDX, a warping mechanism  is performed to pixels using displacement vectors estimated using blurred edge map  and . For chroma component, the downsampled luma edge map is used. There are 4 parameters of the sharpening filter (K, Tr, ShD,ShDch) that should be written to SPS.

# Experimental Conditions

  - Test Sequences, Bit Rates and Coding Conditions

The SHVC common test conditions defined in JCTVC-N1009 shall be used for this CE. In addition, SNR scalablity case and LD-P test configuration are also mandatory to be tested if the tested method is applicable.

  - Software

Reference software SHM-3.0 will be used as the anchor and base software in this CE.

  - Evaluation of CE Results

Results of the CE will be evaluated on the basis of coding efficiency and complexity.

### Coding efficiency assessment

Coding efficiency will be measured based BD-rate saving based on enhancement layer PSNR and total bit rate of base layer and enhancement layer.

### Complexity assessment

Complexity assessment is performed on the basis of encoding abd decoding complexity measurement. To measure software run time, the software of the proposed method and the anchor software shall be tested on homogenous cluster or a single PC.

In addition, methodologies defined in JCTVC-M0455 is used to evaluate memory bandwidth requirement and computational complexity.

# Time-line and Responsibilities

2013-Aug-16:	SCE-3 description finalized and uploaded. Release of the Excel template for Complexity evaluation

2013-Aug-16:	Release of reference software SHM3.0 from AHG ‘SHVC software’

2013-Aug-23:	Release of reference software SHM3.0 with complexity assessment modules

2013-Sep-06:	Cross-verification begins, proponents provide software, draft of contribution text and results to cross-verifiers(s) and CE coordinators

2013-Sep-13:	Proponents provide complexity analysis results to cross-verifiers(s) and CE coordinators

2013-Sep-27:	Cross-verifiers report results to CE participants

2013-Oct-14:	Contribution documents uploaded, the document number and title are reported to coordinators.

2013-Oct-23:	Summary report uploaded

# References

- X. Li, J. Boyce, P. Onno, Y. Ye, “Common SHM test conditions and software reference”, JCTVC-N1009, Vienna, Austria, 18–26 Apr. 2013

- E. Alshina, E. François, “BoG Report on SHVC complexity assessment”, JCTVC-M0455, Incheon, South Korea, 18–26 Apr. 2013