| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N1104 |
| --- | --- |

| Title: | Description of Core Experiment SCE4: Color Gamut and Bit-Depth Scalability |  |  |
| --- | --- | --- | --- |
| Status: | Output Document to JCT-VC |  |  |
| Purpose: | CE description |  |  |
| Author(s) or Contact(s): | Andrew Segall, Philippe Bordes, Cheung Auyeung, Xiang Li, Elena Alshina, Alberto Duenas | Emails: | asegall@sharplabs.com, philippe.bordes@technicolor.com, Cheung.Auyeung@am.sony.com, lxiang@qti.qualcomm.com, elena_a.alshina@samsung.com, alberto@ngcodec.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document provides the description of Core Experiment 4 (SCE4) on Color Gamut and Bit-Depth Scalability in SHVC. It is based on the document BoG report on color gamut scalability [1].

# Introduction

Color Gamut Scalability has been identified as one requirement of Scalable Coding Extension of HEVC. It allows addressing the cases the original Enhancement Layer uses a different color gamut than the Base Layer. This can be useful for instance in case of deployment of UHD services compatible with legacy HD devices: HD is using the Rec.709, while UHD is likely to use some of the parameters defined in the Rec.2020.

The general diagram of a scalable video encoder including a prediction tool for color differences between the base layer (BL) and enhancement layer (EL) is shown in Figure 1.

Figure 1: Color Space Scalable Encoder (courtesy from Sharp).

Basically, the role of the Color Difference Predictor module is to predict the EL color samples from the collocated BL color samples.

During the 14th JCT-VC meeting in Vienna, several Color Difference Predictor models have been proposed that will evaluated in this CE. Additionally, the test conditions and the test material should be adapted to better fit the typical targeted use cases. In particular, this will need to adapt the reference software to support bit-depth scalability.

# Participants

P = Participants (Contribution),

C = Crosscheckers.

| Company | Person | Email address | P | C |
| --- | --- | --- | --- | --- |
| Sharp | A.Segall S.-H. Kim S. Deshpande T. Yamamoto | asegall@sharplabs.com kimse@sharplabs.com sdeshpande@sharplabs.com yamamoto.tomoyuki@sharp.co.jp | X | X |
| Technicolor | P.Bordes P.Andrivon | philippe.bordes@technicolor.com pierre.andrivon@technicolor.com | X | X |
| Sony | C.Auyeung K.Sato | Cheung.Auyeung@am.sony.com Kazushi.Sato@jp.sony.com | X | X |
| Samsung | E.Alshina A.Saxena | elena_a.alshina@samsung.com asaxena@sta.samsung.com | X | X |
| Nokia | K.Ugur | kemal.ugur@nokia.com | X | X |
| NGcodec | A.Duenas | alberto@ngcodec.com |  | X |
| Qualcomm | X. Li J. Chen | lxiang@qti.qualcomm.com cjianle@qti.qualcomm.com |  | X |
| InterDigital | Y. He J. Dong Y. Ye | Yuwen.he@interdigital.com Jie.dong@interdigital.com Yan.ye@interdigital.com |  | X |
| ETRI | J.Lee J. Won H. Lee | jinosoul@etri.re.kr jungwon@etri.re.kr hanilee@etri.re.kr |  | X |
| MediaTek | Shan Liu Yuwen Huang Polin Lai Peter Chuang | shan.liu@mediatek.com yuwen.huang@mediatek.com polin.lai@mediatek.com peter.chuang@mediatek.com |  | X |

# Content creation for the experiments

The test material will be created from the content provided in JCTVC-N0163 [2] and provided by the Ad hoc Group on Color Gamut Scalability.  The Color graded sequences in DCI-P3 will be expressed in BT.2020 gamut, 3840x2160p. The BT.709 sequences will be provided in 1920x1080p (same as in JCTVC-N0163).

The sequences duration will be reduced to 300 frames for 60Hz sequences and 250 frames for 50Hz sequences, representing 5s duration (500 frames originally). However, for the BirthdayFlash sequence, two sequences of 300 frames may be generated.

The BT.2020, 3840x2160p sequences will be 10-bit, the BT.709, 1920x1080p will be provided in 8-bit and 10-bit.

# Reference software

The reference software for the CE will be provided by the Ad hoc group on Color Gamut Scalability.

# Description of Experiments

The test conditions will be 2x scalability with the following contents:

- Test 1: Enhancement layer: 3840x2160 resolution, 10-bit, BT.2020 gamut / Baselayer layer: 1920x1080p, BT.709, 8-bit.

- Test 2: Enhancement layer: 3840x2160 resolution, 10-bit, BT.2020 gamut / Baselayer layer: 1920x1080p, BT.709, 10-bit.

The common SHVC test conditions (QPs) will be used for AI and RA configurations, 2x scalability.

## JCTVC-N0146 (Nokia) and JCTVC-N0218 (Samsung)

The SHM3.0 re-sampling process assumes that input and out-put have the same bit-depth. For example, N bits input of interpolation process goes through horizontal (first) and vertical (second) interpolation, and at the final stage results of interpolation are de-scaled and clipped to N-bit value.

If bit-depth is different for current (nEnhY for Luma and nEnhC for Chroma) and reference layer (nBaseY for Luma and nBaseC for Chroma) then bit-depth increment is needed after re-sampling process (equations G-29’ and G-38’). This design is an anchor for proposed test.

Figure 2: Simplest bit-depth scalability (anchor) and proposed test.

In the proposed test, bit-depth scaling process is merged with re-sampling. It results in modification of de-scaling shift after second step of interpolation only. The goal of proposed modification is to avoid extra interlayer processing step and to minimize accuracy loss.

Specification text changes associated with simplest bit-depth scalability and proposed test are highlighted in yellow in Annex Error: Reference source not found.

## JCTVC-M0214/L0334 (Sharp)

The color prediction process is described by a gain and offset model.  The model operates on individual color planes.  To facilitate integer calculation, a parameter describes the number of fraction bits used in the calculation in the parameter numFractionBits.  For each channel a gain[c] and offset[c] are specified.  The prediction is defined by

Pred[c][x][y] =  (gain[c]*In[x][y] + (1<<(numFractionBits-1))>> numFractionBits + offset[c]

The bit-depth scaling process is the same as proposed in M0214.

## JCTVC-N0168 (Technicolor)

The model is derived from JCTVC-N0168 and is based on the use of a 3D Look-Up Table (LUT) to predict the tri-chromatic color space 2 (EL) samples from the reconstructed BL (color space 1).

## JCTVC-N0271 (Sony)

In this part of the SCE, the two color difference prediction models proposed in JCTVC-N0271 are to be tested with respect to the anchor. In particular, each color difference is predicted independently from other color difference. The color difference prediction models are the followings:

- Piecewise-linear prediction model

- Shift-offset model

The model  parameters are optimized either on-line or off-line.

### Piecewise-linear prediction model (model 1)

Let d[c][x][y] = Pix709[c][x][y] – knot[c]. If d[c][x][y] <= 0

Pix2020[c][x][y] = gain1[c]*d[c][x][y] + offset[c] + (1<<(numFractionBits-1))) >> numFractionBits

else

Pix2020 [c][x][y] = (gain2[c]*d[c][x][y] + offset[c] + (1<<(numFractionBits-1))) >> numFractionBits

The prediction parameters gain1[c], gain2[c], knot[c], and offset[c] are optimized either on-line or off-line for each color component c.

### Shift-offset model (model 2)

Pix2020[c][x][y]  = (Pix709[c][x][y]  << 2) + O[c]

where O[c] are optimized either on-line or off-line for each color component c.

# Cross-checks

- Test 1: Enhancement layer: 3840x2160 resolution, 10-bit, BT.2020 gamut / Baselayer layer: 1920x1080p, BT.709, 8-bit.

- Test 2: Enhancement layer: 3840x2160 resolution, 10-bit, BT.2020 gamut / Baselayer layer: 1920x1080p, BT.709, 10-bit.

| Test num. | Description | Code provided by... | Cross-Checker |
| --- | --- | --- | --- |
| 1 | 5.1-test1 | Samsung | Sony |
| 2 | 5.2-test1 | Sharp | Qualcomm |
| 3 | 5.2-test2 | Sharp | Qualcomm |
| 4 | 5.3-test1 | Technicolor | ETRI |
| 5 | 5.3-test2 | Technicolor | Sharp |
| 6 | 5.4-test1-model1 | Sony | Qualcomm |
| 7 | 5.4-test1-model2 | Sony | Samsung |
| 8 | 5.4-test2-model1 | Sony | Qualcomm |

# Time-line and Responsibilities

2013-August-30:		Test material finalized and up-loaded.

2013- August-30:	Reference software distributed to participants.

2013-Oct-5:		Cross-verification begins: proponents provide software, draft of contribution text and results to cross-verifiers(s) and CE coordinators.

2013-Oct-14: 		Cross-verifiers report results to CE participants.

# References

- A.Segall, “BoG report on color gamut scalability,” JCTVC-N0387, July 2013, Vienna, AT.

- P.Andrivon, P.Bordes, “AHG14: Wide Color Gamut Test Material Creation”, JCTVC-N0163, July 2013, Vienna, AT.

- E. Alshina, A. Alshin, “AhG14: On bit-depth scalability support”, JCTVC-N0218, July 2013, Vienna, AT.

- K. Ugur, A. Aminlou, “AHG14: On resampling & color gamut scalability”, JCTVC-N0146, July 2013, Vienna, AT.

- S.Deshpande, L.Kerofsky, A.Segall, “On Color Gamut Scalable Video Coding,” JCTVC-M0214, April 2013, Incheon, KR.

- P.Bordes, P.Andrivon, F.Hiron, “AHG14: Color Gamut Scalable Video Coding using 3D LUT: New Results”, JCTVC-N0168, July 2013, Vienna, AT.

- C. Auyeung, K.Sato, “Color gamut scalable video coding with piecewise linear predictions and shift-offset model,” JCTVC-N0271, July 2013, Vienna, AT.