| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N1121 |
| --- | --- |

| Title: | HEVC Range Extensions Core Experiment 1 (RCE1): Inter-component Decorrelation Methods |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | CE Description |  |  |
| Author(s) or Contact(s): | Woo-Shik Kim Tung Nguyen | Email: | wooshikk@qti.qualcomm.com tung.nguyen@hhi.fraunhofer.de |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document describes the HEVC Range Extensions (RExt) Core Experiment 1 (RCE1) on inter-component decorrelation methods. The list of participants, contributions, the description on the experiments and the test conditions are given in the subsequent subsections.

# Introduction

The correlation among different color components can be reduced by an adaptive inter-color-component residual prediction approach [1] presented at the 14th JCT-VC meeting resulting in increased compression performance. This CE studies the performance and the complexity associated with the proposal.

JCTVC-N0266 [1] describes an adaptive inter-color-component residual prediction scheme. The proposed technique operates as a prediction applied to chroma residual samples using luma residual samples, which involves addition and bit-shift operations. Experimental results are asserted to show coding efficiency improvements both in RGB and YCbCr 4:4:4 sequences.

# Participants

| Participant | Email address | Affiliation |
| --- | --- | --- |
| W.-S. Kim J. Sole W. Pu | wooshikk@qti.qualcomm.com joels@qti.qualcomm.com wpu@qti.qualcomm.com | Qualcomm |
| T. Nguyen | tung.nguyen@hhi.fraunhofer.de | Fraunhofer HHI |
| N. Saunders K. Sharman | nick.saunders@eu.sony.com karl.sharman@eu.sony.com | Sony |
| Y.-W. Huang T.-D. Chuang C.-W. Hsu S.-T. Hsiang | yuwen.huang@mediatek.com peter.chuang@mediatek.com cw.hsu@mediatek.com shih-ta.hsiang@mediatek.com | MediaTek |
| A. Saxena F. Fernandes H. Chen | asaxena@sta.samsung.com ffernandes@sta.samsung.com haoming.c@sta.samsung.com | Samsung |
| A. Minezawa | Minezawa.Akira@ds.MitsubishiElectric.co.jp | Mitsubishi |
| K. Kawamura | ki-kawamura@kddi.com | KDDI |

# Experiment Description

Below experiments will be performed to evaluate the performance of the inter-color-component residual prediction scheme.

- The performance of JCTVC-N0266 using HM RExt 4.1.
			Proponent:	Qualcomm
			Cross-checker:	Mitsubishi
In this experiment the technique proposed in JCTVC-N0266 will be implemented on top of HM RExt 4.0 reference software and its performance will be evaluated.

- Fast encoding technique applied to JCTVC-N0266.
			Proponent:	Qualcomm
			Cross-checker:	KDDI

In this experiment the fast encoding technique will be tested. For example, the prediction can be performed using the original luma residual samples at the encoder side to facilitate pipelining and parallel processing. The decoding process is the same as Experiment 1.

- Modified signalling scheme of the prediction parameters.
			Proponent:	Qualcomm
			Cross-checker:	HHI

This experiment is to examine the prediction parameter signalling scheme in JCTVC-N0266. Signalling at different data-structure level will be investigated and prediction of parameters will be tested.

- Combination of Experiments 2 and 3.
			Proponent:	Qualcomm
			Cross-checker:	HHI

# Test Conditions

The planned experiments employ the test set and the test condition as described in the RExt common test conditions [2] for RGB and YCbCr 4:4:4 sequences. Test for YCbCr 4:2:2 can be performed optionally. Also the test using the screen content sequences recommended by the AhG8 can be performed optionally.

According to the RExt common test conditions, in case of RGB sequence, the PSNR will be calculated compared to the original RGB sequence, and in case of YCbCr 4:4:4 sequence, the PSNR will be calculated compared to the original YCbCr 4:4:4 sequence. The overall bitrate will be used to measure BD-rate of each color component.

Additional PSNR calculation can be optionally performed as described in the previous RCE1 description [3]. Also chroma QP offset can be optionally adjusted to measure Y BD-rate when the BD-rates of Cb and Cr are zeros.

Additional comparison can be made optionally to evaluate RGB coding performance versus YCbCr coding performance. In this case the decoded YCbCr is converted to RGB, and PSNR is calculated compared to the original RGB sequence.

## Software

The HEVC RExt development reference software in version 4.1 will be used as the anchor for the integration of the tested methods as well as for generation of the anchor results.

## Complexity Assessment

For the complexity measurement, the anchor and the tested method will be executed under similar conditions and the execution time will be measured. A time ratio will then be calculated between the anchor and the tested method.

However, the complexity analysis does not solely rely on the encoding and decoding time measurements of a particular implementation (reference software) of the working draft (e.g., should describe need for LUT, additional logic, memory access, memory storage, etc.).

Core experiment participants who investigate complexity aspects are requested to provide a description showing how the complexity is changed and which architecture(s) would be affected by the proposed scheme. Average and worst case impact from the methods should be considered.

# Suggested Timeline

- 23-08-2013:	Publication of the final RCE1 description.

- 16-09-2013:	Release of HM RExt 4.1.

- 23-09-2013:	Release of core implementation (Experiments 1 and 2). Results and WD text can be provided one week later.

- 26-09-2013:	Release of additional implementation (Experiments 3 and 4).

- 07-10-2013:	Completion of cross-check.

- 14-10-2013:	Input document and summary report for the 15th JCT-VC meeting finalized and 				published.

# References

- W. Pu, W.-S. Kim, J. Chen, K. Rapaka, L. Guo, J. Sole, and M. Karczewicz, “Non RCE1: Inter color component residual prediction,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-N0266, Aug. 2013.

- D. Flynn, and K. Sharman, “Common test conditions and software reference configurations for HEVC range extensions,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-N1006, Aug. 2013.

- T. Nguyen, J. Sole, and J. Kim, “HEVC Range Extensions Core Experiment 1 (RCE1): Inter-component decorrelation methods,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-M1121, Apr. 2013.