| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N1122 |
| --- | --- |

| Title: | HEVC Range Extensions Core Experiment 2 (RCE2): Rice parameter initialisation and update methods |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | CE description |  |  |
| Author(s) or Contact(s): | Chris Rosewarne Joel Sole Karl Sharman S.-H. Kim | Email: | chris.rosewarne@cisra.canon.com.au joel.sole@qti.qualcomm.com karl.sharman@eu.sony.com kimse@sharplabs.com |
| Source: | CE co-ordinators |  |  |

_____________________________

# Abstract

This document is a description of the HEVC Range Extensions Core Experiment 2 on Rice parameter initialisation and update methods.

# Introduction

This document defines HEVC Range Extensions Core Experiment 2 (RCE2) on Rice parameter initialisation and update methods, to be performed for the upcoming October 2013 JCT-VC meeting.

# Participants

| Participant | Email address | Affiliation | P | C |
| --- | --- | --- | --- | --- |
| R. Joshi J. Sole L. Guo M. Karczewicz | rajanj@qti.qualcomm.com joels@qti.qualcomm.com liweig@qti.qualcomm.com martak@qti.qualcomm.com | Qualcomm | X | X |
| K. Sharman N. Saunders | karl.sharman@eu.sony.com nick.saunders@eu.sony.com | Sony Europe | X |  |
| C. Auyeung | cheung.auyeung@am.sony.com | Sony Electronics |  |  |
| C. Rosewarne | chris.rosewarne@cisra.canon.com.au | Canon | X | X |
| S. H. Kim A. Segall | kimse@sharplabs.com asegall@sharplabs.com | Sharp Labs | X | X |
| A. Saxena H. Chen F. Fernandes | asaxena@sta.samsung.com haoming.c@sta.samsung.com ffernandes@sta.samsung.com | Samsung Telecom America |  |  |
| S. Lee | sunil.lee@samsung.com | Samsung |  | X |
| A. Minezawa | minezawa.akira@ds.mitsubishielectric.co.jp | Mitsubishi |  |  |
| Y.-W. Huang T.-D. Chuang C.-W. Hsu S.-T. Hsiang | yuwen.huang@mediatek.com peter.chuang@mediatek.com cw.hsu@mediatek.com shih-ta.hsiang@mediatek.com | Mediatek |  |  |
| H. Yu Z. Ma J. Ye | haoping.yu@huawei.com zhan.ma@huawei.com jing.ye@huawei.com | Huawei |  | X |

# Tools to be tested

Proponents are requested to document any modifications over their original proposals in order support operation at higher bit depths. Significant changes would require non-CE contributions.

## Rice parameter update methods

Test A.1 (N0101, Qualcomm)

One condition check in the HEVC cRiceParam update process is removed. In the preferred variant, the cRiceParam update is performed as follows:

- cRiceParam = Min( cLastRiceParam + ( cLastAbsLevel >> ( 2  +  cLastRiceParam ) ), 4 )

The method is applied to all the blocks. The method is also tested with maximum value of cRiceParam equal to 5. Additional results are also provided for applying the method to only transform-skip and transform-bypass blocks.

Test A.2 (N0281, Sharp)

This contribution presents two modifications to cRiceParam update process of coeff_abs_level_remaining based on rice-parameter signaling method in JCTVC-N0268. The two modifications are: (1) signaling cRiceParam from the set {0, 2, 4, 6} and (2) removing the existing cRiceParam update process. This method is applied only to transform-skip and transform bypass blocks.

## Rice parameter initialisation methods

Test B.1 (N0268, Sharp)

The Rice parameter (cRiceParam) is explicitly signalled for each coefficient sub-block for which at least one coeff_abs_level_remaining syntax element is coded. In the preferred variant, cRiceParam is signalled from the subset {0, 2, 4, 6} using truncated unary (TU) binarization method. The existing cRiceParam update is unchanged. This method is applied only to transform-skip and transform bypass blocks. Additional results are also provided for applying the method to all blocks including transform blocks.

Test B.2 (N0137, Canon)

The update formula for the Rice parameter is unmodified, which is applied once per instance of coeff_abs_level_remaining (up to 16 instances per sub-block).  This contribution proposes to modify the initialisation of the Rice parameter, which is applied once per sub-block parameter for transform skip and transquant bypass blocks. An initial value of 2 is used when the number of greater-than-one coefficients in the current sub-set is equal to or greater than 8.

## Rice parameter adaptive methods

Test C.1 (N0181, Qualcomm)

Modified Rice parameter initialization to be a value from 0 to N, depending upon statistics from previously encoded/decoded blocks. The statistics are updated based upon the coded coefficients of the 4x4 sub-block. The updated statistics are used to derive the initialization value for the next sub-block (i.e. the statistics don’t affect the current sub-block).

Test C.2 (N0189, Sony)

An offset to cRiceParam equal to BF is applied. An algorithm is proposed to allow the value of BF to adaptively change over the course of a slice; conceptually similar to the context variables that are used to adapt CABAC.

## Combinations

Test D.1 Combination of C.1 (N0181) and C.2 (N0189).

Test D.2 Combination of A.1 (N0101) and C.1 (N0181).

Test D.3 Combination of A.1 (N0101) and C.2 (N0189).

Test D.4 Combination of A.1 (N0101) with C.1 (N0181) and C.2 (N0189).

Test D.5 Combination of B.2 (N0137) and C.1 (N0181).

Test D.6 Combination of B.2 (N0137) and C.2 (N0189).

Test D.7 Combination of A.1 (N0101) and B.1(N0268).

# Cross-checks

Table 1 specifies the cross-checker(s) for each subtest.

| Subtest | Cross-checker(s) |
| --- | --- |
| A.1 | Canon |
| A.2 | Canon |
| B.1 | Qualcomm |
| B.2 | Sharp |
| C.1 | Samsung |
| C.2 | Qualcomm |
| D.1 | Canon |
| D.2 | Samsung |
| D.3 | Canon |
| D.4 | Huawei |
| D.5 | Huawei |
| D.6 | Sharp |
| D.7 | Canon |

Table 1: Cross-checks

# Test conditions

## Test condition definition

### AHG8 testing (lossy and lossless)

Lossy and lossless test conditions as defined in AHG8 (see JCTVC-N1123), with the following configurations tested:

- Intra

- Randomaccess

- Lowdelay

### AHG18 testing (lossy high bit-rate)

High bit-rate test conditions are defined as follows:

- ‘GBR’ ordering using the 1080p variants of the ‘SVT Multiformat’ sequences (16-bit precision, 50fps) and Traffic_12bit.

- For each sequence, internal bit-depths to be tested are: 12, 14 and 16-bits.

- Additional 16-bit test sequences may be added to the test conditions of this RCE.

- QPs according to internal bit-depth as follows:

  - 16-bit: −28, −31, −34, −37

  - 14-bit: −16, −19, −22, −25

  - 12-bit: −4 , −7,  −10, −13

- MAX_TRANSFORM_DYNAMIC_RANGE equal to max(15,bitDepth+6)

- ENTROPY_CODING_DYNAMIC_RANGE equal to max(15,bitDepth+6)

- As per AHG5/8 conditions, testing with transform skip enabled for 4x4 blocks only

- Testing with ‘extended precision processing’ enabled:

  - Two cases are defined:

    - Mandatory: Testing with the HM forward transform.

    - Optional: Testing with a ‘high precision’ forward transform (testing with this more precise inversion of the HEVC inverse transform is strongly encouraged).

  - Software settings:

    - Both cases:

      - #define RExt__HIGH_BIT_DEPTH_SUPPORT  1

      - Apply '--ExtendedPrecision=1'

    - Mandatory case:

      - #define RExt__HIGH_PRECISION_FORWARD_TRANSFORM  0

    - Optional case:

      - #define RExt__HIGH_PRECISION_FORWARD_TRANSFORM  1

- RDOQ/RDOQTS enabled

- Results will show BD-rate changes over the 4 QPs and the compression ratio

Other settings as per the ‘encoder_intra_main_rext.cfg’ configuration file (i.e. ‘all-intra’ only).

Use frame count equivalent to 1 second equivalent of each sequence (for SVT sequences, this is 50).

Following are the five SVT sequences being used:

- CrowdRun

- ParkJoy

- DucksTakeOff

- IntoTree

- OldTownCross

## Subtest test conditions

Table 2 specifies the test conditions are used for each subtest.

| Subtest | AHG8 (lossy) | AHG8 (lossless) | AHG18 lossy |
| --- | --- | --- | --- |
| A.1, A.2 | Yes | Yes | Yes |
| B.1, B.2 | Yes | Yes | Yes |
| C.1, C.2 | Yes | Yes | Yes |
| D.1, D.2, D.3, D.4, D.5, D.6, D.7 | Yes | Yes | Yes |

Table 2: Coding conditions to be tested for each subtest

# Evaluation of CE Results

Results of the CE will be evaluated on the basis of bit-rate and complexity. Complexity assessment is performed on the base of hardware and encoding/decoding complexity.

## Complexity assessment

To measure software run times, range extension software with the provided configurations and the proposals implemented on range extension software shall be used. The computational time must be measured for each test sequence and test case for both anchor and proposals. Relative computational time calculated against the anchor must be presented.

Additional evaluation of the HW and SW complexity of the proposed tools for both encoder and decoder is encouraged.

# Timeline

- August 23rd 2013: CE description finalized and uploaded.

- August 23rd 2013: Test sequences and test conditions are finalized with AHG8.

- August 23rd 2013 (tentative): Range extension reference software available.

- Range extension software availability date + 2 weeks: Cross-check begins: Proponents for tests A, B and C provide software, draft specification text and results to CE participants.

- Range extension software availability date + 4 weeks: Proponents for test D provide software, draft specification text and results to CE participants.

- October 14th 2013: Input documents and summary report uploaded.