| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, Austria, 25 Jul. – 2 Aug. 2013 | Document: JCTVC-N1123 |  | Document: JCTVC-Naaaa |
| --- | --- | --- | --- |

| Title: | HEVC Range Extensions Core Experiment 3 (RCE3): Intra Prediction techniques |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | CE description |  |  |
| Author(s) or Contact(s): | Ankur Saxena Do-Kyoung Kwon Matteo Naccari Chao Pang | Email: | asaxena@sta.samsung.com d-kwon@ti.com matteo.naccari@bbc.co.uk cpang@qti.qualcomm.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document is a description of the HEVC Range Extensions Core Experiment 3 on intra prediction techniques for lossless and lossy coding.

# Introduction

This document defines HEVC Range Extensions Core Experiment 3 (RCE3) on prediction techniques for lossless and lossy coding to be performed for the upcoming October 2013 JCTVC meeting.

# Participants

| Participant | Person | Email address |
| --- | --- | --- |
| Samsung | A. Saxena S. Lee H. Chen F. Fernandes E. Alshina | asaxena@sta.samsung.com sunil.lee@samsung.com haoming.c@sta.samsung.com ffernandes@sta.samsung.com elena_a.alshina@samsung.com |
| TI | M. Budagavi Do-Kyoung Kwon | madhukar@ti.com d-kwon@ti.com |
| BBC | M. Naccari M. Mrak R. Weerakkody | matteo.Naccari@bbc.co.uk marta.mrak@bbc.co.uk rajitha.weerakkody@bbc.co.uk |
| Fujitsu | J.Zhu W. Zheng K.Kazui | zhujianqing@cn.fujitsu.com cheng@cn.fujitsu.com kazui.kimihiko@jp.fujitsu.com |
| Qualcomm | C. Pang R. Joshi | cpang@qti.qualcomm.com rajanj@qti.qualcomm.com |
| I2R | C. Yeo Y. H. Tan | chyeo@i2r.a-star.edu.sg yhtan@i2r.a-star.edu.sg |
| Nokia | J. Lainema | Jani.Lainema@nokia.com |
| Huawei | H. Yang Z. Ma X. Wang H. Yu | haitao.yang@huawei.com Zhan.Ma@huawei.com Xian.w@huawei.com haoping.yu@huawei.com |
| Mitsuibishi Electric | A. Minezawa | Minezawa.Akira@ds.MitsubishiElectric.co.jp |
| Mediatek | Y. W. Huang P. Chuang S-T. Hsiang C. Hsu P. Lai S. Liu | yuwen.huang@mediatek.com Peter.Chuang@mediatek.com Shih-ta.Hsiang@mediatek.com cw.hsu@mediatek.com Polin.Lai@mediatek.com Shan.Liu@mediatek.com |
| Microsoft | J. Xu | jzxu@microsoft.com |
| Sony | K. Sato O. Nakagami J. Xu | Kazushi.Sato@jp.sony.com ohji.nakagami@jp.sony.com Jun.Xu@am.sony.com |

# Tools to test

## Prediction techniques

Test A.1 (N0079, Fujitsu, tested for both lossless and lossy)

N0079 proposed a modified version of the sample based intra prediction for lossless coding introduced in JCTVC-M1005. Only 13 out of 35 possible intra prediction modes are used. DC and planar modes are unchanged. 11 angular modes use the nearest reference sample’s value in their direction as prediction value rather than using a filter, shown as Figure 1.

Figure 1: Prediction for angular modes

In this subtest, besides original proposal scheme, some variants will be tested. Lossy coding in intra blocks where transform skip is selected will also be extended and tested. In lossy coding case, angular mode uses the nearest reference sample’s reconstructed value as prediction value.

Test A.1.1 N0079 original. 13 modes of intra prediction for lossless coding

Test A.1.2 N0079_variant 1: Keep all modes of intra prediction in HEVC V1. The simplified SAP is applied for 9 angular modes (mode 2, 6, 10, 14, 18, 22, 26, 30, 34) for lossless coding.

Test A.1.3 N0079_variant 2: Keep all modes of intra prediction in HEVC V1. The simplified SAP is applied for 5 angular modes (mode 2, 10, 18, 26, 34) for lossless coding.

Test A.1.4 N0079 variant 3: Keep all modes of intra prediction in HEVC V1. The simplified SAP is applied for 9 angular modes (mode 2, 6, 10, 14, 18, 22, 26, 30, 34) for lossy coding.

Test A.1.5 N0079_variant 4: Keep all modes of intra prediction in HEVC V1. The simplified SAP is applied for 5 angular modes (mode 2, 10, 18, 26, 34) for lossy coding.

Test A.2 (N0176, N0177, Samsung, tested for both lossless and lossy)

In this subtest, SAP is applied for diagonal modes (modes 2, 18 and 34) for both lossless and lossy settings when transform skip is selected. In addition, supplementary results will also be provided for applying SAP additionally for 4 other pseudo-diagonal modes (modes 6, 14, 22 and 30). Various tests will be performed to find the coding efficiency of SAP for different block sizes from 4x4 to 64x64.

Test A.3 (N0183, Samsung, tested for both lossless and lossy)

In this subtest, nearest neighbor prediction would be used instead of bilinear interpolation for intra prediction. Different selection criteria such as a threshold based pixel difference criteria, or variance or SAD based, or Rate-Distortion would be used to select between the nearest neighbor prediction, or retaining bilinear interpolation. In addition, performance by turning filtering off would also be evaluated in this subtest. Various tests will be performed to find the coding efficiency for different block sizes from 4x4 to 64x64.

3.2 Combination Tests

Test B.1 (N0176/N0177 + N0183): In this combination test, methods in tests A.2 and A.3 described above would be tested jointly for both lossless and lossy settings.

# Cross-check

| Subtest Name | Cross-checker (s) |
| --- | --- |
| A.1 | Samsung, Huawei |
| A.2 | Texas Instruments, Fujitsu |
| A.3 | BBC |
| B.1 | Huawei, Fujitsu |

# Test conditions

## Software

HM range extension software (HM12.0-RExt4.1) released after Vienna meeting will be used for both lossless and lossy coding. For lossless coding, parameter setting ‘--CostMode=lossless ‘should be added in command line.

## Coding Conditions

Table 1 specifies whether the lossy and/or lossless coding conditions are used for each subtest.

| Subtest | Coding conditions to be tested |
| --- | --- |
| A.1, A.2, A.3 | Lossy /Lossless |
| B.1 | Lossy/ Lossless |

Table 1: Coding conditions to be tested for each subtest

In both Lossy and Lossless tests, HM backward compatibility for Ticket #1026 should be disabled (set to 0) in file TypeDef.h

#define RExt__BACKWARDS_COMPATIBILITY_HM_TICKET_1026                           0 ///< Maintain backwards compatibility with HM for ticket 1026 (xGetICRate is deprecated)

Also, for only the lossless settings, intra reference smoothing should be disabled. Specifically, in the config files, the following macro should be set to 0 for lossless tests.

IntraReferenceSmoothing             : 0                # 0: Disable use of intra reference smoothing. 1: Enable use of intra reference smoothing (not valid in V1 profiles)

Internal bitdepth should be set equal to input bitdepth for all test sequences.

### Lossy coding conditions

The recommended configurations and test cases described in range extension common test conditions (JCTVC-N1006) will be used. Results for all the mandatory test cases shall be provided.

- The internal bit-depth should be set equal to the input bit-depth for each sequence.

- The configuration files AI-main_rext, LB-main_rext and RA-main_rext should be used for generating the results.

### Lossless coding conditions

The following configurations are required to achieve lossless coding and also allow comparison between anchor and target tests.

- QP=0

- TransquantBypassEnableFlag=1

- CUTransquantBypassFlagForce=1

For the RGB sequences, the GBR channel order will be used in the encoding process. Thus the following configuration shall be used when encoding the RGB sequences:

- --InputColourSpaceConvert=RGBtoGBR

- --SNRInternalColourSpace (for RGB4:4:4 only) should be set to 1

- --OutputInternalColourSpace=0 (optional)

Note that the setting for SNRInternalColourSpace and OutputInternalColourSpace are optional since they will not affect the coding performance.

The following test cases are mandatory:

- AI-main_rext

- LB-main_rext

- RA-main_rext

### Test sequences

The following test sequences will be used for both lossless and lossy coding conditions. The sequences are based on recommendation from AHG8.

| Class | Input BitDepth | Chroma Format | Sequence Name | # frames | Md5sum |
| --- | --- | --- | --- | --- | --- |
| Screen Content (Class F) | 8-bit | YCbCr 4:2:0 | BasketballDrillText | 500 | 480881343081e3f4adf85075a1ed0f08 |
|  | 8-bit | YCbCr 4:2:0 | ChinaSpeed | 500 | 9dfcbb87d095987891cf90cc593c5ba9 |
|  | 8-bit | YCbCr 4:2:0 | SlideEditing | 300 | dbf1d0e765ba9f775c5208adb4dc6c0c |
|  | 8-bit | YCbCr 4:2:0 | SlideShow | 500 | d6b28f127b9ddf088eb9db9fc5e611bd |
| Class B | 8-bit | YCbCr 4:2:0 | Kimono | 240 | 4a83005bc719012ac148dd3898e5e4ed |
|  | 8-bit | YCbCr 4:2:0 | ParkScene | 240 | b7ada0912d693304165254177d08343d |
| Screen Content (RGB) | 8-bit | RGB 4:4:4 | sc_map | 600 | d83e96251f40e03bc3d27823ab54cba0 |
|  | 8-bit | RGB 4:4:4 | sc_web_browsing | 300 | aa992e5577c7d70d68c6a0f16697d213 |
|  | 8-bit | RGB 4:4:4 | sc_wordEditing | 600 | e4bdfef57a01392e5c04c783989f0cea |
|  | 8-bit | RGB 4:4:4 | SlideShow | 500 | 90bfb5b90419ff16d0490ef919eee3e1 |
|  | 8-bit | RGB 4:4:4 | sc_programming | 600 | dde4b9da34f4b698a9ce984caa4c5034 |
|  | 8-bit | RGB 4:4:4 | ppt_doc_xls | 200 | d638fcc2ab30c95c5c223675a33ad189 |
|  | 8-bit | RGB 4:4:4 | video_conferencing_doc_sharing | 300 | acc5828e173d6d445e5b208b4cb2daae |
| Animation (RGB) | 8-bit | RGB 4:4:4 | sc_robot | 300 | 24455b665c4b8b951170ed8e9972d191 |
|  | 8-bit | RGB 4:4:4 | sc_viking | 300 | 3f4ee8b1da7f8c378a321139cb5b7456 |
|  | 8-bit | RGB 4:4:4 | VenueVue | 150 | 8f98fbad7261bcd78f86d8ba8fd4a19b |
| Screen Content (YCbCr) 444 | 8-bit | YCbCr 4:4:4 | sc_ppt_doc_xls_r1 | 200 | e3bcd5597228302fc0fe528f642338c1 |
|  | 8-bit | YCbCr 4:4:4 | sc_video_conferencing_doc_sharing_r1 | 300 | 49dbe829a58bca7300d7a6bd7c347083 |
|  | 8-bit | YCbCr 4:4:4 | sc_wordEditing | 600 | 63436dc5c1adca429129daf17c6a58e5 |
|  | 8-bit | YCbCr 4:4:4 | SlideShow | 500 | f1a721e59ae9b662d6aefffa6fb64f8d |
|  | 8-bit | YCbCr 4:4:4 | sc_map | 600 | 1b844c4ab08a2885aeafa7a473ec3e16 |
|  | 8-bit | YCbCr 4:4:4 | web_browsing_r1 | 300 | a6926facd648dc46a8c3dd7ae5ddccf8 |
|  | 8-bit | YCbCr 4:4:4 | sc_programming | 600 | 8ffc06bf2873388fbc20e75d4852a21c |
| Animation (YUV) | 10-bit | YCbCr 4:4:4 | VenueVue | 150 | 6f5d8ac82ff84043603025edd3e9a8eb |
|  | 8-bit | YCbCr 4:4:4 | sc_robot | 300 | 15cb0ddd57c9d67bae34e16ce84f5f72 |
|  | 8-bit | YCbCr 4:4:4 | sc_viking | 300 | cca7d59accafc336116553565bc06df2 |
| RExt | 10-bit | YCbCr 4:2:2 | EBUHorse | 250 | 3bef26469ee0d1a3ae5fc972497f17b9 |
|  | 10-bit | YCbCr 4:2:2 | EBUWaterRocks | 250 | b493aa9a920cf8e1490280279432285c |
|  | 10-bit | YCbCr 4:4:4 | EBURainFruits | 250 | 14e011ee5ceb1c8d7a046e2cc0401774 |
|  | 10-bit | YCbCr 4:4:4 | Kimono | 120 | afd06b6a7823d854b89277ccee1e2aa8 |
| Class O (Optional) | 8-bit | RGB 4:4:4 | sc_cad_waveform | 200 | 419d13950c685bb9df2139ff2b37fb74 |
|  | 8-bit | RGB 4:4:4 | sc_twist_tunnel | 300 | d3eb1fcd8995d808bdce33ee3131c8c6 |
|  | 8-bit | RGB 4:4:4 | sc_pcb_layout | 200 | f513b714d3d59432d5c5f532f6b73a0c |
|  | 8-bit | YCbCr 4:4:4 | sc_cad_waveform_r1 | 200 | 6b57b8e56b89dbcf6e9cf1ae16fae3d6 |
|  | 8-bit | YCbCr 4:4:4 | sc_twist_tunnel_r1 | 300 | 885602f7bca5d9293aa4f11d1a0121a3 |
|  | 8-bit | YCbCr 4:4:4 | sc_pcb_layout_r1 | 200 | 12720e1d2c5b571b8b9b7ca30e5b247b |

# Evaluation of CE Results

Results of the CE will be evaluated on the basis of bit rate and complexity. Complexity assessment is performed on the base of hardware and encoding/decoding complexity.

### Complexity assessment

To measure software run times, range extension software with the anchor configuration (the default frame level lossless coding is turned on) and the proposals implemented on range extension software shall be used. The computational time must be measured for each test sequence and test case for both anchor and proposals. Relative computational time calculated against the anchor must be presented.

In addition to the simulation time, the following two metrics shall be provided to measure the decoding implementation complexity.

- Number of operations to decode one sample

- Throughput (e.g., number of residual samples which can be decoded in parallel)

Additional evaluation of the HW and SW complexity of the proposed tools for both encoder and decoder is encouraged.

### Timeline

- Aug 23rd 2013: CE description finalized and uploaded.

- Aug 28th 2013 (Tentative): Range extension reference software available.

- Aug 23rd 2013: Test sequences and test conditions are finalized with AHG8

- Two weeks after (B): Cross-check begins for tests in Category A: Proponents provide software, draft of contribution text and results to CE participants.

- One week after (D): Cross-check begins for tests in Category B: Proponents provide software, draft of contribution text and results to CE participants.

- Sept 30th, 2013: Cross-check complete.

- Oct 14th, 2013: Input documents uploaded.

- Oct 21st, 2013: Summary Report uploaded