| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0004 |
| --- | --- |

| Title: | HEVC conformance test development (AHG4) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki 2-10-1, Osaki, Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2740 teruhikos@jp.sony.com |
|  | Wade Wan 5300 California Ave Irvine, CA 92617 USA | Tel: Email: | wwan@broadcom.com |
| Source: | AHG on HEVC conformance development |  |  |

_____________________________

# Introduction

This AHG was established to develop conformance specification of HEVC.

The mandates of this AHG are:

- Study the requirements of HEVC conformance testing to ensure interoperability.

- Discuss the work plan needed to develop HEVC conformance testing.

- Study potential testing methodology to fulfil the requirements of HEVC conformance testing.

- Establish and coordinate bitstream exchange activities for HEVC.

# Study to develop a potential set of HEVC conformance bitstreams.Activities

## Reflector

The e-mail reflector is [jct-vc@lists.rwth-aachen.de].  To receive email, please subscribe to the e-mail reflector: http://mailman.rwth-aachen.de/mailman/listinfo/jct-vc. For e-mail exchange, it is recommended to put [AHG4] in the subject line for easy grouping.

## ftp site

The ftp site at ITU-T is used to exchange bitstreams. The ftp site for downloading bitstreams is,

http://wftp3.itu.int/av-arch/jctvc-site/bitstream_exchange/

The spreadsheet to summarize the status of bitstream exchange, conformance bitstream generation is available at this directory. It includes the list of bitstreams, codec features and settings, and status of verification.

## Bitstream preparation

The list of the candidate of the conformance bitstream and its volunteers are summarized in Table 2.

So far, 128 bitstreams were collected. The generated bitstreams are available at

http://wftp3.itu.int/av-arch/jctvc-site/bitstream_exchange/draft_conformance/

The features and conformance point of each bitstream are summarized in the attached Excel sheet. Most of them were updated to the HM10 syntax. However, 2 bitstreams are still based on the older version of the spec. Those bitstreams must be updated based on the final spec of the HEVC, as soon as possible. The problems of bitstreams are summarized in Table 1.

| Problems | Bitstreams |
| --- | --- |
| HM10.1 can not decode (CRA/RPS issues, etc) | ENTP_A_LG_2, ENTP_B_LG_2 |
| Profile/level indication | ENTP_C_LG_3 |

Table 1: The list of bitstreams to be revised

|  | Categories | Sub categories | Feature Code | Bitstream features | Volunteers |
| --- | --- | --- | --- | --- | --- |
|  | Intra coding | Intra prediction | IPRED | Functional stage: Decoding of intra PU. Purpose: Check that the decoder can properly decode all values of intraPredMode for different Luma and Chroma transform block sizes. | C.S. Lim, F. Bossen, J. Lainema, K. Sugimoto |
|  |  | Constraint intra prediction | CIP | Functional stage: Decoding of intra PU in non-intra slice when constrained_intra_pred_flag is set to one Purpose: Check that the decoder can properly perform reference sample substitution process. | C.S. Lim, K. Chono |
|  | Inter frame coding | AMVP | AMVP | Check whether motion vector prediction candidate generation and signalling perform correctly. Check the process when neighboring PUs have various partition mode, prediction mode, reference index, inter_pred_idc, etc.. Related syntax element: mvp_l0_flag, mvp_l1_flag | C. Kim, M. Zhou, Y. -W. Huang |
|  |  | MVs | MV | maximum number of motion vectors permitted per CTU |  |
|  |  |  | MV | vectors reference padded picture edges. | M. Coban |
|  |  |  | MV | clipping of motion vector range in MVP/Merge candidates | M. Coban |
|  |  |  | MV | motion vectors are random, maximum page DRAM misses / minimize cache hits (according to memory bandwidth model... e.g. JCTVC-H007) |  |
|  |  | Merge | MERGE | Merge candidate list index signaling: Check that merge candidate list can deal with every merge index (0..4) | B.Bross |
|  |  |  | MERGE | Parallel merge: test all the possible combinations of log2_parallel_merge_level_minus2 and CTB sizes. | M. Zhou, Y. -W. Huang |
|  |  |  | MERGE | number of maximum merge candidates signalling: test all the values of five_minus_max_merge_cand, i.e. 0, 1, 2, 3, 4 for CTB size of 64x64 | M. Zhou, |
|  |  | WP | WP | weighted_pred_flag and/or weighted_bipred_flag are equal to 1. | T. Chujoh |
|  |  |  | WP | various weighted prediction parameter combinations | T. Chujoh |
|  |  | mvd_l1_zero_flag | MVD | Test mvd_l1_zero_flag | F. Bossen |
|  |  | temporal_mvp | TMVP | Test slice_temporal_mvp_enable_flag | J. Xu |
|  |  | Collocated ref idx | RIDX | Test collocated_ref_idx (single slice or multiple slice/picture with different reference lists.) |  |
|  | HL syntax | slices, PSs | PS | bitstreams with stuffed extensions (vps_extension_data_flag=1, sps_extension_data_flag=1, pps_extension_flag=1, ..) | W. Jang, M. Coban |
|  |  | dependent slice | DSLICE | Check: - dependent slices - WPP with dependent slices - tiles with dependent slices | B. Bross |
|  |  | POC | POC | POC increments > 1 | M. Coban |
|  |  | RPS | RPS |  | M. Coban |
|  |  |  | RPS |  | F. Bossen |
|  |  |  | RPS | Slice header RPS with short-term pictures | R. Sjöberg |
|  |  |  | RPS | Slice header RPS with short-term and long-term pictures | R. Sjöberg |
|  |  |  | RPS | PPS RPS with short-term pictures | R. Sjöberg |
|  |  |  | RPS | PPS RPS with short-term pictures and inter-RPS prediction | R. Sjöberg |
|  |  |  | RPS | Mix of PPS and slice header RPS with short-term pictures | R. Sjöberg |
|  |  |  | RPS | Mix of PPS and slice header RPS with short-and long-term pictures | R. Sjöberg |
|  |  | RPL | RPL |  | M.Coban |
|  |  | RAP | RAP | all 6 (currently identified) RAP types | F. Bossen |
|  |  | HRD | HRD | maximum number of pictures in DPB | TBD |
|  |  |  | HRD | bitstream with largest possible CPB delay (maximum number of coded bits per picture but within MinCR) | TBD |
|  |  |  | HRD | bumping DPB | TBD |
|  |  |  | HRD | Low delay, Sub picture | K. Kazui |
|  | Transform | RQT | RQT | Check all depths of inter & intra transform tree: QuadtreeTUMaxDepthInter : 0..4 QuadtreeTUMaxDepthIntra : 0..4 | B. Bross |
|  | Quantization | dQP | DQP | all depths of CU and diff_cu_qp_delta_depth | W. Wan, T. Suzuki |
|  |  | Scaling list | SLIST | non-default scaling lists | T. Suzuki, M. Zhou |
|  | Other coding tools | AMP | AMP | Check all asymetric partition mode enable | X. Zheng, C. Kim |
|  |  | Transform skip | TSKIP |  | J. Xu |
|  |  | I_PCM | IPCM | Functional stage: Parsing of I_PCM size information at SPS and I_PCM flag at CU. Purpose: Check that the decoder can properly decode pcm_flag for different intra CU sizes. Related Syntax: log2_min_pcm_luma_coding_block_size_minus3, log2_diff_max_min_pcm_luma_coding_block_size, pcm_flag. | W. Wan, C.S. Lim, K. Chono |
|  |  |  | IPCM | Functional stage: Parsing and decoding of I_PCM CUs. Purpose: Check that the decoder can properly decode I_PCM CUs with different PCM bit depth. Related syntax: pcm_sample_bit_depth_luma_minus1, pcm_sample_bit_depth_chroma_minus1, pcm_flag | W. Wan, C.S. Lim, K. Chono |
|  |  |  | IPCM | Functional stage: Decoding of I_PCM CUs with loop filter skipping. Purpose: Check that the decoder can properly decode I_PCM CUs by skipping loop filter. Related syntax: pcm_loop_filter_disable_flag, pcm_flag. Note: Combination with cu_transquant_bypass_flag will be tested. | W. Wan, C.S. Lim, K. Chono |
|  |  | Lossless | LOSSLESS | transquant_bypass_enable_flag on, cu_transquant_bypass_flag is set to 0 and 1 randomly at CU level Purpose: Check that the decoder handles lossless coding. | F. Henry |
|  | Parallel processing tools | Tile | TILE | Maximum number of tiles in a random, non-uniform arrangement that changes from picture to picture, with random chosen value for loop_filter_across_tiles_enabled_flag, deblocking_filter_override_flag, slice_disable_deblocking_filter_flag, slice_loop_filter_across_slices_enabled_flag | - Fuldseth, M. Zhou |
|  |  | WPP | WPP | entropy_coding_sync_enabled_flag on. Purpose: Check that the decoder handles wavefront parallel processing | F. Henry |
|  |  | WPP and slices | WPP | entropy_coding_sync_enabled_flag on, number of slice varies randomly between 2 and maximum number of slices permitted, each non-first slice is either regular or dependent in a random way. Purpose: Check that the decoder handles wavefront parallel processing and slices | F. Henry |
|  |  | Tile & WPP | WPP | entry point of WPP | Hendry |
|  | Temporal scalability |  | TSCL |  | W. Jang |
|  |  |  | TSCL | range of nested temporal_id's |  |
|  | Deblocking filter |  | DBLK | various cb, cr qp_offsets | T. Suzuki, R. Sjöberg |
|  |  |  | DBLK | various beta_offset_div2, tc_offset_div2 | T. Suzuki, R. Sjöberg |
|  |  |  | DBLK | trigger all deblocking decisions | T.Suzuki, R. Sjöberg |
|  |  |  | DBLK | deblocking + tile, etc |  |
|  | SAO |  | SAO | Check all CTU sizes with max. SAO information rate. | E. Francois, Y. -W. Huang |
|  |  |  | SAO | Random and extreme SAO offsets values | E. Francois, Y. -W. Huang |
|  |  |  | SAO | Random SAO merge left/up flag values | E. Francois, Y. -W. Huang |
|  |  | Slice- and component-level control | SAO | Pictures divided into x slices / randomly enabling SAO Y and/or SAO UV per slice | E. Francois |
|  |  | Tile- and component-level control | SAO | Pictures divided into x tiles / randomly enabling SAO Y and/or SAO UV per tile | E. Francois |
|  | Entropy coding |  |  |  |  |
|  |  | Sign data hiding | SDH | sign_data_hiding_flag on, transquant_bypass_enable_flag on, cu_transquant_bypass_flag is set to 0 and 1 randomly at CU level Purpose: Check that the decoder handles sign data hiding with lossless coding. | H. Henry |
|  |  | High through put binalizaion | HTB |  | M. Coban |
|  |  | cabac_init_idc | CAINIT |  | - Segall |
|  |  | Max number of bins | MAXBIN |  | V. Sze |
| Corner case bitstream |  |  |  | Maximum number of slices permitted for Profile & Level combination; Each slice has unique short_term_ref_pic_set(), ref_pic_list_modification() and pred_weight_table() with maximum num_long_term_pics | C. Fogg |
|  |  |  |  | CTU size is 64x64 with no cu splits, and only one coded transform unit in the lower right 4x4 pixel corner of the CTU with delaQp signalled (as raised by P. Kaspenberg on the JCT reflector 12-Sept-2012) | C. Fogg |
|  |  |  |  | A wide range of values for pic_crop_left_offset, pic_crop_right_offset, pic_crop_top_offset, pic_crop_bottom_offset | C. Fogg |
|  |  |  |  | inferred splits along non-CTU multiple picture borders |  |
|  |  |  |  | combinations of transform_skip_enabled_flag, transquant_bypass_enable_flag, cu_transquant_bypass_flag |  |
|  |  |  |  | something to test the vast universe that is: POC signalling |  |
|  | Video resolution |  |  | pic_width_in_luma_samples = pic_height_in_luma_samples = (1 << Log2MinCbSize), with various CTU sizes. |  |

Table 2: The list of candidate conformance bitstreams and its volunteer to generate the bitstreams

Looking at the syntax coverage of the bitstreams submitted so far, it would be useful to have additional streams covering the cases listed in Table 3. The volunteers to generate those bitstreams should be identified during Geneva meeting.

| Syntax element | Case to be tested |
| --- | --- |
| sps_video_parameter_set_id | Value other than 0 |
| conf_win_bottom_offset conf_win_top_offset conf_win_left_offset conf_win_right_offset | Values other than 0, preferably not multiples of 4 |
| log2_min_transform_block_size_minus2 | Value other than 0 |
| pcm_sample_bit_depth_luma_minus1 pcm_sample_bit_depth_chroma_minus1 | Values different from each other |
| log2_min_pcm_luma_coding_block_size_minus3 | Value other than 0 |
| num_short_term_ref_pic_sets | Maximum value 64 |
| num_long_term_ref_pic_sps | Value other than 0 |
| sub_layer_profile_present_flag sub_layer_level_present_flag | Values other than 0 |
| use_delta_flag | Value other than 0 |
| output_flag_present_flag | Value other than 0 |
| init_qp_minus26 | Value less than 0 |
| nal_unit_type | Values 38 (filler data) |

Table 3: Syntax elements to be tested

# Related contributions

The following contributions were submitted.

JCTVC-O0084 Editor's proposed draft text of HEVC conformance testing [T. Suzuki, G. Sullivan, W. Wan]

JCTVC-O0104 Corner case conformance streams [C. Fogg (Harmonic)]

# Recommendations

The AHG recommends

- to discuss the plan to revise bitstreams which includes problems

- to continue to collect more bitstreams, especially the bitstreams for missing features summarized in Table 2 and3