| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0007 |
| --- | --- |

| Title: | JCT-VC AHG report: Range extensions software (AHG 7) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Ad-hoc group report |  |  |
| Author(s) or Contact(s): | Karl Sharman David Flynn | Email: | karl.sharman@eu.sony.com dflynn@blackberry.com |
| Source: | AHG7 |  |  |

_____________________________

# Abstract

This report summarizes the activities of Ad Hoc Group 7 on support for range extensions between the 14th and 15th JCT-VC meetings.

# Mandates

The ad hoc group was mandated to:

- Coordinate development of the HM RExt software and its distribution to JCT-VC members.

- Produce documentation of software usage for distribution with the software.

- Prepare and deliver RExt-4.0 software version and the reference configuration encodings according to JCTVC-N1006 [1].

- Prepare and deliver additional “dot” version software releases and software branches as appropriate.

- Perform analysis and reconfirmation checks of the behaviour of the draft design, and report the results of such analysis.

- Suggest configuration files for additional testing of tools.

- Coordinate with AHG6 on range extensions draft text to address any identified issues regarding text and software relationship.

# History of revisions

The HM10.1_RExt3.0 software was upgraded to HM11.0_RExt3.0, then HM11.1_RExt3.0, and then HM12.0_RExt3.0, with all three revisions released on Friday August 23rd, three weeks after the end of the conference. These revisions reflected the changes made to the main HM branch of the software and were tested accordingly.

On the same day, the development code for RExt4.0 with changes for all tools was announced, and a request was made for proponents to verify their tools. A few necessary changes were made, and HM12.0_RExt4.0_rc1 was released on Wednesday August 28th. Further problems were identified by proponents and HM12.0_RExt4.0_rc2 was released on Saturday 31st August, and after more testing, HM12.0_RExt4.0 was released on Thursday 5th September. Unfortunately the intra-reference-smoothing tool was being disabled when it should have been enabled, and vice versa, and HM12.0_RExt4.1 was released on Tuesday September 17th to address this issue, and also an issue regarding distortion costs for inter when FULL_NBIT was enabled (for RCE2/3-AHG18 test conditions).

The changes adopted to this HM12.0_RExt4.1 were due to proposals:

- M0042 - No-Display SEI message.

- RCE2 - Residual DPCM for intra transform-skip coding, and for inter transquant-bypass and transform-skip coding (intra transquant-bypass coding had already been added).

- RCE2 - Residual rotation.

- RCE2 - Single significance map context.

- N0080 - Intra reference smoothing disabled flag (flag off by default).

- N0141 - Use the 4:4:4 chroma qp 1:1 mapping for 4:2:2.

- N0188 - Extended precision processing (off by default).

- N0192 - Derive chroma 32x32 scaling lists for chroma 16x16 scaling lists.

- N0256 - Intra block copy (also known as intra motion vectors).

- N0275 - Transform-skip shift clipping.

- N0288 - Specify transform skip maximum size.

The process of patching the software was made significantly more complicated since there were additional tools added that interacted with other tools. This can be seen in the new flow diagram below, which was also distributed via the reflector (in text form).

Figure 1 - Flow diagram for RExt4

In addition, intra-block-copy utilised inter processing which had not been tested with AHG18 conditions before, and many distortion calculations were found to overflow when FULL_NBIT was set to 1, which led to a new ‘Distortion’ variable type to be used.

Table 1 - Results vs HM12.0_RExt3.0

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | -0.8% | -0.7% | -0.7% | -0.4% | -0.4% | -0.4% | -0.3% | -0.2% | -0.2% |
| YCbCr 4:4:4 | -0.4% | -0.4% | -0.5% | -0.4% | -0.3% | -0.4% | -0.3% | -0.2% | -0.3% |
| YCbCr 4:2:2 | -0.8% | 0.4% | 0.4% | -0.5% | -0.4% | -0.5% | -0.4% | -0.2% | -0.3% |
| Overall | -0.7% | -0.2% | -0.3% | -0.4% | -0.4% | -0.4% | -0.3% | -0.2% | -0.3% |
|  | -0.7% | 0.1% | -0.2% | -0.4% | -0.4% | -0.4% | -0.3% | -0.2% | -0.3% |
| Enc Time[%] | 167% |  |  | 165% |  |  | 161% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 102% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -0.6% | -0.4% | -0.5% | -0.3% | -0.2% | -0.3% |  |  |  |
| YCbCr 4:4:4 | -0.2% | -0.3% | -0.4% | -0.2% | -0.2% | -0.3% |  |  |  |
| YCbCr 4:2:2 | -1.3% | 3.0% | 2.9% | -0.6% | -0.1% | -0.2% |  |  |  |
| Overall | -0.7% | 0.7% | 0.6% | -0.4% | -0.1% | -0.2% |  |  |  |
|  | -0.7% | 0.5% | 0.6% | -0.4% | 0.1% | -0.2% |  |  |  |
| Enc Time[%] | 125% |  |  | 123% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -0.3% | -0.2% | -0.3% | -0.2% | -0.1% | -0.1% |  |  |  |
| YCbCr 4:4:4 | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% |  |  |  |
| YCbCr 4:2:2 | -1.3% | 4.8% | 5.0% | -0.6% | 0.3% | 0.3% |  |  |  |
| Overall | -0.6% | 1.4% | 1.5% | -0.3% | 0.0% | 0.0% |  |  |  |
|  | -0.6% | 1.6% | 1.4% | -0.3% | 0.4% | 0.1% |  |  |  |
| Enc Time[%] | 119% |  |  | 118% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |  |  |  |

The summary of the results for individual tools, as reported by the proponents was also released on the reflector, and is attached to this document for reference. From those results, it can be seen that the significant changes in efficiency are mainly due to intra-block-copy, although this has led to a 67% increase in encoder run-time for intra (this was originally even greater, but reduced by the simpler searching method supplied by the proponents). This intra-block-copy tool has greater benefit when considering screen content using AHG8 test conditions, which is not examined here.

There is also a change in bias for higher QPs, which can be seen in the intra main-tier and all tiers of inter coding, due to the removal of the chroma QP mapping tables.

# Recommendations

•	Continue to develop reference software based on HM and improve its quality.

•	Update encoder and documentation to be consistent with the current usage.

•	Remove macros introduced in previous HM versions before starting integration towards RExt4.0 such  
       as to make the software more readable.

•	Continue to identify bugs and discrepancies with text, and address them.

•	Test reference software more extensively outside of common test conditions.

# References

[1] D. Flynn and C. Rosewarne, “Common test conditions and software reference configurations for HEVC Range Extensions.” JCTVC-L1006, Jan. 2013.