| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-Oxxxx |
| --- | --- |

| Title: | JCT-VC AHG report: Single-Loop Scalability (AHG16) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | M. Wien M. Budagavi K. Misra K. Rapaka K. Ugur X. Xiu | Email: | wien@ient.rwth-aachen.de madhukar@ti.com misrak@sharplabs.com krapaka@qti.qualcomm.com Kemal.Ugur@nokia.com Xiaoyu.Xiu@InterDigital.com |
| Source: | AHG |  |  |

_____________________________

# Abstract

This report summarizes the activities of the Ad Hoc Group on Single-loop Scalability between the 14th JCT-VC meeting held in Vienna, Austria and the 15th JCT-VC meeting held in Geneva, Switzerland.

# Mandates

- Study and compare single-loop and multi-loop scalability (focus on SNR as the more demanding case).

- Analyze and compare the memory bandwidth of single-loop and multi-loop scalability.

- Study the rate-distortion performance and complexity impact of single-loop scalability using residual refinement techniques and by signaling inter-layer prediction restrictions on a multi-loop design.

- Study and analyze the performance and complexity impact of inter-layer inter prediction tools for single-loop and multi-loop scalability.

# Email activity

The AhG used the JCT-VC reflector for discussion.  There was no activity for the AhG on the reflector. The participants in the related core experiment SCE2 on key pictures and single loop scalability had an intense offline discussion with an exchange of more than 300 emails among the proponents and participants.

# Related Contributions

## Proposal documents (SCE2 and HLS)

JCTVC-O0145 [SCE2] Key picture concept and single loop decoding, Christian Feldmann, Mathias Wien (RWTH Aachen University)

JCTVC-O0162 SCE2: Single-loop decoding based SNR scalability for SHVC, X. Xiu, Y. Ye, Y.-W. He, Y. He (InterDigital)

JCTVC-O0175 MV-HEVC/SHVC HLS: Indications related to single-loop decoding, M. M. Hannuksela, H. Roodaki (Nokia)

JCTVC-O0227 SCE2 : On key pictures in SHVC, K. Rapaka, J. Chen, V. Seregin, Y.-K. Wang, M. Karczewicz (Qualcomm)

## Cross-check documents (related to SCE2)

JCTVC-O0146 [SCE2] Cross check for Qualcomm JCTVC-O0227, Christian Feldmann, Mathias Wien (RWTH Aachen University)

JCTVC-O0169 SCE2: Cross-verification of key picture concept and single loop decoding from Aachen, X. Xiu, Y. Ye (InterDigital)

JCTVC-O0231 SCE2 : Cross check for Interdigital JCTVC-O0162, K. Rapaka (Qualcomm)

# Proposals

## High-level summary

The proposal contributions JCTVC-O0145, JCTVC-O0162, and JCTVC-O0227 are responses to SCE2 on key pictures and single-layer decoding. The corresponding crosschecks are listed in Section 3.2 above.

Contribution JCTVC-O0175 contains the high-level syntax proposals to replace the discardable_flag which can be useful for indications related to single-loop decoding.

### Technical contributions regarding single-loop decoding beyond SCE2

Contributions JCTVC-O0145 and JCTVC-O0162 contain results for modified versions of the proposed schemes which are beyond the content of the corresponding proposals to the Vienna meeting, JCTVC-N0202 and JCTVC-N0186, respectively. Therefore, these aspects are not considered to be part of the SCE2 response but are seen improvements relative to the previous proposals which have been tested and evaluated according to the SCE2 conditions. In JCTVC-O0145, results for a modified scheme are presented where the deblocking filter and SAO are enabled for the base layer reconstruction of the key pictures. In JCTVC-O0162, a modified scheme is presented using the compressed BL motion information to generate the alternative picture without using BL residue. As a summary of the performance impact, the results for these proposals are summarized in the tables below, following the reporting format of the SCE2 summary report. It is recommended to synchronize the discussion of these results with the discussion of SCE2.

#### Rate-distortion performance

Comparison of Random Access relative to CTC SHM 3.0.1 Anchor1

| RA Single Loop | BD rate EL [%] |  |  | BD rate BL only [%] |  |  | BD SNR BL only [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | Y | U | V | Y | U | V | Y | U | V |
| JCTVC-O145 no EL constraint | 1,2% | -5,9% | -7,5% | 6,7% | 4,4% | 4,3% | -0,20 | -0,07 | -0,08 |
| JCTVC-O145 key pic BL LF/SAO | -0,7% | -6,8% | -8,1% | 6,5% | 4,2% | 4,2% | -0,20 | -0,06 | -0,08 |
| JCTVC-O0162 no EL constraint | 2,6% | -6,6% | -8,9% | - | - | - | - | - | - |
| JCTVC-O0162 BL compressed MV, no EL residue | 11.6% | 1.8% | -0.7% | - | - | - | - | - | - |

Comparison of Low-Delay B relative to CTC SHM 3.0.1 Anchor1

| LDB Single Loop | BD rate EL [%] |  |  | BD rate BL only [%] |  |  | BD SNR BL only [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | Y | U | V | Y | U | V | Y | U | V |
| JCTVC-O145 no EL constraint | 1,3% | -4,1% | -5,6% | 15,5% | 6,1% | 4,9% | -0,47 | -0,09 | -0,10 |
| JCTVC-O145 key pic BL LF/SAO | 0,0% | -4,7% | -6,0% | 15,4% | 6,1% | 5,0% | -0,47 | -0,09 | -0,10 |
| JCTVC-O0162 no EL constraint | 2,9% | -5,3% | -7,6% | - | - | - | - | - | - |
| JCTVC-O0162 BL compressed MV, no EL residue | 12.7% | 4.5% | 1.8% | - | - | - | - | - | - |

#### Memory bandwidth

Comparison of Random Access relative to CTC SHM 3.0.1 Anchor1

| RA Single Loop |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test case | Pure | DDR2 | DDR3 | Mults |
| JCTVC-O145 no EL constraint | 59% | 60% | 62% | 68% |
| JCTVC-O145 key pic BL LF/SAO | 59% | 60% | 62% | 68% |
| JCTVC-O0162 no EL constraint | 63% | 67% | 67% | 61% |
| JCTVC-O0162 BL compressed MV, no EL residue | 56% | 59% | 60% | 60% |

Comparison of Low-Delay B relative to CTC SHM 3.0.1 Anchor1

| LDB Single Loop |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test case | Pure | DDR2 | DDR3 | Mults |
| JCTVC-O145 no EL constraint | 63% | 63% | 63% | 76% |
| JCTVC-O145 key pic BL LF/SAO | 62% | 62% | 63% | 75% |
| JCTVC-O0162 no EL constraint | 67% | 70% | 70% | 68% |
| JCTVC-O0162 BL compressed MV, no EL residue | 56% | 58% | 59% | 63% |

#### Pixel-level complexity

Comparison of Random Access relative to CTC SHM 3.0.1 Anchor1

| RA Single Loop |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | Pix Inter | Pix Intra | Pix MV Full Sum | Pix MV Half | Pix MV Quarter | Pix MV Full BL | Pix Inv Transf Y | Pix Deblocking Y | Pix SAO Y |
| JCTVC-O145 no EL constraint | 82% | 85% | 51% | 64% | 66% | 5% | 98% | 66% | 102% |
| JCTVC-O145 key pic BL LF/SAO | 79% | 83% | 48% | 61% | 61% | 5% | 98% | 63% | 102% |
| JCTVC-O0162 no EL constraint | 71% | 85% | 44% | 51% | 52% | 20% | 57% | 88% | 88% |
| JCTVC-O0162 BL compressed MV, no EL residue | 63% | 105% | 44% | 52% | 53% | 19% | 71% | 59% | 80% |

Comparison of Low-Delay B relative to CTC SHM 3.0.1 Anchor1

| LDB Single Loop |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | Pix Inter | Pix Intra | Pix MV Full Sum | Pix MV Half | Pix MV Quarter | Pix MV Full BL | Pix Inv Transf Y | Pix Deblocking Y | Pix SAO Y |
| JCTVC-O145 no EL constraint | 85% | 78% | 56% | 79% | 75% | 2% | 95% | 67% | 99% |
| JCTVC-O145 key pic BL LF/SAO | 85% | 77% | 56% | 78% | 74% | 2% | 95% | 67% | 96% |
| JCTVC-O0162 no EL constraint | 71% | 62% | 50% | 62% | 62% | 3% | 90% | 50% | 81% |
| JCTVC-O0162 BL compressed MV, no EL residue | 58% | 103% | 46% | 60% | 60% | 3% | 63% | 53% | 71% |

## JCTVC-O0145 [SCE2] Key picture concept and single loop decoding (RWTH Aachen University)

In this document a performance and complexity analysis of single loop decoding in SHVC using the key picture concept is presented in response to SCE2 test 2.3. The scheme has similarity with MGS as used in SVC. The enhancement layer rate-distortion performance is reported to be in the range of 1.2 % BD delta rate comparable to the multi-loop SHM CTC configuration while the reconstruction of the base layer only is reportedly reduced by 0.20 dB BD-PSNR for RA and 0.47 dB BD-PSNR for the LDB configuration. The AHG17 measurements of the presented single loop decoding scheme are reported to be in the range of 59-73 % of the memory access of SHM3.0.1 under CTC.

Additionally, results for a modified scheme are presented where the deblocking filter and SAO are enabled for the base layer reconstruction of the key pictures.  The results show comparable performance to the multi-loop SHM CTC configuration for the full reconstruction while the drift for reconstruction of the base layer only is reported to be in the range of 0.19 dB BD-PSNR for RA and 0.47 dB BD-PSNR for the LDB configuration. The AHG17 measurements are reported to be in the range of 59-69 % of the memory access of SHM3.0.1 under CTC.

## JCTVC-O0162 SCE2: Single-loop decoding based SNR scalability for SHVC (InterDigital)

This document reports the performance of the single-loop decoding scheme for SNR scalability of SHVC, which was originally proposed in JCTVC-N0186 and included in SCE2. In the current SHVC design, the reconstructed base layer (BL) picture (after up-sampling if necessary) is used for the inter-layer prediction (ILP) of enhancement layer (EL) coding. This implies that multiple motion compensation (MC) operations have to be performed for all dependent layers. In JCTVC-N0186, a single-loop decoding scheme is proposed by introducing an alternative ILP picture by applying BL motion information on EL temporal reference pictures. In addition, BL residue could be further added to the alternative ILP picture for quality improvement. As both BL motion information and BL residue could be obtained without the full reconstruction of BL picture, the single-loop decoding requirement is fulfilled by replacing the conventional ILP picture with the alternative ILP picture for EL prediction.

Experimental results shows that compared to SHM 3.0.1 anchor, the proposed sing-loop decoding scheme can reportedly reduce the average memory access by 35%, with the average BD-rate increase of 2.7% for RA/LDB. When temporal scalability is enabled, the same reduced memory access can also be achieved, with average BD-rate increase of 3.0% for RA/LDB, compared to SHM 3.0.1 temporal scalability anchor. In addition, when the compressed BL motion information is used to generate the alternative picture without using BL residue, the average savings of the memory access is increased to 44% and 43% for non-temporal scalability case and temporal scalability case, respectively.

## JCTVC-O0175 MV-HEVC/SHVC HLS: Indications related to single-loop decoding (Nokia)

It is proposed to replace the discardable_flag syntax element with a 2-bit indication, pic_ref_idc, having values with the following semantics:

- 3 = discardable picture.

- 2 = intra-layer non-reference picture. Specifies that the picture is not used as a reference picture for inter prediction of subsequent pictures in decoding order within the same layer and that the picture may be used as an inter-layer prediction reference.

- 1 = non-output-layer skip (NOLS) picture, which may be used as a reference for other NOLS and discardable pictures in the same layer but is not used as a reference for inter-layer prediction.

- 0 specifies that the picture may be used as a reference for inter prediction of subsequent pictures in decoding order within the same layer and may be used as a reference for inter-layer prediction.

It is asserted that encoders may use prediction structures including intra-layer non-reference pictures and/or NOLS pictures to omit the decoding of some parts of the base layer and to reduce the DPB memory usage for the decoded base layer pictures.

## JCTVC-O0227 SCE2 : On key pictures in SHVC (Qualcomm)

This contribution proposes a design to enable using the decoded pictures from higher quality layer as reference for lower layer pictures for SNR scalability in SHVC. The document proposes

- Signaling a flag in VPS to specify that immediate higher layer picture is used as reference for lower layer

- The concept of key picture to reduce error drift for base layer decoding

- Reference picture set (RPS) and reference picture list (RPL) initialization process is modified for base layer pictures to include BL reference picture into the RPL and use it as collocated picture for TMVP derivation. This is to ensure that there is no drift introduced to motion information.

With respect to SHM 3.0.1 temporal scalability anchor, the proposed method reports an average of −3.1%, −3.3% and −3.5% luma BD-rate reductions (EL + BL) for RA-SNR, LD-B and LD-P SNR scalability cases respectively. It is also reported that over SHM 3.0.1 anchor an average of −2.9%, 1.3% and −0.2% luma BD-rate reductions (EL + BL) is obtained for RA-SNR, LD-B and LD-P SNR scalability cases respectively.

# Recommendations

The AHG recommends to

- study the contributions related to the AHG

- study the contributions related SCE2 and beyond

- study the application of single-loop decoding as well as the application of key-pictures in SHVC.