| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0017 |
| --- | --- |

| Title: | JCT-VC AHG report: SHVC complexity assessment (AHG17) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E. Alshina, M. Budagavi, J. Dong, E. François, J. Kang, X. Li, A. Tabatabai | Email: | elena_a.alshina@samsung.com, madhukar@ti.com, Edouard.Francois@technicolor.com, Jie.Dong@InterDigital.com, Ali.Tabatabai@am.sony.com, lxiang@qti.qualcomm.com, jungwon@etri.re.kr |
| Source: | AhG 17 |  |  |

_____________________________

# Abstract

This document reports on the work of the JCT-VC AHG on SHVC complexity assessment (AHG17) between the 14th JCT-VC meeting in Vienna, AT, (25 July – 2 Aug. 2013) and 15th JCT-VC meeting in Geneva, Switzerland, (22 Oct – 1 Nov. 2013), and lists the related input documents.

# Mandate

SHVC complexity assessment (AHG17)

(jct-vc@lists.rwth-aachen.de)

- Study memory bandwidth, memory usage and computational complexity of scalable tools and methodologies to evaluate them.

- Study restrictions to reduce the complexity of scalable tools and evaluate the impact of such restrictions on coding efficiency.

- Prepare a report analyzing performance and complexity of single-layer, simulcast, and scalable coding configurations for 1.0, 1.5, and 2.0 and arbitrary scalability ratio (e.g. ~1.75) cases.

- Coordinate with SCE developments and provide a template to be used for complexity analysis of SHM3.0, single-layer coding, and SCE techniques.

# Activity related to mandates and test results summary

Following process are taken into account during complexity assessment:

- Memory access for reconstructed frame writing

- Memory access during interpolation process (motion compensation or re-sampling for inter-layer texture prediction).

- Multiplication operations during interpolation process (motion compensation or re-sampling for inter-layer texture prediction).

Block based inter-layer prediction model is taken into account during complexity assessment.

Complexity assessment s/w module was modified compare to previous version [1]-[3] as follows:

- The measurement for number of multiplication was added

- Both base layer and enhancement layer memory access are counted (bug fix).

Complexity assessment materials (after all bug fixes) were released to SCE coordinators at the middle of September.

In the worst case 2 layers SHM3.0 decoder is 2 times more complex compare to HEVC single layer decoder (both from view point of memory access and number of multiplications).

Graphs and tables below summarize the complexity measurement for 2 layers SHM3.0 decoder compare to HEVC single layer.

In so-called “all intra” test scenario all frames of base layer are Intra coded (it means there is no memory access or multiplications associated with interpolation process). But for enhancement layer interpolation process is needed for inter-layer texture prediction.

In this case an average memory access for scalable system is more than double (spatial ratio ×1.5) compare to single layer coding. For expected to be used the most frequently spatial ratio ×2 memory access in so-called “all intra” test scenario is ~150% compare to HEVC single layer.

Complexity increment in this test scenario is the highest among all. But the BD-rate drop of 2 layers scalable system compare to HEVC single layer is 11-14% (the smallest among all tests).

Fig. 1 Complexity measurement results for SHM3.0 vs HEVC single layer (“all intra” test).

In tests scenarios with motion compensation average memory access of 2 layers scalable system compare to single layer is from ~120% (spatial ratio ×2)  to ~190% (SNR test == spatial ratio ×1). Multiplications number is from 124% (spatial ratio ×2) to ~176% (SNR test == spatial ratio ×1).  The BD-rate drop 2 layers scalable system shows compare to HEVC single layer is 18%, 25% and 27% for random access, low-delay-B and low-delay P configurations correspondently.

Fig. 2 Complexity measurement results for SHM3.0 vs HEVC single layer (“motion compensation” tests).

# New aspects

Following statists was collected during SCE2 experiments. The number of pixels processed by motion compensation, intra prediction, SAO and de-blocking was reported and analysed both for anchor and proposed tests.

# List of related input documents

The following documents are related to AHG17.

- JCTVC-O0032 Mathias Wien, Krishna Rapaka, Xiaoyu Xiu, “SCE2 Summary Report”.

- JCTVC-O00323 XXX, “SCE3 Summary Report”

- JCTVC-O0115 W. Hamidouche, M. Raulet . O. Deforges “Pipeline and parallel architecture for the SHVC decoder”

- JCTVC-O0165 S. Gudumasu , Y. He , Y. Ye , Y. He (InterDigital) “Real time SHVC software decoding with multi-threaded parallel processing”

- JCTVC-O0165 S. Gudumasu , Y. He , Y. Ye , Y. He (InterDigital) “Real time SHVC software decoding with multi-threaded parallel processing”

# Recommendations

The SHVC complexity assessment AHG recommends the JCT-VC to:

- Review all input documents listed in Section 4.

# References

- E.Alshina, A.Alshin “AhG17: complexity analysis of SHM2.0”, JCTVC-N0150, Vienna, Austria, Jul. 2013.

- E. Alshina, E. François “BoG Report on SHVC complexity assessment”, JCTVC-M0455, Incheon, Korea, Apr. 2013.

- E. François, A. Tabatabai, E. Alshina “BoG report: Methodology for evaluating complexity of combined and residual prediction methods in SHVC”, JCTVC-L00440, Geneva, Switzerland, Jan. 2013.