| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0018 |
| --- | --- |

| Title: | JCT-VC AHG report: high bit-rate and bit-depth operating points (AHG 18) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Ad-hoc group report |  |  |
| Author(s) or Contact(s): | Karl Sharman Hui Yong Kim | Email: | karl.sharman@eu.sony.com hykim5@etri.re.kr |
| Source: | AHG18 |  |  |

_____________________________

# Abstract

This report summarizes the activities of the Ad Hoc Group on high bit-rate and bit-depth operating points between the 14th JCT-VC meeting held in Vienna, Austria and the 15th JCT-VC meeting held in Geneva, Switzerland.

# Mandates

- Study the accuracy needed for internal processing elements to support bit depths up to 16-bits per sample.

- Study relationship to lossless coding capability.

- Verify rate-distortion optimization behaviour for high bit rates and high bit depths.

- Study entropy coding operation and throughput at high bit rates and high bit depths and potential needs for associated design modification.

- Identify test sequences and test conditions for testing high bit rate and high bit depth coding behaviour.

- Prepare software implementation for technical investigation of new features intended for high bit rates and high bit depths.

- Study coding performance at high bit-rate and high bit depth operating points and investigate the benefit over existing standards.

# Reflector activity

The AhG intended to use the JCT-VC reflector for discussion, although there was no activity for this AhG. There have been some private discussions.

A bug was immediately identified and reported in the DAM text to the editors, and more recently a bug report was filed (#1185). This relates to the DAM text not including the increased precision during the inverse-transform stage – the DAM text permits coefficients of bitDepth+7 bits (signed) in the bit-stream and at the output of the inverse-quantiser, although it currently downshifts all bitDepth+7 data to 16 bits (signed) prior to the inverse transform, thereby only increasing the accuracy of the inverse-quantiser and applying an unbalanced clipping scheme at the output of the inverse-quantiser. However, bitDepth+7 was so designed because the transformed coefficients could be at most bitDepth+7 in size, and therefore reduction of the precision before the inverse transform would negate any benefit of having bigger values in the bit-stream. The RExt4 model utilises bitDepth+7 throughout.

However, a comment on #1185 has been made on this issue requesting the C model to be changed to convert all data back to 16 bit (signed).

# Related Contributions

RCE2 and non-RCE2 contributions.

Source video test material:

- JCTVC-O0069 AHG5 and AHG18: 16-bit Test Sequences for Range Extensions [K. Sharman, N. Saunders, J. Gamei (Sony)]

- JCTVC-O0172 AHG22: Sequences with medical mixed content and medical visual content with bit depth beyond 10 bits for HEVC development [P. Amon, A. Hutter, U.-E. Martin, B. Heigl (Siemens)]

- JCTVC-O0173 Coding of medical mixed content and medical visual content with bit depth beyond 10 bits [P. Amon, A. Hutter, U.-E. Martin, N. Wirsz (Siemens)]

Throughput:

- JCTVC-O0046 AHG5 and AHG18: Entropy Coding Throughput for High Bit Depths [K. Sharman, N. Saunders, J. Gamei (Sony)]

- JCTVC-O0207 AhG5 and AhG18: Bypass bins alignment depending on the Rice parameter [J. Sole, R. Joshi, M. Karczewicz (Qualcomm)]

- JCTVC-O0208 AhG5 and AhG18: Bypass of the coefficient level flags for higher throughput [J. Sole, R. Joshi, M. Karczewicz (Qualcomm)]

  - JCTVC-O0313 Cross-check of JCTVC-O0208 [C. Rosewarne, M. Maeda (Canon)] [late] [miss]

- JCTVC-O0209 AHG5 and AHG18: Bypass of the significance and coefficient level flags for higher throughput [R. Joshi, J. Sole, M. Karczewicz (Qualcomm)]

- JCTVC-O0236 CABAC Bypass Throughput Improvement for High Bit Depths [W. Pu (Qualcomm)] [miss]

Precision analysis:

- JCTVC-O0068 AHG5 and AHG18: Transform Matrix Precision for High Bit Depths [K. Sharman, N. Saunders, J. Gamei (Sony)]

- JCTVC-O0259 AHG18: On Supporting Higher Bit Depth With Transform Skip Only Changes [K. Misra, S.-H. Kim (Sharp)]

  - JCTVC-O0314 Cross-check of JCTVC-O0259 on supporting higher bit-depth with transform skip only changes [S. Lee, C. Park, C. Kim (Samsung)] [late] [miss]

Other:

- JCTVC-O0090 AhG 5 and 18: High Bit-depth Coding Using Auxiliary Picture [W.-S. Kim, W. Pu, J. Chen, Y.-K. Wang, J. Sole, M. Karczewicz (Qualcomm)]

- JCTVC-O0091 AhG 5 and 18: Sample Adaptive Offset on High Bit-depth [W.-S. Kim, J. Sole, M. Karczewicz (Qualcomm)]

# Recommendations

The AHG recommends studying the contributions related to the AHG.