| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0031r1 |
| --- | --- |

| Title: | SCE1: Summary report of SHVC Core Experiment on support for arbitrary ratio scalability |  |  |
| --- | --- | --- | --- |
| Status: | CE report input to JCT-VC |  |  |
| Purpose: | CE Summary |  |  |
| Author(s) or Contact(s): | Edouard François (primary) Elena Alshina Jianle Chen | Tel: Email: | edouard.francois@crf.canon.fr elena_a.alshina@samsung.com cjianle@qti.qualcomm.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document summarizes the activities and test results performed in SCE1 on arbitrary ratio scalability. Two proposals have been evaluated on the test conditions defined in document JCTVC-N1101.

# List of proposals

SCE1 aims at evaluating different candidate upsampling filters for spatial scalability with any ratio between 1x and 2x. Two proposals are considered in this SCE.

Table 1: list of SCE1 proposals

| Proposal | Initial proposal | Proposal documents | Cross-checking documents |
| --- | --- | --- | --- |
| 1.1 | JCTVC-N0219: Non-SCE1: On arbitrary spatial ratio scalability in SHVC | JCTVC-O0076 (Qualcomm, Samsung) | JCTVC-O0124 (Canon) |
| 1.2 | JCTVC-N0273: On the selection of fixed filters for upsampling | JCTVC-O0052 (Arris) | JCTVC-O0281 (Samsung) |

Table 2 and Table 3 provide the 16-phases upsampling filters for Luma and Chroma for the reference (used as anchor, cf JCTVC-N0375), and for proposals 1.1 and 1.2. Filters for phase p > 8 are not depicted since they are the symmetrical version of filters of phase (8-p). Filters from proposals differing from the reference filters are highlighted in yellow.

Table 2: luma upsampling filters

| Luma filters |  |  |  |
| --- | --- | --- | --- |
| phase | Reference (N0375) | Proposal 1.1 (O0076) | Proposal 1.2 (O0052) |
| 0/16 | 0,0, 0,64, 0, 0,0, 0 | 0,0, 0,64, 0, 0,0, 0 | 0,0, 0,64, 0, 0,0, 0 |
| 1/16 | 0,1, -3,63, 4, -2,1, 0 | 0,1, -3,63, 4, -2,1, 0 | 0,1, -3,63, 4, -2,1, 0 |
| 2/16 | -1,2, -5,62, 8, -3,1, 0 | -1,2, -5,62, 8, -3,1, 0 | -1,3, -7,63, 8, -3,1, 0 |
| 3/16 | -1,3, -8,60,13, -4,1, 0 | -1,3, -8,60,13, -4,1, 0 | -1,3, -8,60,13, -4,1, 0 |
| 4/16 | -1,4,-10,58,17, -5,1, 0 | -1,4,-10,58,17, -5,1, 0 | -1,4,-10,58,17, -5,1, 0 |
| 5/16 | -1,4,-11,52,26, -8,3,-1 | -1,4,-11,52,26, -8,3,-1 | -1,4,-11,54,23, -7,3,-1 |
| 6/16 | -1,3, -9,47,31,-10,4,-1 | -1,4,-11,50,28, -8,2, 0 | -1,3, -9,47,31,-10,4,-1 |
| 7/16 | -1,4,-11,45,34,-10,4,-1 | -1,4,-11,45,34,-10,4,-1 | -1,4,-11,45,34,-10,4,-1 |
| 8/16 | -1,4,-11,40,40,-11,4,-1 | -1,4,-11,40,40,-11,4,-1 | -1,4,-11,40,40,-11,4,-1 |

Table 3: chroma upsampling filters

| Chroma filters |  |  |  |
| --- | --- | --- | --- |
| phase | Reference (N0375) | Proposal 1.1 (O0076) | Proposal 1.2 (O0052) |
| 0/16 | 0, 64, 0, 0 | 0, 64, 0, 0 | 0, 64, 0, 0 |
| 1/16 | -2, 62, 4, 0 | -2, 62, 4, 0 | 0, 58, 8, -2 |
| 2/16 | -2, 58, 10, -2 | -2, 58, 10, -2 | -2, 58, 10, -2 |
| 3/16 | -4, 56, 14, -2 | -4, 56, 14, -2 | -2, 53, 17, -4 |
| 4/16 | -4, 54, 16, -2 | -4, 54, 16, -2 | -4, 54, 16, -2 |
| 5/16 | -6, 52, 20, -2 | -6, 52, 20, -2 | -4, 48, 24, -4 |
| 6/16 | -6, 46, 28, -4 | -6, 46, 28, -4 | -4, 44, 28, -4 |
| 7/16 | -4, 42, 30, -4 | -4, 42, 30, -4 | -6, 44, 30, -4 |
| 8/16 | -4, 36, 36, -4 | -4, 36, 36, -4 | -4, 36, 36, -4 |

Reference filters are integrated in the SHM3.0.1 (enabled by macro ARBITRARY_SPATIAL_RATIO).

# SCE test conditions

Three ratios are considered in SCE1 (cf JCTVC-N0375): ratios 1.5x and 2x already used in Common Test Conditions, and a ratio close to 1.75x (adjusted in order to ensure picture size multiple of 8). This later ratio enables to test all of the 16 phases.

4 types of data are provided in SCE1, summarized in Table 4.

Table 4: cases considered in SCE1

| Downsampler | SHVC downsampler | JSVM downsampler |
| --- | --- | --- |
| Ratios |  |  |
| 1.5x and 2x | Case 1 (CTCs) | Case 2 (CTCs with new BL) |
| ~1.75x | Case 3 | Case 4 |

## Downsamplers

Two downsampler versions have been used to generate the base layer sequences:

- So-called SHVC downsampler ; for ratios 1.5x and 2x, it corresponds to the downsampler used for the Common Test Conditions.

- JSVM downsampler, modified to use phase 0 alignment between top left base and enhancement luma samples.

Both downsamplers are implemented in the JSVM software. The patch corresponding to the modifications is attached to this document. The modified software contains two new macros:

- #define ZERO_PHASE: when set to 1, top left base and enhancement luma samples are aligned.

- #define SHVC_DOWNSAMPLER: when set to 0, JSVM downsampling filters are used; when set to 1, so-called SHVC downsampling filters are used.

The filters coefficients corresponding to the SHVC and JSVM downsamplers are given in Annex.

The md5 sums of the different base layer sequences are also provided in Annex.

The BD-rate performance drop of SHVC vs HM11.0 single layer for different base-layers have been evaluated. Results for luma are summarized in Table 5. For ratio ~1.75x the reference upsampling filters (JCTVC-N0375) are used. These results have been generated by Samsung and cross-checked by Canon.

This table shows coherent results between the two downsamplers. According to these results, the "arbitrary ratio" downsamplers work adequately.

Table 5: Luma BD-rate performance drop vs HM11.0 single layer for different base-layers.

|  | ×2 SHVC | ×2 JSVM | ~1,75 SHVC | ~1,75 JSVM | ×1,5 SHVC | ×1,5 JSVM |
| --- | --- | --- | --- | --- | --- | --- |
| AI | 12.8% | 14.1% | 14.0% | 15.5% | 10.5% | 13.3% |
| RA | 19.0% | 19.4% | 19.5% | 20.3% | 16.2% | 17.3% |
| LD-B | 28.3% | 28.0% | 28.5% | 28.7% | 24.8% | 24.9% |
| LD-P | 26.5% | 26.1% | 26.6% | 26.6% | 22.8% | 22.8% |

## Performed tests

The four cases summarized in Table 4 correspond to the following settings:

- Performance using non-modified base layer with ratios 2x and 1.5x 
This corresponds to the Common test conditions, without any change to the base layer sequences.

- Performance using base layer with ratios 2x and 1.5x generated by JSVM downsampler 
This corresponds to following modified JSVM settings:

- #define ZERO_PHASE        1

- #define SHVC_DOWNSAMPLER  0

- Performance using base layer with ratio ~1.75x generated by SHVC downsampler
This corresponds to following modified JSVM settings:

- #define ZERO_PHASE        1

- #define SHVC_DOWNSAMPLER  1

- Performance using base layer with ratio ~1.75x generated by JSVM downsampler
This corresponds to following modified JSVM settings:

- #define ZERO_PHASE        1

- #define SHVC_DOWNSAMPLER  0

## Summary of Results

Results are summarized in Table 6, using SHM3.0.1 with filters of JCTVC-N0375 as reference (the 2 digits table in shown in Annex section).

Table 6: summary of coding performance and complexity measures.

|  |  | Proposal 1.1 - JCTVC-O0076 |  |  |  |  | Proposal 1.2 - JCTVC-O0052 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | mem | mult | Y | U | V | mem | mult |
| Case1 | AI | 0.0% | 0.0% | 0.0% | 100% | 100% | 0.0% | 0.0% | 0.0% | 100% | 100% |
| CTC | RA | 0.0% | 0.0% | 0.0% | 100% | 100% | 0.0% | 0.0% | 0.0% | 100% | 100% |
| 2x | LB | 0.0% | 0.0% | 0.0% | 100% | 100% | 0.0% | 0.0% | 0.0% | 100% | 100% |
|  | LP | 0.0% | 0.0% | 0.0% | 100% | 100% | 0.0% | 0.0% | -0.1% | 100% | 100% |
| Case1 | AI | 0.0% | 0.0% | 0.0% | 100% | 100% | 0.5% | 0.3% | 0.3% | 100% | 100% |
| CTC | RA | 0.0% | 0.0% | 0.0% | 100% | 100% | 0.4% | 0.1% | 0.1% | 100% | 100% |
| 1.5x | LB | 0.0% | 0.0% | 0.0% | 100% | 100% | 0.3% | 0.1% | 0.0% | 100% | 100% |
|  | LP | 0.0% | 0.0% | 0.0% | 100% | 100% | 0.3% | 0.1% | 0.1% | 100% | 100% |
| Case2 | AI | 0.0% | 0.0% | 0.0% | 100% | 100% | 0.0% | 0.0% | 0.0% | 100% | 100% |
| JSVM | RA | 0.0% | 0.0% | 0.0% | 100% | 100% | 0.0% | 0.0% | 0.0% | 100% | 100% |
| 2x | LB | 0.0% | 0.0% | 0.0% | 100% | 100% | 0.0% | -0.1% | 0.0% | 100% | 100% |
|  | LP | 0.0% | 0.0% | 0.0% | 100% | 100% | 0.0% | -0.1% | -0.0% | 100% | 100% |
| Case2 | AI | 0.0% | 0.0% | 0.0% | 100% | 100% | 0.2% | 0.0% | 0.0% | 100% | 100% |
| JSVM | RA | 0.0% | 0.0% | 0.0% | 100% | 100% | 0.2% | 0.0% | 0.0% | 100% | 100% |
| 1.5x | LB | 0.0% | 0.0% | 0.0% | 100% | 100% | 0.1% | 0.0% | -0.2% | 100% | 100% |
|  | LP | 0.0% | 0.0% | 0.0% | 100% | 100% | 0.1% | 0.0% | -0.1% | 100% | 100% |
| Case3 | AI | 0.0% | 0.0% | 0.0% | 100% | 100% | -0.1% | -0.2% | -0.2% | 100% | 100% |
| SHVC | RA | 0.0% | 0.0% | 0.0% | 100% | 100% | 0.0% | -0.1% | -0.1% | 100% | 100% |
| ~1.75x | LB | 0.0% | -0.1% | 0.0% | 100% | 100% | 0.0% | -0.1% | -0.2% | 100% | 100% |
|  | LP | 0.0% | 0.0% | 0.0% | 100% | 100% | 0.0% | -0.1% | -0.1% | 100% | 100% |
| Case4 | AI | 0.0% | 0.0% | 0.0% | 100% | 100% | 0.0% | -0.2% | -0.2% | 100% | 100% |
| JSVM | RA | 0.0% | 0.0% | 0.0% | 100% | 100% | 0.0% | -0.2% | -0.2% | 100% | 100% |
| ~1.75x | LB | 0.0% | -0.1% | 0.0% | 100% | 100% | 0.0% | -0.1% | -0.1% | 100% | 100% |
|  | LP | 0.0% | 0.0% | 0.0% | 100% | 100% | 0.0% | -0.1% | -0.1% | 100% | 100% |
| Overall summary |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | U | V | mem | mult | Y | U | V | mem | mult |
| Case1 | avg | 0.0% | 0.0% | 0.0% | 100% | 100% | 0.2% | 0.1% | 0.0% | 100% | 100% |
| Case2 | avg | 0.0% | 0.0% | 0.0% | 100% | 100% | 0.1% | 0.0% | 0.0% | 100% | 100% |
| Case3 | avg | 0.0% | 0.0% | 0.0% | 100% | 100% | 0.0% | -0.2% | -0.2% | 100% | 100% |
| Case4 | avg | 0.0% | 0.0% | 0.0% | 100% | 100% | 0.0% | -0.2% | -0.1% | 100% | 100% |

# ANNEX

## Filters coefficients of the downsamplers

### SHVC downsampler

Filters below are based on the SHVC dowmsampler filters, as described in MPEG documents M23485 and M24499.

if (  3 * EL_width >  5 * BL_width)   // cases 2x, 1.75x

2,  -3, -9,	 6, 39, 58, 39,  6, -9,	-3,  2,  0        // 0

2,  -3, -9,	 4, 38, 58, 43,  7, -9,	-4,  1,  0        // 1

2,  -2, -9,	 2, 35, 58, 44,  9, -8,	-4,  1,  0        // 2

1,  -2, -9,	 1, 34, 58, 46, 11, -8,	-5,  1,  0        // 3

1,  -1, -8,	-1, 31, 57, 47, 13, -7,	-5,  1,  0        // 4

1,  -1, -8,	-2, 29, 56, 49, 15, -7,	-6,  1,  1        // 5

1,   0, -8,	-3, 26, 55, 51, 17, -7,	-6,  1,  1        // 6

1,   0, -7,	-4, 24, 54, 52, 19, -6,	-7,  1,  1        // 7

1,   0, -7,	-5, 22, 53, 53, 22, -5,	-7,  0,  1        // 8

1,   1, -7,	-6, 19, 52, 54, 24, -4,	-7,  0,  1        // 9

1,   1, -6,	-7, 17, 51, 55, 26, -3,	-8,  0,  1        // 10

1,   1, -6,	-7, 15, 49, 56, 29, -2,	-8, -1,  1        // 11

0,   1, -5,	-7, 13, 47, 57, 31, -1,	-8, -1,  1        // 12

0,   1, -5,	-8, 11, 46, 58, 34,  1,	-9, -2,  1        // 13

0,   1, -4,	-8,  9, 44, 58, 35,  2,	-9, -2,  2        // 14

0,   1, -4,	-9,  7, 43, 58, 38,  4,	-9, -3,  2        // 15

Else if (  4 * EL_width >  5 * BL_width)   // case 1.5x

0, 5, -6,	-10,	37,	76, 37, -10,   -6,  5,	0,  0        //0

0, 5, -4,	-11,	33,	76, 40,  -9,   -7,  5,	0,  0        //1

-1, 5, -3,	-12,  29,	75, 45,  -7,   -8,  5,	0,  0        //2

-1, 4, -2,	-13,	25,	75, 48,  -5,   -9,  5,	1,  0        //3

-1, 4, -1,	-13,	22,	73, 52,  -3,  -10,  4,	1,  0        //4

-1, 4,  0,  -13,	18,	72, 55,  -1,  -11,  4,	2, -1        //5

-1, 4,  1,  -13,	14,	70, 59,   2,  -12,  3,	2, -1        //6

-1, 3,  1,  -13,	11,	68, 62,   5,  -12,  3,	2, -1        //7

-1, 3,  2,  -13,	 8,	65, 65,   8,  -13,  2,	3, -1        //8

-1, 2,  3,  -12,	 5,	62, 68,  11,  -13,  1,	3, -1        //9

-1, 2,  3,  -12,	 2,	59, 70,  14,  -13,  1,	4, -1        //10

-1, 2,  4,  -11,	-1,	55, 72,  18,  -13,  0,	4, -1        //11

0, 1,  4,  -10,	-3,	52, 73,  22,  -13, -1,	4, -1        //12

0, 1,  5,   -9,  -5,	48, 75,  25,  -13, -2,	4, -1        //13

0, 0,  5,   -8,  -7,	45, 75,  29,  -12, -3,	5, -1        //14

0, 0,  5,   -7,	-9,	40, 76,  33,  -11, -4,	5,  0        //15

### JSVM downsampler

if (  3 * EL_width >  5 * BL_width)   // cases 2x, 1.75x

0,  -4,  -7,  11,  38,  52,  38,  11,  -7,  -4,   0,   0

0,  -4,  -7,   9,  37,  51,  40,  13,  -6,  -7,   2,   0

0,  -3,  -7,   8,  35,  51,  41,  14,  -5,  -7,   1,   0

0,  -2,  -8,   6,  33,  51,  42,  16,  -5,  -7,   2,   0

0,  -2,  -8,   5,  32,  50,  43,  18,  -4,  -8,   2,   0

0,  -2,  -8,   4,  30,  50,  45,  19,  -3,  -8,   1,   0

0,  -1,  -8,   2,  28,  49,  46,  21,  -2,  -8,   1,   0

0,  -1,  -8,   1,  26,  49,  47,  23,  -1,  -8,   0,   0

0,   0,  -8,   0,  24,  48,  48,  24,   0,  -8,   0,   0

0,   0,  -8,  -1,  23,  47,  49,  26,   1,  -8,  -1,   0

0,   1,  -8,  -2,  21,  46,  49,  28,   2,  -8,  -1,   0

0,   1,  -8,  -3,  19,  45,  50,  30,   4,  -8,  -2,   0

0,   2,  -8,  -4,  18,  43,  50,  32,   5,  -8,  -2,   0

0,   2,  -7,  -5,  16,  42,  51,  33,   6,  -8,  -2,   0

0,   1,  -7,  -5,  14,  41,  51,  35,   8,  -7,  -3,   0

0,   2,  -7,  -6,  13,  40,  51,  37,   9,  -7,  -4,   0

Else if (  4 * EL_width >  5 * BL_width)   // case 1.5x

2,   0, -10,   0,  40,  64,  40,   0, -10,   0,   2,   0

2,   1,  -9,  -2,  37,  64,  42,   2, -10,  -1,   2,   0

2,   1,  -9,  -3,  34,  64,  44,   4, -10,  -1,   2,   0

2,   1,  -8,  -5,  31,  63,  47,   6, -10,  -2,   3,   0

1,   2,  -8,  -6,  29,  62,  49,   8, -10,  -2,   3,   0

1,   2,  -7,  -7,  26,  61,  52,  10, -10,  -3,   3,   0

1,   2,  -6,  -8,  23,  60,  54,  13, -10,  -4,   3,   0

1,   2,  -6,  -9,  20,  59,  56,  15, -10,  -4,   3,   1

1,   2,  -5,  -9,  18,  57,  57,  18,  -9,  -5,   2,   1

1,   3,  -4, -10,  15,  56,  59,  20,  -9,  -6,   2,   1

0,   3,  -4, -10,  13,  54,  60,  23,  -8,  -6,   2,   1

0,   3,  -3, -10,  10,  52,  61,  26,  -7,  -7,   2,   1

0,   3,  -2, -10,   8,  49,  62,  29,  -6,  -8,   2,   1

0,   3,  -2, -10,   6,  47,  63,  31,  -5,  -8,   1,   2

0,   2,  -1, -10,   4,  44,  64,  34,  -3,  -9,   1,   2

0,   2,  -1, -10,   2,  42,  64,  37,  -2,  -9,   1,   2

## Md5 sum of base layer sequences

### Using SHVC downsampler

The downsampler settings are as follows:

- #define ZERO_PHASE        1

- #define SHVC_DOWNSAMPLER  1

The obtained md5 sums with these settings are as follows:

2x: base layer sequences are same as SHVC CTCs

~1.75x:

b150bc33cef23b91b4ab99bd5878b70a Traffic_1456x912_30_crop_shvc.yuv

30364ec0b2270a4e0d0e78482b32ca99 PeopleOnStreet_1456x912_30_crop_shvc.yuv

c13d7f6c3bb339f38580c5db6999bec6 Kimono1_1096x616_24_shvc.yuv

9bbc033618a01570df633d1e6a29af68 ParkScene_1096x616_24_shvc.yuv

ac13b953eed1c61c670a4f7e1ff4d7a5 Cactus_1096x616_50_shvc.yuv

708367ffafc654e86739aaf8f9f2f219 BasketballDrive_1096x616_50_shvc.yuv

deb6a2891b4e130e1272683430875e04 BQTerrace _1096x616_60_shvc.yuv

1.5x: base layer sequences are same as SHVC CTCs

### Using JSVM downsampler

The downsampler settings are as follows:

- #define ZERO_PHASE        1

- #define SHVC_DOWNSAMPLER  0

The obtained md5 sums with these settings are as follows:

2x:

4d492c990810d0ebb0fee55ba6fd30ba basketballdrive_960x540_50_jsvm.yuv

908de20f99954a9257e5d4274bd2faa4 bqterrace_960x540_60_jsvm.yuv

2905f2ada4dc957e2a66e8e9e794eb24 cactus_960x540_50_jsvm.yuv

26f47201197da99ae35afa332c2475f6 kimono1_960x540_24_jsvm.yuv

c40ed8818caf50b54df1843620b740e0 parkscene_960x540_24_jsvm.yuv

00d997669643badfc0c4015335d9c006 peopleonstreet_1280x800_30_crop_jsvm.yuv

45d12bc319d6da1346daef21ce61b31f traffic_1280x800_30_crop_jsvm.yuv

~1.75x:

0a59327663de1fad8af27fa2d88e8609 basketballdrive_1096x616_50_jsvm.yuv

aae12198debe84a2bcdb760d1e2bbd2f bqterrace_1096x616_60_jsvm.yuv

5719143e66fe919f925f3b6dca2947a8 cactus_1096x616_50_jsvm.yuv

a494ebc576a17d3174ea72b8f7108008 kimono1_1096x616_24_jsvm.yuv

9926a403dcacd88a4f8bb1616a142b81 parkscene_1096x616_24_jsvm.yuv

8d6da49491cd1157965efeb213100071 peopleonstreet_1456x912_30_crop_jsvm.yuv

2ec4794bac611ea56824625c890bb178 traffic_1456x912_30_crop_jsvm.yuv

1.5x:

e27ff2157280c1760c5c4059a4410849 basketballdrive_1280x720_50_jsvm.yuv

8afd8fe00fb1a091d43e468847187969 bqterrace_1280x720_60_jsvm.yuv

574a55bc5c96eceaf83a6865ecac8d2d cactus_1280x720_50_jsvm.yuv

b1d5ed59ce57f9262f5ba7a6d7707230 kimono1_1280x720_24_jsvm.yuv

b5a804be5a20944fff8b59201d311a37 parkscene_1280x720_24_jsvm.yuv

## Results with 2 digits

|  |  | Proposal 1.1 - JCTVC-O0076 |  |  |  |  | Proposal 1.2 - JCTVC-O0052 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | mem | mult | Y | U | V | mem | mult |
| Case1 | AI | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.01% | -0.03% | -0.03% | 100% | 100% |
| CTC | RA | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.01% | 0.01% | -0.01% | 100% | 100% |
| 2x | LB | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | -0.01% | -0.04% | 100% | 100% |
|  | LP | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | -0.07% | 100% | 100% |
| Case1 | AI | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.48% | 0.25% | 0.26% | 100% | 100% |
| CTC | RA | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.37% | 0.15% | 0.07% | 100% | 100% |
| 1.5x | LB | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.28% | 0.07% | 0.02% | 100% | 100% |
|  | LP | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.30% | 0.07% | 0.07% | 100% | 100% |
| Case2 | AI | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.01% | -0.03% | -0.03% | 100% | 100% |
| JSVM | RA | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.01% | -0.01% | -0.03% | 100% | 100% |
| 2x | LB | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | -0.08% | 0.04% | 100% | 100% |
|  | LP | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | -0.05% | -0.05% | 100% | 100% |
| Case2 | AI | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.23% | -0.01% | -0.01% | 100% | 100% |
| JSVM | RA | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.18% | 0.00% | 0.04% | 100% | 100% |
| 1.5x | LB | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.13% | -0.02% | -0.18% | 100% | 100% |
|  | LP | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.11% | -0.05% | -0.14% | 100% | 100% |
| Case3 | AI | 0.00% | -0.02% | -0.02% | 100% | 100% | -0.07% | -0.24% | -0.25% | 100% | 100% |
| SHVC | RA | 0.01% | 0.00% | 0.02% | 100% | 100% | -0.03% | -0.15% | -0.11% | 100% | 100% |
| ~1.75x | LB | 0.01% | -0.07% | 0.04% | 100% | 100% | -0.02% | -0.09% | -0.18% | 100% | 100% |
|  | LP | 0.02% | 0.00% | 0.00% | 100% | 100% | -0.03% | -0.09% | -0.15% | 100% | 100% |
| Case4 | AI | 0.00% | -0.03% | -0.03% | 100% | 100% | -0.03% | -0.17% | -0.19% | 100% | 100% |
| JSVM | RA | 0.00% | -0.01% | 0.00% | 100% | 100% | -0.01% | -0.16% | -0.15% | 100% | 100% |
| ~1.75x | LB | 0.01% | -0.09% | 0.02% | 100% | 100% | -0.01% | -0.13% | -0.12% | 100% | 100% |
|  | LP | 0.02% | -0.01% | -0.03% | 100% | 100% | -0.01% | -0.14% | -0.14% | 100% | 100% |
| Overall summary |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | U | V | mem | mult | Y | U | V | mem | mult |
| Case1 | avg | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.18% | 0.06% | 0.03% | 100% | 100% |
| Case2 | avg | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.08% | -0.03% | -0.05% | 100% | 100% |
| Case3 | avg | 0.01% | -0.02% | 0.01% | 100% | 100% | -0.04% | -0.19% | -0.19% | 100% | 100% |
| Case4 | avg | 0.01% | -0.03% | -0.01% | 100% | 100% | -0.01% | -0.15% | -0.15% | 100% | 100% |