| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0032 |
| --- | --- |

| Title: | SCE2 Summary Report: Key picture concept and single loop decoding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Mathias Wien (RWTH Aachen University) Krishna Rapaka (Qualcomm Inc.) Xiaoyu Xiu (InterDigital Communications Inc.) | Tel: Email: | wien@ient.rwth-aachen.de krapaka@qti.qualcomm.com Xiaoyu.Xiu@InterDigital.com |
| Source: | SCE Coordinators |  |  |

_____________________________

# Abstract

The SCE2 activities are summarized. The report includes the results reported by the proponents in response to the testing conditions as defined in the SCE2 document JCTVC-N1102. The reported results include the rate-distortion performance assessment using the RD reporting sheets provided with the JCT-VC CTC (JCTVC-N1009) and the memory access assessment as defined by AHG17, see document JCTVC-N0017. In addition to these measurements, pixel level processing measurements have been performed according to the SCE2 test definition.

# Introduction

This document summarizes the activities in SCE2. The report includes the results reported by the proponents in response to the testing conditions as defined in the SCE2 CE document JCTVC-N1102. The reported results include the rate-distortion performance assessment using the RD reporting sheets provided with the JCT-VC CTC (JCTVC-N1009) and the memory access assessment as defined by AHG17, see document JCTVC-N0017. In addition to these measurements, pixel level processing measurements have been performed according to the SCE2 test definition.

The SCE had 11 participating companies and three proponents. More than 300 emails have been exchanged in the SCE2 time period. The proposal documents in response to SCE2 and the corresponding crosscheck documents are listed in the table below.

In section 4 of this document the rate-distortion results and the complexity measurements in SCE2 are reported.

## Subscribed participants

The following participants have registered for participation within the editing period of the SCE document. Input documents in response to the SCE are listed in the table.

| Participants | Affiliation | SCE Response | Crosscheck |
| --- | --- | --- | --- |
| Krishna Rapaka (krapaka@qti.qualcomm.com), Jianle Chen (cjianle@qti.qualcomm.com) | Qualcomm | JCTVC-O0227 | JCTVC-O0146 (by RWTH) |
| Christian Feldmann (feldmann@ient.rwth-aachen.de), Mathias Wien (wien@ient.rwth-aachen.de) | RWTH Aachen University | JCTVC-O0145 | JCTVC-O0169 (by InterDigital) |
| Xiaoyu Xiu (Xiaoyu.Xiu@InterDigital.com), Yan Ye (Yan.Ye@InterDigital.com), Yuwen He (Yuwen.He@InterDigital.com) | InterDigital | JCTVC-O0162 | JCTVC-O0231 (by Qualcomm) |
| Kemal Ugur (Kemal.Ugur@nokia.com) | Nokia |  |  |
| Elena Alshina (elena_a.alshina@samsung.com) Ankur Saxena (asaxena@sta.samsung.com ) | Samsung |  |  |
| Jill Boyce (jill@vidyo.com) | Vidyo |  |  |
| Yu-Wen Huang (yuwen.huang@mediatek.com), Peter Chuang (peter.chuang@mediatek.com), Shan Liu (shan.liu@mediatek.com) | Mediatek |  |  |
| Tomoyuki Yamamoto (yamamoto.tomoyuki@sharp.co.jp), Takeshi Tsukuba (tsukuba.takeshi@sharp.co.jp) | Sharp Corporation |  |  |
| Hahyun Lee (hanilee@etri.re.kr) | ETRI |  |  |

## Additional participants to the email discussion

| Participants | Affiliation |
| --- | --- |
| Kiran Mishra (misrak@sharplabs.com) | Sharp Labs America |
| Kazushi Sato (Kazushi.Sato@jp.sony.com) Cheung Auyeung (Cheung.Auyeung@am.sony.com) Ohij Nakagami (Ohji.Nakagami@jp.sony.com) | Sony |

# Technology under evaluation in the SCE

## JCTVC-N0161: Using decoded pictures from higher layer as references in SHVC (Qualcomm)

This contribution proposes a design to enable using the decoded pictures from higher quality layer as reference for lower layer pictures for SNR scalability in SHVC. The document uses the concepts of key pictures to reduce error drift that is induced in the base layer.

Further, this contribution proposes few encoder only constraints to allow single-loop decoding design for SHVC SNR scalability. Based on the key picture framework, separate encoder only constraints for key and non-key pictures are proposed to avoid the need of motion compensation in the reference layer.

## JCTVC-N0186: Single-loop decoding of SNR scalability for refIdx based SHVC (InterDigital)

This contribution describes a single-loop decoding scheme for SNR scalability of reference index based SHVC. In the current reference index based SHVC, the reconstructed base layer (BL) picture (after up-sampling if necessary) is used for the inter-layer prediction (ILP) of enhancement layer (EL) coding. This implies that multiple motion compensation (MC) operations have to be performed for all dependent layers. In this contribution, a single-loop decoding scheme is achieved by introducing an alternative ILP picture by applying BL motion information on EL temporal reference pictures. In addition, BL residue is added to the alternative ILP picture for quality improvement. As both BL motion information and BL residue could be obtained without the full reconstruction of BL picture, the single-loop decoding requirement is fulfilled by replacing the conventional ILP picture with the alternative ILP picture for EL prediction. Compared to the existing single-loop decoding approaches studied in AHG16, the proposed scheme does not require any low-level changes to single-layer HEVC.

## JCTVC-N0202: Key picture concept and single loop decoding (RWTH)

In this document a performance analysis of the key picture concept in SHVC is presented. The scheme has similarity with MGS as used in SVC. Key-pictures are introduced, that only utilize other key-pictures as reference. For non-key-pictures a prediction from a higher layer reconstruction is performed. The proposal increases the overall performance but also introduces drift when decoding only the base layer.

As a modification of the scheme, a variant is proposed that adds further restrictions to the encoder. Namely these restrictions are the use of constrained intra prediction in the base layer and utilization of the unfiltered BL reconstruction for inter layer prediction. These restrictions enable single loop decoding for the enhancement layer.

# Anchors

## Multi-loop anchors

The multi-loop anchors were generated by InterDigital and cross-checked by Qualcomm.

- Anchor1: SHM3.0.1 according to SHM CTC JCTVC-N1009
Note: Prediction structure includes reference across hierarchy layers and does not allow for temporal scalability

- Anchor2: SHM3.0.1 CTC with modified configuration to enable temporal scalability (according to JCTVC-L0322)

  - Key-pictures period of 8 pictures for random access

  - Key-picture period of 4 pictures for low-delay

## Single-loop anchor

The single-loop anchor using of IRAP-only single-loop scalable coding based on max_tid_il_ref_pics_plus1 was provided to the SCE participants by Nokia and crosschecked by Qualcomm.

# Evaluation of simulation results

## Rate-distortion assessment

### Comparison to SHM3.0.1 CTC Anchor1

#### Single-loop decoding

Comparison of Random Access relative to CTC SHM 3.0.1 Anchor1

| RA Single Loop | BD rate EL [%] |  |  | BD rate BL only [%] |  |  | BD SNR BL only [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | Y | U | V | Y | U | V | Y | U | V |
| JCTVC-O145 no EL constraint | 1,2% | -5,9% | -7,5% | 6,7% | 4,4% | 4,3% | -0,20 | -0,07 | -0,08 |
| JCTVC-O0162 no EL constraint | 2,6% | -6,6% | -8,9% | - | - | - | - | - | - |
| JCTVC-O145 EL constraint | 2,0% | -5,7% | -7,6% | 9,1% | 4,8% | 4,8% | -0,28 | -0,07 | -0,09 |
| JCTVC-O0162 EL constraint | 3,4% | -5,9% | -8,2% | - | - | - | - | - | - |

Comparison of Low-Delay B relative to CTC SHM 3.0.1 Anchor1

| LDB Single Loop | BD rate EL [%] |  |  | BD rate BL only [%] |  |  | BD SNR BL only [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | Y | U | V | Y | U | V | Y | U | V |
| JCTVC-O145 no EL constraint | 1,3% | -4,1% | -5,6% | 15,5% | 6,1% | 4,9% | -0,47 | -0,09 | -0,10 |
| JCTVC-O0162 no EL constraint | 2,9% | -5,3% | -7,6% | - | - | - | - | - | - |
| JCTVC-O145 EL constraint | 2,5% | -3,6% | -5,5% | 15,5% | 6,1% | 5,0% | -0,47 | -0,09 | -0,10 |
| JCTVC-O0162 EL constraint | 4,9% | -3,1% | -5,5% | - | - | - | - | - | - |

#### Key picture concept (Multi-loop)

Comparison of Random Access relative to CTC SHM 3.0.1 Anchor1

| RA Multi Loop | BD rate EL [%] |  |  | BD rate BL only [%] |  |  | BD SNR BL only [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | Y | U | V | Y | U | V | Y | U | V |
| JCTVC-O0227 | -2,9% | -8,3% | -9,4% | 4.1% | 2.2% | 2.1% | -0.12 | -0.03 | -0.04 |

### Comparison to SHM3.0.1 Anchor2 with temporal scalability

#### Single-loop decoding

Comparison of Random Access relative to CTC SHM 3.0.1 Anchor2

| RA Single Loop TS | BD rate EL [%] |  |  | BD rate BL only [%] |  |  | BD SNR BL only [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | Y | U | V | Y | U | V | Y | U | V |
| JCTVC-O145 no EL constraint |  |  |  |  |  |  |  |  |  |
| JCTVC-O0162 no EL constraint | 2.5% | -6.9% | -9.2% | - | - | - | - | - | - |
| JCTVC-O145 EL constraint |  |  |  |  |  |  |  |  |  |
| JCTVC-O0162 EL constraint | 3.4% | -6.1% | -8.5% | - | - | - | - | - | - |

Comparison of Low-Delay B relative to CTC SHM 3.0.1 Anchor2

| LDB Single Loop TS | BD rate EL [%] |  |  | BD rate BL only [%] |  |  | BD SNR BL only [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | Y | U | V | Y | U | V | Y | U | V |
| JCTVC-O145 no EL constraint |  |  |  |  |  |  |  |  |  |
| JCTVC-O0162 no EL constraint | 3.6% | -6.1% | -9.1% | - | - | - | - | - | - |
| JCTVC-O145 EL constraint |  |  |  |  |  |  |  |  |  |
| JCTVC-O0162 EL constraint | 4.6% | -4.8% | -7.9% | - | - | - | - | - | - |

#### Key picture concept (Multi-loop)

Comparison of Random Access relative to CTC SHM 3.0.1 Anchor2

| RA Multi Loop TS | BD rate EL [%] |  |  | BD rate BL only [%] |  |  | BD SNR BL only [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | Y | U | V | Y | U | V | Y | U | V |
| JCTVC-O0227 | -3,1% | -8,4% | -9,5% | 3,9% | 2,1% | 2,0% | -0,12 | -0,03 | -0,04 |

Comparison of Low-Delay relative to CTC SHM 3.0.1 Anchor2

| Multi Loop TS | BD rate EL [%] |  |  | BD rate BL only [%] |  |  | BD SNR BL only [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | Y | U | V | Y | U | V | Y | U | V |
| JCTVC-O0227 LDB | -3,3% | -6,8% | -7,4% | 3,3% | 1,1% | 1,0% | -0,10 | -0,02 | -0,02 |
| JCTVC-O0227 LDP | -3,5% | -7,1% | -7,7% | 3,4% | 0,6% | 0,1% | -0,10 | -0,01 | -0,01 |

### Comparison to SHM3.0.1 anchor with IRAP-only single-loop scalable coding

Single-loop decoding is enabled in SHVC via the max_tid_il_ref_pics_plus1 syntax element. The results below have been provided by Nokia and crosschecked by Qualcomm.

| RA Single Loop (max TID) | BD rate EL [%] |  |  |
| --- | --- | --- | --- |
| Test case (vs. SHM 3.0.1) | Y | U | V |
| Temporal Scalabilty | 0,2% | 0,0% | 0,0% |
| MAX TID - ILP every 2nd picture | 1,3% | 0,3% | 0,1% |
| MAX TID - ILP every 4th picture | 4,0% | 1,1% | 0,3% |
| MAX TID - ILP every 8th picture | 6,8% | 1,0% | -0,5% |
| MAX TID - ILP every IDR picture | 16,3% | 5,8% | 3,3% |

## Subjective assessment

Informal subjective assessment of visual quality of BL-only reconstruction to be conducted if different from the single loop BL reconstruction, reporting to be included in the CE responses. The method of subjective evaluation shall be reported by participants.

## Complexity assessment

### Memory bandwidth

#### Comparison to SHM3.0.1 CTC Anchor 1

##### Single-loop decoding

Comparison of Random Access relative to CTC SHM 3.0.1 Anchor1

| RA Single Loop |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test case | Pure | DDR2 | DDR3 | Mults |
| JCTVC-O145 no EL constraint | 59% | 60% | 62% | 68% |
| JCTVC-O0162 no EL constraint | 63% | 67% | 67% | 61% |
| JCTVC-O145 EL constraint | 57% | 59% | 60% | 65% |
| JCTVC-O0162 EL constraint | 60% | 63% | 64% | 57% |

Comparison of Low-Delay B relative to CTC SHM 3.0.1 Anchor1

| LDB Single Loop |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test case | Pure | DDR2 | DDR3 | Mults |
| JCTVC-O145 no EL constraint | 63% | 63% | 63% | 76% |
| JCTVC-O0162 no EL constraint | 67% | 70% | 70% | 68% |
| JCTVC-O145 EL constraint | 61% | 62% | 62% | 73% |
| JCTVC-O0162 EL constraint | 62% | 64% | 65% | 61% |

##### Key picture concept: (Multi-Loop)

Comparison of Random Access relative to CTC SHM 3.0.1 Anchor1

| RA Multi Loop |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test case | Pure | DDR2 | DDR3 | Mults |
| JCTVC-O0227 | 94% | 94% | 95% | 81% |

Comparison of Low-Delay B relative to CTC SHM 3.0.1 Anchor1

| Multi Loop |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test case | Pure | DDR2 | DDR3 | Mults |
| JCTVC-O0227 LDB | 97% | 97% | 98% | 81% |
| JCTVC-O0227 LDP | 94% | 94% | 95% | 82% |

#### Comparison to SHM3.0.1 Anchor2 with temporal scalability

##### Single-loop decoding

Comparison of Random Access relative to CTC SHM 3.0.1 Anchor2

| RA Single Loop |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test case | Pure | DDR2 | DDR3 | Mults |
| JCTVC-O145 no EL constraint |  |  |  |  |
| JCTVC-O0162 no EL constraint | 63% | 67% | 67% | 61% |
| JCTVC-O145 EL constraint |  |  |  |  |
| JCTVC-O0162 EL constraint | 60% | 63% | 64% | 58% |

Comparison of Low-Delay B relative to CTC SHM 3.0.1 Anchor2

| LDB Single Loop |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test case | Pure | DDR2 | DDR3 | Mults |
| JCTVC-O145 no EL constraint |  |  |  |  |
| JCTVC-O0162 no EL constraint | 67% | 71% | 71% | 69% |
| JCTVC-O145 EL constraint |  |  |  |  |
| JCTVC-O0162 EL constraint | 63% | 66% | 67% | 64% |

##### Key picture concept: (Multi-loop)

Comparison of Random Access relative to CTC SHM 3.0.1 Anchor2

| RA Multi Loop |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test case | Pure | DDR2 | DDR3 | Mults |
| JCTVC-O0227 | 94% | 94% | 94% | 81% |

Comparison of Low-Delay B relative to CTC SHM 3.0.1 Anchor2

| Multi Loop |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test case | Pure | DDR2 | DDR3 | Mults |
| JCTVC-O0227 LDB | 95% | 95% | 95% | 82% |
| JCTVC-O0227 LDP | 98% | 97% | 97% | 84% |

### Pixel-level processing

#### Comparison to SHM3.0.1 CTC Anchor 1

##### Single-loop decoding

Comparison of Random Access relative to CTC SHM 3.0.1 Anchor1

| RA Single Loop |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | Pix Inter | Pix Intra | Pix MV Full Sum | Pix MV Half | Pix MV Quarter | Pix MV Full BL | Pix Inv Transf Y | Pix Deblocking Y | Pix SAO Y |
| JCTVC-O145 no EL constraint | 82% | 85% | 51% | 64% | 66% | 5% | 98% | 66% | 102% |
| JCTVC-O0162 no EL constraint | 71% | 85% | 44% | 51% | 52% | 20% | 94% | 57% | 88% |
| JCTVC-O145 EL constraint | 79% | 86% | 48% | 61% | 62% | 5% | 99% | 64% | 104% |
| JCTVC-O0162 EL constraint | 67% | 87% | 40% | 46% | 46% | 20% | 93% | 55% | 90% |

Comparison of Low-Delay B relative to CTC SHM 3.0.1 Anchor1

| LDB Single Loop |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | Pix Inter | Pix Intra | Pix MV Full Sum | Pix MV Half | Pix MV Quarter | Pix MV Full BL | Pix Inv Transf Y | Pix Deblocking Y | Pix SAO Y |
| JCTVC-O145 no EL constraint | 85% | 78% | 56% | 79% | 75% | 2% | 95% | 67% | 99% |
| JCTVC-O0162 no EL constraint | 71% | 62% | 50% | 62% | 62% | 3% | 90% | 50% | 81% |
| JCTVC-O145 EL constraint | 84% | 82% | 54% | 74% | 71% | 2% | 95% | 67% | 103% |
| JCTVC-O0162 EL constraint | 66% | 69% | 43% | 51% | 51% | 3% | 85% | 50% | 83% |