| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0033 |
| --- | --- |

| Title: | SCE3: Summary Report of SHVC Core Experiment on Inter-layer Filtering |  |  |
| --- | --- | --- | --- |
| Status: | CE report input to JCT-VC |  |  |
| Purpose: | CE Summary |  |  |
| Author(s) or Contact(s): | Jianle Chen Elena Alshina Jie Dong Maxim Sychev | Email: | cjianle@qti.qualcomm.com elena_a.alshina@samsung.com jie.dong@interdigital.com Sychev.Maxim@huawei.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document summarizes the activities and test results performed in SCE3 on inter-layer filtering. Two tools have been evaluated on the test conditions defined in document JCTVC-N1103.

# List of test methods

Two tools listed in the table below were evaluated in this CE.

Table 1 summary of SCE3 documents

| Test | Methods | Proposal documents | Cross-checking documents |
| --- | --- | --- | --- |
| 3.1 | Region based cross-color inter-layer filter | JCTVC-O0078 (Qualcomm, Samsung, Interdigital) | JCTVC-O0151 (Huawei) |
| 3.2 | Inter-layer prediction modes based on base layer sharpness filter | JCTVC-O0163 (Huawei) | JCTVC-O0282 (Samsung) |

# Summary of test methods and results

## Region based cross-color inter-layer filter (JCTVC-O0078)

In this test, the chroma component of inter layer reference picture is enhanced by adding an offset which is derived by applying a high pass filter on the base layer luma component. The main features of this method are as follows:

- Filter shape: 8 tap 2D non-separable filter,

- Region-based: each chroma color plane is divided up to 16 equal-size rectangle region,

- Input of high pass filter is base layer Luma signal before up-sampling so that chroma and luma can be re-sampled independently,

- Picture level on/off, filter parameters are signaled at picture level,

- BD rate saving: best gain in 2.0x spatial ratio:

  - Spatial scalability: −0.8%(Y)/ −11.7% (U) /−21.7%(V)

  - SNR scalability: −0.5%(Y)/ −10.7% (U) / −19.8%(V)

- Complexity measurement:

  - Memory access compare to anchor is 100% for spatial and 101% for SNR scalability,

  - Worst case memory access is not changed.

## Inter-layer prediction modes based on base layer sharpness filter (JCTVC-O0163)

In this test, a method of sharpening filter is tested. The main features of this method are as follows:

- Sharpening filter process is a following step of um-sampling filter process,

- Two interlayer reference pictures are inserted reference picture list (current SHVC only insert the first one):

  - Normal up-sampled base layer picture,

  - Up-sampled base layer picture with additional sharpen filter,

- The sharpening filter process consists of following ordered steps:

  - Vertical gradient map calculation,

  - Horizontal gradient map calculation,

  - Gradient vector magnitude is estimated,

()

  - Blurring filter  are applied in horizontal and vertical directions to smooth vector gradient map,

  - Displacement vector evaluation,

  - Bi-linear interpolation in fractional position given by displacement vector

  - All the above steps are applied cascaded, i.e. output of one step is the input for the next step,

    - According the cross-check report, the worst case memory access is much higher than that of the current SHVC,

- BD rate saving:

  - Spatial scalability: −1.0%(Y)/ −2.3% (U) /−2.3%(V),

  - SNR scalability: −0.5%(Y)/ −1.6% (U) / −1.4%(V).

# List of non-CE proposals

Non-SCE3 proposals are listed in the table below.

Table 2 list of non-SCE3 proposals

| Proposal documents | Proposal Title | Proponents |
| --- | --- | --- |
| JCTVC-O0189 | non-SCE3: Combined inter-layer prediction: sharpness and region-based cross-color filters | M. Sychev, V. Anisimovskiy (Huawei) |
| JCTVC-O0191 | non-SCE3: Reduced complexity for inter-layer sharpness prediction mode | M. Sychev, V. Anisimovskiy, S. Ikonin (Huawei) |