| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0034 |
| --- | --- |

| Title: | SCE4: Summary Report of Colour Gamut and Bit Depth Scalability |  |  |
| --- | --- | --- | --- |
| Status: | CE report input to JCT-VC |  |  |
| Purpose: | CE Summary |  |  |
| Author(s) or Contact(s): | Andrew Segall Philippe Bordes, C.Auyeung, X. Li, Elena Alshina, A. Duenas | Emails | asegall@sharplabs.com, philippe.bordes@technicolor.com, Cheung.Auyeung@am.sony.com, lxiang@qti.qualcomm.com, elena_a.alshina@samsung.com, alberto@ngcodec.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document summarizes the activities and test results performed in SCE4 and Non-SCE4 on Colour Gamut and Bit Depth Scalability. The evaluated tools and the test conditions are defined in document JCTVC-N1104.

# Introduction

The complete description of the SCE4 is JCTVC-N1104 [1].

The test material has been created from the content provided in JCTVC-N0163 [2] and provided by the Ad hoc Group on Color Gamut Scalability.  The Color graded sequences in DCI-P3 have been expressed in BT.2020 gamut, 3840x2160p. The BT.709 sequences have been provided in 1920x1080p (same as in JCTVC-N0163).

The sequences duration have been reduced to 300 frames for 60Hz sequences and 250 frames for 50Hz sequences, representing 5s duration (500 frames originally). However, for the BirthdayFlash sequence, two sequences of 300 frames have been generated: BirthdayFlashPart1 and BirthdayFlashPart2.

The BT.2020, 3840x2160p sequences are 10-bit, the BT.709, 1920x1080p have been provided in 8-bit and 10-bit.

The reference software (bit depth scalability) SHM3.0.1-bitdepth for the CE has been provided by Samsung and InterDigital within the Ad hoc group on Color Gamut Scalability.

# Bit depth scalability

The reference software implementing bit-depth scalability on top of SHM3.0.1 and to be used for the SCE4 anchors.

Table 1: summary of SCE4 bit-depth scalability documents.

| Test num. | Method | Proposal documents | Cross-checking doc. |
| --- | --- | --- | --- |
| (1) 5.1-test1 | Combined up-sampling and bit-depth increase | JCTVC-O0194 (Nokia, Samsung) | JCTVC-O0210 (Sony) |

# Colour gamut and bit depth scalability

The test conditions are 2x scalability with the following contents:

- Test 1: Enhancement layer: 3840x2160 resolution, 10-bit, BT.2020 gamut / Baselayer layer: 1920x1080p, BT.709, 8-bit.

- Test 2: Enhancement layer: 3840x2160 resolution, 10-bit, BT.2020 gamut / Baselayer layer: 1920x1080p, BT.709, 10-bit.

The common SHVC test conditions (QPs) have been used for AI and RA configurations, 2x scalability.

Table 2: summary of SCE4 and non-SCE4 colour gamut and bit-depth scalability documents.

| SCE4 test number |  | Method | Proposal documents | Cross-checking documents |
| --- | --- | --- | --- | --- |
| 1 | 5.1-test1 | Weighted Prediction | JCTVC-O0194 (Nokia, Samsung) | JCTVC-O0210 (Sony) |
| 2 | 5.2-test1 | Gain-Offset model | JCTVC-O0201 (Sharp) | JCTVC-O0241 (Qualcomm) |
| 3 | 5.2-test2 |  |  |  |
| 4 | 5.3-test1 | 3D LUT | JCTVC-O0159 (Technicolor) | JCTVC-O0130 (ETRI) |
| 5 | 5.3-test2 |  |  | JCTVC-O0261 (Sharp) |
| 6 | 5.4-test1-model1 | Piecewise-linear | JCTVC-O0196 (Sony) | JCTVC-O0242 (Qualcomm) |
| 7 | 5.4-test1-model2 | Shift-offset | Withdraw | Samsung |
| 8 | 5.4-test2-model1 | Piecewise-linear | JCTVC-O0196 (Sony) | JCTVC-O0242 (Qualcomm) |

# Summary of Results

# Test 1: Base Layer 8-bit, Enhancement Layer 10-bit

|  | SCE4 | AI HEVC 2x 8-bit base |  |  |  |  | RA HEVC 2x 8-bit base |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate |  |  | Time |  | BD-rate |  |  | Time |  |
|  | Technology | Y | U | V | Enc | Dec | Y | U | V | Enc | Dec |
| 1 | WP (adp.) | -6.6% | -5.5% | -9.5% | 97.5% | 97.1% | -3.4% | -1.0% | -4.4% | 145.9% | 111.6% |
|  | WP (adp+fixed) | -6.6% | -5.5% | -9.5% |  |  | -3.7% | -1.3% | -4.7% |  |  |
| 2 | GO | -4.6% | -3.3% | -7.3% |  |  | -2.9% | -0.2% | -3.7% |  |  |
| 4 | LUT | -12.3% | -9.9% | -16.0% | 98.0% | 90.1% | -8.2% | -3.0% | -9.9% | 98.9% | 91.8% |
| 6 | Piecewise-linear | -4.4% | -3.0% | -7.9% |  |  | -3.3% | -0.1% | -4.6% |  |  |

# Test 2: Base Layer 10-bit, Enhancement Layer 10-bit

|  | SCE4 | AI HEVC 2x 10-bit base |  |  |  |  | RA HEVC 2x 10-bit base |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate |  |  | Time |  | BD-rate |  |  | Time |  |
|  | Technology | Y | U | V | Enc | Dec | Y | U | V | Enc | Dec |
| 3 | GO | -4.0% | -2.3% | -6.0% |  |  | -2.5% | -0.1% | -3.7% |  |  |
| 5 | LUT | -12.2% | -9.6% | -14.9% | 97.9% | 95.8% | -8.5% | -3.4% | -10.1% | 98.7% | 101.9% |
| 8 | Piecewise-linear | -4.1% | -2.8% | -6.8% |  |  | -3.1% | -0.6% | -4.7% |  |  |

# Appendix: Non-SCE4

| Non-SCE4 |  | Method | Proposal documents | Cross-checking documents |
| --- | --- | --- | --- | --- |
| 9 | test1 (lut 8bits) | Combined bit-depth and color gamut conversion with 3D LUT | JCTVC-O0161 (InterDigital) | JCTVC-O0160 (Technicolor) |
| 10 | test1 (lut 10bits) |  |  |  |
| 11 | test2 (lut 10bits) |  |  |  |
| 12 | test1 | Picture and region adaptive Gain-Offset | JCTVC-O0195 (Sony) | JCTVC-O0298 (Sharp) |
| 13 | test2 |  |  |  |
| 14 | test1 | Weighted Prediction | JCTVC-O0180 (Qualcomm) | JCTVC-O0284 (Samsung) |
| 15 | test2 |  |  |  |

## Test 1: Base Layer 8-bit, Enhancement Layer 10-bit

In the case the Enc/Dec times are not available or those given by the proponents are not reliable, the cross-checker times have been given.

|  | Non-SCE4 | AI HEVC 2x 8-bit base |  |  |  |  | RA HEVC 2x 8-bit base |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate |  |  | Time |  | BD-rate |  |  | Time |  |
|  | Technology | Y | U | V | Enc | Dec | Y | U | V | Enc | Dec |
| 9 | LUT (8-bit) | -13.2% | -13.2% | -18.9% | 97.1%* | 97.6% | -9.1% | -7.3% | -13.2% | 97.6%* | 106.5% |
| 10 | LUT (10-bit) | -15.3% | -15.7% | -22.9% | 96.7%* | 94.6% | -10.0% | -8.7% | -16.6% | 115.7%* | 105.6% |
| 12 | GO Adaptive | -6.9% | -5.6% | -10.1% |  |  | -3.0% | -0.4% | -3.7% |  |  |
| 14 | WP | -6.0% | -4.7% | -9.0% | 100.5% | 105.6% | -3.5% | -1.1% | -4.9% | 144.5% | 101.6% |

*cross-checker times

## Test 2: Base Layer 10-bit, Enhancement Layer 10-bit

|  | Non-SCE4 | AI HEVC 2x 10-bit base |  |  |  |  | RA HEVC 2x 10-bit base |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate |  |  | Time |  | BD-rate |  |  | Time |  |
|  | Technology | Y | U | V | Enc | Dec | Y | U | V | Enc | Dec |
| 11 | LUT (10-bit) | -15.4% | -15.5% | -22.2% | 97.0%* | 95.6% | -10.5% | -9.1% | -17.2% | 96.7%* | 105.6% |
| 13 | GO Adaptive | -7.7% | -6.2% | -9.8% |  |  | -3.3% | -0.9% | -4.2% |  |  |
| 15 | WP | -6.5% | -5.2% | -8.5% | 100.1% | 102.9% | -4.0% | -1.6% | -5.3% | 147.8% | 111.0% |

*cross-checker times

# References

- A.Segall, P.Bordes, C.Auyeung, X.Li, E.Alshina, A.Duenas, “Description of Core Experiment SCE4: Color Gamut and Bit-Depth Scalability,” JCTVC-N1104, July 2013, Vienna, AT.

- P.Andrivon, P.Bordes, “AHG14: Wide Color Gamut Test Material Creation”, JCTVC-N0163, July 2013, Vienna, AT.