| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0035 |
| --- | --- |

| Title: | RCE1: Summary Report of HEVC Range Extensions Core Experiment 1 on Inter-Component Decorrelation Methods |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | CE Report |  |  |
| Author(s) or Contact(s): | Woo-Shik Kim Tung Nguyen | Email: | wooshikk@qti.qualcomm.com tung.nguyen@hhi.fraunhofer.de |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

HEVC Range Extensions (RExt) Core Experiment 1 (RCE1) was formed to study proposals on inter-component decorrelation methods as described in RCE1 description, JCTVC-N1121, where four Experiments are included to be tested. In this document, the results of four Experiements are summarized and their performance is studied.

# Introduction

High Efficiency Video Coding (HEVC) version 1 [1] has been developed mainly targeting video in 4:2:0 chroma format, while its Range Extension (RExt) work considers video in 4:2:2 and 4:4:4 formats as well [2]. In case of 4:4:4, test sequences in both YCbCr and RGB color spaces are included in the common test conditions [3].

The correlation among different color components can be reduced by an adaptive inter-color-component residual prediction approach [4] presented at the 14th JCT-VC meeting resulting in increased compression performance. In the RExt Core Experiment 1 (RCE1) description [5], four Experiments are included to be tested. In this document, the results of four Experiements are summarized and their performance is studied.

# Description of RCE1 Experiments

Four Experiments are included in RCE1 [5] to study the performance of the inter-component decorrelation methods as described below.

## RCE1 Experiment 1

An adaptive inter color component residual prediction scheme is proposed in JCTVC-N0266 [1], where a prediction is performed between luma residual signal and chroma residual signal. The predictor is generated using simple operations like one multiplication and one bit-shift operation. Specifically, the chroma residual signal is predicted using the luma residual signal at the encoder side as:

ΔrC(x,y)=rC(x,y)−α×rL'(x,y)>>3					 (1)

and it is compensated at the decoder side as:

rC'(x,y)=ΔrC'(x,y)+α×rL'(x,y)>>3	 				(2)

where rC denotes the chroma residual sample at a position ,
rL' denotes the reconstructed residual sample of the luma component, 
ΔrC denotes the predicted signal using inter-color prediction,

ΔrC'denotes the reconstructed signal after coding and decoding ΔrC, and

rC' denotes the reconstructed chroma residual signal.
α is allowed to be chosen from −8, −4, −2, −1, 0, 1, 2, 4, 8, inclusively.

The scaling parameter α is calculated at the encoder side and signalled for each TU if luma cbf is not zero. The decoder does not need to re-calculate α. In case of intra coding, only those with DM chroma mode are allowed to use this prediction.

## RCE1 Experiment 2

In this experiment, at the encoder side, when calculating the α, the luma residual signal before scaling and transformation (i.e., the difference between original luma sample and its prediction) is used rather than the reconstructed luma residual signal as in Experiment 1. Specifically, at the encoder side instead of (1) below equation is used.

ΔrC(x,y)=rC(x,y)−α×rL(x,y)>>3					 (3)

This can reduce the implementation complexity by facilitating pipelining and parallel processing as the prediction can be performed without waiting until reconstruction of luma residual signal. This requires only encoder side modification, and the decoding part is the same as Experiment 1.

## RCE1 Experiment 3

For inter prediction, α is calculated for each PU rather than for each TU, and signaled in the first TU in each PU whose corresponding luma cbf is not equal to 0.

## RCE1 Experiment 4

This experiment is a combination of Experiment 2 and Experiment 3.

# Experimental Results

The experiments employ the test set and the test condition as described in the RExt common test conditions [3] for YCbCr 4:4:4 and RGB test sequences. The HEVC RExt reference software version 4.1 was used as the anchor for the integration of the tested methods as well as for generation of the anchor results. Full results can be found in [6].

## Experiment 1

In this experiment, the performance of JCTVC-N0266 is evaluated using HM 12.0 RExt-4.1. Table 1 is the summary results of Experiment 1.

Table 1. Performance of RCE1 Experiment 1 vs. HEVC RExt4.1 anchor.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | -18.1% | -16.6% | -17.3% | -14.2% | -13.5% | -13.8% | -10.8% | -10.4% | -10.5% |
| YCbCr 4:4:4 | -1.4% | -6.5% | -6.7% | -1.7% | -4.4% | -6.1% | -1.9% | -3.2% | -4.4% |
| Overall | -10.3% | -11.9% | -12.4% | -8.4% | -9.3% | -10.2% | -6.7% | -7.0% | -7.6% |
|  | -10.3% | -12.0% | -12.5% | -8.4% | -9.4% | -10.4% | -6.6% | -7.1% | -7.3% |
| Enc Time[%] | 112% |  |  | 114% |  |  | 115% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 101% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -14.5% | -11.9% | -13.7% | -11.4% | -8.7% | -10.7% |  |  |  |
| YCbCr 4:4:4 | -0.5% | -8.5% | -7.3% | -0.9% | -6.2% | -7.6% |  |  |  |
| Overall | -8.0% | -10.3% | -10.7% | -6.5% | -7.6% | -9.2% |  |  |  |
|  | -7.9% | -11.0% | -10.5% | -6.7% | -9.3% | -9.6% |  |  |  |
| Enc Time[%] | 113% |  |  | 118% |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -13.6% | -9.2% | -10.8% | -10.4% | -7.0% | -8.5% |  |  |  |
| YCbCr 4:4:4 | -0.2% | -6.1% | -5.8% | -0.7% | -4.2% | -6.4% |  |  |  |
| Overall | -7.3% | -7.8% | -8.5% | -5.9% | -5.7% | -7.5% |  |  |  |
|  | -7.2% | -6.7% | -8.0% | -6.1% | -6.5% | -7.5% |  |  |  |
| Enc Time[%] | 106% |  |  | 108% |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |  |  |  |

## Experiment 2

Table 2 is the summary results of Experiment 2.

Table 2. Performance of RCE1 Experiment 2 vs. HEVC RExt4.1 anchor.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | -18.1% | -16.6% | -17.3% | -14.2% | -13.5% | -13.8% | -10.8% | -10.4% | -10.5% |
| YCbCr 4:4:4 | -1.4% | -6.6% | -6.9% | -1.8% | -4.4% | -6.2% | -1.9% | -3.2% | -4.4% |
| Overall | -10.3% | -11.9% | -12.4% | -8.4% | -9.3% | -10.3% | -6.7% | -7.1% | -7.7% |
|  | -10.3% | -12.0% | -12.5% | -8.4% | -9.4% | -10.4% | -6.7% | -7.2% | -7.3% |
| Enc Time[%] | 116% |  |  | 118% |  |  | 119% |  |  |
| Dec Time[%] | 104% |  |  | 103% |  |  | 103% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -14.1% | -11.7% | -13.5% | -11.1% | -8.6% | -10.5% |  |  |  |
| YCbCr 4:4:4 | -0.5% | -8.2% | -7.2% | -0.9% | -6.0% | -7.5% |  |  |  |
| Overall | -7.8% | -10.1% | -10.6% | -6.3% | -7.4% | -9.1% |  |  |  |
|  | -7.7% | -10.7% | -10.4% | -6.5% | -9.5% | -9.4% |  |  |  |
| Enc Time[%] | 111% |  |  | 114% |  |  |  |  |  |
| Dec Time[%] | 104% |  |  | 103% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -12.6% | -8.7% | -10.1% | -9.8% | -6.5% | -7.9% |  |  |  |
| YCbCr 4:4:4 | -0.2% | -5.8% | -5.4% | -0.6% | -3.9% | -6.0% |  |  |  |
| Overall | -6.8% | -7.3% | -7.9% | -5.5% | -5.3% | -7.0% |  |  |  |
|  | -6.7% | -6.5% | -7.6% | -5.6% | -6.0% | -7.0% |  |  |  |
| Enc Time[%] | 110% |  |  | 112% |  |  |  |  |  |
| Dec Time[%] | 105% |  |  | 105% |  |  |  |  |  |

## Experiment 3

Table 3 is the summary results of Experiment 3.

Table 3. Performance of RCE1 Experiment 3 vs. HEVC RExt4.1 anchor.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | -18.1% | -16.6% | -17.3% | -14.2% | -13.5% | -13.8% | -10.8% | -10.4% | -10.5% |
| YCbCr 4:4:4 | -1.4% | -6.5% | -6.7% | -1.7% | -4.4% | -6.1% | -1.9% | -3.2% | -4.4% |
| Overall | -10.3% | -11.9% | -12.4% | -8.4% | -9.3% | -10.2% | -6.7% | -7.0% | -7.6% |
|  | -10.3% | -12.0% | -12.5% | -8.4% | -9.4% | -10.4% | -6.6% | -7.1% | -7.3% |
| Enc Time[%] | 127% |  |  | 130% |  |  | 130% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 99% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -14.2% | -11.8% | -13.6% | -11.2% | -8.7% | -10.5% |  |  |  |
| YCbCr 4:4:4 | -0.5% | -8.3% | -7.2% | -0.8% | -6.1% | -7.3% |  |  |  |
| Overall | -7.8% | -10.2% | -10.6% | -6.4% | -7.5% | -9.0% |  |  |  |
|  | -7.8% | -11.0% | -10.4% | -6.5% | -9.4% | -9.3% |  |  |  |
| Enc Time[%] | 154% |  |  | 158% |  |  |  |  |  |
| Dec Time[%] | 104% |  |  | 102% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -13.0% | -9.1% | -10.5% | -10.0% | -6.8% | -8.1% |  |  |  |
| YCbCr 4:4:4 | -0.1% | -6.2% | -5.6% | -0.5% | -4.3% | -5.9% |  |  |  |
| Overall | -7.0% | -7.7% | -8.2% | -5.6% | -5.6% | -7.1% |  |  |  |
|  | -6.9% | -7.1% | -7.8% | -5.8% | -6.3% | -7.0% |  |  |  |
| Enc Time[%] | 140% |  |  | 145% |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |  |  |  |

## Experiment 4

Table 4 is the summary results of Experiment 4.

Table 4. Performance of RCE1 Experiment 4 vs. HEVC RExt4.1 anchor.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | -18.1% | -16.6% | -17.3% | -14.2% | -13.5% | -13.8% | -10.8% | -10.4% | -10.5% |
| YCbCr 4:4:4 | -1.4% | -6.6% | -6.9% | -1.8% | -4.4% | -6.2% | -1.9% | -3.2% | -4.4% |
| Overall | -10.3% | -11.9% | -12.4% | -8.4% | -9.3% | -10.3% | -6.7% | -7.1% | -7.7% |
|  | -10.3% | -12.0% | -12.5% | -8.4% | -9.4% | -10.4% | -6.7% | -7.1% | -7.3% |
| Enc Time[%] | 131% |  |  | 132% |  |  | 132% |  |  |
| Dec Time[%] | 103% |  |  | 103% |  |  | 103% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -13.9% | -11.8% | -13.4% | -11.0% | -8.6% | -10.4% |  |  |  |
| YCbCr 4:4:4 | -0.5% | -8.3% | -7.1% | -0.9% | -6.1% | -7.1% |  |  |  |
| Overall | -7.7% | -10.1% | -10.5% | -6.3% | -7.4% | -8.9% |  |  |  |
|  | -7.6% | -10.8% | -10.3% | -6.3% | -9.2% | -9.2% |  |  |  |
| Enc Time[%] | 146% |  |  | 150% |  |  |  |  |  |
| Dec Time[%] | 110% |  |  | 109% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -12.3% | -8.8% | -10.0% | -9.6% | -6.5% | -7.8% |  |  |  |
| YCbCr 4:4:4 | -0.1% | -5.9% | -5.3% | -0.4% | -4.2% | -5.6% |  |  |  |
| Overall | -6.6% | -7.4% | -7.8% | -5.3% | -5.4% | -6.8% |  |  |  |
|  | -6.5% | -6.7% | -7.5% | -5.4% | -6.0% | -6.7% |  |  |  |
| Enc Time[%] | 137% |  |  | 140% |  |  |  |  |  |
| Dec Time[%] | 110% |  |  | 107% |  |  |  |  |  |

## Performance evaluation of Experiment 2 on screen contents

Additional experiments are performed to evaluate the performance of Experiment 2 on screen contents coding. AhG8 test condition is followed along with the recommended test sequences (only 4:4:4 sequences are tested). Table 5 is the summary results.

Table 5. Performance of RCE1 Experiment 2 vs. HEVC RExt4.1 anchor on screen contents.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U |  | V |  |
| SC RGB 444 | -26.0% | -25.7% | -26.0% | -24.3% | -24.0% | -24.1% | -22.6% | -22.2% |  | -22.3% |  |
| Animation RGB 444 | -25.9% | -25.7% | -24.5% | -25.2% | -24.3% | -23.5% | -22.5% | -21.1% |  | -20.6% |  |
| SC YUV 444 | -4.5% | -8.0% | -7.9% | -4.6% | -6.4% | -7.0% | -4.2% | -5.2% |  | -5.7% |  |
| Animation YUV 444 | -2.6% | -11.5% | -7.2% | -3.1% | -9.3% | -6.7% | -3.3% | -7.0% |  | -5.7% |  |
| RangeExt | -1.7% | -5.6% | -11.1% | -2.0% | -4.4% | -9.2% | -2.2% | -3.7% |  | -6.8% |  |
| SC(444) GBR Optional | -24.2% | -24.5% | -24.8% | -24.8% | -25.0% | -25.7% | -24.9% | -25.1% |  | -25.7% |  |
| SC(444) YUV Optional | -7.2% | -8.3% | -7.3% | -6.5% | -5.8% | -6.4% | -5.5% | -4.5% |  | -5.5% |  |
| Enc Time[%] | 113% |  |  | 114% |  |  | 114% |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 101% |  |  |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |  |  |
| SC RGB 444 | -23.2% | -23.1% | -23.5% | -22.3% | -22.2% | -22.4% |  |  |  |  |  |
| Animation RGB 444 | -20.0% | -19.9% | -18.4% | -20.7% | -19.8% | -18.7% |  |  |  |  |  |
| SC YUV 444 | -3.5% | -7.8% | -7.8% | -3.7% | -6.3% | -7.0% |  |  |  |  |  |
| Animation YUV 444 | -1.0% | -11.1% | -6.3% | -1.5% | -10.3% | -6.6% |  |  |  |  |  |
| RangeExt | -0.8% | -8.5% | -11.2% | -1.0% | -5.9% | -10.7% |  |  |  |  |  |
| SC(444) GBR Optional | -23.4% | -23.5% | -23.7% | -24.4% | -24.5% | -25.0% |  |  |  |  |  |
| SC(444) YUV Optional | -6.0% | -7.9% | -6.4% | -5.7% | -6.0% | -5.8% |  |  |  |  |  |
| Enc Time[%] | 109% |  |  | 110% |  |  |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |  |  |
| SC RGB 444 | -20.9% | -20.3% | -20.8% | -20.5% | -20.0% | -20.4% |  |  |  |  |  |
| Animation RGB 444 | -18.9% | -16.9% | -15.6% | -19.4% | -17.0% | -16.0% |  |  |  |  |  |
| SC YUV 444 | -2.7% | -6.8% | -7.1% | -3.2% | -5.4% | -6.1% |  |  |  |  |  |
| Animation YUV 444 | -0.4% | -9.3% | -4.1% | -1.0% | -8.8% | -4.7% |  |  |  |  |  |
| RangeExt | -0.4% | -5.2% | -7.5% | -0.6% | -3.0% | -7.5% |  |  |  |  |  |
| SC(444) GBR Optional | -21.1% | -21.1% | -21.5% | -23.0% | -22.9% | -23.6% |  |  |  |  |  |
| SC(444) YUV Optional | -8.6% | -10.2% | -9.1% | -7.5% | -7.8% | -7.8% |  |  |  |  |  |
| Enc Time[%] | 107% |  |  | 108% |  |  |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |  |  |  |  |  |

# Comments on Results

## Benefit in RGB coding

RExt common test condition includes RGB sequences, as some application requires RGB coding, so that better color fidelity can be achieved. Theoretically without coding distortion, the distortion from color conversion can be derived. For example, if RGB is converted to YCbCr and converted back to RGB, the forward color transform will introduce 1/12 rounding error due to the uniform rounding quantization error. Then, during the backward transform the rounding operations introduce a2/12, where a is the coefficient of the color conversion matrix.

When the Rec. BT 709 is used, the backward transform is defined as below.

RGB=1.00.01.57481.0−0.1873−0.46811.01.85560.0YCbCr

In this case, for each color component, the overall conversion error becomes as below.

eR=11212+12+02+1.57482,eG=11212+12+0.18732+0.46812,eG=11212+12+1.85562+02.

Table 6 shows the numbers when it is represented in terms of 8 bit and 10 bit PSNRs.

Table 6. Distortion due to YCbCr color conversion.

|  | 8 bit | 10 bit |
| --- | --- | --- |
| R | 52.41 | 64.48 |
| G | 55.39 | 67.46 |
| B | 51.56 | 63.63 |
| average | 53.48 | 65.55 |

This reveals that there is limit in quality that YCbCr coding can achieve. In JCTVC-N0115 [7], experimental results on screen contents can be found, in which the performance of YCbCr coding and RGB coding is compared.

It is noted that the inter-component prediction method can provide good coding efficiency without sacrificing color fidelity unlike YCbCr coding. Below rate-distortion curve is one example, where the performance of RGB coding and YCbCr coding is compared to that of RCE1 Experiment 2. It can be noticed that RCE1 Experiment 2 shows the best coding efficiency performance, while it does not sacrifice color fidelity.

Fig. 1. Rate-distortion curves of RGB, YCbCr coding results and results of RCE1 Experiment 2.

## Interpretation of chroma coding gain in YCbCr 4:4:4 coding

In case of YCbCr 4:4:4 coding, RCE1 results show better coding gain in chroma components than in luma component. As there is no norm to measure the overall performance when there is such bias, it is studied how to interpret the overall performance when there is larger gain in chroma components.

During the course of HEVC development, BD-bitrate has been widely used, which uses the overall bitrate and the PSNR of each color component to calculate BD-bitrate of each color component. In RCE1 results it can be noticed that chroma PSNR is usually larger than the anchor. Therefore, if the chroma components are coded with less quality, it will reduce the overall bitrate, which would results in increment of BD-bitrate of Y component. Note that this is only for the measurement purpose. It is not suggested to sacrifice quality of chroma components, which would harm overall visual quality.

There can be two ways to control chroma bitrate – change of chroma QP and chroma Lagrange multiplier. Change of slice QP can results in large bitrate change. Therefore, in this experiment the chroma Lagrange multiplier is modified. As adjusting these parameters for each sequence and for each color component will require too many simulations, it is tried to apply the same parameter for all the sequences and components and the average performance is measured.

Table 7 shows the results of RCE1 Experiment 2, where there is no change in luma Lagrange multiplier and the chroma Lagrange multiplier is 1.5 times larger than that of luma.

Table 7. Results of RCE1 Experiment 2 with 1.5 times larger chroma Lagrange multiplier.

|  | All Intra HE Main-tier |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| YCbCr 4:4:4 | -5.4% | -0.2% | -1.3% |
|  | Random Access HE Main-tier |  |  |
|  | Y | U | V |
| YCbCr 4:4:4 | -4.4% | -3.8% | -2.3% |
|  | Low delay B HE Main-tier |  |  |
|  | Y | U | V |
| YCbCr 4:4:4 | -6.3% | -0.6% | -0.2% |

These results show that the gain in chroma components can have large impact on overall performance in case of 4:4:4 coding.

## Complexity

The experimental results show similar performance among the Experiments 1 to 4. However, they have different complexity.

In all the methods, the scaling factor is calculated at the encoder side only, and there is no need for such calculation at the decoder side. In case of intra coding, it is applied only when DM mode is used for chroma, which can further reduce the complexity at the encoder side.

It is noted that Experiment 2 can have less implementation complexity. It uses non-reconstructed luma residual signal for prediction, which can facilitate a parallel processing.

Experiment 3 and 4 tested CU based signalling for inter coding, which revealed no performance improvement on Experiment 1. For consistency between intra and inter coding, it is suggested to keep TU based signalling as in Experiment 1 and 2.

## Related Non-RCE1 contributions

There are 2 Non-RCE1 contributions in this meeting, which are listed below.

- A. Khairat, T. Nguyen, M. Siekmann, and D. Marpe, “Non-RCE1: Extended adaptive inter-component prediction,” JCTVC-O0150, Oct. 2013.

- K. Kawamura and S. Naito, “Non-RCE1: Inter colour-component residual coefficients prediction,” JCTVC-O0263, Oct. 2013.

# Conclusion

In this document the description of RCE1 experiments are given with the results. Among four experiments, Experiment 2 shows good coding efficiency improvement with reduced implementation complexity as it can facilitate parallel processing.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, Y.-K. Wang, and T. Wiegand, “High efficiency video coding (HEVC) text specification draft 10,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-L1003, Jan. 2013.

- D. Flynn, J. Sole, and T. Suzuki, “High Efficiency Video Coding (HEVC) Range Extensions text specification: Draft 4,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-N1005, Jul. 2013.

- D. Flynn, K. Sharman, and C. Rosewarne, “Common test conditions and software reference configurations for HEVC range extensions,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-N1006, Jul. 2013.

- W. Pu, W.-S. Kim, J. Chen, K. Rapaka, L. Guo, J. Sole, and M. Karczewicz, “Non-RCE1: Inter color component residual prediction,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-N0266, Jul. 2013.

- W.-S. Kim, and T. Nguyen, “HEVC Range Extensions Core Experiment 1 (RCE1): Inter-component decorrelation methods,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-N1121, Jul. 2013.

- W. Pu, W.-S. Kim, J. Chen, J. Sole, and M. Karczewicz, “RCE1: Descriptions and results for Experiments 1, 2, 3, and 4,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-O0202, Oct. 2013.

- A. Minezawa, S. Sekiguchi, and T. Murakami, “AHG5/AHG8: On RGB to YCbCr conversion for screen contents,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-N0115, Jul. 2013.