| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0036 |
| --- | --- |

| Title: | RCE2: Summary report on HEVC Range Extensions Core Experiment 2 (RCE2) on Rice parameter initialisation and update methods |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chris Rosewarne Joel Sole Karl Sharman S.-H. Kim | Email: | chris.rosewarne@cisra.canon.com.au joels@qti.qualcomm.com karl.sharman@eu.sony.com kimse@sharplabs.com |
| Source: | CE coordinators |  |  |

_____________________________

# Introduction

This is a summary report on HEVC Range Extensions Core Experiment 2 on Rice parameter initialisation and update methods. The core experiment investigated coding techniques for transformed, transform-skip and transform-bypass blocks. The core experiment consisted of subtests that investigated initialization and update of the Rice parameter for level coding. Performance of the proposed methods as well as their combinations was evaluated for lossy and lossless configurations based on the test conditions and sequences described in JCTVC-N1122 and JCTVC-N1123.

# Document List

## Subtest A

### CE contributions

- JCTVC-O0206: “RCE2 Test A1: Simplified update of the coefficient level Rice parameter” (Qualcomm)

  - Cross-check: JCTVC-O0240: “Cross-check of ‘RCE2 Test A1: Simplified update of the coefficient level Rice parameter’ (O0206) by Qualcomm” (Canon)

- JCTVC-O0246: “RCE2: Test A2. Rice parameter signaling for transform-skip blocks” (Sharp Labs)

  - Cross-check: JCTVC-O0250: “Cross-check of ‘RCE2: Test A2. Rice parameter signaling for transform-skip blocks’ (JCTVC-O0246) by Sharp Labs” (Canon)

## Subtest B

### CE contributions

- JCTVC-O0247: “RCE2: Test B1. Rice parameter signaling for transform-skip blocks” (Sharp Labs)

  - Cross-check: JCTVC-O0294: “RCE2: Cross-check of Test B1 JCTVC-O0247 Rice parameter signaling for transform-skip blocks” (Qualcomm)

- JCTVC-O0114: “RCE2: Test B2. Golomb-rice parameter initialization for transform-skip and transquant-bypass modes” (Canon)

  - Cross-check: JCTVC-O0290: “Cross-check of ‘RCE2: Test B2. Golomb-rice parameter initialization for transform-skip and transquant-bypass modes” (Sharp)

## Subtest C

### CE contributions

- JCTVC-O0219: “RCE2: Results of Test C1 on Rice Parameter Initialization” (Qualcomm)

  - Cross-check: JCTVC-O0283: “RCE2: Verification of test C1 on Rice parameter initialization” (Samsung)

- JCTVC-O0065: “RCE2: C2 - Entropy Coding Compression Efficiency for High Bit Depths” (Sony Europe)

  - Cross-check: JCTVC-O0295: “RCE2: Cross-check of Test C2 JCTVC-O0065 Entropy Coding Compression Efficiency for High Bit Depths” (Qualcomm)

## Subtest D

### CE contributions

- JCTVC-O0239: “RCE2: Results of Test D1 on Rice Parameter Initialization” (Qualcomm and Sony Europe)

  - Cross-check: JCTVC-O0283: “RCE2: Verification of test C1 on Rice parameter initialization” (Samsung)

  - Cross-check: JCTVC- O0257: “Cross-check of 'RCE2: Results of Test D1 on Rice Parameter Initialization' (JCTVC-O0239) by Qualcomm and Sony” (Canon)

## Related non-CE contributions

- JCTVC-O0129: “Non-RCE2 and AHG18: Increase in the maximum value of Rice parameter for high bit-depth support” (Samsung)

  - Cross-check: JCTVC-O0312: “Cross-check of ‘Non-RCE2 and AHG18: Increase in the maximum value of Rice parameter for high bit-depth support’ (JCTVC-O0129) by Samsung” (Canon)

- JCTVC-O0327: “Non-RCE2: Rice parameter initialization for higher bit depth coding” (Sharp Labs)

  - Cross-check: JCTVC- Oxxxx

# Tested methods

Note from CE description (JCTVC-N1122): Proponents are requested to document any modifications over their original proposals in order support operation at higher bit depths. Significant changes would require non-CE contributions.

## Rice parameter update methods

Test A.1 (JCTVC-N0101/JCTVC-O0206, Qualcomm)

One condition check in the HEVC cRiceParam update process is removed. In the preferred variant, the cRiceParam update is performed as follows:

- cRiceParam = Min( cLastRiceParam + ( cLastAbsLevel >> ( 2  +  cLastRiceParam ) ), 4 )

The method is applied to all the blocks. The method is also tested with maximum value of cRiceParam equal to 7 and, for high-bit depths, 16. Additional results are also provided for applying the method to only transform-skip and transform-bypass blocks.

Test A.2 (JCTVC-N0281/JCTVC-O0246, Sharp)

This contribution presents two modifications to cRiceParam update process: (1) signaling cRiceParam from the set {0, 2, 4, 6} and (2) removing the existing cRiceParam update process. This method is applied only to transform-skip and transquant bypass blocks. The method is also tested with maximum value of cRiceParam equal to 16 for high-bit depths.

## Rice parameter initialisation methods

Test B.1 (JCTVC-N0268/JCTVC-O0247, Sharp)

The Rice parameter (cRiceParam) is explicitly signalled for each sub-block. The existing cRiceParam update is unchanged. This method is applied only to transform-skip and transquant bypass blocks. Additional results are also provided for applying the method to all blocks including transform blocks. The method is also tested with maximum value of cRiceParam equal to 16 for high-bit depths.

Test B.2 (JCTVC-N0137/JCTVC-O0114, Canon)

The update formula for the Rice parameter is unmodified, which is applied once per instance of coeff_abs_level_remaining (up to 16 instances per sub-block).  This contribution proposes to modify the initialisation of the Rice parameter, which is applied once per sub-block parameter for transform skip and transquant bypass blocks. An initial value of 2 is used when the number of greater-than-one coefficients in the current sub-set is equal to or greater than 8.

## Rice parameter adaptive methods

Test C.1 (JCTVC-N0181/JCTVC-O0219, Qualcomm)

4x4 sub-blocks are divided into different categories “sbType” depends on the transform skip flag and/or whether it is luma block. Statistic statCoeff is maintained for each sbType, and is updated using the the first remaining level coefficient of each sub-block

statCoeff += (uiLevel == statCoeff) ? 0 : ( ( uiLevel < statCoeff ) ? -1 : 1);

statCoeff = min(320, statCoeff);

The initial rice parameter for the next sub-block of the same sbType is calculated as follows

cRiceParam = statCoeff<16? (statCoeff+1)>>2: 4+(statCoeff+40)>>6);

The calculated cRiceParam is further clipped,

cRiceParam = Clip3 (minRicePara, maxRicePara, cRiceParam)

with minRicePara = 0 for internal bit-depth equal to or smaller than 10, and minRicePara =2 otherwise.

From JCTVC-O0219:

In the proposed scheme, the 4×4 sub-blocks are divided into different categories (“sbType”). For the current sub-block, its initial Rice parameter is derived based on the previously coded sub-blocks in the same category.

The categorization is based on whether the block is transform skip block or not (isTSFlag). A PPS flag useSeparateStatsFlag is introduced to enable further splitting sub-blocks based on whether it is the first color component or not (isLumaFlag)

sbType = (useSeparateStatsFlag? 2*isLumaFlag:0) + isTSFlag;

Statistics are maintained for each type of sub-block depending on the absolute coefficient value (uiLevel):

statCoeff += (uiLevel == statCoeff) ? 0 : ( ( uiLevel < statCoeff ) ? -1 : 1);

statCoeff = min(320, statCoeff);

This variable is updated at most once per 4×4 sub-block using the value of the first coded coeff_abs_level_remaining of the sub-block. The entries of statCoeff are 9-bit and they are reset to 0 at the beginning of the slice (like the CABAC contexts).

The value of statCoeff is used to initialize the Rice parameter at the beginning of each 4×4 sub-block as follows:

cRiceParam = statCoeff<16? (statCoeff+1)>>2: 4+(statCoeff+40)>>6);

The calculated cRiceParam is further clipped,

cRiceParam = Clip3 (minRicePara, maxRicePara, cRiceParam)

with minRicePara = 0 for internal bit-depth equal to or smaller than 10, and minRicePara =2 otherwise.

Test C.2 (JCTVC-N0189/JCTVC-O0065, Sony)

An offset to cRiceParam equal to BF is applied. An algorithm is proposed to allow the value of BF to adaptively change over the course of a slice; conceptually similar to the context variables that are used to adapt CABAC.

From JCTVC-O0065:

Currently, when coding an escape code, the value to be coded is split into a prefix and a suffix using either a Golomb-Rice or exponential-Golomb-order-k scheme, with the suffix including a number cRiceParam of least-significant bits of the coded value. For example, to code the value 13 (0b1101) with cRiceParam = 2, the LSBs of the suffix would be equal to the 2 LSBs 0b01 of the coded value. The value of cRiceParam is maintained by the entropy coder; being reset to 0 at the start of each coefficient group and conditionally incremented after each escape coding operation.

An algorithm is proposed to apply a Golomb-Rice parameter offset BF to the standard HEVC cRiceParam, which will adaptively change over the course of a slice; conceptually similar to the context variables that are used to adapt CABAC.

The scheme is enabled with a flag in the SPS; no additional signalling is required.

- The value of BF is initialised to 0 at the start of each slice.

- For each TU encoded/decoded:

  - The variables undershootCount, overshootCount and totalCount are each initialised to 0.

  - For each significant coefficient, if an escape code escapeCode is coded for that coefficient, steps are taken as defined by the following pseudo-code:

totalCount++

if (escapeCode > (3 << cRiceParam))
{
  undershootCount++
}
else if ((escapeCode * 3) < (1 << cRiceParam))
{
  overshootCount++
}

  - At the end of encoding/decoding the TU, BF is adjusted according to the following pseudo-code:

if       ((undershootCount * 4) > totalCount)              BF++	[Note ]
else if (((overshootCount  * 2) > totalCount) && (BF > 0)) BF--

Conceptually, the adaptation algorithm determines if the current value of BF has undershot or overshot the ideal value for each coefficient. The end-of-TU update condition causes an increment if more than 25% of coefficients undershot the ideal value of BF and decrement if more than 50% of coefficients overshot the ideal value of BF. The reason for this asymmetry is that undershooting causes more inefficiency than overshooting.

## Combinations

Test D.1 Combination of C.1 and C.2.

From JCTVC-O0239:

4x4 sub-blocks are divided into different categories “sbType” depends on the transform skip flag and/or whether it is luma block. Statistic statCoeff is maintained for each sbType, and is updated using the first remaining level coefficient of each sub-block

if (uiLevel >=3*(1<<(statCoeff /4)))            statCoeff ++;

else if ((2* uiLevel)<(1<<( statCoeff/4)))           statCoeff --;

The initial rice parameter for the next sub-block is calculated as follows

cRiceParam = Min(maxRicePara, statCoeff/4).

Test results with different max rice parameter values are provided.

In the proposed scheme, the 4×4 sub-blocks are divided into different categories (“sbType”). For each sub-block, the initial Rice parameter is derived based on previously coded sub-blocks in the same category. The categorization is based on whether the block is a transform-skip block (“isTSFlag”). A PPS flag (“useSeparateStatsFlag”) is introduced to enable further separation of the sub-blocks based on whether they belong to the first color component or not (“isLumaFlag”)

sbType = (useSeparateStatsFlag? 2*isLumaFlag:0) + isTSFlag;

Stats statCoeff  are maintained for each sub-block type (sbType) depending on the absolute coefficient value (uiLevel):

if (uiLevel >=3*(1<<(statCoeff /4)))            statCoeff ++;

else if ((2* uiLevel)<(1<<( statCoeff/4)))           statCoeff --;

This variable is updated at most once per 4×4 sub-block using the value of the first coded coeff_abs_level_remaining of the sub-block. The entries of statCoeff are 8-bit and they are reset to 0 at the beginning of the slice (like the CABAC context variables).

The value of statCoeff is used to initialize the Rice parameter at the beginning of each 4×4 sub-block as

cRiceParam = Min(maxRicePara, statCoeff/4).

Results are reported for the following settings:

Setting 1: In this test, the maximum rice parameter (MAX_RICE_PARAM) is set to be 7 for AHG8 test and 16 for AHG18 test. Extra results for AHG8 test with  MAX_RICE_PARAM = 4 can be found in uploaded Excel files. The useSeparateStatsFlag is set to be 1 for YUV sequences and 0 for RGB sequences. Extra results for always (for both YUV and RGB) setting useSeparateStatsFlag = 1 can be found in the uploaded Excel files.

Setting 2: In this test, MAX_RICE_PARAM is set to equal to MAX_TR_DYNAMIC_RANGE. The Rice parameter update will never be triggered once uiGoRiceParam reaches MAX_TR_DYNAMIC_RANGE, and thus the clipping in the update process can be removed,

i.e., the update process

uiGoRiceParam = min<UInt>(uiGoRiceParam+1, MAX_RICE_PARAM);

can be simplified as

uiGoRiceParam++;

Setting 3: In the previous two settings, the initial value of the Rice parameter is set to 0 for RDOQ. In this setting, a value derived using previously coded coefficients is used as the initial Rice parameter in RDOQ for a TU. The setting of MAX_RICE_PARAM is the same as Setting 2 (MAX_RICE_PARAM = MAX_TR_DYNAMIC_RANGE).

Test D.2 Combination of A.1 and C.1 - withdrawn.

Test D.3 Combination of A.1 and C.2 - withdrawn.

Test D.4 Combination of A.1 with C.1 and C.2 - withdrawn.

Test D.5 Combination of B.2 and C.1 – withdrawn.

Test D.6 Combination of B.2 and C.2 – withdrawn.

Test D.7 Combination of A.1 and B.1 - withdrawn.

# Summary of Results

The proposed methods were implemented on top of HM-12.1-RExt-4.1 and simulated under the RCE2/3 common test conditions [1] [2].

## Subtest A

### Lossy

|  |  | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A1 max=4 no_reset=1 Qualcomm | Class F | -0.2% | -0.3% | -0.5% | -0.1% | -0.1% | 0.0% | 0.0% |
|  | Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC RGB 444 | -0.4% | -1.0% | -1.7% | -0.4% | -0.9% | 0.3% | -0.3% |
|  | Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC YUV 444 | -0.2% | -0.4% | -0.8% | 0.0% | -0.3% | -0.3% | -0.4% |
|  | Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC(444) GBR Optional | -1.8% | -3.3% | -4.9% | -1.6% | -3.0% | -0.5% | -3.1% |
|  | SC(444) YUV Optional | -0.5% | -1.2% | -2.4% | -0.5% | -1.1% | -1.2% | -1.2% |
|  | Enc Time[%] | 98% | 97% | 97% | 99% | 99% | 99% | 98% |
|  | Dec Time[%] | 96% | 97% | 97% | 96% | 96% | 97% | 96% |

|  |  | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A2 Sharp | Class F | 0.4% | 0.4% | 0.2% | 0.3% | 0.3% | 0.4% | 0.4% |
|  | Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
|  | SC RGB 444 | 0.7% | 0.1% | -1.1% | 0.4% | -0.1% | 0.9% | 0.1% |
|  | Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC YUV 444 | 0.7% | 0.6% | 0.3% | 0.3% | 0.3% | 0.2% | 0.3% |
|  | Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC(444) GBR Optional | -1.8% | -4.5% | -8.1% | -1.8% | -4.4% | -1.7% | -4.5% |
|  | SC(444) YUV Optional | 0.7% | 0.0% | -2.2% | 0.6% | 0.1% | -0.9% | -1.1% |
|  | Enc Time[%] | 73% | 73% | 74% | 89% | 87% | 106% | 115% |
|  | Dec Time[%] | 101% | 100% | 99% | 99% | 99% | 104% | 103% |

### Lossless

|  |  | AI |  |  |  | RA |  |  |  | LB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving | Total | Average | Min | Max | Total | Average | Min | Max | Total | Average | Min | Max |
| A1 max=7 no_reset=1 Qualcomm | Class F | 0.7% | 0.9% | 0.1% | 2.1% | -0.1% | 0.3% | -0.4% | 1.4% | -0.2% | 0.1% | -0.5% | 0.5% |
|  | Class B | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC RGB 444 | 4.0% | 5.7% | 0.7% | 13.5% | 1.6% | 4.5% | 0.4% | 13.5% | 0.9% | 3.8% | 0.3% | 13.3% |
|  | Animation RGB 444 | -0.1% | -0.1% | -0.2% | 0.0% | -0.2% | -0.2% | -0.4% | 0.0% | -0.3% | -0.3% | -0.5% | 0.0% |
|  | SC YUV 444 | 1.9% | 2.7% | 0.2% | 7.9% | 0.8% | 2.2% | 0.1% | 7.8% | 0.5% | 2.0% | 0.2% | 7.5% |
|  | Animation YUV 444 | 0.1% | 0.1% | -0.1% | 0.4% | 0.1% | 0.0% | -0.2% | 0.3% | 0.1% | 0.0% | -0.3% | 0.3% |
|  | RangeExt | 2.5% | 2.3% | 0.6% | 4.0% | 2.4% | 2.3% | 0.3% | 3.7% | 2.4% | 2.3% | 0.3% | 3.8% |
|  | SC(444) GBR Optional | 11.5% | 12.8% | 8.1% | 15.2% | 7.6% | 12.4% | 6.9% | 15.2% | 7.1% | 13.4% | 6.8% | 16.9% |
|  | SC(444) YUV Optional | 8.5% | 8.6% | 7.5% | 9.4% | 7.4% | 9.0% | 6.9% | 10.3% | 6.9% | 7.3% | 5.0% | 10.1% |
|  | Enc Time[%] | 99% |  |  |  | 97% |  |  |  | 104% |  |  |  |
|  | Dec Time[%] | 99% |  |  |  | 102% |  |  |  | 102% |  |  |  |

|  |  | AI |  |  |  | RA |  |  |  | LB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving | Total | Average | Min | Max | Total | Average | Min | Max | Total | Average | Min | Max |
| A2 Sharp | Class F | -0.6% | -0.6% | -1.8% | 0.4% | -0.5% | -0.4% | -0.7% | 0.1% | 0.0% | 0.2% | -0.1% | 0.6% |
|  | Class B | -1.4% | -1.4% | -2.5% | -0.2% | 1.0% | 1.0% | 0.8% | 1.2% | 0.0% | 0.0% | 0.0% | 0.1% |
|  | SC RGB 444 | 3.5% | 6.1% | -0.7% | 17.3% | 0.4% | 4.5% | -1.1% | 17.3% | 1.0% | 2.4% | 0.5% | 5.8% |
|  | Animation RGB 444 | -3.4% | -3.3% | -4.4% | -1.5% | -0.8% | -0.8% | -1.2% | -0.3% | -0.1% | -0.1% | -0.3% | 0.0% |
|  | SC YUV 444 | 0.7% | 2.2% | -1.3% | 9.9% | -0.6% | 1.8% | -1.7% | 9.9% | 0.4% | 1.6% | 0.3% | 5.2% |
|  | Animation YUV 444 | -3.0% | -2.8% | -3.7% | -2.3% | -1.7% | -1.2% | -2.5% | -0.5% | 0.0% | 0.0% | -0.2% | 0.1% |
|  | RangeExt | -1.8% | -2.5% | -5.2% | -0.3% | -3.5% | -3.9% | -5.7% | -1.4% | 0.6% | 0.5% | 0.1% | 1.4% |
|  | SC(444) GBR Optional | 15.0% | 17.1% | 9.6% | 21.1% | 9.1% | 16.5% | 8.0% | 21.0% | 3.5% | 5.3% | 3.4% | 8.5% |
|  | SC(444) YUV Optional | 12.3% | 12.7% | 9.2% | 15.5% | 8.7% | 12.3% | 7.6% | 16.2% | 3.3% | 3.5% | 1.3% | 6.0% |
|  | Enc Time[%] | 68% |  |  |  | 74% |  |  |  | 90% |  |  |  |
|  | Dec Time[%] | 103% |  |  |  | 101% |  |  |  | 103% |  |  |  |

### High bit depth

|  |  | All Intra |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y | U | V |
| A1 max=9 no_reset=1 Qualcomm | 12-bit | -3.6% | -3.3% | -3.5% |
|  | 14-bit | -11.3% | -11.2% | -11.3% |
|  | 16-bit | -23.7% | -23.6% | -23.7% |
|  | Enc Time[%] | 91% |  |  |
|  | Dec Time[%] | 85% |  |  |

|  |  | All Intra |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y | U | V |
| A2 Sharp | 12-bit | -3.7% | -3.5% | -3.5% |
|  | 14-bit | -12.5% | -12.6% | -12.5% |
|  | 16-bit | -25.9% | -25.8% | -25.8% |
|  | Enc Time[%] | 160% |  |  |
|  | Dec Time[%] | 88% |  |  |

## Subtest B

### Lossy

|  |  | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B1 Sharp All blocks | Class F | 0.0% | -0.2% | -0.4% | 0.1% | 0.0% | 0.3% | 0.2% |
|  | Class B | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% | 0.1% |
|  | SC RGB 444 | -0.5% | -1.5% | -2.8% | -0.4% | -1.2% | -0.1% | -0.8% |
|  | Animation RGB 444 | 0.2% | 0.1% | 0.1% | 0.2% | 0.1% | 0.2% | 0.2% |
|  | SC YUV 444 | 0.1% | -0.3% | -0.8% | 0.0% | -0.1% | 0.1% | 0.0% |
|  | Animation YUV 444 | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% | 0.2% |
|  | RangeExt | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% | 0.1% |
|  | SC(444) GBR Option | -3.5% | -6.2% | -9.5% | -3.3% | -6.0% | -2.4% | -6.2% |
|  | SC(444) YUV Option | -0.3% | -1.6% | -3.9% | -0.3% | -1.4% | -1.5% | -2.1% |
|  | Enc Time[%] | 82% | 84% | 89% | 101% | 97% | 122% | 128% |
|  | Dec Time[%] | 100% | 100% | 100% | 98% | 99% | 99% | 99% |

|  |  | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B1 Sharp TS blocks | Class F | -0.1% | -0.3% | -0.6% | 0.0% | -0.1% | 0.1% | 0.1% |
|  | Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC RGB 444 | -0.6% | -1.6% | -2.9% | -0.5% | -1.3% | 0.1% | -0.8% |
|  | Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC YUV 444 | 0.0% | -0.4% | -0.9% | 0.0% | -0.2% | -0.1% | -0.4% |
|  | Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC(444) GBR Option | -3.5% | -6.2% | -9.5% | -3.2% | -5.8% | -2.3% | -5.4% |
|  | SC(444) YUV Option | -0.4% | -1.6% | -4.0% | -0.5% | -1.3% | -1.2% | -1.7% |
|  | Enc Time[%] | 105% | 102% | 98% | 126% | 122% | 89% | 90% |
|  | Dec Time[%] | 99% | 99% | 99% | 102% | 102% | 99% | 98% |

|  |  | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B2 Canon | Class F | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% |
|  | Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC RGB 444 | -0.1% | -0.2% | -0.4% | 0.1% | -0.2% | 0.0% | 0.0% |
|  | Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC YUV 444 | 0.0% | -0.1% | -0.2% | 0.1% | 0.1% | -0.1% | -0.1% |
|  | Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC(444) GBR Optional | -0.1% | -0.3% | -0.5% | 0.0% | -0.1% | 1.5% | 0.9% |
|  | SC(444) YUV Optional | 0.0% | -0.1% | -0.3% | -0.1% | -0.1% | -0.4% | 0.0% |
|  | Enc Time[%] | 102% | 101% | 101% | 101% | 101% | 98% | 98% |
|  | Dec Time[%] | 102% | 102% | 102% | 101% | 100% | 96% | 95% |

### Lossless

|  |  | AI |  |  |  | RA |  |  |  | LB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving | Total | Average | Min | Max | Total | Average | Min | Max | Total | Average | Min | Max |
| B1 Sharp All blocks | Class F | 1.2% | 1.3% | 0.3% | 2.5% | 0.1% | 0.6% | -0.1% | 1.6% | 0.0% | 0.2% | -0.1% | 0.6% |
|  | Class B | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
|  | SC RGB 444 | 5.5% | 7.3% | 1.3% | 14.8% | 2.4% | 5.8% | 0.8% | 14.8% | 1.6% | 5.2% | 0.6% | 14.6% |
|  | Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
|  | SC YUV 444 | 3.0% | 4.1% | 0.5% | 10.4% | 1.2% | 3.3% | 0.3% | 10.4% | 0.6% | 2.8% | 0.3% | 10.4% |
|  | Animation YUV 444 | 0.1% | 0.0% | -0.1% | 0.3% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% | 0.0% |
|  | RangeExt | 2.4% | 2.1% | 0.5% | 4.0% | 1.5% | 1.3% | 0.0% | 3.1% | 1.5% | 1.3% | -0.1% | 3.1% |
|  | SC(444) GBR Optional | 12.9% | 14.3% | 9.0% | 18.1% | 8.3% | 13.7% | 7.6% | 17.9% | 7.6% | 13.5% | 7.4% | 18.8% |
|  | SC(444) YUV Optional | 12.5% | 12.8% | 9.2% | 15.4% | 8.6% | 12.3% | 7.4% | 16.1% | 7.7% | 11.8% | 7.2% | 17.4% |
|  | Enc Time[%] | 128% |  |  |  | 55% |  |  |  | 97% |  |  |  |
|  | Dec Time[%] | 100% |  |  |  | 99% |  |  |  | 100% |  |  |  |

|  |  | AI |  |  |  | RA |  |  |  | LB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving | Total | Average | Min | Max | Total | Average | Min | Max | Total | Average | Min | Max |
| B1 Sharp TS blocks | Class F | 1.2% | 1.3% | 0.3% | 2.5% | 0.1% | 0.6% | -0.1% | 1.6% | 0.0% | 0.2% | -0.1% | 0.6% |
|  | Class B | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
|  | SC RGB 444 | 5.5% | 7.3% | 1.3% | 14.8% | 2.4% | 5.8% | 0.8% | 14.8% | 1.6% | 5.2% | 0.6% | 14.6% |
|  | Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
|  | SC YUV 444 | 3.0% | 4.1% | 0.5% | 10.4% | 1.2% | 3.3% | 0.3% | 10.4% | 0.6% | 2.8% | 0.3% | 10.4% |
|  | Animation YUV 444 | 0.1% | 0.0% | -0.1% | 0.3% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% | 0.0% |
|  | RangeExt | 2.4% | 2.1% | 0.5% | 4.0% | 1.5% | 1.3% | 0.0% | 3.1% | 1.5% | 1.3% | -0.1% | 3.1% |
|  | SC(444) GBR Optional | 12.9% | 14.3% | 9.0% | 18.1% | 8.3% | 13.7% | 7.6% | 17.9% | 7.6% | 13.5% | 7.4% | 18.8% |
|  | SC(444) YUV Optional | 12.5% | 12.8% | 9.2% | 15.4% | 8.6% | 12.3% | 7.4% | 16.1% | 7.7% | 11.8% | 7.2% | 17.4% |
|  | Enc Time[%] | 100% |  |  |  | 47% |  |  |  | 93% |  |  |  |
|  | Dec Time[%] | 99% |  |  |  | 100% |  |  |  | 99% |  |  |  |

|  |  | AI |  |  |  | RA |  |  |  | LB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving | Total | Average | Min | Max | Total | Average | Min | Max | Total | Average | Min | Max |
| B2 Canon | Class F | 0.4% | 0.4% | 0.2% | 0.6% | 0.0% | 0.2% | 0.0% | 0.4% | 0.0% | 0.1% | 0.0% | 0.2% |
|  | Class B | 0.0% | 0.0% | -0.1% | 0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
|  | SC RGB 444 | 1.4% | 1.3% | 0.6% | 1.8% | 0.8% | 0.8% | 0.1% | 1.4% | 0.8% | 0.8% | 0.2% | 1.2% |
|  | Animation RGB 444 | 0.1% | 0.0% | -0.1% | 0.2% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
|  | SC YUV 444 | 0.8% | 0.9% | 0.3% | 1.4% | 0.6% | 0.8% | 0.1% | 1.7% | 0.4% | 0.6% | 0.1% | 1.3% |
|  | Animation YUV 444 | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% | -0.2% | 0.0% | -0.2% | -0.1% | -0.3% | 0.0% |
|  | RangeExt | 2.3% | 2.1% | 0.6% | 3.3% | 1.6% | 1.6% | -0.1% | 2.8% | 1.6% | 1.6% | -0.1% | 2.8% |
|  | SC(444) GBR Optional | 0.7% | 0.8% | 0.4% | 1.4% | 0.3% | 0.7% | 0.3% | 1.4% | 0.3% | 1.2% | -0.8% | 4.0% |
|  | SC(444) YUV Optional | 1.1% | 1.0% | 0.6% | 1.6% | 0.4% | 0.7% | 0.2% | 1.4% | 0.1% | -0.9% | -3.3% | 0.2% |
|  | Enc Time[%] | 103% |  |  |  | 101% |  |  |  | 101% |  |  |  |
|  | Dec Time[%] | 101% |  |  |  | 100% |  |  |  | 100% |  |  |  |

### High bit depth

|  |  | All Intra |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y | U | V |
| B1 Sharp All blocks | 12-bit | -6.4% | -6.0% | -6.0% |
|  | 14-bit | -17.8% | -17.4% | -17.4% |
|  | 16-bit | -27.9% | -27.7% | -27.8% |
|  | Enc Time[%] | 103% |  |  |
|  | Dec Time[%] | 100% |  |  |

|  |  | All Intra |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y | U | V |
| B1 Sharp TS blocks | 12-bit | -4.9% | -4.4% | -4.4% |
|  | 14-bit | -16.9% | -16.5% | -16.4% |
|  | 16-bit | -27.8% | -27.6% | -27.6% |
|  | Enc Time[%] | 105% |  |  |
|  | Dec Time[%] | #VALUE! |  |  |

|  |  | All Intra |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y | U | V |
| B2 Canon | 12-bit | -2.0% | -1.7% | -1.7% |
|  | 14-bit | -3.3% | -3.3% | -3.1% |
|  | 16-bit | -2.9% | -3.0% | -3.0% |
|  | Enc Time[%] | 100% |  |  |
|  | Dec Time[%] | 98% |  |  |

## Subtest C

### Lossy

For C1 lossy and lossless, cMaxRiceParam = 7 and for high bit-depth, cMaxRiceParam = 16.

|  |  | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C1 Qualcomm | Class F | -0.4% | -0.7% | -1.1% | -0.2% | -0.5% | -0.2% | -0.3% |
|  | Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC RGB 444 | -1.3% | -2.5% | -3.9% | -1.2% | -2.2% | -0.8% | -1.5% |
|  | Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC YUV 444 | -0.5% | -1.1% | -1.8% | -0.4% | -0.8% | -0.4% | -0.5% |
|  | Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC(444) GBR Optional | -5.5% | -8.9% | -12.4% | -5.3% | -8.5% | -4.9% | -9.0% |
|  | SC(444) YUV Optional | -1.6% | -3.4% | -5.9% | -1.5% | -3.3% | -1.6% | -2.7% |
|  | Enc Time[%] | 98% | 99% | 98% | 98% | 98% | 94% | 94% |
|  | Dec Time[%] | 100% | 100% | 100% | 100% | 100% | 100% | 100% |

|  |  | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C2 Sony Europe | Class F | -0.2% | -0.5% | -0.7% | -0.1% | -0.3% | -0.1% | -0.2% |
|  | Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC RGB 444 | -1.3% | -2.7% | -4.1% | -1.0% | -2.2% | -0.6% | -1.6% |
|  | Animation RGB 444 | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
|  | SC YUV 444 | -0.2% | -0.6% | -1.2% | 0.0% | -0.2% | -0.1% | 0.0% |
|  | Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC(444) GBR Optional | -5.4% | -8.7% | -12.2% | -5.1% | -8.3% | -4.3% | -8.1% |
|  | SC(444) YUV Optional | -1.0% | -2.6% | -5.1% | -0.7% | -2.1% | -0.7% | -2.0% |
|  | Enc Time[%] | 101% | 101% | 101% | 101% | 101% | 100% | 100% |
|  | Dec Time[%] | 100% | 100% | 100% | 100% | 100% | 100% | 100% |

### Lossless

|  |  | AI |  |  |  | RA |  |  |  | LB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving | Total | Average | Min | Max | Total | Average | Min | Max | Total | Average | Min | Max |
| C1 Qualcomm | Class F | 1.3% | 1.4% | 0.2% | 3.1% | 0.2% | 0.7% | 0.0% | 1.8% | 0.1% | 0.4% | 0.0% | 1.0% |
|  | Class B | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC RGB 444 | 6.3% | 8.6% | 1.3% | 18.4% | 2.8% | 7.0% | 1.0% | 18.3% | 1.9% | 6.3% | 0.8% | 18.3% |
|  | Animation RGB 444 | 0.2% | 0.2% | 0.1% | 0.4% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC YUV 444 | 3.8% | 5.2% | 0.7% | 12.9% | 1.6% | 4.3% | 0.4% | 12.9% | 1.0% | 3.9% | 0.4% | 12.7% |
|  | Animation YUV 444 | 0.3% | 0.2% | 0.0% | 0.5% | 0.2% | 0.1% | 0.0% | 0.3% | 0.1% | 0.1% | 0.0% | 0.3% |
|  | RangeExt | 3.5% | 3.1% | 1.0% | 5.3% | 2.4% | 2.3% | 0.2% | 4.5% | 2.4% | 2.3% | 0.2% | 4.5% |
|  | SC(444) GBR Optional | 18.1% | 20.6% | 12.1% | 26.4% | 11.7% | 20.2% | 10.5% | 27.0% | 10.7% | 19.9% | 10.3% | 27.2% |
|  | SC(444) YUV Optional | 15.2% | 15.4% | 12.0% | 18.0% | 11.6% | 15.2% | 10.4% | 18.5% | 10.6% | 14.3% | 10.2% | 19.7% |
|  | Enc Time[%] | 98% |  |  |  | 101% |  |  |  | 98% |  |  |  |
|  | Dec Time[%] | 98% |  |  |  | 99% |  |  |  | 98% |  |  |  |

|  |  | AI |  |  |  | RA |  |  |  | LB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving | Total | Average | Min | Max | Total | Average | Min | Max | Total | Average | Min | Max |
| C2 Sony Europe | Class F | 1.1% | 1.1% | 0.3% | 2.1% | 0.1% | 0.6% | 0.0% | 1.4% | 0.1% | 0.3% | 0.0% | 0.6% |
|  | Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC RGB 444 | 6.8% | 9.1% | 1.4% | 19.2% | 2.9% | 7.3% | 1.1% | 19.2% | 1.9% | 6.5% | 0.9% | 19.1% |
|  | Animation RGB 444 | 0.1% | 0.1% | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC YUV 444 | 3.3% | 4.8% | 0.6% | 12.0% | 1.4% | 4.0% | 0.4% | 12.0% | 0.8% | 3.7% | 0.3% | 12.2% |
|  | Animation YUV 444 | 0.2% | 0.1% | 0.0% | 0.4% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% |
|  | RangeExt | 2.7% | 2.2% | 0.4% | 4.2% | 1.9% | 1.6% | 0.0% | 4.1% | 1.9% | 1.6% | 0.0% | 4.1% |
|  | SC(444) GBR Optional | 18.4% | 20.7% | 12.8% | 25.7% | 12.0% | 20.2% | 10.9% | 26.2% | 11.1% | 20.6% | 10.6% | 26.3% |
|  | SC(444) YUV Optional | 14.4% | 14.6% | 11.7% | 16.6% | 11.0% | 14.2% | 10.0% | 17.5% | 10.2% | 13.9% | 9.8% | 19.2% |
|  | Enc Time[%] | 101% |  |  |  | 101% |  |  |  | 100% |  |  |  |
|  | Dec Time[%] | 100% |  |  |  | 99% |  |  |  | 100% |  |  |  |

### High bit-depth

|  |  | All Intra |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y | U | V |
| C1 Qualcomm | 12-bit | -7.1% | -6.8% | -6.8% |
|  | 14-bit | -18.7% | -18.4% | -18.4% |
|  | 16-bit | -28.4% | -28.2% | -28.2% |
|  | Enc Time[%] | 94% |  |  |
|  | Dec Time[%] | 86% |  |  |

|  |  | All Intra |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y | U | V |
| C2 Sony Europe | 12-bit | -7.0% | -6.7% | -6.7% |
|  | 14-bit | -18.4% | -18.1% | -18.0% |
|  | 16-bit | -28.2% | -28.0% | -28.1% |
|  | Enc Time[%] | 93% |  |  |
|  | Dec Time[%] | 91% |  |  |

## Subtest D

### Lossy

|  |  | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| D1 Setting 1 Max=4 (supplemental result) Qualcomm and Sony Europe | Class F | -0.5% | -0.9% | -1.3% | -0.3% | -0.6% | -0.1% | -0.3% |
|  | Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC RGB 444 | -1.5% | -2.9% | -4.3% | -1.3% | -2.4% | -0.6% | -1.4% |
|  | Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC YUV 444 | -0.6% | -1.3% | -2.1% | -0.6% | -1.0% | -0.8% | -1.0% |
|  | Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC(444) GBR Optional | -6.0% | -9.4% | -12.0% | -5.7% | -8.9% | -5.8% | -9.5% |
|  | SC(444) YUV Optional | -1.7% | -3.7% | -6.4% | -1.7% | -3.3% | -1.8% | -2.7% |
|  | Enc Time[%] | 96% | 96% | 96% | 96% | 96% | 96% | 96% |
|  | Dec Time[%] | 100% | 100% | 100% | 100% | 100% | 100% | 100% |

|  |  | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| D1 Setting 1 Max=7 Qualcomm and Sony Europe | Class F | -0.5% | -0.9% | -1.3% | -0.3% | -0.6% | -0.1% | -0.3% |
|  | Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC RGB 444 | -1.5% | -2.9% | -4.4% | -1.3% | -2.4% | -0.7% | -1.5% |
|  | Animation RGB 444 | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% |
|  | SC YUV 444 | -0.6% | -1.3% | -2.2% | -0.6% | -1.0% | -0.7% | -1.0% |
|  | Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC(444) GBR Optional | -6.0% | -9.5% | -13.0% | -5.7% | -9.0% | -5.6% | -9.1% |
|  | SC(444) YUV Optional | -1.7% | -3.7% | -6.4% | -1.7% | -3.3% | -1.8% | -2.6% |
|  | Enc Time[%] | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
|  | Dec Time[%] | 100% | 100% | 100% | 100% | 100% | 100% | 100% |

|  |  | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| D1 Setting 2 Qualcomm and Sony Europe | Class F | -0.5% | -0.9% | -1.3% | -0.3% | -0.6% | -0.1% | -0.3% |
|  | Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC RGB 444 | -1.5% | -2.9% | -4.4% | -1.3% | -2.4% | -0.7% | -1.5% |
|  | Animation RGB 444 | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% |
|  | SC YUV 444 | -0.6% | -1.3% | -2.2% | -0.6% | -1.0% | -0.7% | -1.0% |
|  | Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC(444) GBR Optional | -6.0% | -9.5% | -12.9% | -5.7% | -9.0% | -5.6% | -9.1% |
|  | SC(444) YUV Optional | -1.7% | -3.7% | -6.4% | -1.7% | -3.3% | -1.8% | -2.6% |
|  | Enc Time[%] | 99% | 99% | 98% | 101% | 101% | 101% | 101% |
|  | Dec Time[%] | 100% | 100% | 100% | 99% | 99% | 100% | 100% |

|  |  | AI-MT | AI-HT | AI_SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| D1 Setting 3 Qualcomm and Sony Europe | Class F | -0.5% | -0.9% | -1.3% | -0.3% | -0.6% | -0.2% | -0.3% |
|  | Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC RGB 444 | -1.6% | -3.0% | -4.4% | -1.4% | -2.5% | -1.0% | -2.0% |
|  | Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC YUV 444 | -0.6% | -1.3% | -2.2% | -0.6% | -1.0% | -0.2% | -0.5% |
|  | Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC(444) GBR Optional | -6.1% | -9.5% | -13.0% | -5.8% | -9.2% | -5.3% | -9.2% |
|  | SC(444) YUV Optional | -1.8% | -3.8% | -6.5% | -1.9% | -3.6% | -2.1% | -3.3% |
|  | Enc Time[%] | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
|  | Dec Time[%] | 99% | 100% | 100% | 99% | 99% | 100% | 100% |

### Lossless

|  |  | AI |  |  |  | RA |  |  |  | LB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving | Total | Average | Min | Max | Total | Average | Min | Max | Total | Average | Min | Max |
| D1 Setting 1 max = 4 (supplemental result) Qualcomm and Sony Europe | Class F | 1.2% | 1.3% | 0.1% | 2.8% | 0.2% | 0.7% | 0.0% | 1.8% | 0.1% | 0.4% | 0.0% | 0.9% |
|  | Class B | 0.1% | 0.1% | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC RGB 444 | 5.1% | 6.4% | 1.4% | 11.4% | 2.4% | 5.1% | 1.1% | 11.4% | 1.9% | 4.7% | 0.9% | 11.1% |
|  | Animation RGB 444 | 0.3% | 0.3% | 0.1% | 0.5% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC YUV 444 | 3.6% | 4.8% | 0.7% | 10.8% | 1.7% | 4.0% | 0.6% | 10.7% | 1.0% | 3.5% | 0.5% | 10.6% |
|  | Animation YUV 444 | 0.3% | 0.3% | 0.1% | 0.6% | 0.2% | 0.1% | 0.0% | 0.4% | 0.2% | 0.1% | 0.0% | 0.3% |
|  | RangeExt | 3.3% | 3.1% | 1.0% | 4.8% | 2.6% | 2.5% | 0.3% | 4.2% | 2.6% | 2.5% | 0.3% | 4.2% |
|  | SC(444) GBR Optional | 10.6% | 11.4% | 8.5% | 13.6% | 7.9% | 10.8% | 7.4% | 13.4% | 7.5% | 9.9% | 7.3% | 11.7% |
|  | SC(444) YUV Optional | 11.8% | 12.0% | 9.0% | 13.9% | 8.2% | 11.5% | 7.1% | 14.5% | 7.3% | 10.5% | 7.0% | 15.5% |
|  | Enc Time[%] | 100% |  |  |  | 95% |  |  |  | 96% |  |  |  |
|  | Dec Time[%] | 100% |  |  |  | 100% |  |  |  | 100% |  |  |  |

|  |  | AI |  |  |  | RA |  |  |  | LB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving | Total | Average | Min | Max | Total | Average | Min | Max | Total | Average | Min | Max |
| D1 Setting 1 max = 7 Qualcomm and Sony Europe | Class F | 1.4% | 1.5% | 0.1% | 3.4% | 0.2% | 0.7% | 0.0% | 2.0% | 0.1% | 0.4% | 0.0% | 1.0% |
|  | Class B | 0.1% | 0.1% | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC RGB 444 | 6.8% | 9.2% | 1.5% | 19.4% | 3.1% | 7.5% | 1.2% | 19.4% | 2.1% | 6.6% | 1.0% | 19.3% |
|  | Animation RGB 444 | 0.3% | 0.3% | 0.1% | 0.5% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC YUV 444 | 4.2% | 5.6% | 0.7% | 13.5% | 1.7% | 4.5% | 0.5% | 13.5% | 1.1% | 4.1% | 0.5% | 13.3% |
|  | Animation YUV 444 | 0.4% | 0.3% | 0.1% | 0.7% | 0.2% | 0.1% | 0.0% | 0.4% | 0.2% | 0.1% | 0.0% | 0.3% |
|  | RangeExt | 3.8% | 3.6% | 1.2% | 5.7% | 2.8% | 2.7% | 0.3% | 5.0% | 2.8% | 2.7% | 0.3% | 5.0% |
|  | SC(444) GBR Optional | 18.8% | 21.2% | 13.0% | 26.7% | 12.4% | 20.7% | 11.2% | 27.3% | 11.4% | 21.1% | 10.9% | 27.4% |
|  | SC(444) YUV Optional | 15.4% | 15.7% | 12.4% | 18.1% | 11.8% | 15.2% | 10.7% | 18.6% | 10.9% | 14.5% | 10.5% | 19.6% |
|  | Enc Time[%] | 103% |  |  |  | 103% |  |  |  | 102% |  |  |  |
|  | Dec Time[%] | 98% |  |  |  | 101% |  |  |  | 99% |  |  |  |

|  |  | AI |  |  |  | RA |  |  |  | LB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving | Total | Average | Min | Max | Total | Average | Min | Max | Total | Average | Min | Max |
| D1 Setting 2 max = 7 Qualcomm and Sony Europe | Class F | 1.4% | 1.5% | 0.1% | 3.4% | 0.2% | 0.7% | 0.0% | 2.0% | 0.1% | 0.4% | 0.0% | 1.0% |
|  | Class B | 0.1% | 0.1% | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC RGB 444 | 6.8% | 9.2% | 1.5% | 19.4% | 3.1% | 7.5% | 1.2% | 19.4% | 2.1% | 6.6% | 1.0% | 19.3% |
|  | Animation RGB 444 | 0.3% | 0.3% | 0.1% | 0.5% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC YUV 444 | 4.2% | 5.6% | 0.7% | 13.5% | 1.7% | 4.5% | 0.5% | 13.5% | 1.1% | 4.1% | 0.5% | 13.3% |
|  | Animation YUV 444 | 0.4% | 0.3% | 0.1% | 0.7% | 0.2% | 0.1% | 0.0% | 0.4% | 0.2% | 0.1% | 0.0% | 0.3% |
|  | RangeExt | 3.8% | 3.6% | 1.2% | 5.7% | 2.8% | 2.7% | 0.3% | 5.0% | 2.8% | 2.7% | 0.3% | 5.0% |
|  | SC(444) GBR Optional | 18.8% | 21.2% | 13.0% | 26.7% | 12.4% | 20.7% | 11.2% | 27.3% | 11.4% | 21.1% | 10.9% | 27.4% |
|  | SC(444) YUV Optional | 15.4% | 15.7% | 12.4% | 18.1% | 11.8% | 15.2% | 10.7% | 18.6% | 10.9% | 14.5% | 10.5% | 19.6% |
|  | Enc Time[%] | 100% |  |  |  | 99% |  |  |  | 96% |  |  |  |
|  | Dec Time[%] | 98% |  |  |  | 99% |  |  |  | 98% |  |  |  |

|  |  | AI |  |  |  | RA |  |  |  | LB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving | Total | Average | Min | Max | Total | Average | Min | Max | Total | Average | Min | Max |
| D1 Setting 3 Qualcomm and Sony Europe | Class F | 1.4% | 1.5% | 0.1% | 3.4% | 0.2% | 0.7% | 0.0% | 2.0% | 0.1% | 0.4% | 0.0% | 1.0% |
|  | Class B | 0.1% | 0.1% | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC RGB 444 | 6.8% | 9.2% | 1.5% | 19.4% | 3.1% | 7.5% | 1.2% | 19.4% | 2.1% | 6.6% | 1.0% | 19.3% |
|  | Animation RGB 444 | 0.3% | 0.3% | 0.1% | 0.5% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC YUV 444 | 4.2% | 5.6% | 0.7% | 13.5% | 1.7% | 4.5% | 0.5% | 13.5% | 1.1% | 4.1% | 0.5% | 13.3% |
|  | Animation YUV 444 | 0.4% | 0.3% | 0.1% | 0.7% | 0.2% | 0.1% | 0.0% | 0.4% | 0.2% | 0.1% | 0.0% | 0.3% |
|  | RangeExt | 3.8% | 3.6% | 1.2% | 5.7% | 2.8% | 2.7% | 0.3% | 5.0% | 2.8% | 2.7% | 0.3% | 5.0% |
|  | SC GBR 444 Optional | 18.8% | 21.2% | 13.0% | 26.7% | 12.4% | 20.7% | 11.2% | 27.3% | 11.4% | 21.1% | 10.9% | 27.4% |
|  | SC YUV 444 Optional | 15.4% | 15.7% | 12.4% | 18.1% | 11.8% | 15.2% | 10.7% | 18.6% | 10.9% | 14.5% | 10.5% | 19.6% |
|  | Enc Time[%] | 100% |  |  |  | 100% |  |  |  | 100% |  |  |  |
|  | Dec Time[%] | 98% |  |  |  | 98% |  |  |  | 99% |  |  |  |

### High bit depth

|  |  | All Intra |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y | U | V |
| D1 Setting 1 max = 16 Qualcomm and Sony Europe | 12-bit | -7.1% | -6.8% | -6.8% |
|  | 14-bit | -18.7% | -18.4% | -18.3% |
|  | 16-bit | -28.6% | -28.5% | -28.5% |
|  | Enc Time[%] | 86% |  |  |
|  | Dec Time[%] | 84% |  |  |

|  |  | All Intra |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y | U | V |
| D1 Setting 2 Qualcomm and Sony Europe | 12-bit | -7.1% | -6.8% | -6.8% |
|  | 14-bit | -18.7% | -18.4% | -18.3% |
|  | 16-bit | -28.6% | -28.5% | -28.5% |
|  | Enc Time[%] | 88% |  |  |
|  | Dec Time[%] | 92% |  |  |

|  |  | All Intra |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y | U | V |
| D1 Setting 3 Qualcomm and Sony Europe | 12-bit | -7.2% | -6.8% | -6.9% |
|  | 14-bit | -18.7% | -18.4% | -18.3% |
|  | 16-bit | -28.6% | -28.5% | -28.5% |
|  | Enc Time[%] | 94% |  |  |
|  | Dec Time[%] | 88% |  |  |

# References

- C. Rosewarne, J. Sole, K. Sharman, S.-H. Kim, “HEVC Range Extensions Core Experiment 2 (RCE2): Rice parameter initialisation and update methods,” JCTVC-N1122, Vienna, AT, July 2013.

- A. Saxena, D.-K. Kwon, M. Naccari, C. Pang, “HEVC Range Extensions Core Experiment 3 (RCE3): Intra Prediction techniques,” JCTVC-N1123, Vienna, AT, July 2013.

BF is limited to ENTROPY_CODING_DYNAMIC_RANGE (as defined in JCTVC-N0188)