| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0041v2 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0031v2 |
| --- | --- |

| Title: | REXT/MV-HEVC/SHVC HLS: auxiliary picture layers |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

JCT-VC agreed to take contribution JCTVC-N0063/JCT3V-E0049 “REXT/MV-HEVC/SHVC HLS: Auxiliary picture layers” as a starting point for strong consideration and further development in AHGs on SHVC HLS and RExt. This contribution can be regarded as a revision of JCTVC-N0063/JCT3V-E0049.

Version 2 of the contribution adds a constraint to use monochrome pictures for alpha planes and depth auxiliary pictures and includes an introduction that provides a motivation of the auxiliary picture layer design in contrast to using new NAL unit type(s) for auxiliary pictures.

# Introduction

The support for auxiliary pictures could be realized using either of the following two approaches:

- Specifying new VCL NAL unit type(s) for carriage of auxiliary pictures. This approach was used in H.264/AVC.

- Specifying new scalability dimension for a layer carrying auxiliary pictures. This approach is proposed in this contribution.

It is asserted that specifying auxiliary pictures as layers (approach 2) has the following benefits over approach 1:

- HEVC v1 provides mechanisms to provide CPB HRD parameters separately for the base layer only (containing only “texture” video) and for the entire bitstream (also containing auxiliary pictures). Hence, decoders can choose the CPB HRD parameters that suit best to their use. In other words, decoders can choose either to extract and decode the base-layer sub-bitstream only and use the HRD parameters indicated for the base layer in the decoding process, or to use the HRD parameters indicated for the entire bitstream. If new VCL NAL unit types are used for auxiliary pictures, the CPB HRD parameters always include both texture video and auxiliary pictures.

- HRD parameters can be provided for only the primary picture layers by using layer sets only containing primary picture layers with VPS hrd_parameters( ) and with scalable nesting SEI messages containing buffering period, picture timing and decoding unit information SEI messages. Handling of auxiliary pictures in the HRD with approach 1 requires the specified bitrates and CPB sizes to take auxiliary pictures always into account.

- The bitstream extraction process can be used to exclude auxiliary picture layers from the bitstream for those decoders, which are not able to make use of them.

- The presence of auxiliary layers can be indicated through VPS and hence be used in session negotiation and other systems functionalities.

- No new VCL NAL unit type(s) are needed.

# Proposal

The proposal is split into four technical areas, which are dealt in the respective sub-sections below:

- Using the layer mechanism for multiplexing auxiliary pictures into the bitstream

- Indication of the type of auxiliary pictures

- Constraints for auxiliary pictures

- Indication of the characteristics of auxiliary pictures

The sub-sections provide the motivation and the general idea of the proposed changes. When convenient, also the specification text for the change is provided in the sub-sections. All the proposed specification text changes are provided in the accompanying document.

## Auxiliary picture layers

The support for auxiliary pictures could be realized using either of the following two approaches:

- Specifying new VCL NAL unit type(s) for carriage of auxiliary pictures. This approach was used in H.264/AVC.

- Specifying new scalability dimension for a layer carrying auxiliary pictures. This approach is proposed in this contribution.

It is asserted in this contribution that specifying auxiliary pictures as layers (approach 2) has the following benefits over approach 1:

- HEVC v1 provides mechanisms to provide CPB HRD parameters separately for the base layer only (containing only the “primary” video) and for the entire bitstream (also containing auxiliary pictures). Hence, decoders can choose the CPB HRD parameters that suit best to their use. In other words, decoders can choose either to extract and decode the base-layer sub-bitstream only and use the HRD parameters indicated for the base layer in the decoding process, or to use the HRD parameters indicated for the entire bitstream. If new VCL NAL unit types are used for auxiliary pictures, the CPB HRD parameters always include both texture video and auxiliary pictures.

- The bitstream extraction process can be used to exclude auxiliary picture layers from the bitstream for those decoders, which are not able to make use of them.

- The presence of auxiliary layers can be indicated through VPS and hence be used in session negotiation and other systems functionalities.

- No new VCL NAL unit type(s) are needed.

It is therefore proposed to specify auxiliary pictures as a new scalability dimension.

The proposed specification text is indicated in red below.

Table Error: Reference source not found-1 – Mapping of ScalabiltyId to scalability dimensions

| scalability mask index | Scalability dimension | ScalabilityId mapping |
| --- | --- | --- |
| … |  |  |
| 3 | Auxiliary | AuxId |
| … |  |  |

## Indication of the type of auxiliary pictures

It is proposed that one layer carries only one type of auxiliary pictures and that the value of the scalability dimension, referred to as AuxId, indicates the type of auxiliary pictures within the layer. This design enables the use of the type of auxiliary pictures in systems functions, such as session negotiation. It can be useful for example to negotiate that depth maps are transmitted rather than alpha planes.

It is proposed to specify auxiliary picture types for alpha planes, depth pictures, and chroma enhancement pictures. All of these auxiliary picture types have been discussed and agreed earlier:

- In the Incheon meeting MPEG Requirements met jointly with JCT-VC and JCT-3V and concluded that the same mechanism is intended to be used for both alpha channel and depth map sequences. This position was reinforced in the Vienna meeting (e.g. MPEG N13767 “Disposition of Comments on ISO/IEC 23008-2:201x/PDAM2”).

- As response to contribution JCTVC-N0145 “AHG20: Backwards compatible enhancement of chroma format”, JCT-VC concluded that the proposed functionality is useful and that the use of the auxiliary picture mechanism would be the desirable way out of the solutions suggested in JCTVC-N0145.

The proposed specification text is indicated in red below.

The variable ScalabilityId[ i ][ smIdx ] specifying the identifier of the smIdx-th scalability dimension type of the i-th layer, the variable ViewOrderIdx[ layer_id_in_nuh[ i ] ] specifying the view order index of the i-th layer, and the variable ViewScalExtLayerFlag specifying whether the i-th layer is a view scalability extension layer are derived as follows:

NumViews = 1

for( i = 0; i  <=  vps_max_layers_minus1; i++ ) {

lId = layer_id_in_nuh[ i ]

for( smIdx= 0, j = 0; smIdx < 16; smIdx++ )

if( scalability_mask_flag[ smIdx ] )

ScalabilityId[ i ][ smIdx ] = dimension_id[ i ][ j++ ]

ViewOrderIdx[ lId ] = ScalabilityId[ i ][ 1 ]

if( i > 0  &&  ( ViewOrderIdx[ lId ]  !=  ScalabilityId[ i – 1][ 1 ] ) )

NumViews++

ViewScalExtLayerFlag[ lId ] = ( ViewOrderIdx[ lId ] > 0 )

AuxId[ lId ] = ScalabilityId[ i ][ 3 ]

}

AuxId[ lId ] equal to 0 specifies the layer with nuh_layer_id equal to lId does not contain auxiliary pictures. AuxId[ lId ] greater than 0 specifies the type of auxiliary pictures in layer with nuh_layer_id equal to lId as specified in Table Error: Reference source not found-2.

Table Error: Reference source not found-2 – Mapping of AuxId to the type of auxiliary pictures

| AuxId | Name of AuxId | Type of auxiliary pictures |
| --- | --- | --- |
| 0 | - | Not an auxiliary picture |
| 1 | AUX_ALPHA | Alpha plane |
| 2 | AUX_DEPTH | Depth picture |
| 3 | AUX_CB | Monochrome picture representing a Cb sample array that enhances the Cb sample array of the associated primary picture |
| 4 | AUX_CR | Monochrome picture representing a Cr sample array that enhances the Cr sample array of the associated primary picture |
| 5-255 |  | Reserved |

AuxId[ lId ] shall be in the range of 0 to 4, inclusive, for bitstreams conforming to this version of this Specification. Although the value of AuxId[ lId ] shall be in the range of 0 to 4, inclusive, in this version of this Specification, decoders shall allow values of AuxId[ lId ] in the range of 0 to 255, inclusive.

## Constraints for auxiliary pictures

The following constraints are proposed for auxiliary pictures:

- An auxiliary picture has no normative effect on the decoding process of primary pictures.

- An auxiliary picture is not required to be decoded by any of the profiles of the present amendments.

- No prediction takes place between layers with a different value of AuxId. Prediction between layers of the same value of AuxId may be allowed for example to enable inter-view prediction of multiple depth views coded as auxiliary picture layers.

- An auxiliary picture and an associated primary picture has the same ScalabilityId values (e.g. the same view order index in multiview coding) expect for AuxId. It is proposed that for alpha planes and chroma enhancement pictures, there shall be an associated primary picture, while that is not required for depth pictures. This is to enable unpaired multiview-video-plus-depth use cases, where e.g. depth pictures may represent a viewpoint of a range sensing camera, while the layers containing primary pictures may represent conventional cameras.

- The alpha plane, depth, and chroma enhancement auxiliary picture layers shall contain monochrome pictures.

- The chroma enhancement auxiliary picture layers shall be paired, i.e. there shall be both a Cb layer and a Cr layer.

The proposed specification text is provided in the accompanying document.

## Indication of the characteristics of auxiliary pictures

It is proposed that each type of auxiliary picture may have a different SEI message to indicate characteristics of the auxiliary pictures. A separate SEI message per each auxiliary picture type is justified, as the characteristics of the auxiliary pictures are specific to the type – for example, alpha planes have different parameters than depth maps.

The SEI message approach is justified, as the characteristics of auxiliary pictures do not affect the decoding process. Moreover, the SEI message approach allows changing the characteristics at any picture. For example, in some sequences it is desirable to change depth range parameters picture by picture.

The specification text for the SEI messages is proposed in other contributions.

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).