| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0046 |
| --- | --- |

| Title: | AHG5 and AHG18: Entropy Coding Throughput for High Bit Depths |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Sharman, N. Saunders, J. Gamei Sony BPRL Jays Close, Viables, Basingstoke RG22 4SB, UK | Tel: Email: | +44 1256 355 011 karl.sharman@eu.sony.com nick.saunders@eu.sony.com |
| Source: | Sony Europe Ltd |  |  |

_____________________________

# Abstract

A method is presented for aligning the CABAC process prior to coding bypass data, reportedly allowing easy, simultaneous decoding of multiple bypass bins; the number of CABAC encoded bins is described as being bounded to 25 bins per coefficient group.

The cost of alignment has been reduced through refinements and further reduced at low operating points via conditional application. The losses due to the alignment are observed to be 0.5%-0.6% for the intra Range Extensions test conditions. Losses of 0.2% and less, along with a 7%-8% decrease in decoding time are observed at the more negative QPs at which this throughput tool is targeted.

# Introduction

With higher bit depths and higher operating points (and faster frame rates) comes an increased demand on codec throughput. The current CABAC loop, wherein the evaluation of each iteration is dependent on the outcome of the previous iteration, constitutes an inherent bottleneck in the processing of the bit stream. Through a simple modification to the coding process, bypass-encoded data can be made to appear in raw binary form at the point of decoding. This allows quicker (and parallel) decoding of the bypass data, since the values can now be read directly out of the stream, removing the requirement for long-division when decoding bypass bins.

The modification introduces an additional stage prior to encoding/decoding bypass data for each coefficient group causing the CABAC stream to be “bit-aligned”.

The presented algorithm was previously proposed (see JCTVC-N0190) at the 14th JCT-VC meeting in Vienna. At that time, the decision was made that the algorithm should be studied in an ad-hoc group with a view to adoption at the next meeting.

# HEVC CABAC Entropy Coding of EP bins

The current HEVC CABAC entropy coder decodes one EP bin as follows:

ivlOffset = (ivlOffset<<1) | read_bits(1)

if ( ivlOffset >= ivlCurrRange )

the decoded EP value (binVal) is a 1. ivlOffset -= ivlCurrRange

else

the decoded EP value (binVal) is a 0.

A worked example of this process, decoding two bins is:

let ivlCurrRange=458 and ivlOffset=303, and the next two bits in the stream are both 1.

cycle 1:

ivlOffset = (303 * 2) + 1 = 607

ivlOffset >= 458, so the EP value is 1. ivlOffset = (ivlOffset – 459) = 149

cycle 2:

ivlOffset = (149 * 2) + 1 = 299

ivlOffset < 458, so the EP value is 0 (ivlOffset unchanged)

As can be seen, to decode two EP values, this sequential process is followed, akin to a long division.

However, for the cases where ivlCurrRange happens to be the value 256 at the point of decoding EP bins, then the algorithm can be simplified to the following:

To read n EP bins,

EP bins = top n bits but 1 of ivlOffset.

Apply a left shift of n to ivlOffset (except for most significant bit, which must remain as a 0), and read in n bits from the stream.

# Bit-Alignment Algorithm

This algorithm is equivalent to that presented in JCTVC-M0178 Section 6.2.

If ivlCurrRange is always made to be 256 before encoding/decoding bypass data, the simple decoding process described above in Section 2 could be used to read multiple bypass bins in parallel. For example, all of the sign bits for a coefficient group or the worst-case exponential-Golomb escape code could be read and decoded in a single operation.

The only step required to bit-align the stream is:

ivlCurrRange = 256		(in both the encoder and decoder)

As already stated, the result of this is that the encoding and decoding process is significantly simplified; allowing the binary data to be read directly out of ivlOffset in raw form, and therefore many bits at a time can simultaneously be processed by the decoder.

However, note that the act of setting ivlCurrRange to 256 incurs a loss of, on average, 0.55 bits:

if ivlCurrRange was already 256, there is no loss;
if ivlCurrRange was 510, then approximately 1 bit will be lost;
the average over all valid values of ivlCurrRange is 0.55 bits (the loss is not exactly linear with the value of ivlCurrRange).

Applying this algorithm prior to each coefficient group’s bypass (and sign) data would give a noticeable loss at the normal HEVC 8-bit operating points; however, when considering higher bit depths where there is considerably more bypass data, this loss is less significant, and the requirement to increase throughput is more important. A flag has been added to the sequence parameter set so that the encoder may disable alignment where the operating point is low enough for it not to be needed.

The effects on coding efficiency of this algorithm can be seen in the accompanying Excel spreadsheets; graphs are not included here as the differences are negligible.

To increase the range of operating points where this alignment can be used, the algorithm is further developed, as described in the next section.

## Algorithm Refinement

The algorithm described in Section 3 has been further refined to reduce the average cost from 0.55 bits to 0.30 bits (and further).

In the above algorithm, ivlCurrRange was always aligned to 256. However, as an alternative, ivlCurrRange can be aligned to one of a number N of predetermined values V={vi}. If the values are regularly spaced over the range [256, 512), (for example V={256, 384} for N=2), then the loss-per-alignment is approximately (0.5 / N) + 0.05.

Since ivlCurrRange is no longer a simple power of 2, this would still require division, although the use of look-up tables could be employed. However, to circumvent this need for division, the bin (or bins) immediately following the alignment process can be coded using (unequal) symbol ranges that are powers of two. This would not result in any further loss in bit efficiency (i.e. the loss-per-alignment is still only (0.5 / N) + 0.05).

The proposed refinement is to align to 256 and 384 (N=2): when ivlCurrRange is less than 384 the same process as in Section  is used, otherwise the following process is used:

When aligning to 384, the symbol ranges for [0,1] for the subsequent bin are [256,128].
If a 0 is coded, ivlCurrRange is set to 256, making the cost to encode the bin 0.5 bits.
If a 1 is coded, the range is set to 128 (and 256 is added to ivlOffset) and the system is renormalised (again, ivlCurrRange becomes 256), making the cost to encode the bin 1.5 bits.

Since 0 and 1 are expected with equal probability, the average cost to encode the bin immediately following alignment is still 1 bit. For equi-probable (EP) bins, 1 bit is expected per bin.

This proposed refinement is illustrated in Figure 1, which gives a loss of 0.30 (=(0.5 / N) + 0.05) bits for alignment. This loss is confirmed by experiment, and can be explored in the accompanying Excel spreadsheet.

Figure 1 - Bit-alignment procedure and subsequent symbol ranges for alignment case where N = 2

The algorithm for the decoder to align using the N=2 refinement can be expressed as:

if (ivlCurrRange >= 384)

if (ivlOffset < 256)		nextEPBitIsZero = true			[Note ]

else 				ivlOffset -= 128				[Note ]

ivlCurrRange = 256

In addition, Figure 2 illustrates the case where N=4 (i.e. V={256, 320, 384, 448}), which gives an average loss of 0.18 bits per alignment. A similar decoder algorithm can be constructed, although the implementation complexity is increased, and is therefore not recommended.

Figure 2 - Bit-alignment procedure and subsequent symbol ranges for alignment case where N = 4

## Conditional Application of Alignment

As noted in Section 3, the cost of alignment becomes more significant at lower operating points (higher QPs). To mitigate this problem, the alignment algorithm can be modified to apply only where a significant quantity of bypass data is present.

While it is not possible for a decoder to know in advance exactly how many bypass bins are to be decoded, an estimate can be made based on the usage of escape codes. Escape codes make up a significant part of the bypass data for a coefficient group due to the unary-coded nature of the Golomb-Rice/exponential-Golomb prefix. At low operating points, the majority of the coefficients are small and escape codes are comparatively rarely used, whereas at high operating points, escape codes are much more prevalent.

In this conditional method, alignment is enabled for each coefficient group if any escape codes are present in that group.

This system limits the number of CABAC encoded bins per 16-coefficient group to just:

16 (significance map) + 8 (greater-than-one map) + 1 (greater-than-two flag) = 25

if there are escape coded values, or

16 (significance map) + 8 (greater-than-one map) + 1 (greater-than-two flag) + 16 EP (sign bits) = 41

if there are no escape coded values.

At standard RExt (AHG5) operating points, the BD-rate changes when using conditional alignment are illustrated in Table 1:

Table 1 - Effects of conditional alignment at standard RExt intra operating points. Intra results are presented here – inter results are similar.

For AHG18 high bit depth test conditions (as defined in RCE2), the BD-rate changes are illustrated in Table 2:

Table 2 - Effects of conditional alignment at standard RCE2 AHG18 test conditions

# Conclusion

A method has been presented for aligning the CABAC process prior to coding bypass data, allowing easy, simultaneous decoding of multiple bypass bins. The cost of alignment has been reduced through refinements presented in Section 3.1 and further reduced at low operating points via conditional application.

It is proposed that the refined alignment method with N = 2 presented in Section 3.1 and the alignment conditions presented in Section 3.2 be adopted into HEVC Range Extensions. The losses due to this alignment system are 0.5%-0.6% for the AHG5 test conditions (see Table 1), though the encoder may opt not to apply alignment at these operating points. For AHG18 high bit depth test conditions (see Table 2), losses of 0.2% and less, along with a 7%-8% decrease in decoding time are observed at the more negative QPs at which this throughput tool is targeted.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix A: Working Draft Text

Changes for basic alignment (see Section 3) are shown in yellow.

Additional changes for refined alignment with N=2 (see Section 3.1) are shown in green.

Additional changes for conditional alignment (see Section 3.2) are shown in cyan.

#### 7.3.2.2	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_video_parameter_set_id | u(4) |
| … |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| align_cabac_before_bypass_data_flag | u(1) |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| … | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

7.3.8.11	Residual coding syntax

| residual_coding( x0, y0, log2TrafoSize, cIdx ) { | Descriptor |
| --- | --- |
| … |  |
| lastScanPos = 16 |  |
| escapeDataPresent = 0 |  |
| lastSubBlock = ( 1 << ( log2TrafoSize − 2 ) ) * ( 1 << ( log2TrafoSize − 2 ) ) − 1 |  |
| … |  |
| for( i = lastSubBlock; i >= 0; i− − ) { |  |
| … |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ( xS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 1 ] |  |
| if( sig_coeff_flag[ xC ][ yC ] ) { |  |
| if( numGreater1Flag < 8 ) { |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| numGreater1Flag++ |  |
| if( coeff_abs_level_greater1_flag[ n ] && lastGreater1ScanPos = = −1 ) |  |
| lastGreater1ScanPos = n |  |
| else if ( coeff_abs_level_greater1_flag[ n ]) |  |
| escapeDataPresent = 1 |  |
| } |  |
| else |  |
| escapeDataPresent = 1 |  |
| if( lastSigScanPos = = −1 ) |  |
| lastSigScanPos = n |  |
| firstSigScanPos = n |  |
| } |  |
| } |  |
| signHidden = ( lastSigScanPos − firstSigScanPos > 3 && !cu_transquant_bypass_flag ) |  |
| if( lastGreater1ScanPos != −1 ) { |  |
| coeff_abs_level_greater2_flag[ lastGreater1ScanPos ] | ae(v) |
| if ( coeff_abs_level_greater2_flag[ lastGreater1ScanPos ] ) |  |
| escapeDataPresent = 1 |  |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ( xS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 1 ] |  |
| if( sig_coeff_flag[ xC ][ yC ] && ( !sign_data_hiding_enabled_flag ¦ ¦ !signHidden ¦ ¦ ( n != firstSigScanPos ) ) ) |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| numSigCoeff = 0 |  |
| sumAbsLevel = 0 |  |
| … |  |
| } |  |
| } |  |

NOTE – when alignment is enabled and escapeDataPresent is 1, the entropy coder is limited to coding 16 (significance map) + 8 (greater-than-one map) + 1 (greater-than-two flag) = 25 CABAC bins per coefficient group; all escape bins are available raw at the point of decoding. When alignment is enabled and escapeDataPresent is 0, the entropy coder is limited to coding 25 CABAC bins and 16 EP CABAC bins per coefficient group.

7.4.3.2	Sequence parameter set RBSP semantics

…

bit_depth_chroma_minus8 specifies the bit depth of the samples of the chroma arrays BitDepthC and the value of the chroma quantization parameter range offset QpBdOffsetC as follows:

BitDepthC      = 8 + bit_depth_chroma_minus8		(7-1)

QpBdOffsetC = 6 * bit_depth_chroma_minus8		(7-2)

bit_depth_chroma_minus8 shall be in the range of 0 to 6, inclusive.

align_cabac_before_bypass_data_flag specifies the use of aligned bypass coding during the entropy coding process.

When align_cabac_before_bypass_data_flag is not present, it is inferred to be equal to 0.

…

[ Note that existing profiles would be amended to include the clause “The syntax element align_cabac_before_bypass_data_flag shall not be present in the bit stream.” ]

9.3.4	Decoding process flow

9.3.4.1	General

Inputs to this process are all bin strings of the binarization of the requested syntax element as specified in subclause Error: Reference source not found.

Output of this process is the value of the syntax element.

This process specifies how each bin of a bin string is parsed for each syntax element. After parsing each bin, the resulting bin string is compared to all bin strings of the binarization of the syntax element and the following applies:

–	If the bin string is equal to one of the bin strings, the corresponding value of the syntax element is the output.

–	Otherwise (the bin string is not equal to one of the bin strings), the next bit is parsed.

While parsing each bin, the variable binIdx is incremented by 1 starting with binIdx being set equal to 0 for the first bin.

The parsing of each bin is specified by the following two ordered steps:

1.	The derivation process for ctxTable, ctxIdx, and bypassFlag, and alignFlag as specified in subclause  is invoked with binIdx as input and ctxTable, ctxIdx, and bypassFlag, and alignFlag as outputs.

2.	The arithmetic decoding process as specified in subclause Error: Reference source not found is invoked with ctxTable, ctxIdx, and bypassFlag, and alignFlag as inputs and the value of the bin as output.

9.3.4.2	 Derivation process for ctxTable, ctxIdx and, bypassFlag and alignFlag

9.3.4.2.1	General

Input to this process is the position of the current bin within the bin string, binIdx.

Outputs of this process are ctxTable, ctxIdx, and bypassFlag and alignFlag.

The values of ctxTable, ctxIdx, and bypassFlag and alignFlag are derived as follows based on the entries for binIdx of the corresponding syntax element in Table 3:

- If the entry in Table 3 is not equal to "bypass", "bypass_align", "terminate", and "na", the values of binIdx are decoded by invoking the DecodeDecision process as specified in subclause Error: Reference source not found and the following applies:

- ctxTable is specified in Error: Reference source not found.

- The variable ctxInc is specified by the corresponding entry in Table 3 and when more than one value is listed in Table 3 for a binIdx, the assignment process for ctxInc for that binIdx is further specified in the subclauses given in parenthesis.

- The variable ctxIdxOffset is specified by the lowest value of ctxIdx in Error: Reference source not found depending on the current value of initType.

- ctxIdx is set equal to the sum of ctxInc and ctxIdxOffset.

- bypass Flag is set equal to 0.

- alignFlag is set equal to 0.

- Otherwise, if the entry in Table 3 is equal to "bypass", the values of binIdx are decoded by invoking the DecodeBypass process as specified in subclause  and the following applies:

- ctxTable is set equal to 0.

- ctxIdx is set equal to 0.

- bypassFlag is set equal to 1.

- alignFlag is set equal to 0.

- Otherwise, if the entry in Table 9-37 is equal to "bypass_align", the values of binIdx are decoded by invoking the DecodeBypass process as specified in subclause 9.3.4.3.4 and the following applies:

  - ctxTable is set equal to 0.

  - ctxIdx is set equal to 0.

  - bypassFlag is set equal to 1.

  - If escapeDataPresent is equal to 1 then alignFlag is set equal to the value of align_cabac_before_bypass_data_flag.

  - Otherwise if escapeDataPresent is not equal to 1, alignFlag is set equal to 0.

- Otherwise, if the entry in Table 3 is equal to "terminate", the values of binIdx are decoded by invoking the DecodeTerminate process as specified in subclause Error: Reference source not found and the following applies:

- ctxTable is set equal to 0.

- ctxIdx is set equal to 0.

- bypassFlag is set equal to 0.

- alignFlag is set equal to 0.

- Otherwise (the entry in Table 3 is equal to "na"), the values of binIdx do not occur for the corresponding syntax element.

| Table 3 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| end_of_slice_segment_flag | terminate | na | na | na | na | na |
| … | … | … | … | … | … | … |
| coeff_abs_level_remaining[ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| coeff_sign_flag[ ] | bypass | na | na | na | na | na |
| coeff_abs_level_remaining[ ] | bypass_align | bypass_align | bypass_align | bypass_align | bypass_align | bypass_align |
| coeff_sign_flag[ ] | bypass_align | na | na | na | na | na |

9.3.4.3.1	General

Inputs to this process are ctxTable, ctxIdx, and bypassFlag and alignFlag, as derived in subclause , and the state variables ivlCurrRange and ivlOffset of the arithmetic decoding engine.

Output of this process is the value of the bin.

Figure 9-3 illustrates the whole arithmetic decoding process for a single bin. For decoding the value of a bin, the context index table ctxTable and the ctxIdx are passed to the arithmetic decoding process DecodeBin( ctxTable, ctxIdx ), which is specified as follows:

–	If bypassFlag is equal to 1, DecodeBypass( alignFlag ) as specified in subclause  is invoked.

–	Otherwise, if bypassFlag is equal to 0, ctxTable is equal to 0, and ctxIdx is equal to 0, DecodeTerminate( ) as specified in subclause Error: Reference source not found is invoked.

–	Otherwise (bypassFlag is equal to 0 and ctxTable is not equal to 0), DecodeDecision( ) as specified in subclause Error: Reference source not found is invoked.

Figure 9-3 – Overview of the arithmetic decoding process for a single bin (informative)

9.3.4.3.4	Bypass decoding process for binary decisions

Inputs to this process are bits from slice segment data and the variables ivlCurrRange, and ivlOffset and alignFlag.

Outputs of this process are the updated variables ivlOffset and ivlCurrRange and the decoded value binVal.

The bypass decoding process is invoked when bypassFlag is equal to 1. Figure 9-4 shows a flowchart of the corresponding process.

If alignFlag is equal to 1:

–	If ivlCurrRange is greater than or equal to 384, then

–	If ivlOffset is greater than or equal to 256, ivlOffset is decremented by 128.

–	Otherwise (ivlOffset is less than 256), the variable binVal is set equal to 0, ivlCurrRange is set to 256 and the DecodeBypass process is finished.

–	ivlCurrRange is set to 256.

ThenFirst, the value of ivlOffset is doubled, i.e. left-shifted by 1 and a single bit is shifted into ivlOffset by using read_bits( 1 ). Then, the value of ivlOffset is compared to the value of ivlCurrRange and further steps are specified as follows:

–	If ivlOffset is greater than or equal to ivlCurrRange, the variable binVal is set equal to 1 and ivlOffset is decremented by ivlCurrRange.

–	Otherwise (ivlOffset is less than ivlCurrRange), the variable binVal is set equal to 0.

The bitstream shall not contain data that result in a value of ivlOffset being greater than or equal to ivlCurrRange upon completion of this process.

NOTE – when ivlCurrRange is 256, the process can be simplified: ivlOffset and the bit-stream can be considered as a shift register, and binVal is the most-significant-bit-but-1 (the most significant bit is always 0 due to above restriction of ivlOffset being less than ivlCurrRange).

Figure 9-4 – Flowchart of bypass decoding process

9.3.5.5	Bypass encoding process for binary decisions (informative)

This subclause does not form an integral part of this Specification.

Inputs to this process are the variables binVal, alignFlag, ivlLow, ivlCurrRange, bitsOutstanding, and BinCountsInNalUnits.

Output of this process is a bit written to the RBSP and the updated variables ivlLow, bitsOutstanding, and BinCountsInNalUnits.

This encoding process applies to all binary decisions with bypassFlag equal to 1. Renormalization is included in the specification of this process as given in Figure 9-5.

Figure 9-5 – Flowchart of encoding bypass

This stage actually decodes a 0 from the bit-stream, which will be the next bit returned by the equi-probable bit decoding process.

The full process to decode a 1 with these symbol ranges would be to subtract 256 (the symbol range for 0) from ivlOffset, set ivlCurrRange to 128 (the symbol range for 1) and renormalise. At this point, the 1 is partially decoded by subtracting half of the symbol range for 0. The remaining half-subtraction and renormalisation are postponed until the 1 is actually decoded later when ivlCurrRange = 256.