| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013; Document: JCTVC-Oxxxx. | Document: JCTVC-O0049 |
| --- | --- |

| Title: | RCE3: Nearest-neighbor intra prediction for screen content video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Haoming Chen, Ankur Saxena and Felix C. Fernandes | Email: Tel: | haoming.c@sta.samsung.com, asaxena@sta.samsung.com, ffernandes@sta.samsung.com 1-972-761-7761 |
| Source: | Samsung Electronics, Co., Ltd. |  |  |

_____________________________

# Abstract

In intra prediction, some oblique modes use the interpolated sample as a predictor, which works well for natural content. However, for the computer-generated screen content, the smoothed predictor is not good, as there are sharp edges, and features. In this contribution, bilinear interpolation is disabled for the intra prediction based on the variance of the reference samples. The proposed scheme achieves bit-rate savings of 1.2%, 0.9%, and 0.6% for All Intra, Random Access, and Low Delay settings for lossless screen content YUV444 coding. For the lossy scenario, BD-Rate savings of 1.8%, 1.6% and 1.3% are achieved for All Intra, Radom Access and Low Delay-high tier settings.

# Introduction

In unified angular intra prediction method in HEVC, for some angular modes, bilinear interpolation of neighboring reference samples is used as the predictor for pixels as shown in Fig. 1. This kind of smoothening scheme is suitable for nature content since motion; noise and digital image sensor tends to smooth edges in such content.

Figure 1: Bilinear interpolation of angular intra prediction. X is a sample in a PU, A and B are two neighbouring reference samples along the angle of the prediction.

However, for the computer-generated screen content or graphics content, the sharp edges are preserved. Therefore, the smoothed intra prediction may not be suitable. In this proposal, we adaptively disable the interpolation scheme in the intra prediction based on the variance of the reference samples. The results show that our proposed scheme significantly improved the intra prediction for screen content video, thereby improving the compression efficiency.

# Proposed Scheme

The proposed scheme is shown in Fig. 2. We replace the bilinear interpolation with a change as follows:

pred(x) = A if d < 16, otherwise B.

Figure2: Non-interpolation intra prediction method. The predictor of sample X is the copy of the reference that is closer to the predicted position (dotted circle in the figure). Dashed indicates the prediction direction.

Note that this method does not affect the horizontal, vertical and three strictly diagonal modes since for these modes, the predictor is from the integral pixel location. The rationale for introducing such an enhanced prediction scheme is that for screen content, there are sharp edges, which do not generally exist in natural camera-captured video content. As an example, we show blocks from natural and screen content, which clearly shows that screen content has sharp edges.

Figure 3: Blocks of natural and screen coding video content

Obviously, the method in its current form might not be suitable for the natural content, so a strategy should be used to indicator whether the method should be applied or not. We propose two different approaches: Rate-Distortion based; and threshold based to decide whether or not to perform the bilinear interpolation.

Approach 1: Rate Distortion Based

At the encoder side, both the bilinear interpolation and the non-interpolation methods are tested for each mode, and their Sum of Absolute Difference (SAD) cost is computed. The best mode out of bilinear interpolation (coming from oblique mode) and the proposed nearest neighbor intra mode combination, which results in the minimum rate-distortion cost, will be selected at the encoder. To indicate the type of mode used at the decoder sider, one flag is included in the bit stream for each (interpolation, nearest neighbor intra mode) combination. The following operations are also employed at the encoder:

- Skip the search between choosing the normal HEVC mode, and proposed nearest neighbor prediction mode for the Planar, DC, Horizontal, Vertical and three diagonal modes, since for these modes, no bilinear interpolation is performed. Since the intra mode information is derived prior to interpolation information in the decoder side, no extra bit (flag) is needed for the Planar, DC, Horizontal, Vertical and three diagonal modes.

- For the chroma components, the same interpolation can be used as corresponding luma component, when the “derived mode” (same as luma intra mode) is selected. Hence, no extra flag is needed for chroma components to distinguish between the bilinear interpolation mode as in HEVC, or proposed nearest neighbor prediction mode.

When encoding the flag of interpolation, a syntax adaptive binary arithmetic coding (SBAC) method can be used. For the initialization of the contexts for the “flag” for enhanced intra prediction, the following table is used as initialization for various frames (0 is Intra; 1 is normal Inter P frame; and 2 is Bi-directional B Inter frame).

Table A – Values of initValue for ctxIdx of intra_luma_pred_interpolation_mode

| Initialization variable | ctxIdx of prev_intra_luma_pred_mode |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 184 | 154 | 183 |

Approach 2:

- We can use a very simplistic “threshold” between two neighboring pixels to decide whether to use bilinear interpolation. For example, in Fig. 2, we can have the following criteria for deciding for the predictor of pixel X:

if (abs (A-B) >= thr )

prediction(X) = A if d < 16, otherwise B.

else

Retain bilinear interpolation method using pixels A and B from HEVC for prediction of pixel X.

In the above, abs denotes the absolute value, and “thr” is some threshold. We use thr value of 128 for 8-bit video, and 512 for 10-bit video sequences.

# Test settings and experimental results

In this section, coding results for the proposed scheme are presented according to the lossless and lossy test conditions used as stipulated in RCE3 [1]. The anchor is the range extensions software HM 12.0-RExt-4.1 for lossless and lossy settings respectively.

We first enumerate the tests that we have performed. Detailed results are provided in attached excel sheets, while the summary is given next in the tables 1 to 8.

- Test 1: Lossless coding results for Rate-Distortion based technique on all blocks sizes from 4×4 to 64x64.

- Test 2: Lossless coding results for Rate-Distortion based technique applied on block size 4×4 only.

- Test 3: Lossless coding results for Threshold based technique on all blocks sizes from 4×4 to 64x64

- Test 4: Lossless coding results for Threshold based technique on block sizes 4x4 only.

- Test 5: Lossy coding results for Rate-Distortion based technique on all blocks sizes from 4×4 to 64x64.

- Test 6: Lossy coding results for Rate-Distortion based technique applied on block size 4×4 only.

- Test 7: Lossy coding results for Threshold based technique on all blocks sizes from 4×4 to 64x64.

- Test 8: Lossy coding results for Threshold based technique on block sizes 4x4 only.

According to our analysis, applying the proposed technique on only 4×4 block achieves most of the gain.

Table 1: Lossless coding results Rate-Distortion based technique on all blocks sizes

Table 2: Lossless coding results for Rate-Distortion based technique applied on block size 4×4 only

.

Table 3: Lossless coding results for Threshold based technique on all blocks sizes

Table 4: Lossless coding results for Threshold based technique on block sizes 4x4 only.

Table 5: Lossy coding results for Rate-Distortion based technique on all blocks sizes

Table 6: Lossy coding results for Rate-Distortion based technique applied on block size 4×4 only.

Table 7: Lossy coding results for Threshold based technique on all blocks sizes

Table 8: Lossy coding results for Threshold based technique on blocks size 4x4

# Complexity Considerations

For the Rate-distortion scheme, the additional encoder complexity is in the search of best intra prediction mode. At the decoder, using the transmitted “flag”, the decoder decides whether to perform traditional bilinear interpolation in HEVC, or perform the proposed nearest neighbor interpolation. For each pixel, there would be a condition check to find the nearest neighbor, but the bilinear interpolation process would be skipped.

For the threshold based scheme, the additional complexity is negligible at both the encoder and decoder. Specifically, for each pixel when the proposed scheme is used, there are two condition checks to find the prediction. However, the bilinear interpolation operations from pixels A and B are skipped in this case.

For throughput, in both R-D, and Threshold based scheme, there is no difference, as the scheme can be applied on all the pixels in a parallel fashion at both the encoder and the decoder.

# Conclusion

In this contribution, the standard bilinear interpolation in intra prediction is skipped adaptively, and a nearest neighbor interpolation technique is used. The coding efficiency of screen content video is improved by 1.2% in lossless coding and 1.8% in lossy coding All Intra scenario for Screen Content YUV444 coding. We recommend adopting this proposal in the committee draft of the HEVC-Range Extensions.

# References

- A. Saxena, D. Kwon, M. Naccari and C. Pang, “HEVC Range Extensions Core Experiment 3 (RCE3): Intra Prediction techniques,” JCTVC-N1123, Vienna, Austria, July 2013.

# Working Draft Text

Please refer to attached files: WD_Threshold for WD Text of Threshold based technique; and WD_RD for WD text of R-D based scheme.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd., may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).