| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0052 |
| --- | --- |

| Title: | SCE1: Results of Test 1.2 on the selection of fixed filters for upsampling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Koohyar Minoo, David Baylon 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1 (858) 404-2521 koohyar.minoo@arrisi.com david.baylon@arrisi.com |
| Source: | ARRIS Group, Inc. |  |  |

_____________________________

# Abstract

This document reports the results using fixed filters proposed in [1] for upsampling to support an arbitrary scalability ratio.  In the experiments, JSVM and CTC BL sequences were used for 2x, ~1.75x, and 1.5x scalability.  For the case of JSVM BL, the average luma (chroma) BD-rate using the proposed filters compared to the anchor is 0.00% (-0.03%), -0.01% (-0.15%), and 0.16% (-0.05%) for 2x, ~1.75x, and 1.5x, respectively.  For the case of CTC BL, the average luma (chroma) BD-rate using the proposed filters compared to the anchor is 0.00% (-0.02%), -0.04% (-0.15%), and 0.36% (0.12%) for 2x, ~1.75x, and 1.5x, respectively.  The loss for the 1.5x case is expected as the proposed fixed filters have 1/16 (luma) uniform phase spacing and do not favor the 2/3 phase offset.

# Introduction

For upsampling filtering the following filters [1] for the selected luma and chroma phases are tested.

Luma:

Phase 2    	{-1     3    -7    63     8    -3     1     0}

Phase 5    	{-1     4   -11    54    23    -7     3    -1}

Phase 6    	{-1     3    -9    47    31   -10     4    -1}

Phase 10 	{-1     4   -10   31    47     -9     3     -1}

Phase 11 	{-1     3     -7   23    54    -11    4    -1}

Phase 14 	{0     1     -3     8    63      -7    3    -1}

Chroma:

Phase 1    	{0    58     8    -2}

Phase 3    	{-2    53    17    -4}

Phase 5    	{-4    48    24    -4}

Phase 6    	{-4    44    28    -4}

Phase 7    	{-6    44    30    -4}

Phase 9    	{-4   30   44   -6}

Phase 10 	{-4   28   44   -4}

Phase 11 	{-4   24   48   -4}

Phase 13 	{-4   17   53   -2}

Phase 15 	{-2    8    58   0}

# Simulation results

Results of the proposed filters with Test 1.2 compared to SHM-3.0.1 are shown in Table 1 and Table 2 for JSVM BL and CTC BL, respectively.  For the case of JSVM BL, the average luma (chroma) BD-rate using the proposed filters compared to the anchor is 0.00% (-0.03%), -0.01% (-0.15%), and 0.16%	 (-0.05%) for 2x, ~1.75x, and 1.5x, respectively.  For the case of CTC BL, the average luma (chroma) BD-rate using the proposed filters compared to the anchor is 0.00% (-0.02%), -0.04% (-0.15%), and 0.36% (0.12%) for 2x, ~1.75x, and 1.5x, respectively.  The loss for the 1.5x case is expected as the proposed fixed filters have 1/16 (luma) uniform phase spacing and do not favor the 2/3 phase offset.

|  | AI HEVC 2x |  |  | AI HEVC ~1.75x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.01% | -0.03% | -0.03% | -0.04% | -0.21% | -0.18% |  |  |  |
| Class B | 0.01% | -0.03% | -0.02% | -0.03% | -0.16% | -0.19% | 0.23% | -0.01% | -0.01% |
| Overall (Test vs Ref) | 0.01% | -0.03% | -0.03% | -0.03% | -0.17% | -0.19% | 0.23% | -0.01% | -0.01% |
| Overall (Test vs single layer) | 14.1% | 16.4% | 16.1% | 15.5% | 17.0% | 16.5% | 13.5% | 13.1% | 12.9% |
| Overall (Ref vs single layer) | 14.1% | 16.4% | 16.1% | 15.5% | 17.2% | 16.7% | 13.3% | 13.2% | 13.0% |
| EL only (Test vs Ref) | 0.0% | 0.0% | 0.0% | -0.1% | -0.3% | -0.3% | 0.5% | 0.2% | 0.2% |
| Enc Time[%] | 90.1% |  |  | 80.8% |  |  | 91.9% |  |  |
| Dec Time[%] | 97.6% |  |  | 61.8% |  |  | 99.0% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x |  |  | RA HEVC ~1.75x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.01% | 0.03% | -0.01% | -0.01% | -0.15% | -0.11% |  |  |  |
| Class B | 0.00% | -0.03% | -0.03% | -0.01% | -0.16% | -0.17% | 0.18% | 0.00% | 0.04% |
| Overall (Test vs Ref) | 0.01% | -0.01% | -0.03% | -0.01% | -0.16% | -0.15% | 0.18% | 0.00% | 0.04% |
| Overall (Test vs single layer) | 19.4% | 32.8% | 31.2% | 20.3% | 33.0% | 31.7% | 17.5% | 28.6% | 28.7% |
| Overall (Ref vs single layer) | 19.4% | 32.8% | 31.3% | 20.3% | 33.3% | 31.9% | 17.3% | 28.6% | 28.7% |
| EL only (Test vs Ref) | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | -0.2% | 0.3% | 0.1% | 0.2% |
| Enc Time[%] | 108.6% |  |  | 82.5% |  |  | 109.5% |  |  |
| Dec Time[%] | 97.8% |  |  | 51.4% |  |  | 99.3% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC ~1.75x |  |  | LD-B HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.01% | 0.08% | -0.02% | 0.01% | -0.13% | -0.06% |  |  |  |
| Class B | 0.00% | -0.15% | 0.07% | -0.01% | -0.14% | -0.14% | 0.13% | -0.02% | -0.18% |
| Overall (Test vs Ref) | 0.00% | -0.08% | 0.04% | -0.01% | -0.13% | -0.12% | 0.13% | -0.02% | -0.18% |
| Overall (Test vs single layer) | 28.0% | 37.7% | 38.6% | 28.7% | 38.0% | 39.0% | 25.1% | 31.9% | 34.2% |
| Overall (Ref vs single layer) | 28.0% | 37.8% | 38.5% | 28.7% | 38.2% | 39.2% | 24.9% | 32.0% | 34.5% |
| EL only (Test vs Ref) | 0.0% | -0.1% | 0.0% | 0.0% | -0.2% | -0.2% | 0.3% | 0.1% | -0.1% |
| Enc Time[%] | 105.0% |  |  | 83.2% |  |  | 104.2% |  |  |
| Dec Time[%] | 94.8% |  |  | 51.5% |  |  | 94.5% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

|  | LD-P HEVC 2x |  |  | LD-P HEVC ~1.75x |  |  | LD-P HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.01% | 0.11% | -0.01% | -0.02% | -0.12% | -0.15% |  |  |  |
| Class B | -0.01% | -0.11% | -0.07% | -0.01% | -0.14% | -0.14% | 0.11% | -0.05% | -0.14% |
| Overall (Test vs Ref) | 0.00% | -0.05% | -0.05% | -0.01% | -0.14% | -0.14% | 0.11% | -0.05% | -0.14% |
| Overall (Test vs single layer) | 26.1% | 36.8% | 37.8% | 26.6% | 37.0% | 38.3% | 22.9% | 31.3% | 34.1% |
| Overall (Ref vs single layer) | 26.1% | 36.9% | 37.9% | 26.6% | 37.2% | 38.4% | 22.8% | 31.4% | 34.3% |
| EL only (Test vs Ref) | 0.0% | 0.0% | -0.1% | -0.1% | -0.2% | -0.2% | 0.2% | 0.0% | -0.1% |
| Enc Time[%] | 101.4% |  |  | 82.3% |  |  | 103.5% |  |  |
| Dec Time[%] | 94.9% |  |  | 53.7% |  |  | 95.3% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Table 1.  Performance with JSVM BL

|  | AI HEVC 2x |  |  | AI HEVC ~1.75x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.01% | -0.04% | -0.04% | -0.09% | -0.31% | -0.28% |  |  |  |
| Class B | 0.01% | -0.03% | -0.03% | -0.06% | -0.21% | -0.24% | 0.48% | 0.25% | 0.26% |
| Overall (Test vs Ref) | 0.01% | -0.03% | -0.03% | -0.07% | -0.24% | -0.25% | 0.48% | 0.25% | 0.26% |
| Overall (Test vs single layer) | 12.8% | 14.8% | 14.5% | 13.9% | 15.0% | 14.5% | 11.0% | 10.1% | 9.6% |
| Overall (Ref vs single layer) | 12.8% | 14.9% | 14.6% | 14.0% | 15.3% | 14.8% | 10.5% | 9.8% | 9.3% |
| EL only (Test vs Ref) | 0.0% | 0.0% | 0.0% | -0.2% | -0.4% | -0.4% | 1.3% | 1.1% | 1.1% |
| Enc Time[%] | 92.6% |  |  | 90.6% |  |  | 91.6% |  |  |
| Dec Time[%] | 101.2% |  |  | 97.2% |  |  | 99.4% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x |  |  | RA HEVC ~1.75x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.01% | 0.08% | -0.05% | -0.04% | -0.28% | -0.17% |  |  |  |
| Class B | 0.01% | -0.01% | 0.00% | -0.03% | -0.09% | -0.08% | 0.37% | 0.15% | 0.07% |
| Overall (Test vs Ref) | 0.01% | 0.01% | -0.01% | -0.03% | -0.15% | -0.11% | 0.37% | 0.15% | 0.07% |
| Overall (Test vs single layer) | 19.0% | 33.1% | 31.8% | 19.5% | 33.0% | 31.9% | 16.6% | 29.0% | 29.3% |
| Overall (Ref vs single layer) | 19.0% | 33.1% | 31.8% | 19.5% | 33.2% | 32.1% | 16.2% | 28.9% | 29.2% |
| EL only (Test vs Ref) | 0.0% | 0.0% | 0.0% | -0.1% | -0.2% | -0.2% | 0.8% | 0.6% | 0.5% |
| Enc Time[%] | 108.4% |  |  | 106.2% |  |  | 107.7% |  |  |
| Dec Time[%] | 97.9% |  |  | 95.1% |  |  | 98.9% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC ~1.75x |  |  | LD-B HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.02% | 0.01% | 0.00% | -0.03% | -0.14% | -0.15% |  |  |  |
| Class B | -0.01% | -0.02% | -0.05% | -0.02% | -0.07% | -0.18% | 0.28% | 0.07% | 0.02% |
| Overall (Test vs Ref) | 0.00% | -0.01% | -0.04% | -0.02% | -0.09% | -0.18% | 0.28% | 0.07% | 0.02% |
| Overall (Test vs single layer) | 28.3% | 38.9% | 39.7% | 28.5% | 38.5% | 39.6% | 25.1% | 33.2% | 36.0% |
| Overall (Ref vs single layer) | 28.3% | 38.9% | 39.7% | 28.5% | 38.6% | 39.9% | 24.8% | 33.2% | 36.0% |
| EL only (Test vs Ref) | 0.0% | 0.0% | 0.0% | -0.1% | -0.2% | -0.3% | 0.7% | 0.4% | 0.4% |
| Enc Time[%] | 107.0% |  |  | 107.2% |  |  | 104.2% |  |  |
| Dec Time[%] | 94.3% |  |  | 94.5% |  |  | 92.1% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

|  | LD-P HEVC 2x |  |  | LD-P HEVC ~1.75x |  |  | LD-P HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.02% | -0.02% | -0.10% | -0.04% | -0.29% | -0.23% |  |  |  |
| Class B | 0.00% | 0.01% | -0.06% | -0.03% | -0.01% | -0.11% | 0.30% | 0.07% | 0.07% |
| Overall (Test vs Ref) | 0.00% | 0.00% | -0.07% | -0.03% | -0.09% | -0.15% | 0.30% | 0.07% | 0.07% |
| Overall (Test vs single layer) | 26.5% | 37.9% | 38.9% | 26.6% | 37.8% | 39.3% | 23.2% | 32.9% | 35.8% |
| Overall (Ref vs single layer) | 26.5% | 37.9% | 38.9% | 26.6% | 38.0% | 39.5% | 22.8% | 32.8% | 35.6% |
| EL only (Test vs Ref) | 0.0% | 0.0% | -0.1% | -0.1% | -0.2% | -0.2% | 0.7% | 0.4% | 0.4% |
| Enc Time[%] | 105.8% |  |  | 102.5% |  |  | 103.7% |  |  |
| Dec Time[%] | 96.1% |  |  | 93.9% |  |  | 95.2% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Table 2.  Performance with CTC BL

# Reference

[1]   K. Minoo and D. Baylon, “On the selection of fixed filters for upsampling”, JCTVC-N0273, 14th JCT-VC Meeting, Vienna, AT, July 2013.

# Patent rights declaration(s)

ARRIS Group, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).