| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0063 |
| --- | --- |

| Title: | HLS: Extensions to Temporal Motion-constrained tile sets SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sally Hattori, Ohji Nakagami, Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 shinobu.hattori@jp.sony.com ohji.nakagami@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution proposes an extension to a temporal motion-constrained tile sets SEI message to indicate the level information for a decoder to decode each defined motion-constrained tile set. The proposal provides flexibility for HEVC tile structure to be applied for various applications such as in interactive UHDTV application, dynamic high-quality zoom-in application and interactive on-demand e-learning etc. The idea is initially proposed in JCTVC-N0117. This contribution further clarifies the specification text on the definitions of level constraints for motion-constrained tile sets.

# Introduction

The proposal is to enable an application in which only the partial region of a picture are delivered to the decoder such as tiled streaming applications. In order to realize this application, it is necessary that partially delivered tile data can be decoded such that level indication such partial regions can be helpful information to determine the capability of decoding the partial regions. The characteristics of tiled streaming applications are all tiles are independently decodable, all regions may be of interest for the users and the resolution of the video to be decoded varies depending on the display devices.

Figure 1 Example image of Tiled Streaming Application

# Proposal

The proposed syntax and semantics are as follows. The proposal extends the a temporal motion-constrained tile sets SEI message to signal a level for motion-constrained tile set. This extension enables to signal required level to be supported by the decoder to decode indicated motion-constrained tile sets.

- Temporal motion-constrained tile sets SEI message syntax

| temporal_motion _constrained_tile_sets ( payloadSize ) { | Descriptor |
| --- | --- |
| mc_all_tiles_exact_sample_value_match_flag | u(1) |
| each_tile_one_tile_set_flag | u(1) |
| mcts_level_idc_present_flag | u(1) |
| if ( !each_tile_one_tile_set_flag ) { |  |
| num_sets_in_message_minus1 | ue(v) |
| for( i = 0; i <= num_sets_in_message_minus1; i++) { |  |
| mcts_id[ i ] | ue(v) |
| num_tile_rects_in_set_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_tile_rects_in_set_minus1[ i ]; j++) { |  |
| top_left_tile_index[ i ][ j ] | ue(v) |
| bottom_right_tile_index[ i ][ j ] | ue(v) |
| } |  |
| if ( ! mc_all_tiles_exact_sample_value_match_flag ) |  |
| exact_sample_value_match_flag[ i ] | u(1) |
| if ( mcts_level_idc_present_flag ) |  |
| mcts_level_idc[ i ] | u(8) |
| } |  |
| } else { |  |
| if ( mcts_level_idc_present_flag ) |  |
| max_mcts_level_idc | u(8) |
| } |  |
| } |  |

- Temporal motion-constrained tile sets SEI message semantics

add the following paragraph after the semantics of each_tile_one_tile_set_flag:

mcts_level_idc_present_flag equal to 1 indicates that level information is present for motion-constrained tile sets defined in the SEI message. mcts_level_idc_present_flag equal to 0 indicates that level information is not present for motion-constrained tile sets is present in the SEI message. mcts_level_idc_present_flag equal 0 indicates that level information for motion-constrained tile sets inferrs to be equal to the value of general_level_idc.

add the following paragraph after the semantics of exact_sample_value_match_flag[ i ]:

mcts_level_idc[ i ] is same as the syntax element general_level_idc, but applied to the representation of the i-th identified motion-constrained tile sets. The value of mcts_level_idc indicates a level to which the motion-constrained tile sets conforms as specified in Annex A with the following additions.

–	The variables PicSizeInSamplesY is inferred to be equal to total number of luma samples in the rectanguler region for the defined motion-constrained tile set.

NOTE – A rectanguler region for a motion-constrained tile set is a region with  minimum width and height which covers all tiles defined for each motion-constrained tile set as illustrated in Figure 2. In case the region of motion-constrained tile sets is not in rectanguler shape, the width and height are inferred to be equal to the value of a rectangle which covers the entire tile sets horizontally and vertically.

–	The value of pic_width_in_luma_samples and pic_height_in_luma_samples are inferred to be equal to the width and height of rectanguler region for motion-constrained tile sets in units of luma samples, respectively.

–	The value of num_tile_columns_minus1 plus 1 and num_tile_rows_minus1 plus 1 are inferred to be equal to the total number of tile columns and rows in the rectanguler region for the defined motion-constrained tile set.

–	For the VCL HRD parameters and NAL HRD parameters, the values should be inferred to be equal to the values when the HRD parameters are not present for an indicated level.

Figure 2 Examples of rectangular regions for motion-constrained tile sets

max_mcts_level_idc is same as the syntax element mcts_level_idc[ i ] for i-th identified motion-constrained tile set but applied to the representation of motion-constrained tile sets which is composed of one tile per one tile set. The value of max_mcts_level_idc should be the maximum mcts_level_idc value in a picture.

Figure 3 Example of max_mcts_level_idc

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).