| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0064 |
| --- | --- |

| Title: | HLS: SEI message for transfer function information |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sally Hattori, Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 shinobu.hattori@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution proposes a new SEI message to signal knee function information to enable display devices to process decompression or compression of colour samples of the decoded pictures to customize the picture for a particular display environment.

# Introduction

Today, the video cameras typically ameliorate their sensor’s harsh clipping with a knee function which allows compression of the upper portion of the dynamic range. The compression characteristic of the knee function is shown in Figure 1. As shown in Figure 1, a knee function is composed of two lines which connects following three points normalized in range of 0.0 to 1.0; 1) origin (0,0), 2) knee point (a,b) and 3) (1,1). The D range (Dynamic range) represents the peak signal level of the original picture in relative to the nominal luminance level.

Figure 1 Example of Knee Function and Dynamic range

Using knee function by camera or the video production process is widely being practiced today. The purpose of using the knee function has been to fit the dynamic range of the pictures to the display brightness capability which is lower than what can be captured. However, with the recent evolvement of the display brightness, many displays are becoming to be capable of displaying wider dynamic range with brighter luminance. For these displays, providing parameters to enable decompressing the dynamic range to derive higher dynamic range can be very beneficial to make full use of the display capability.

# Proposal

The proposed syntax and semantics are as follows. Figure 2 shows example of knee function parameters proposed in the SEI message for the purpose of decompression process. Figure 3 shows example of knee function parameters proposed in the SEI message for the purpose of compression process.

Figure 2 Example of knee function information for decompression of colour samples

Figure 3 Example of knee function information for compression of colour samples

- Knee function information SEI message syntax

| knee_function_info ( payloadSize ) { | Descriptor |
| --- | --- |
| knee_function_id | ue(v) |
| knee_function_cancel_flag | u(1) |
| input_knee_point | u(16) |
| output_knee_point | u(16) |
| d_range | u(32) |
| d_range_disp_luminance | u(32) |
| } |  |

- Knee function information SEI message semantics

This SEI message provides information to enable decompression or compression of the colour samples of the ouput decoded pictures by using knee function for customization to particular display environments. The decompression or compression process maps the white level of sample values in the normalized linear RGB colour space. The compression/decompression should be applied to each RGB component produced by colour space conversion of the decoded image accordingly.

A knee function is a function which is composed of two lines which connects three points normalized in range of 0.0 to 1.0; 1) origin, 2) knee point and 3) (1,1) as shown in the Figure x-x.

Figure x-x Knee Function

knee_function_id contains an identifying number that may be used to identify the purpose of the knee functions. The value of knee_function_id shall be in the range of 0 to 232 − 2, inclusive.

Values of knee_function_id from 0 to 255 and from 512 to 231 − 1 may be used as determined by the application. Values of knee_function_id from 256 to 511, inclusive, and from 231 to 232 – 2, inclusive are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all knee function information SEI messages containing a value of knee_function_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, and bitstreams shall not contain such values.

NOTE 1 – The knee_function_id can be used to support knee function process that are suitable for different display scenarios. For example, different values of knee_function_id may correspond to different display bit depths.

knee_function_cancel_flag equal to 1 indicates that the knee function information SEI message cancels the persistence of any previous knee function information SEI message in output order. knee_function_cancel_flag equal to 0 indicates that knee function information follows.

input_knee_point specifies the luminane level of a point at which the decompression or compression starts to flatten the contrast curve for the inut picture. The luminance level of the knee point for input picture is normalized in range of 0 to 1.0 in units of 0.1%. The value of input_knee_point is expressed in range of 0 to 1000.

output_knee_point specifies the luminane level of a point at which the decompression or compression starts to flatten the contrast curve for the output picture. The luminance level of the knee point for output picture is normalized in range of 0 to 1.0 in units of 0.1%. The value of output_knee_point is expressed in range of 0 to 1000.

NOTE 2 – The coordinate of the knee point is defined as (input_knee_point, output_knee_point).

d_range specifies the peak luminance level of either the decompressed picture or unompressed picture in relative to the nominal luminance level in units of 0.1%.

The value of d_range specifies the peak luminanve level of the decompressed picture (output picture of the knee process) when the value of input_knee_point is greater than or equal to the value of ouput_knee_point. The value of d_range specifies the peak luminance level of the uncompressed picture (input picture of the knee process) when the value of input_knee_point is less than the value of output_knee_point.

d_range_disp_luminance specifies the expected display brightness of peak luminance level specified as d_range. The value of d_range_disp_luminance is in units of candera per square metre.

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).