| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0066 |
| --- | --- |

| Title: | AHG5: Unifying DPCM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Sharman, N. Saunders, J. Gamei Sony BPRL Jays Close, Viables, Basingstoke RG22 4SB, UK | Tel: Email: | +44 1256 355 011 karl.sharman@eu.sony.com nick.saunders@eu.sony.com |
| Source: | Sony Europe Ltd |  |  |

_____________________________

# Abstract

At the previous meeting, a number of proposals were accepted that in effect provided RPDCM coding for inter and intra (and intra-block-copy) prediction modes, for transform-skipped lossy and transquant-bypass lossless processing modes. In addition, an independent rotation function for transform-skip and transquant-bypass was introduced. The resulting algorithm differed for inter and intra processing. This proposal examines the current algorithm, indicates the relative merits of the differences, and then describes two methods by which the algorithm can be unified.

BD-rate changes for the CDPCM-only method versus the anchor (HM12.0_RExt4.1) are reported to range from 0.0% (for non-screen-content) to -4.1% (for low-delay high tier GBR screen-content).

# Introduction

During the Incheon meeting (JCTVC-M), sample-adaptive intra predict (SAIP) was added to the intra transquant-bypass processing path. In short, this method involves horizontal or vertical differential coding of the residual and is used when the intra-prediction direction is horizontal (mode 10) or vertical (mode 26) respectively (the differential coding was not used for other intra prediction modes).

During the Vienna meeting (JCTVC-N), this SAIP algorithm was extended so that it was applied to intra transform-skip and also to inter (both transquant-bypass and transform-skip paths). The inter path was also used for the intra-block-copy tool added during the same meeting. The resulting new algorithm was called Residual DPCM (RDPCM).

For the inter path, there is of course no intra-prediction direction available to select the type of differential coding. The encoder must therefore test whether to use horizontal or vertical or no differential coding and signal this to the decoder on a per-TU basis.

When encoding RDPCM, a difference is calculated between the current residual value and the reconstructed version of the previously encoded residual value either to the left (for horizontal RDPCM) or above (for vertical RDPCM). This means that new feedback loops in the encoder are required in the processing. Processing of RDPCM coefficients is now a sample-by-sample process, requiring one sample to be fully encoded and decoded before the next can begin. This is not the case for transquant-bypass TUs since the reconstructed TU is the same as the source TU, and therefore the feedback loop can effectively be removed.

In addition, at the Vienna meeting, transform skip was extended so that it could be used for TUs larger than 4x4 (as dictated by a picture parameter set (PPS) parameter), and 4x4 transform-skipped TUs were optionally rotated through 180o (as dictated by a sequence parameter set (SPS) flag).

This has resulted in a system described by the flow charts shown in Figure 1, where n represents the bit depth and MTDR is the maximum transform dynamic range (also equal to the maximum entropy coding dynamic range):

Figure 1 - HM12.0_RExt4.0 RDPCM configuration

## Problem 1 – Differences in Configuration Between Inter and Intra

From an implementation standpoint, the differences between the inter and intra paths are unwelcome. The use of rotation prior to the encoder’s inter-RDPCM is also less than ideal.

In effect, the inter path should be considered to be a coefficient DPCM (CDPCM) because it is applied after the transform skip in the encoder.

As argued by the proponents, the inter path benefits from the increased accuracy of the different ordering (i.e. DPCM after transform-skip). It also benefits from a shortened encoder feedback path.

## Problem 2 – Application of RDOQ.

The rate-distortion-optimised quantisation (RDOQ) algorithm cannot be applied to the inter-case – the differential coding is applied in forward scan order, whereas the entropy coder (which RDOQ models), will process the coefficients in reverse scan order. RDOQ can actually be used for the intra case, but only if rotation is used (and then therefore only for 4x4 transform-skip TUs).

## Problem 3 – Numerical Limits and Clipping

There are issues in both cases regarding the numerical limits. The bit-stream is restricted in that it may not contain values that are more than (MTDR + 1) bits (signed) (typically 16-bit) in size. The only other clipping is at the output of the inverse quantiser (which also clips to (MTDR + 1) bits (signed) for non-transquant-bypass TUs) and when the reconstruction value is formed. In the intra-configuration in Figure 1, this means that the inputs to the inverse RDPCM could be (MTDR + 1) bits (signed) in size, even though in reality the values should be of the order of residual coefficients, i.e. (n + 1) bits (signed) in size. In addition, the inverse RDPCM module utilises an accumulator, and therefore theoretically, the number of bits in the output (and accumulator) could be as many as (input_rdpcm_bit_depth + log2(size)). For 32x32 TUs, this can be 5 bits more than the input, which could be as many as 21 bits (signed).

## Problem 4 – Parallelisation

When SAIP was originally introduced, parallelisation was trivial as the process was only applied to lossless TUs. An encoder could easily generate differences between the source residual data; a decoder could easily generate accumulations, either column-by-column or row-by-row (i.e. for horizontal RDPCM, the data could be processed column by column). Note, however, that the TU could not start to be decoded until the top-left coefficient was decoded, which since the order is reverse-scan, is decoded last. Hence a buffer to temporarily hold the TU is required.

For the current RDPCM system, the available degree of parallelisation is limited. Again, an encoder can, for a horizontal RDPCM arrangement, process the data column by column. However, as before, a decoder must wait until the top-left coefficient is decoded; for non 4x4 TUs or when rotation is disabled this will be the last in the TU. In addition, for inter-processing, the scan order is always diagonal rather than horizontal or vertical, and therefore coefficients will not be decoded in the optimal order.

Where rotation is applied (for 4x4 intra TUs), the coefficients are decoded by the entropy decoder in an optimal order. For horizontal RDPCM for example, the coefficients are decoded column by column (because the intra prediction direction that indicates the use of horizontal RDPCM also indicates the use of the vertical scan order). The use of rotation means that the first set of values decoded is actually the first column, and therefore RDPCM can be processed column-by-column without delay.

# Unification

It is proposed to unify the inter and intra cases using one of the methods described in Sections 2.1 and 2.2, giving an encoder the option to apply RDOQ for the transform-skip case and to improve the processing order of coefficients to increase parallelisation and reduce buffer usage.

In these configurations, always applying rotation after RDPCM allows RDOQ to be used – the order the coefficients are encoded in is now effectively from the top left down to the bottom right (either horizontal, vertical or diagonal scan), which is the order in which RDPCM can produce coefficients.

In addition, to simplify the processing of inter-RDPCM coefficients, the encoding of inter RDPCM coefficients is no longer limited to using diagonal scan, but instead uses vertical scanning for horizontal inter RDPCM, and horizontal scanning for vertical inter RPCDM (similar to the intra behaviour).

Finally, to prevent the accumulators in the IRDPCM module being excessively large, a clipping operation is used.

Essentially in both of the configurations the full transform-skip processing chain (including transform) could be applied within the quantiser (e.g. in the HM_RExt C functions xQuant and xRateDistOptQuant) for the encoder, or inverse quantiser for the decoder.

## Unified RDPCM

In this method, RDPCM is applied to the residual if required, and then the TU is rotated if RDPCM was applied (or if required by the SPS rotation flag). Following this, the TU is transformed (which for transform-skip comprises only a left-shift) and quantised. Clipping would be applied during the accumulation of RDPCM values in the IRDPCM module, limiting the accumulator and downstream values to just (n + 1) signed bits. Note that rotation could be applied after the quantiser, or can even be implemented as a reversal of the scan order in the entropy coder (i.e. scanning in forward order rather than reverse). This can be seen in Figure 2:

Figure 2 - Proposal 1 - Unified RDPCM

## Unified CDPCM

In CDPCM, the TU is first transformed (which for transform-skip comprises only a left shift), then CDPCM is applied (if required), then the TU is rotated (again only if CDPCM was applied, or if required by the SPS rotation flag), and quantised. Again, clipping would be applied during the accumulation of CDPCM values in the ICDPCM module, limiting the accumulator and downstream values to just (MTDR + 1) signed bits. Note that rotation could be applied after the quantiser, or can even be implemented as a reversal of the scan order in the entropy coder (i.e. scanning in forward order rather than reverse). This can be seen in Figure 3:

Figure 3 - Proposal 2 - Unified CDPCM

# Results

Each of the proposed systems detailed in Sections 2.1 and 2.2 has been tested under AHG8 lossy test conditions. Results are presented as follows:

## Unified RDPCM with modified inter-scan-orders

The system tested was as detailed in Section 2.1. The results for high-tier operating points are presented in Table 1. Results at other operating points may be found in the accompanying Excel spreadsheets.

Table 1 – BD-rate changes for unified RDPCM (as detailed in Section 2.1) under AHG8 lossy test conditions

## CDPCM with modified inter-scan-orders

The system tested was as detailed in Section 2.2. The results for high-tier operating points are presented in Table 2. Results at other operating points may be found in the accompanying Excel spreadsheets.

Table 2 – BD-rate changes for unified CDPCM (as detailed in Section 2.2) under AHG8 lossy test conditions

## Unified RDPCM with unmodified inter-scan-orders

The system tested was as detailed in Section 2.1, but with no modification of the inter scan-orders. The results for high-tier operating points are presented inTable 3. Results at other operating points may be found in the accompanying Excel spreadsheets.

Table 3 – BD-rate changes for unified RDPCM (as detailed in Section 2.1) without inter scan-order modification under AHG8 lossy test conditions

## CDPCM with unmodified inter-scan-orders

The system tested was as detailed in Section 2.2, but with no modification of the inter scan-orders. The results for high-tier operating points are presented in Table 4. Results at other operating points may be found in the accompanying Excel spreadsheets.

Table 4 – BD-rate changes for unified CDPCM (as detailed in Section 2.2) without inter scan-order modification under AHG8 lossy test conditions

# Conclusion

The Range Extensions model has been analysed, and a number of suggestions have been made regarding DPCM and rotation, namely: use of either RDPCM or CDPCM for both intra and inter, applying rotation before and adding a clipping operation to the inverse DPCM stage, always enabling rotation for DPCM when used with transform-skip (and potentially when used with transquant-bypass), and using the DPCM direction to control the inter scan-order to aid decoding.

The result of these changes is to unify the intra and inter processing paths, prevent internal numeric ranges from becoming too large, and process coefficients in a manner by which the DPCM operation is trivial and which allows the simple application of RDOQ.

There is little difference in the application of the inter scan-order modification to the general results – some sequences improve slightly, some degrade slightly. The only significant difference is to low-delay-B lossless coding of the “SC 444 YUV Optional” sequence, cad_waveform, where only a small amount of noise to the result can produce huge differences, due to the compression ratio being ~2200:1. This modification is only intended for simplification of decoding.

The CDPCM-only method (Proposal 2) is preferred as the feedback loop required is significantly shorter. It also provides an improvement over the RDPCM-only method (Proposal 1), observed for screen-content sequences. Gains for the CDPCM-only method over the anchor (HM12.0_RExt4.1) range from 0.0% (for non-screen-content) to -4.1% (for GBR screen-content).

For lossless, there is no difference between the RDPCM and CDPCM results (as expected because there is no difference between the two paths). There are only marginal differences in the results, which are due to the inter (and intra-block-copy) scan direction being affected by the DPCM direction, which of course suits some screen-content sequences more than others. Without the additional scan orders for inter, the results are identical to anchor. Always applying rotation to transquant-bypass TUs where DPCM is used (which may help reduce internal buffering) is still being studied.

The software has been written so that the changes to apply DPCM are minimised, removing the necessity of the additional two pairs of variants of xQuant and xDequant that were added during the previous meeting cycle.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix A: Working Draft Text

(relative to JCTVC-N1005_v3)

7.3.8.11	Residual coding syntax

| residual_coding( x0, y0, log2TrafoSize, cIdx ) { | Descriptor |
| --- | --- |
| if( transform_skip_enabled_flag && !cu_transquant_bypass_flag && ( log2TrafoSize <= Log2MaxTransformSkipSize ) ) |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTER && residual_dpcm_inter_enabled_flag && ( transform_skip_flag[ x0 ][ y0 ][ cIdx ] ¦ ¦ cu_transquant_bypass_flag ) ) { |  |
| inter_rdpcm_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| if( inter_rdpcm_flag[ x0 ][ y0 ][ cIdx ] ) |  |
| inter_rpdcm_dir_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| } |  |
| last_sig_coeff_x_prefix | ae(v) |
| … |  |

7.4.9.11	Residual coding semantics

…

transform_skip_flag[ x0 ][ y0 ][ cIdx ] specifies whether a transform is applied to the associated transform block or not: The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index cIdx specifies an indicator for the colour component; it is equal to 0 for luma, equal to 1 for Cb, and equal to 2 for Cr. transform_skip_flag[ x0 ][ y0 ][ cIdx ] equal to 1 specifies that no transform is applied to the current transform block. transform_skip_flag[ x0 ][ y0 ][ cIdx ] equal to 0 specifies that the decision whether transform is applied to the current transform block or not depends on other syntax elements. When transform_skip_flag[ x0 ][ y0 ][ cIdx ] is not present, it is inferred to be equal to 0.

inter_rdpcm_flag[ x0 ][ y0 ][ cIdx ] specifies whether the residual modification process for blocks using a transform bypass is applied to the associated transform block or not. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index cIdx specifies an indicator for the colour component; it is equal to 0 for luma, equal to 1 for Cb, and equal to 2 for Cr. inter_rdpcm_flag[ x0 ][ y0 ][ cIdx ] equal to 1 specifies that the residual modification process is applied to the current transform block. inter_rdpcm_flag[ x0 ][ y0 ][ cIdx ] equal to 0 specifies that no residual modification process is applied to the current transform block. When inter_rdpcm_flag[ x0 ][ y0 ][ cIdx ] is not present, it is inferred to be equal to 0.

inter_rdpcm_dir_flag[ x0 ][ y0 ][ cIdx ] specifies the direction to be used by the residual modification process for the associated transform block. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index cIdx specifies an indicator for the colour component; it is equal to 0 for luma, equal to 1 for Cb, and equal to 2 for Cr. When inter_rdpcm_dir_flag[ x0 ][ y0 ][ cIdx ] is not present, it is inferred to be equal to 0.

last_sig_coeff_x_prefix specifies the prefix of the column position of the last significant coefficient in scanning order within a transform block. The values of last_sig_coeff_x_prefix shall be in the range of 0 to ( log2TrafoSize  <<  1 ) − 1, inclusive.

…

8.4.4.1	General decoding process for intra blocks

…

- The variable nTbS is set equal to 1  <<  log2TrafoSize.

- The variable yTbOffset is set equal to blkIdx * nTbS.

- The variable residualDpcm is derived as follows:

  - If residual_dpcm_intra_enabled_flag is equal to 1, and predModeIntra is equal to 10 or 26, and one or more of the following conditions are true, residualDpcm is set equal to 1.

    - cu_transquant_bypass_flag is equal to 1

    - transform_skip_flag[ xTb0 ][ yTb0 + yTbOffset ][ cIdx ] is equal to 1

  - Otherwise, residualDpcm is set equal to 0.

- Depending upon the value of predModeIntraBc, the following applies

–	When predModeIntraBc is equal to 0, the general intra sample prediction process as specified in subclause Error: Reference source not found is invoked with the transform block location ( xTb0, yTb0 + yTbOffset ), the intra prediction mode predModeIntra, the transform block size nTbS, and the variable cIdx as inputs, and the output is an (nTbS)x(nTbS) array predSamples.

–	Otherwise (predModeIntraBc is equal to 1), the intra block copying process as specified in subclause Error: Reference source not found is invoked with the transform block location ( xTb0, yTb0 + yTbOffset ), the transform block size nTbS, the variable bvIntra, and the variable cIdx as inputs, and the output is an (nTbS)x(nTbS) array predSamples.

- The scaling and transformation process as specified in subclause Error: Reference source not found is invoked with the luma location ( xTbY, yTbY + yTbOffset * SubHeightC ), the variable trafoDepth, the variable cIdx, and the transform size trafoSize set equal to nTbS, and the variable DPCMEnabledFlag set equal to residualDpcm as inputs, and the output is an (nTbS)x(nTbS) array resSamples.

- When residualDpcm is equal to 1 and either predModeIntra is equal to 10, or predModeIntra is equal to 26, the directional residual modification process for blocks using a transform bypass as specified in subclause Error: Reference source not found is invoked with the variable mDir set equal to predModeIntra / 26, the variable nTbS, the variable cIdx, the location ( xTb0, yTb0 ), and the (nTbS)x(nTbS) array r set equal to the array resSamples as inputs, and the output is a modified (nTbS)x(nTbS) array resSamples.

- The picture reconstruction process prior to in-loop filtering for a colour component as specified in subclause Error: Reference source not found is invoked with the transform block location ( xTb0, yTb0 + yTbOffset ), the variables nCurrSw and nCurrSh both set equal to nTbS, the variable cIdx, the (nTbS)x(nTbS) array predSamples, and the (nTbS)x(nTbS) array resSamples as inputs.

8.5.4.2	Decoding process for luma residual blocks

…

–	Otherwise (split_transform_flag[ xCb + xB0 ][ yCb + yB0 ][ trafoDepth ] is equal to 0), the following ordered steps apply:

- The variable nTbS is set equal to 1  <<  log2TrafoSize.

- The scaling and transformation process as specified in subclause 8.6.2 is invoked with the luma location ( xCb + xB0, yCb + yB0 ), the variable trafoDepth, the variable cIdx set equal to 0, and the transform size trafoSize set equal to nTbS, and the variable DPCMEnabledFlag set equal to inter_rdpcm_flag[ xCb + xB0 ][ yCb + yB0 ][ 0 ] as inputs, and the output is an (nTbS)x(nTbS) array transformBlock.

- When residual_dpcm_inter_enabled_flag is equal to 1 and transform_skip_flag inter_rdpcm_flag[ xCb + xB0 ][ yCb + yB0 ][ 0 ] is equal to 1, the directional residual modification process for blocks using a transform bypass as specified in subclause 8.6.5 is invoked with the variable mDir set equal to inter_rdpcm_dir_flag[ xCb + xB0 ][ yCb + yB0 ][ 0 ], the variable nTbS, the variable cIdx set equal to 0, the location ( xCb + xB0, yCb + yB0 ), and the (nTbS)x(nTbS) array r set equal to the array transformBlock as inputs, and the output is a modified (nTbS)x(nTbS) array transformBlock.

- The (nCbS)x(nCbS) residual sample array of the current coding block resSamples is modified as follows:

resSamples[ xB0 + i, yB0 + j ] = transformBlock[ i, j ], with i = 0..nTbS − 1, j = 0..nTbS − 1

8.5.4.3	Decoding process for chroma residual blocks

…

–	Otherwise (splitChromaFlag is equal to 0), for the variable blkIdx proceeding over the values 0..( ChromaArrayType  = =  2 ? 1 : 0 ), the following ordered steps apply:

- The variable nTbS is set equal to ( 1  <<  log2TrafoSize ) / SubWidthC.

- The variable yBN is set equal to yB0 + blkIdx * nTbS * SubHeightC.

- The scaling and transformation process as specified in subclause 8.6.2 is invoked with the luma location ( xCb + xB0, yCb + yBN ), the variable trafoDepth, the variable cIdx, and the transform size trafoSize set equal to nTbS, and the variable DPCMEnabledFlag set equal to inter_rdpcm_flag[ xCb + xBN ][ yCb + yBN ][ cIdx ] as inputs, and the output is an (nTbS)x(nTbS) array transformBlock.

- When residual_dpcm_inter_enabled_flag is equal to 1 and transform_skip_flag inter_rdpcm_flag[ xCb + xBN ][ yCb + yBN ][ cIdx ] is equal to 1, the directional residual modification process for blocks using a transform bypass as specified in subclause 8.6.5 is invoked with the variable mDir set equal to inter_rdpcm_dir_flag[ xCb + xBN ][ yCb + yBN ][ cIdx ], the variable nTbS, the variable cIdx, the location ( xCb + xB0, yCb + yBN ), and the (nTbS)x(nTbS) array r set equal to the array transformBlock as inputs, and the output is a modified (nTbS)x(nTbS) array transformBlock.

- The (nCbS)x(nCbS) residual sample array of the current coding block resSamples is modified as follows, for i = 0..nTbS − 1, j = 0..nTbS − 1:

resSamples[ ( xCb + xB0 ) / SubWidthC + i, ( yCb + yBN ) / SubHeightC + j ] = transformBlock[ i, j ]

8.6.2	Scaling and transformation process

Inputs to this process are:

–	a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

–	a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding block,

–	a variable cIdx specifying the colour component of the current block,

–	a variable nTbS specifying the size of the current transform block,

–	a variable DPCMEnabledFlag specifying whether or not DPCM is enabled for the current block.

Output of this process is the (nTbS)x(nTbS) array of residual samples r with elements r[ x ][ y ].

…

–	The variable tsShift is derived as follows:

tsShift = extended_precision_processing_flag ? Max( 7, bdShift ) : 7	(8-272)

–	If transform_skip_rotation_enabled_flag is equal to 1 and nTbS is equal to 4,

–	If DPCMEnabledFlag is equal to 1,

r[ x ][ y ] = d[ nTbS − x −1 ][ nTbS − y − 1 ]	(8-273)

–	Otherwise,

r[ x ][ y ] = ( d[ nTbS − x −1 ][ nTbS − y − 1 ]  <<  tsShift )	(8-274)

–	Otherwise,

–	If DPCMEnabledFlag is equal to 1,

r[ x ][ y ] = d[ x ][ y ]		(8-275)

–	Otherwise,

r[ x ][ y ] = ( d[ x ][ y ]  <<  tsShift )	(8-276)

- Otherwise (transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 0), the transformation process for scaled transform coefficients as specified in subclause 8.6.4 is invoked with the transform block location ( xTbY, yTbY ), the size of the transform block nTbS, the colour component variable cIdx, and the (nTbS)x(nTbS) array of scaled transform coefficients d as inputs, and the output is an (nTbS)x(nTbS) array of residual samples r.

…

8.6.5	Residual modification process for blocks using a transform bypass

Inputs to this process are:

–	a variable mDir specifying the residual modification direction,

–	a variable nTbS specifying the transform block size,

–	an (nTbS)x(nTbS) array of residual samples r with elements r[ x ][ y ],

–	a variable cIdx specifying the colour component of the current block,

–	a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture.

Output of this process is the modified (nTbS)x(nTbS) array of residual samples .

Depending upon the value of mDir, the (nTbS)x(nTbS) array of samples r is modified as follows:

–	If mDir is equal to 0 (horizontal direction), the array values r[ x ][ y ], for x = 1..nTbS − 1, y = 0..nTbS − 1, are modified as follows:

r[ x ][ y ] += r[ x − 1 ][ y ]	(8-297)

–	Otherwise (vertical direction), the array values r[ x ][ y ], for x = 0..nTbS − 1, y = 1..nTbS − 1, are modified as follows:

r[ x ][ y ] += r[ x ][ y − 1 ]	(8-298)

If cu_transquant_bypass_flag is equal to 0, the (nTbS)x(nTbS) array of samples r is modified as follows:

–	The variable bitDepth is derived as follows:

bitDepth = ( cIdx  = =  0 ) ? BitDepthY : BitDepthC

–	The variables bdShift and tsShift are derived as follows:

bdShift = ( extended_precision_processing_flag ? Max( 15, ( bitDepth + 6 ) ) : 15 ) + 5 − bitDepth

tsShift = extended_precision_processing_flag ? Max( 7, bdShift ) : 7

–	The array values r[ x ][ y ], for x = 0..nTbS − 1, y = 1..nTbS − 1, are modified as follows:

r[ x ][ y ] = ( r[ x ][ y ]  <<  tsShift )