| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0067 |
| --- | --- |

| Title: | Transform Skip and Scaling Lists |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Sharman, N. Saunders, J. Gamei Sony BPRL Jays Close, Viables, Basingstoke RG22 4SB, UK K. Sato 2-10-1, Osaki, Shinagawa-ku, Tokyo, 141-8610 JAPAN | Tel: Email: | +44 1256 355 011 karl.sharman@eu.sony.com nick.saunders@eu.sony.com kazushi.sato@jp.sony.com |
| Source: | Sony Europe Ltd and Sony Corporation |  |  |

_____________________________

# Abstract

The use of larger transform-skipped TUs, it is claimed, potentially causes problems when considering scaling lists: scaling lists are currently common to transformed and transform-skipped TUs. Scaling lists, when not flat, would normally be designed so that the bottom right coefficients are quantised more heavily than the top left coefficients (DC); for transform-skipped TUs, it is suggested that quantising the spatial coefficients using frequency-based scaling lists is not ideal.

It is proposed that scaling lists can be disabled by a PPS flag for transform-skipped TUs.

# Introduction

At the Vienna meeting (JCTVC-N), a number of proposals on transform-skip TU sizes were considered, which resulted in a new PPS parameter indicating the maximum transform-skip TU size. Prior to this, as in the HEVC Standard, only 4x4 TU sizes could be transform-skipped.

The quantiser currently applies scaling lists to transformed and transform-skipped TUs, with the same scaling lists applied to both. The scaling lists would commonly be designed to be flat, or designed so that the high-frequency coefficients were quantised more heavily than the low-frequency coefficients. In the latter case, applying the shaped scaling lists to a mixture of transformed and transform-skipped TUs will cause unusual noise characteristics in transform-skipped TUs, where samples nearer the bottom right edge will be more heavily quantised. Transform-skipped TUs consist of spatial coefficients, rather than frequency coefficients, and applying frequency-based scaling lists to such data is not ideal.

In the case where the maximum transform-skipped TU size is 4x4 (as in the HEVC Standard), an encoder would be likely to use flat 4x4 scaling lists, thereby mitigating this unusual effect.

However, now that larger TUs can be used, the use of the same scaling lists on, for example, 32x32 transformed TUs as 32x32 transform-skipped TUs is either likely to cause unusual effects for transform-skipped TUs, or will make scaling lists ineffective since they will all have to be flat.

# Transform-skip scaling lists

## Proposed solutions

It is proposed that transform-skip scaling lists can be made to be independent from transformed-TUs. This can be achieved in one of five ways:

- Do not use scaling lists for any transform-skipped TUs.

- Do not use scaling lists for any transform-skipped TUs, other than 4x4.

- Use a flat scaling list for transform-skipped TUs, with the value used being (for example) the DC value of the counterpart scaling list for transformed TUs.

- Use a flat scaling list for transform-skipped TUs, with the value for each size / component sent in the PPS header.

- Use specific scaling list(s) for transform-skipped TUs, as specified in the PPS header.

These options could be configured, either by detecting that the maximum transform-skip size is not 4x4, or by a separate enable flag in the PPS.

## Background

Discussions were held on the scaling list adaptation for transform-skip TUs at the 10th JCTVC meeting in Stockholm. The relevant contributions are as follows:

- Using a flat scaling list for transform-skipped TUs, with the value used being 16 (JCTVC-J0237 part2, JCTVC-J0184 part1, and JCTVC-J0201) [similar to solution 1 or 2.]

- Using a flat scaling list for transform-skipped TUs, with the value used being the DC value (JCTVC-J0092 and JCTVC-J0184 part3) [similar to solution 3.]

- Using a flat scaling list for transform-skipped TUs, with the value used being explicitly signalled to the decoder (JCTVC-J0184 part2) [similar to solution 4.]

However, the HEVC version 1 specification only allows transform skip to be applied to 4x4 TUs, and at the 10th JCTVC meeting, concern was raised that switching scaling list operation at 4x4 granularity would be an unnecessary complexity overhead. For this reason, it was decided that a flat matrix would be defined as the default scaling list for 4x4 TUs, regardless of the value of the transform_skip_enabled_flag in PPS.

In Range Extensions, as transform skip can now be applied for larger TUs than 4x4, the complexity overhead of switching the use of scaling lists would be smaller. In addition, scaling lists are more useful for transformed TUs with larger sizes to reduce redundant high frequency components such as noise.

## Summary

It is recommended to re-consider the adoption of the solutions stated above. Of these options, we recommend that a flag is added to the PPS that indicates that scaling lists are not used for any transform-skipped TU.

# Conclusion

The use of larger transform-skipped TUs potentially causes problems when considering scaling lists. Methods to overcome these problems are considered, with the recommendation being that a flag is added to the PPS that indicates that scaling lists are not used for any transform-skipped TU.

# References

[1] “Transform Skipping in the presence of Scaling Lists”, D. Flynn, M. Mrak, M. Naccari, A. Gabriellini (BBC), JCTVC-J0092, July 2012

[2] “On Transform Skip”, JCTVC-J0184, Y Morigami, K Sato (Sony), T Sugio, H Sasai (Panasonic), July 2012

[3] “QM bypass for transform skip mode”, JCTVC-J0201, H. Y. Kim, S.-C. Lim (ETRI), K. Y. Kim, G. H. Park (KHU), J. Lee, J. S. Choi (ETRI), July 2012

[4] “Inter transform skipping”, JCTVC-J0237, X. Peng (Microsoft), C. Lan (Xidian Univ.), J. Xu, G. J. Sullivan (Microsoft), July 2012

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix A: Draft Text For Option 1 With Flag

(relative to JCTVC-N1005_v3)

7.3.2.3	Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| dependent_slice_segments_enabled_flag | u(1) |
| … |  |
| slice_segment_header_extension_present_flag | u(1) |
| pps_extension1_flag | u(1) |
| if( pps_extension1_flag ) { |  |
| if( transform_skip_enabled_flag ) { |  |
| log2_transform_skip_max_size_minus2 | ue(v) |
| enable_scaling_list_for-transform_skip_flag | u(1) |
| } |  |
| pps_extension2_flag | u(1) |
| } |  |
| if( pps_extension2_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

7.3.2.3	Picture parameter set RBSP semantics

…

pps_extension1_flag equal to 1 specifies that log2_max_transform_skip_size_minus2 and pps_extension2_flag are present in the PPS RBSP syntax structure. pps_extension1_flag equal to 0 specifies that these syntax elements are not present.

log2_max_transform_skip_size_minus2 plus 2 specifies the maximum transform unit size for which transform_skip_flag may be present. When not present, the value of log2_max_transform_skip_size_minus2 is inferred to be equal to 0. When present, the value of log2_max_transform_skip_size_minus2 shall be less than or equal to Log2MaxTrafoSize − 2.

The variable Log2MaxTransformSkipSize is derived as follows:

Log2MaxTransformSkipSize = log2_max_transform_skip_size_minus2 + 2	(7-1)

enable_scaling_list_for_transform_skip_flag equal to 1 specifies that scaling lists shall be applied for TUs using the transform-skip path in the same way as for TUs using the transform path. When not present, the value of enable_scaling_list_for_transform_skip_flag is inferred to be equal to 1.

pps_extension2_flag equal to 0 specifies that no pps_extension_data_flag syntax elements are present in the PPS RBSP syntax structure. When present, pps_extension2_flag shall be equal to 0 in bitstreams conforming to this version of this Specification. When not present, the value of pps_extension2_flag is inferred to be equal to 0. The value of 1 for pps_extension2_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall allow the value of pps_extension_flag (when present) to be equal to 1 and shall ignore all pps_extension_data_flag syntax elements that follow the value 1 for pps_extension2_flag in a PPS NAL unit.

…

8.6.3	Scaling process for transform coefficients

…

For the derivation of the scaled transform coefficients d[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1, the following applies:

–	The scaling factor m[ x ][ y ] is derived as follows:

–	If scaling_list_enabled_flag is equal to 0, or transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1 and enable_scaling_list_for_transform_skip_flag is equal to 0,

m[ x ][ y ] = 16		(8-2)

–	Otherwise (scaling_list_enabled_flag is equal to 1),

m[ x ][ y ] = ScalingFactor[ sizeId ][ matrixId ][ x ][ y ]	(8-3)

…

# Appendix B: Draft Text For Option 2 Without Flag

(relative to JCTVC-N1005_v3)

8.6.3	Scaling process for transform coefficients

…

For the derivation of the scaled transform coefficients d[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1, the following applies:

–	The scaling factor m[ x ][ y ] is derived as follows:

–	If one or more of the following conditions is true, then m[ x ][ y ] is set equal to 16:

–	scaling_list_enabled_flag is equal to 0.

–	nTbS is greater than 4 and transform_skip_flag[ xTbY  ][ yTbY ][ cIdx ] is equal to 1.

–	Otherwise m[ x ][ y ] is set equal to ScalingFactor[ sizeId ][ matrixId ][ x ][ y ]

–	If scaling_list_enabled_flag is equal to 0,

m[ x ][ y ] = 16		(8-4)

–	Otherwise (scaling_list_enabled_flag is equal to 1),

m[ x ][ y ] = ScalingFactor[ sizeId ][ matrixId ][ x ][ y ]	(8-5)

…