| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0070 |
| --- | --- |

| Title: | Fixed MPM set in intra mode coding for screen contents |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | proposal |  |  |
| Author(s) or | Xiaozhong Xu Shan Liu Junghye Min Sunil Lee Elena Alshina Chanyul Kim | Email: | xiaozhong.xu@mediatek.com shan.liu@mediatek.com jh643.min@samsung.com sunil.lee@samsung.com elena_a.alshina@samsung.com dionism@samsung.com |
| Source: | MediaTek, Inc., Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution is a follow-up document of JCTVC-N0231. It presents an intra mode coding method for screen contents. 3 MPMs (Most Probable Mode) are set fixed without referring intra modes of neighbouring prediction units. 3 variations of such fixed MPM strategy are described in this document. The proposed method provides performance gains of 0.3%~0.5% for both screen content sequences with RGB and YUV formats for AI lossless testing conditions. For AI lossy testing conditions, gains of 0.8 ~1.0 % are observed for screen content sequences with RGB and YUV formats. Additional flag in SPS can be employed to indicate whether proposed derivation method will be used or not.

# Introduction

It has been proven that intra prediction mode coding using most probable modes from neighbouring prediction units are efficient for coding efficiency of intra prediction. But derivation process of most probable modes in the HM-12.0+RExt-4.1 is complicated as follows.

- For X being replaced by either A or B, the variables candIntraPredModeX are derived as follows:

- The availability derivation process for a block in z-scan order as specified in subclause Error: Reference source not found is invoked with the location ( xCurr, yCurr ) set equal to ( xPb, yPb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbX, yNbX ) as inputs, and the output is assigned to availableX.

- The candidate intra prediction mode candIntraPredModeX is derived as follows:

- If availableX is equal to FALSE, candIntraPredModeX is set equal to INTRA_DC.

- Otherwise, if CuPredMode[ xNbX ][ yNbX ] is not equal to MODE_INTRA or pcm_flag[ xNbX ][ yNbX ] is equal to 1, candIntraPredModeX is set equal to INTRA_DC,

- Otherwise, if X is equal to B and yPb − 1 is less than ( ( yPb  >>  CtbLog2SizeY )  <<  CtbLog2SizeY ), candIntraPredModeB is set equal to INTRA_DC.

- Otherwise, candIntraPredModeX is set equal to IntraPredModeY[ xNbX ][ yNbX ].

- The candModeList[ x ] with x = 0..2 is derived as follows:

- If candIntraPredModeB is equal to candIntraPredModeA, the following applies:

  - If candIntraPredModeA is less than 2 (i.e. equal to INTRA_PLANAR or INTRA_DC), candModeList[ x ] with x = 0..2 is derived as follows:

candModeList[ 0 ] = INTRA_PLANAR	(8-1)

candModeList[ 1 ] = INTRA_DC		(8-2)

candModeList[ 2 ] = INTRA_ANGULAR26	(8-3)

  - Otherwise, candModeList[ x ] with x = 0..2 is derived as follows:

candModeList[ 0 ] = candIntraPredModeA	(8-4)

candModeList[ 1 ] = 2 + ( ( candIntraPredModeA + 29 ) % 32 )	(8-5)

candModeList[ 2 ] = 2 + ( ( candIntraPredModeA − 2 + 1 ) % 32 )	(8-6)

- Otherwise (candIntraPredModeB is not equal to candIntraPredModeA), the following applies:

  - candModeList[ 0 ] and candModeList[ 1 ] are derived as follows:

candModeList[ 0 ] = candIntraPredModeA	(8-7)

candModeList[ 1 ] = candIntraPredModeB	(8-8)

  - If neither of candModeList[ 0 ] and candModeList[ 1 ] is equal to INTRA_PLANAR, candModeList[ 2 ] is set equal to INTRA_PLANAR,

  - Otherwise, if neither of candModeList[ 0 ] and candModeList[ 1 ] is equal to INTRA_DC, candModeList[ 2 ] is set equal to INTRA_DC,

  - Otherwise, candModeList[ 2 ] is set equal to INTRA_ANGULAR26.

For screen contents coding, real time encoding/decoding is more critical than any other applications. A simplified prediction process is desirable for intra coding of screen contents. As in following figure, one of characteristics of screen contents is correlation of patterns along horizontal and vertical directions.

Figure 1. Example of screen contents

Simple derivation method for most probable intra modes utilizing the characteristics of screen contents is proposed in the next section.

# Proposed method

Without referring to neighbouring prediction units, three most probable modes (candModeList[ x ], x=0,1,2) are set as follows in 3 variations.

Variation 1:

candModeList[ 0 ] = INTRA_HOR

candModeList[ 1 ] = INTRA_VER

candModeList[ 2 ] = INTRA_ANGULAR_18

Variation 2:

candModeList[ 0 ] = INTRA_HOR

candModeList[ 1 ] = INTRA_VER

candModeList[ 2 ] = INTRA_DC

Variation 3:

candModeList[ 0 ] = INTRA_HOR

candModeList[ 1 ] = INTRA_VER

candModeList[ 2 ] = INTRA_PLANAR

Variation 1 is just the method we used in previous JCTVC-N0231 document. Variation 2 and 3 come from the observation that in the intra prediction mode distribution—for screen contents, horizontal, vertical, planar and DC modes are the top 4 modes in terms of number of occurrences. Fig.2 shows some examples of such statistics. In Fig. 2, the y axis is the intra mode value; the x axis is the index of occurrence. So the wider the horizontal line segment is, statistically the more chance this mode appears. In HEVC, Planar mode = 0, DC mode = 1, Horizontal mode = 10 and Vertical mode = 26.

Figure 2. Example of intra prediction mode distributions (YUV format, 1st frame)

# Signalling of MPM derivation method

Derivation method for MPM can be signalled in SPS as follows. This flag indicates which derivation method is used for most probable mode of intra mode.

| sps_extension1_flag | u(1) |
| --- | --- |
| if( sps_extension1_flag ) { |  |
| transform_skip_rotation_enabled_flag | u(1) |
| transform_skip_context_enabled_flag | u(1) |
| intra_block_copy_enabled_flag | u(1) |
| residual_dpcm_intra_enabled_flag | u(1) |
| residual_dpcm_inter_enabled_flag | u(1) |
| extended_precision_processing_flag | u(1) |
| intra_smoothing_disabled_flag | u(1) |
| sps_extension2_flag | u(1) |
| sps_fixed_intra_mpm_derivation_flag | u(1) |
| } |  |

sps_fixed_intra_mpm_derivation_flag equal to 1 specifies 3 fixed modes are set to most probable modes for intra without referring to neighbouring prediction units. sps_fixed_intra_mpm_derivation_flag equal to 0 specifies that derivation of 3 most probable modes are derived by referring intra prediction modes of neighbouring prediction units. When not present, the value of sps_fixed_intra_mpm_derivation_flag is inferred to be equal to 0.

# Experimental results

Proposed methods were implemented on top of HM12.0_RExt4.1 s/w under common test condition described in JCTVC-N1123 [2].  Table 1~3 show the performance gains for lossless testing condition

.

Table 1 – Test results for lossless case, variation 1 (Angular mode 18)

Table 2 – Test results for lossless case, variation 2 (DC)

Table 3 – Test results for lossless case, variation 3 (Planar)

Table 4~6 show the performance gains for lossy testing condition.

Table 4 – Test results for lossy case, variation 1 (Angular mode 18)

Table 5 – Test results for lossy case, variation 2 (DC)

Table 6 – Test results for lossy case, variation 3 (Planar)

# Conclusions

This contribution provides a simplified intra mode coding method for screen contents. Derivation process of most probable modes is removed and three MPMs are set to (1) horizontal, vertical and angular_18 mode; (2) horizontal, vertical and DC mode; (3) horizontal, vertical and planar mode. The proposed method provides performance gains for screen contents for both lossy and lossless test conditions. Additional flag in SPS can be employed to indicate whether proposed derivation method will be used or not.

# References

[1] Junghye Min, Sunil Lee and Chanyul Kim, AHG 8: Intra mode coding for screen contents, JCTVC-N0231, 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013.

[2] Ankur Saxena, Do-Kyoung Kwon, Matteo Naccari, Chao Pang, HEVC Range Extensions Core Experiment 3 (RCE3): Intra Prediction techniques, JCTVC-N1123, 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013.

[3] David Flynn, Joel Sole, Teruhiko Suzuki, Range Extensions Draft 4, JCTVC-N1123, 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013.

# Patent rights declaration(s)

MediaTek Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).