| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0071 |
| --- | --- |

| Title: | AhG13: Performance analysis of scalable systems with different down-samplers |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | E. Alshina, A. Alshin 416, Maetan 3-dong, Yeongtong-gu, Suwon, Korea, 443-742 Xiang Li Jianle Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA Jie Dong, Yan Ye 9710 Scranton Rd, Ste 250, San Diego, CA 92121, USA Edouard Francois | Tel: Email: Tel: Email: Tel: Email: Email: | +82 10 3026 1305 elena_a.alshina@samsung.com alexander_b.alshin@samsung.com +1-858-651-8028 lxiang@qti.qualcomm.com cjianle@qti.qualcomm.com martak@qti.qualcomm.com +1 858.210.4809 jie.dong@interdigital.com Edouard.Francois@technicolor.com |
| Source: | Samsung Electronics, Ltd,Qualcomm Incorporated, InterDigital Communications, Inc, Technicolor |  |  |

_____________________________

# Abstract

This contribution compares performance of scalable systems in which 2 different down-samplers are used.  Since base layer is different, BD-rate performance is reported compare to the single layer HEVC. Depending on scalability ratio performance drop of scalable system is 10.5%...14.0% (AI), 16.2%...19.0% (RA), 24.8%...28.5% (LD-B) and 22.8%...26.6% (LD-P) when so-called SHVC down-sampler and 13.5%...15.5% (AI), 17.3%...20.3% (RA), 24.9%...28.7% (LD-B) and 22.8%...26.6% (LD-P) when JSVM down-sampler is used. So the usage of so-called SHVC down-sampler is preferable for all-intra, random access and low-delay-B configurations.

# Introduction

Based on JCTVC 14th meeting decision down-sampler was integrated into the reference s/w (SHM3.0.1). The down-sampler originally was developed by Inter Digital and supported scalability ratio ×2 and ×1.5. For purpose of SCE1 (arbitrary scalability ratio support) down-sampler was generalized and ratios different from ×2 and ×1,5 are supported now. This down-sampler was implemented by Qualcomm.

The goal of this contribution is to verify the performance of scalable system if this newly developed down-sampler is used. In order to do it we compared the performance with well-known JSVM down-sampler (just borrowed from JSVM s/w).

In both cases zero-phase shift between enhancement and base layer picture was used. Tests were performed for scalability ratios ×2 and ×1.5. Additionally scalability ratio ~×1.75 was tested.

# Down-sampler description

## SHVC downsampler

Filters below are so-called SHVC downsampler. It was proposed in MPEG documents M23485 [2] and M24499 [1].

if (  3 * EL_width >  5 * BL_width)   // cases 2x, 1.75x

2,  -3, -9,	 6, 39, 58, 39,  6, -9,	-3,  2,  0        // 0

2,  -3, -9,	 4, 38, 58, 43,  7, -9,	-4,  1,  0        // 1

2,  -2, -9,	 2, 35, 58, 44,  9, -8,	-4,  1,  0        // 2

1,  -2, -9,	 1, 34, 58, 46, 11, -8,	-5,  1,  0        // 3

1,  -1, -8,	-1, 31, 57, 47, 13, -7,	-5,  1,  0        // 4

1,  -1, -8,	-2, 29, 56, 49, 15, -7,	-6,  1,  1        // 5

1,   0, -8,	-3, 26, 55, 51, 17, -7,	-6,  1,  1        // 6

1,   0, -7,	-4, 24, 54, 52, 19, -6,	-7,  1,  1        // 7

1,   0, -7,	-5, 22, 53, 53, 22, -5,	-7,  0,  1        // 8

1,   1, -7,	-6, 19, 52, 54, 24, -4,	-7,  0,  1        // 9

1,   1, -6,	-7, 17, 51, 55, 26, -3,	-8,  0,  1        // 10

1,   1, -6,	-7, 15, 49, 56, 29, -2,	-8, -1,  1        // 11

0,   1, -5,	-7, 13, 47, 57, 31, -1,	-8, -1,  1        // 12

0,   1, -5,	-8, 11, 46, 58, 34,  1,	-9, -2,  1        // 13

0,   1, -4,	-8,  9, 44, 58, 35,  2,	-9, -2,  2        // 14

0,   1, -4,	-9,  7, 43, 58, 38,  4,	-9, -3,  2        // 15

else if (  4 * EL_width >  5 * BL_width)   // case 1.5x

0, 5, -6,	-10,	37,	76, 37, -10,   -6,  5,	0,  0        //0

0, 5, -4,	-11,	33,	76, 40,  -9,   -7,  5,	0,  0        //1

-1, 5, -3,	-12,  29,	75, 45,  -7,   -8,  5,	0,  0        //2

-1, 4, -2,	-13,	25,	75, 48,  -5,   -9,  5,	1,  0        //3

-1, 4, -1,	-13,	22,	73, 52,  -3,  -10,  4,	1,  0        //4

-1, 4,  0,  -13,	18,	72, 55,  -1,  -11,  4,	2, -1        //5

-1, 4,  1,  -13,	14,	70, 59,   2,  -12,  3,	2, -1        //6

-1, 3,  1,  -13,	11,	68, 62,   5,  -12,  3,	2, -1        //7

-1, 3,  2,  -13,	 8,	65, 65,   8,  -13,  2,	3, -1        //8

-1, 2,  3,  -12,	 5,	62, 68,  11,  -13,  1,	3, -1        //9

-1, 2,  3,  -12,	 2,	59, 70,  14,  -13,  1,	4, -1        //10

-1, 2,  4,  -11,	-1,	55, 72,  18,  -13,  0,	4, -1        //11

0, 1,  4,  -10,	-3,	52, 73,  22,  -13, -1,	4, -1        //12

0, 1,  5,   -9,  -5,	48, 75,  25,  -13, -2,	4, -1        //13

0, 0,  5,   -8,  -7,	45, 75,  29,  -12, -3,	5, -1        //14

0, 0,  5,   -7,	-9,	40, 76,  33,  -11, -4,	5,  0        //15

## JSVM downsampler

Filters below are downsampler from JSVM software. For a fair comparison, the JSVM software is further modified to use phase 0 alignment between top left base and enhancement luma samples.

if (  3 * EL_width >  5 * BL_width)   // cases 2x, 1.75x

0,  -4,  -7,  11,  38,  52,  38,  11,  -7,  -4,   0,   0

0,  -4,  -7,   9,  37,  51,  40,  13,  -6,  -7,   2,   0

0,  -3,  -7,   8,  35,  51,  41,  14,  -5,  -7,   1,   0

0,  -2,  -8,   6,  33,  51,  42,  16,  -5,  -7,   2,   0

0,  -2,  -8,   5,  32,  50,  43,  18,  -4,  -8,   2,   0

0,  -2,  -8,   4,  30,  50,  45,  19,  -3,  -8,   1,   0

0,  -1,  -8,   2,  28,  49,  46,  21,  -2,  -8,   1,   0

0,  -1,  -8,   1,  26,  49,  47,  23,  -1,  -8,   0,   0

0,   0,  -8,   0,  24,  48,  48,  24,   0,  -8,   0,   0

0,   0,  -8,  -1,  23,  47,  49,  26,   1,  -8,  -1,   0

0,   1,  -8,  -2,  21,  46,  49,  28,   2,  -8,  -1,   0

0,   1,  -8,  -3,  19,  45,  50,  30,   4,  -8,  -2,   0

0,   2,  -8,  -4,  18,  43,  50,  32,   5,  -8,  -2,   0

0,   2,  -7,  -5,  16,  42,  51,  33,   6,  -8,  -2,   0

0,   1,  -7,  -5,  14,  41,  51,  35,   8,  -7,  -3,   0

0,   2,  -7,  -6,  13,  40,  51,  37,   9,  -7,  -4,   0

else if (  4 * EL_width >  5 * BL_width)   // case 1.5x

2,   0, -10,   0,  40,  64,  40,   0, -10,   0,   2,   0

2,   1,  -9,  -2,  37,  64,  42,   2, -10,  -1,   2,   0

2,   1,  -9,  -3,  34,  64,  44,   4, -10,  -1,   2,   0

2,   1,  -8,  -5,  31,  63,  47,   6, -10,  -2,   3,   0

1,   2,  -8,  -6,  29,  62,  49,   8, -10,  -2,   3,   0

1,   2,  -7,  -7,  26,  61,  52,  10, -10,  -3,   3,   0

1,   2,  -6,  -8,  23,  60,  54,  13, -10,  -4,   3,   0

1,   2,  -6,  -9,  20,  59,  56,  15, -10,  -4,   3,   1

1,   2,  -5,  -9,  18,  57,  57,  18,  -9,  -5,   2,   1

1,   3,  -4, -10,  15,  56,  59,  20,  -9,  -6,   2,   1

0,   3,  -4, -10,  13,  54,  60,  23,  -8,  -6,   2,   1

0,   3,  -3, -10,  10,  52,  61,  26,  -7,  -7,   2,   1

0,   3,  -2, -10,   8,  49,  62,  29,  -6,  -8,   2,   1

0,   3,  -2, -10,   6,  47,  63,  31,  -5,  -8,   1,   2

0,   2,  -1, -10,   4,  44,  64,  34,  -3,  -9,   1,   2

0,   2,  -1, -10,   2,  42,  64,  37,  -2,  -9,   1,   2

# Test results summary

Performance test results are reported compare to HEVC single layer. HM11.0 was used for single layer performance generation.

Table 1. Scalable system with SHVC down-sampler performance compared to single layer.

| BD-rate vs HM11.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Y | U | V | Y | U | V | Y | U | V |
| AI HEVC 2x |  |  | AI HEVC ~1.75x |  |  | AI HEVC 1.5x |  |  |
| 12,8% | 14,9% | 14,6% | 14,0% | 15,3% | 14,8% | 10,5% | 9,8% | 9,3% |
| RA HEVC 2x |  |  | RA HEVC ~1.75x |  |  | RA HEVC 1.5x |  |  |
| 19,0% | 33,1% | 31,8% | 19,5% | 33,2% | 32,1% | 16,2% | 28,9% | 29,2% |
| LD-B HEVC 2x |  |  | LD-B HEVC ~1.75x |  |  | LD-B HEVC 1.5x |  |  |
| 28,3% | 38,9% | 39,7% | 28,5% | 38,6% | 39,9% | 24,8% | 33,2% | 36,0% |
| LD-P HEVC 2x |  |  | LD-P HEVC ~1.75x |  |  | LD-P HEVC 1.5x |  |  |
| 26,5% | 37,9% | 38,9% | 26,6% | 38,0% | 39,5% | 22,8% | 32,8% | 35,6% |

Table 2. Scalable system with JSVM down-sampler performance compared to single layer.

| BD-rate vs HM11.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Y | U | V | Y | U | V | Y | U | V |
| AI HEVC 2x |  |  | AI HEVC ~1.75x |  |  | AI HEVC 1.5x |  |  |
| 14,1% | 16,4% | 16,1% | 15,5% | 17,2% | 16,7% | 13,3% | 13,2% | 13,0% |
| RA HEVC 2x |  |  | RA HEVC ~1.75x |  |  | RA HEVC 1.5x |  |  |
| 19,4% | 32,8% | 31,3% | 20,3% | 33,3% | 31,9% | 17,3% | 28,6% | 28,7% |
| LD-B HEVC 2x |  |  | LD-B HEVC ~1.75x |  |  | LD-B HEVC 1.5x |  |  |
| 28,0% | 37,8% | 38,5% | 28,7% | 38,2% | 39,2% | 24,9% | 32,0% | 34,5% |
| LD-P HEVC 2x |  |  | LD-P HEVC ~1.75x |  |  | LD-P HEVC 1.5x |  |  |
| 26,1% | 36,9% | 37,9% | 26,6% | 37,2% | 38,4% | 22,8% | 31,4% | 34,3% |

Table 1 summarizes results for the case when SHVC down-sampler was used for base layer generation. Table 2 summarizes results for the case when JSVM down-sampler was used for base layer generation. These tests confirm that the usage of SHVC down-sampler is clearly beneficial in case of all-intra, random access and low-delay-B configurations. In low-delay-P configuration the usage of JSVM down-sampler is slightly better. Performance reported was used as an anchor for SCE1 tests. Details of test results can be found in SCE contribution [3].

# Conclusion

The performance of scalable system using newly developed so-called SHVC down-sampler is verified in this contribution. Scalability ratios ×2, ×1.5 and ~×1.75 were studied. Performance was compared with scalable system which uses JSVM down-sampler and SHVC down-sampler is found to be better for all-intra, random access and low-delay-B configurations. Samsung,Qualcomm, InterDigital and Technicolor propose to include the code of SHVC downsampler into the next release of SHM software.

# References

[1] Jie  Dong, Yong He, and Yan Ye,  "Downsampling filter for anchor generation for scalable

extensions of HEVC," m24499, 100th MPEG meeting, Geneva, Switzerland, May 2012.

[2] Jie  Dong, Yong He, and Yan Ye,  "Downsampling filter for anchor generation for scalable

extensions of HEVC," m23485, 99th MPEG meeting, San José, USA, Feb. 2012

[3] E.Alshina, A.Alshin ,J.Chen, X.Li, M.Karczewicz, “SCE1: performance and complexity of test 1.1”, JCTVC-O0071, Geneva, Switzerland, Oct. 2013

# Patent rights declarations

Samsung Electronics, Ltd. does not have any current or pending patent rights relating to the technology described in this contribution.

Qualcomm Incorporated does not have any current or pending patent rights relating to the technology described in this contribution.

InterDigital, Ltd. does not have any current or pending patent rights relating to the technology described in this contribution.

Technicolor Incorporated does not have any current or pending patent rights relating to the technology described in this contribution.