| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0072 |
| --- | --- |

| Title: | AhG13: On Chroma accurate position alignment during re-sampling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | E. Alshina, A. Alshin 416, Maetan 3-dong, Yeongtong-gu, Suwon, Korea, 443-742 | Tel: Email: | +82 10 3026 1305 elena_a.alshina@samsung.com alexander_b.alshin@samsung.com |
| Source: | Samsung Electronics, Ltd. |  |  |

_____________________________

# Abstract

Contribution presents the set performance tests with misaligned Chroma position during re-sampling process. If accurate Chroma position is not taken into account then re-sampling process can be simplified: reference position calculation for both Luma and Chroma, vertical and horizontal interpolation becomes the same. This simplification leads to no performance degradation in terms of Luma BD-rate (0,1% Chroma BD-rate drop is observed).

# Introduction

In 4:2:0 color format Chroma samples position relatively to Luma samples can be one of 6 shown in Figure 1. In SHM3.0.1 reference s/w and current specification draft case “b” (0 horizontal displacement and ½ pel vertical displacements between Luma and Chroma positions) is assumed. The chroma position during 4:4:4 to 4:2:0 conversion is not necessarily under user’s control. There are two corner cases: to ignore accurate chroma position during re-sampling or to take it into account and signal to the decoder.

This contribution investigates performance effect of chroma position misalignment during re-sampling.  Without test content modification we force to use cases (a)…(f) during re-sampling and run spatial scalability tests (×2 and ×1,5).

Average performance variation compare to anchor is summarized in Table 1.

Table 1. Brief summary of performance test results.

| Chroma position assumed during re-sampling | Avg. BD-rate |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| a | 0,0% | 0,1% | 0,1% |
| b | 0,0% | 0,0% | 0,0% |
| c | 0,1% | 0,1% | 0,1% |
| d | 0,1% | 0,0% | 0,0% |
| e | 0,2% | 0,1% | 0,0% |
| f | 0,1% | 0,2% | 0,1% |

Since no critical performance change is observed then is seems reasonable to simplify re-sampling process and ignore accurate chroma position alignment during re-sampling.

Figure 1. Chroma samples position relatively to Luma in 4:2:0 color format.

# Specification text modification

Specification text simplifications are highlighted below.

H6.2. Derivation process for reference layer sample location used in resampling

Inputs to this process are

–	a variable cIdx specifying the color component index,

–	a sample location ( xP, yP ) relative to the top-left sample of the color component of the current picture specified by cIdx.

Output of this process is a sample location ( xRef16, yRef16 ) specifying the reference layer sample location in units of 1/16-th sample relative to the top-left sample of the reference layer picture.

The variables offsetX and offsetY are derived as follows:

offsetX = ScaledRefLayerLeftOffset / ( ( cIdx = = 0)  ?  1 :  SubWidthC)	(H-1)
offsetY = ScaledRefLayerTopOffset / ( ( cIdx = = 0)  ?  1 :  SubHeightC)	(H-2)

The variables phaseY and addY are derived as follows:

phaseY = (cIdx = = 0) ? 0 : 1		(H-3)

addY = ( ScaleFactorY * phaseY + 2 ) >> 2		(H-4)

The variables xRef16 and yRef16 are derived as follows:

xRef16 = ( ( ( xP – offsetX ) * ScaleFactorX  + ( 1 << 11 ) ) >> 12 ) 	(H-5)
yRef16 = ( ( ( yP – offsetY ) * ScaleFactorY + addY + ( 1 << 11 ) ) >> 12 ) –  ( phaseY << 2 )	(H-6)

# Conclusion

Based on demonstrated performance test Samsung Electronics proposes simplification of re-ampling process by not taking into account accurate chroma alignement.

# Patent rights declarations

Samsung Electronics, Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Test results

Reference s/w version 3.0.1 was used as an anchor.

Table 2. Chroma position “a” is assumed during re-sampling.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | 0,1% | 0,2% |  |  |  |
| Class B | 0,1% | 0,1% | 0,1% | 0,0% | 0,0% | 0,0% |
| Overall (Test vs Ref) | 0,1% | 0,1% | 0,1% | 0,0% | 0,0% | 0,0% |
| Overall (Test vs single layer) | 12,8% | 15,0% | 14,7% | 10,6% | 9,8% | 9,3% |
| Overall (Ref vs single layer) | 12,8% | 14,9% | 14,6% | 10,5% | 9,8% | 9,3% |
| EL only (Test vs Ref) | 0,1% | 0,2% | 0,2% | 0,1% | 0,1% | 0,1% |
| Enc Time[%] | 98,5% |  |  | 98,6% |  |  |
| Dec Time[%] | 98,4% |  |  | 97,9% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | -0,1% | 0,2% |  |  |  |
| Class B | 0,1% | 0,2% | 0,2% | 0,0% | 0,0% | -0,1% |
| Overall (Test vs Ref) | 0,1% | 0,1% | 0,2% | 0,0% | 0,0% | -0,1% |
| Overall (Test vs single layer) | 19,1% | 33,3% | 32,1% | 16,2% | 28,9% | 29,1% |
| Overall (Ref vs single layer) | 19,0% | 33,1% | 31,8% | 16,2% | 28,9% | 29,2% |
| EL only (Test vs Ref) | 0,1% | 0,2% | 0,2% | 0,1% | 0,1% | 0,0% |
| Enc Time[%] | 96,6% |  |  | 96,2% |  |  |
| Dec Time[%] | 94,1% |  |  | 93,4% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | 0,0% | 0,5% |  |  |  |
| Class B | 0,0% | 0,3% | 0,3% | 0,0% | 0,2% | 0,0% |
| Overall (Test vs Ref) | 0,0% | 0,2% | 0,4% | 0,0% | 0,2% | 0,0% |
| Overall (Test vs single layer) | 28,4% | 39,1% | 40,3% | 24,8% | 33,4% | 36,0% |
| Overall (Ref vs single layer) | 28,3% | 38,9% | 39,7% | 24,8% | 33,2% | 36,0% |
| EL only (Test vs Ref) | 0,1% | 0,2% | 0,4% | 0,1% | 0,2% | 0,1% |
| Enc Time[%] | 96,9% |  |  | 95,7% |  |  |
| Dec Time[%] | 93,7% |  |  | 92,1% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
| Optional Tests |  |  |  |  |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | -0,1% | 0,4% |  |  |  |
| Class B | 0,1% | 0,3% | 0,3% | 0,0% | 0,1% | 0,2% |
| Overall (Test vs Ref) | 0,0% | 0,2% | 0,3% | 0,0% | 0,1% | 0,2% |
| Overall (Test vs single layer) | 26,5% | 38,2% | 39,4% | 22,9% | 33,0% | 35,9% |
| Overall (Ref vs single layer) | 26,5% | 37,9% | 38,9% | 22,8% | 32,8% | 35,6% |
| EL only (Test vs Ref) | 0,1% | 0,2% | 0,3% | 0,1% | 0,2% | 0,3% |
| Enc Time[%] | 96,2% |  |  | 95,8% |  |  |
| Dec Time[%] | 94,3% |  |  | 92,1% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Table 3. Chroma position “c” is assumed during re-sampling.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,1% | 0,0% | 0,0% |  |  |  |
| Class B | 0,2% | 0,0% | 0,0% | 0,2% | 0,1% | 0,1% |
| Overall (Test vs Ref) | 0,2% | 0,0% | 0,0% | 0,2% | 0,1% | 0,1% |
| Overall (Test vs single layer) | 13,0% | 14,9% | 14,6% | 10,7% | 9,9% | 9,4% |
| Overall (Ref vs single layer) | 12,8% | 14,9% | 14,6% | 10,5% | 9,8% | 9,3% |
| EL only (Test vs Ref) | 0,3% | 0,1% | 0,1% | 0,4% | 0,4% | 0,4% |
| Enc Time[%] | 98,5% |  |  | 98,6% |  |  |
| Dec Time[%] | 98,1% |  |  | 97,7% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,1% | -0,1% | 0,0% |  |  |  |
| Class B | 0,1% | 0,0% | 0,0% | 0,1% | 0,2% | 0,2% |
| Overall (Test vs Ref) | 0,1% | 0,0% | 0,0% | 0,1% | 0,2% | 0,2% |
| Overall (Test vs single layer) | 19,2% | 33,1% | 31,8% | 16,3% | 29,2% | 29,4% |
| Overall (Ref vs single layer) | 19,0% | 33,1% | 31,8% | 16,2% | 28,9% | 29,2% |
| EL only (Test vs Ref) | 0,2% | 0,1% | 0,1% | 0,3% | 0,4% | 0,4% |
| Enc Time[%] | 96,5% |  |  | 96,1% |  |  |
| Dec Time[%] | 93,9% |  |  | 93,4% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | -0,1% | 0,2% |  |  |  |
| Class B | 0,1% | 0,2% | 0,1% | 0,1% | 0,2% | 0,2% |
| Overall (Test vs Ref) | 0,1% | 0,1% | 0,1% | 0,1% | 0,2% | 0,2% |
| Overall (Test vs single layer) | 28,4% | 39,0% | 39,9% | 24,9% | 33,4% | 36,3% |
| Overall (Ref vs single layer) | 28,3% | 38,9% | 39,7% | 24,8% | 33,2% | 36,0% |
| EL only (Test vs Ref) | 0,1% | 0,2% | 0,2% | 0,3% | 0,3% | 0,4% |
| Enc Time[%] | 96,8% |  |  | 95,7% |  |  |
| Dec Time[%] | 94,3% |  |  | 92,9% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
| Optional Tests |  |  |  |  |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | -0,1% | 0,0% |  |  |  |
| Class B | 0,1% | 0,1% | 0,1% | 0,1% | 0,2% | 0,2% |
| Overall (Test vs Ref) | 0,1% | 0,1% | 0,1% | 0,1% | 0,2% | 0,2% |
| Overall (Test vs single layer) | 26,6% | 38,0% | 39,0% | 22,9% | 33,0% | 36,0% |
| Overall (Ref vs single layer) | 26,5% | 37,9% | 38,9% | 22,8% | 32,8% | 35,6% |
| EL only (Test vs Ref) | 0,1% | 0,1% | 0,1% | 0,3% | 0,4% | 0,4% |
| Enc Time[%] | 96,1% |  |  | 95,7% |  |  |
| Dec Time[%] | 94,7% |  |  | 91,2% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Table 4. Chroma position “d” is assumed during re-sampling.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,1% | -0,1% | -0,1% |  |  |  |
| Class B | 0,1% | -0,1% | -0,1% | 0,1% | 0,0% | 0,1% |
| Overall (Test vs Ref) | 0,1% | -0,1% | -0,1% | 0,1% | 0,0% | 0,1% |
| Overall (Test vs single layer) | 12,9% | 14,8% | 14,4% | 10,7% | 9,8% | 9,4% |
| Overall (Ref vs single layer) | 12,8% | 14,9% | 14,6% | 10,5% | 9,8% | 9,3% |
| EL only (Test vs Ref) | 0,2% | 0,0% | 0,0% | 0,4% | 0,3% | 0,3% |
| Enc Time[%] | 99,4% |  |  | 98,9% |  |  |
| Dec Time[%] | 99,0% |  |  | 97,9% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,1% | 0,2% | 0,0% |  |  |  |
| Class B | 0,1% | -0,1% | -0,1% | 0,1% | 0,2% | 0,2% |
| Overall (Test vs Ref) | 0,1% | 0,0% | -0,1% | 0,1% | 0,2% | 0,2% |
| Overall (Test vs single layer) | 19,1% | 33,1% | 31,7% | 16,3% | 29,1% | 29,4% |
| Overall (Ref vs single layer) | 19,0% | 33,1% | 31,8% | 16,2% | 28,9% | 29,2% |
| EL only (Test vs Ref) | 0,2% | 0,1% | 0,0% | 0,2% | 0,3% | 0,4% |
| Enc Time[%] | 97,6% |  |  | 97,5% |  |  |
| Dec Time[%] | 95,0% |  |  | 94,6% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | 0,1% | 0,0% |  |  |  |
| Class B | 0,1% | 0,0% | -0,2% | 0,1% | 0,0% | 0,0% |
| Overall (Test vs Ref) | 0,0% | 0,0% | -0,2% | 0,1% | 0,0% | 0,0% |
| Overall (Test vs single layer) | 28,4% | 38,9% | 39,5% | 24,9% | 33,2% | 35,9% |
| Overall (Ref vs single layer) | 28,3% | 38,9% | 39,7% | 24,8% | 33,2% | 36,0% |
| EL only (Test vs Ref) | 0,1% | 0,0% | -0,1% | 0,2% | 0,1% | 0,1% |
| Enc Time[%] | 98,2% |  |  | 96,4% |  |  |
| Dec Time[%] | 95,1% |  |  | 93,3% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
| Optional Tests |  |  |  |  |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | 0,0% | 0,0% |  |  |  |
| Class B | 0,1% | 0,1% | -0,1% | 0,1% | 0,0% | 0,0% |
| Overall (Test vs Ref) | 0,1% | 0,1% | -0,1% | 0,1% | 0,0% | 0,0% |
| Overall (Test vs single layer) | 26,5% | 38,0% | 38,8% | 22,9% | 32,8% | 35,7% |
| Overall (Ref vs single layer) | 26,5% | 37,9% | 38,9% | 22,8% | 32,8% | 35,6% |
| EL only (Test vs Ref) | 0,1% | 0,1% | 0,0% | 0,2% | 0,2% | 0,1% |
| Enc Time[%] | 97,4% |  |  | 96,8% |  |  |
| Dec Time[%] | 95,6% |  |  | 93,3% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Table 5. Chroma position “e” is assumed during re-sampling.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,3% | 0,0% | -0,1% |  |  |  |
| Class B | 0,3% | 0,0% | -0,1% | 0,3% | 0,1% | 0,1% |
| Overall (Test vs Ref) | 0,3% | 0,0% | -0,1% | 0,3% | 0,1% | 0,1% |
| Overall (Test vs single layer) | 13,2% | 14,8% | 14,5% | 10,8% | 9,9% | 9,5% |
| Overall (Ref vs single layer) | 12,8% | 14,9% | 14,6% | 10,5% | 9,8% | 9,3% |
| EL only (Test vs Ref) | 0,6% | 0,2% | 0,1% | 0,8% | 0,6% | 0,6% |
| Enc Time[%] | 98,4% |  |  | 98,4% |  |  |
| Dec Time[%] | 97,8% |  |  | 97,9% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,2% | 1,1% | 0,3% |  |  |  |
| Class B | 0,2% | 0,1% | -0,1% | 0,2% | 0,5% | 0,5% |
| Overall (Test vs Ref) | 0,2% | 0,4% | 0,0% | 0,2% | 0,5% | 0,5% |
| Overall (Test vs single layer) | 19,3% | 33,7% | 31,9% | 16,4% | 29,5% | 29,9% |
| Overall (Ref vs single layer) | 19,0% | 33,1% | 31,8% | 16,2% | 28,9% | 29,2% |
| EL only (Test vs Ref) | 0,4% | 0,6% | 0,2% | 0,5% | 0,8% | 0,9% |
| Enc Time[%] | 96,4% |  |  | 96,0% |  |  |
| Dec Time[%] | 93,8% |  |  | 93,3% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,1% | 0,7% | 0,2% |  |  |  |
| Class B | 0,1% | -0,2% | -0,4% | 0,1% | 0,0% | 0,1% |
| Overall (Test vs Ref) | 0,1% | 0,1% | -0,2% | 0,1% | 0,0% | 0,1% |
| Overall (Test vs single layer) | 28,5% | 39,0% | 39,4% | 24,9% | 33,1% | 36,1% |
| Overall (Ref vs single layer) | 28,3% | 38,9% | 39,7% | 24,8% | 33,2% | 36,0% |
| EL only (Test vs Ref) | 0,2% | 0,2% | -0,1% | 0,4% | 0,2% | 0,3% |
| Enc Time[%] | 96,7% |  |  | 95,6% |  |  |
| Dec Time[%] | 93,4% |  |  | 92,3% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
| Optional Tests |  |  |  |  |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,1% | 0,7% | -0,1% |  |  |  |
| Class B | 0,2% | -0,1% | -0,3% | 0,1% | 0,0% | 0,1% |
| Overall (Test vs Ref) | 0,2% | 0,2% | -0,2% | 0,1% | 0,0% | 0,1% |
| Overall (Test vs single layer) | 26,7% | 38,2% | 38,6% | 23,0% | 32,8% | 35,7% |
| Overall (Ref vs single layer) | 26,5% | 37,9% | 38,9% | 22,8% | 32,8% | 35,6% |
| EL only (Test vs Ref) | 0,3% | 0,3% | -0,1% | 0,4% | 0,3% | 0,3% |
| Enc Time[%] | 96,0% |  |  | 95,6% |  |  |
| Dec Time[%] | 95,1% |  |  | 92,3% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Table 6. Chroma position “f” is assumed during re-sampling.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,2% | 0,1% | 0,0% |  |  |  |
| Class B | 0,2% | 0,1% | 0,0% | 0,1% | 0,0% | 0,1% |
| Overall (Test vs Ref) | 0,2% | 0,1% | 0,0% | 0,1% | 0,0% | 0,1% |
| Overall (Test vs single layer) | 13,0% | 14,9% | 14,6% | 10,7% | 9,8% | 9,4% |
| Overall (Ref vs single layer) | 12,8% | 14,9% | 14,6% | 10,5% | 9,8% | 9,3% |
| EL only (Test vs Ref) | 0,3% | 0,2% | 0,2% | 0,4% | 0,3% | 0,4% |
| Enc Time[%] | 98,2% |  |  | 98,6% |  |  |
| Dec Time[%] | 97,9% |  |  | 98,4% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,1% | 0,9% | 0,2% |  |  |  |
| Class B | 0,1% | 0,2% | 0,0% | 0,1% | 0,3% | 0,3% |
| Overall (Test vs Ref) | 0,1% | 0,4% | 0,1% | 0,1% | 0,3% | 0,3% |
| Overall (Test vs single layer) | 19,2% | 33,7% | 31,9% | 16,3% | 29,2% | 29,6% |
| Overall (Ref vs single layer) | 19,0% | 33,1% | 31,8% | 16,2% | 28,9% | 29,2% |
| EL only (Test vs Ref) | 0,2% | 0,5% | 0,2% | 0,3% | 0,5% | 0,5% |
| Enc Time[%] | 96,3% |  |  | 96,3% |  |  |
| Dec Time[%] | 93,5% |  |  | 92,9% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,1% | 0,6% | 0,2% |  |  |  |
| Class B | 0,1% | 0,0% | -0,1% | 0,1% | 0,1% | 0,2% |
| Overall (Test vs Ref) | 0,1% | 0,1% | 0,0% | 0,1% | 0,1% | 0,2% |
| Overall (Test vs single layer) | 28,4% | 39,1% | 39,7% | 24,9% | 33,3% | 36,2% |
| Overall (Ref vs single layer) | 28,3% | 38,9% | 39,7% | 24,8% | 33,2% | 36,0% |
| EL only (Test vs Ref) | 0,1% | 0,2% | 0,1% | 0,2% | 0,2% | 0,3% |
| Enc Time[%] | 96,6% |  |  | 95,6% |  |  |
| Dec Time[%] | 94,0% |  |  | 92,1% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
| Optional Tests |  |  |  |  |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,1% | 0,5% | -0,1% |  |  |  |
| Class B | 0,1% | 0,0% | 0,0% | 0,1% | 0,0% | 0,2% |
| Overall (Test vs Ref) | 0,1% | 0,2% | -0,1% | 0,1% | 0,0% | 0,2% |
| Overall (Test vs single layer) | 26,6% | 38,2% | 38,9% | 22,9% | 32,8% | 35,9% |
| Overall (Ref vs single layer) | 26,5% | 37,9% | 38,9% | 22,8% | 32,8% | 35,6% |
| EL only (Test vs Ref) | 0,2% | 0,2% | 0,0% | 0,2% | 0,1% | 0,3% |
| Enc Time[%] | 96,0% |  |  | 95,8% |  |  |
| Dec Time[%] | 94,2% |  |  | 91,6% |  |  |
| BL Match | Matched |  |  | Matched |  |  |