| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0073 |
| --- | --- |

| Title: | AhG5: On context modelling simplification for Intra_bc_flag coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | E. Alshina, A. Alshin 416, Maetan 3-dong, Yeongtong-gu, Suwon, Korea, 443-742 | Tel: Email: | +82 10 3026 1305 elena_a.alshina@samsung.com alexander_b.alshin@samsung.com |
| Source: | Samsung Electronics, Ltd. |  |  |

_____________________________

# Abstract

Contribution present performance test results for Intra Block Copy signalling simplification. In current implementation context for this flag is predicted from above and left. 3 context models are used in total. If both predictions from left and above are not used then 2 context models are removed from design and lien buffer for Intra Block Copy flag storage is not needed. The performance change in loss-less test is 0,0% for all test classes (for one class even 0,2% gain is observed). In lossy test configuration average performance change is 0,0% for all 3 color components.

# Introduction

In HEVC design only 2 flags (split and skip) use context prediction from left and above block. Recently added into the design Intra Block Copy (IntraBC) is enabled at CU level and 1 bit flag is encoded. 3 context models are used for this flag encoding. Switching between them is perfumed based on IntraBC flag value in above and left blocks. This requires the storage of IntraBC flags in the whole line of CTUs above.

In this contribution we propose to remove context model for IntraBC flag coding selection depending on left and above block. 2 context models are removed from the design and line buffer for IntraBC flags storage is not needed. For fair comparison initialization of context model used for IntraBC flag coding was un-changed compare to anchor.

# Specification text modification

Specification text modifications are described below.

Table 9-1 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

…

| coding_unit( ) | cu_transquant_bypass_flag | Error: Reference source not found | 0 | 1 | 2 |
| --- | --- | --- | --- | --- | --- |
|  | cu_skip_flag | Error: Reference source not found |  | 0..2 | 3..5 |
|  | intra_bc_flag[ ][ ] | Table 9-2 | 00..2 | 13..5 | 26..8 |
|  | pred_mode_flag | Error: Reference source not found |  | 0 | 1 |
|  | part_mode | Error: Reference source not found | 0 | 1..4 | 5..8 |
|  | prev_intra_luma_pred_flag[ ][ ] | Error: Reference source not found | 0 | 1 | 2 |
|  | intra_chroma_pred_mode[ ][ ] | Error: Reference source not found | 0 | 1 | 2 |
|  | rqt_root_cbf | Error: Reference source not found |  | 0 | 1 |

Table 9-2 – Values of initValue for ctxIdx of intra_bc_flag

| Initialization variable | ctxIdx of intra_bc_flag |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 13 | 4 | 5 | 26 | 7 | 8 |
| initValue | 185 | 185 | 201 | 197 | 197 | 185 | 197 | 197 | 185 |

# Test results

REXT s/w version 4.1 with default settings agreed in RCE2 and RCE3 was used as an anchor in our test. Performance test results for lossy configuration are summarized in Table 1. Performance test results for loss-less configuration are summarized in Table 2.

Table 1. Performance test results IntraBC context modelling simplification (lossy configuration).

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | 0,1% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class B | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| SC RGB 444 | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Animation RGB 444 | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| SC YUV 444 | 0,1% | 0,1% | 0,1% | 0,1% | 0,1% | 0,1% | 0,0% | 0,1% | 0,1% |
| Animation YUV 444 | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| RangeExt | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| SC(444) GBR | 0,1% | 0,0% | 0,0% | 0,2% | 0,0% | 0,1% | 0,1% | 0,0% | 0,1% |
| SC(444) YUV | 0,1% | 0,1% | 0,1% | 0,1% | 0,1% | 0,1% | 0,1% | 0,1% | 0,1% |
| Enc Time[%] | 101% |  |  | 101% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  | 100% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0,1% | 0,1% | 0,0% | 0,1% | 0,1% | 0,1% |  |  |  |
| Class B | 0,0% | 0,0% | 0,1% | 0,0% | 0,0% | 0,0% |  |  |  |
| SC RGB 444 | 0,1% | 0,0% | 0,1% | -0,1% | -0,1% | -0,1% |  |  |  |
| Animation RGB 444 | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |  |  |  |
| SC YUV 444 | 0,0% | -0,1% | 0,0% | 0,0% | -0,1% | 0,0% |  |  |  |
| Animation YUV 444 | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |  |  |  |
| RangeExt | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |  |  |  |
| SC(444) GBR | 0,1% | 0,0% | -0,1% | 0,1% | 0,1% | 0,1% |  |  |  |
| SC(444) YUV | -0,2% | -0,2% | -0,3% | 0,0% | -0,2% | -0,2% |  |  |  |
| Enc Time[%] | 101% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0,1% | -0,6% | -0,7% | 0,1% | 0,0% | -0,4% |  |  |  |
| Class B | 0,0% | -0,1% | -0,2% | 0,0% | -0,1% | 0,0% |  |  |  |
| SC RGB 444 | 0,6% | 0,6% | 0,6% | 0,4% | 0,5% | 0,3% |  |  |  |
| Animation RGB 444 | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |  |  |  |
| SC YUV 444 | -0,2% | -0,2% | -0,4% | -0,2% | -0,2% | -0,3% |  |  |  |
| Animation YUV 444 | 0,0% | -0,1% | 0,0% | 0,0% | -0,2% | -0,1% |  |  |  |
| RangeExt | 0,0% | 0,1% | 0,0% | 0,0% | 0,0% | -0,1% |  |  |  |
| SC(444) GBR | -0,6% | -0,5% | -0,6% | 0,2% | 0,3% | 0,3% |  |  |  |
| SC(444) YUV | -0,6% | -0,4% | -0,5% | 0,0% | 0,1% | 0,0% |  |  |  |
| Enc Time[%] | 101% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

Table 2. Performance test results IntraBC context modelling simplification (loss-less configuration).

|  | AI Main |  | RA Main |  | LB Main |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Total) | Bit-rate saving (Average) |
| Class F | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class B | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| SC RGB 444 | 0,0% | 0,0% | 0,0% | -0,1% | 0,0% | 0,0% |
| Animation RGB 444 | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| SC YUV 444 | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Animation YUV 444 | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| RangeExt | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| SC GBR 444 Optional | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | -0,2% |
| SC YUV 444 Optional | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Enc Time[%] | 100% |  | 100% |  | 100% |  |
| Dec Time[%] | 99% |  | 99% |  | 98% |  |

In both tests as average performance change is under noise level (0,0X%).

# Conclusion

Based on demonstrated no performance degradation Samsung Electronics proposes simplification of intra_bc_flag coding by removing context dependency from left and above block.  Line buffer for intra_bc_flag storage and 2 conexts models will removed from the design.

# Patent rights declarations

Samsung Electronics, Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).