| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0074 |
| --- | --- |

| Title: | AhG5: Intra block copy within one LCU |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | E. Alshina, A. Alshin, S. Lee 416, Maetan 3-dong, Yeongtong-gu, Suwon, Korea, 443-742 | Tel: Email: | +82 10 3026 1305 elena_a.alshina@samsung.com alexander_b.alshin@samsung.com sunil.lee@samsung.com |
| Source: | Samsung Electronics, Ltd. |  |  |

_____________________________

# Abstract

Contribution presents performance and complexity of intra block copy depending on intra motion vector range. If intra block copy is allowed within only current LCU then BD-rate gain from this tool shown for screen content is 19.7% in all-intra, 15.9% in random access and low-delay tests. This is 71-72% of intra block copy gain shown in default configuration (27.4% in all-intra, 22.3% in random access and low-delay tests) in lossy test. In lossless test situation is very similar 18,2% (from 25,0%) in all-intra test and 14,4% (21,0%) in MC test IntraBC gain is preserved if search range is restricted for one LCU. The restriction of intra block copy within current LCU only is highly preferable for HW implementation.

# Introduction

Intra block copy is well known technique. It was included into the draft of HEVC range extension  standard at JCTVC 14th meeting.

For Intra motion vector the HEVC syntax for motion vector difference is utilized. So in current design Intra motion vector value is not limited. This introduces 2 problems:

- Intra motion vector might point to the region of Intra frame which is not decoded yet (for example (0,0) vector points to the current block). Decoder behavior is not specified in this case.

- In HEVC implementation decoder stores current LCU plus 4 lines of above LCU and 4 columns of left LCU (for de-blocking and SAO).

Intra motion vector range is controlled only by encoder so far.

In order not to point to non-decoder yet region Intra motion vector (iMVx, iMVy) shall be restricted as follows:

- iMVx * iMVy  ≤ 0 (cannot be both positive);

- |iMVx|+|iMVy| ≥ CUsize (CUsize× CUsize region shall be available for copy).

In default reference s/w configuration intra motion vector search range is restricted to the current LCU plus LCU at the left (Figure 1). It means that decoder should keep in the memory one extra LCU (left LCU).

When adoption of intra block copy was discussed quite interesting modification of design was demonstrated. In simplified version intra block copy was restricted to the current LCU plus 4 columns of left LCU. These samples are storied in decoder any way for de-blocking and SAO.

Here we utilize the same concept. The intra motion vector is restricted to be within current LCU plus 4 columns of left LCU and 4 rows of LCU above (Figure 2). As mentioned above these samples are anyway available due to de-blocking and SAO.

Figure 1. Intra block copy range in default s/w configuration.

Figure 2. Proposed Intra block copy range.

# About non-available sample handling

One way to handle non-available for Intra block copy samples is described in Fig.3. Proposed above restriction is used for maximum value displacement vector restriction.

Green square area is possible area intra block copy displacement vector might point.

At the beginning of LCU encoding/decoding all samples in memory assigned for reconstructed signal storage are filled with (1<<(bit_depth-1)) value (middle grey). After each block reconstruction is finished middle grey samples are replaced with actual reconstructed samples. In this case whole current LCU plus 4 columns of left LCU and 4 rows of above LCU are available for intra block copy and description for allowed values of intra block copy displacement vector is clean and short.

Figure 3. Proposed Intra block copy design.

# Test results

Test results in this section are presented as follows. The reference s/w v4.1 was modified as agreed in RCE2 and RCE3. In the anchor below intra block copy is disabled. In first test intra block copy is enabled (with all default fast search settings in 4.1 s/w) and search range is current LCU plus LCU at the left. In second test intra block copy is enable (with all default fast search settings in 4.0 s/w) but search change is current LCU plus 4 lines of LCU above and 2 columns of LCU at the left.

Table 1 shows average across screen content test sequences performance and run-time numbers for both tests. Table 2 shows details of test 1, Table 3 shows details of test 2.

Table 1. Brief summary of performance and run-time for intra block copy (average across screen content).

| Search Range additionally to current LCU | All Intra |  | RA&LD |  |
| --- | --- | --- | --- | --- |
|  | Y, BD-rate | Enc time | Y, BD-rate | Enc time |
| Lossy test |  |  |  |  |
| LCU at the left | -27,4% | 139% | -22,3% | 114% |
| 4 lines above LCU + 4 columns left LCU | -19,7% | 126% | -15,9% | 109% |
| Lossless test |  |  |  |  |
| LCU at the left | 25,0% | 156% | 19,9% | 111% |
| 4 lines above LCU + 4 columns left LCU | 18,2% | 135% | 14,4% | 108% |

Table 2. Intra block copy is disabled for anchor, but enabled for test. Search region is current LCU plus LCU at the left (lossy test).

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -9,3% | -9,2% | -9,3% | -8,2% | -8,4% | -8,4% | -7,1% | -7,3% | -7,3% |
| Class B | -0,2% | -0,3% | -0,4% | -0,2% | -0,2% | -0,2% | -0,1% | -0,1% | -0,1% |
| SC RGB 444 | -26,1% | -26,3% | -26,3% | -25,5% | -25,8% | -25,7% | -25,1% | -25,1% | -25,2% |
| Animation RGB 444 | -1,1% | -1,1% | -1,0% | -0,7% | -0,8% | -0,7% | -0,4% | -0,5% | -0,5% |
| SC YUV 444 | -25,3% | -24,9% | -25,1% | -25,3% | -24,9% | -25,0% | -25,0% | -24,7% | -24,5% |
| Animation YUV 444 | -0,9% | -1,1% | -1,1% | -0,7% | -0,8% | -0,9% | -0,5% | -0,6% | -0,7% |
| RangeExt | -0,1% | -0,1% | -0,2% | 0,0% | -0,1% | -0,1% | 0,0% | 0,0% | 0,0% |
| SC(444) GBR | -42,0% | -41,9% | -42,4% | -42,1% | -42,5% | -43,1% | -42,6% | -42,4% | -42,7% |
| SC(444) YUV | -44,5% | -43,6% | -43,7% | -44,8% | -44,3% | -44,2% | -45,3% | -44,9% | -44,9% |
| Enc Time[%] | 141% |  |  | 139% |  |  | 136% |  |  |
| Dec Time[%] | 90% |  |  | 89% |  |  | 89% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -6,8% | -7,4% | -7,3% | -6,1% | -6,6% | -6,6% |  |  |  |
| Class B | 0,0% | -0,2% | -0,3% | 0,0% | -0,2% | -0,2% |  |  |  |
| SC RGB 444 | -21,0% | -20,5% | -21,0% | -19,3% | -18,8% | -19,5% |  |  |  |
| Animation RGB 444 | -0,6% | -0,7% | -0,6% | -0,4% | -0,4% | -0,4% |  |  |  |
| SC YUV 444 | -20,2% | -20,3% | -20,7% | -20,4% | -20,1% | -20,4% |  |  |  |
| Animation YUV 444 | -0,4% | -0,7% | -0,6% | -0,3% | -0,5% | -0,5% |  |  |  |
| RangeExt | 0,0% | -0,1% | -0,1% | 0,0% | -0,1% | 0,0% |  |  |  |
| SC(444) GBR | -37,2% | -37,0% | -37,3% | -37,5% | -37,5% | -37,9% |  |  |  |
| SC(444) YUV | -39,2% | -38,4% | -38,8% | -40,0% | -39,4% | -39,6% |  |  |  |
| Enc Time[%] | 116% |  |  | 115% |  |  |  |  |  |
| Dec Time[%] | 103% |  |  | 103% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -4,0% | -4,2% | -3,9% | -3,8% | -3,9% | -3,6% |  |  |  |
| Class B | 0,0% | -0,2% | 0,0% | 0,0% | -0,1% | -0,1% |  |  |  |
| SC RGB 444 | -16,2% | -15,5% | -15,5% | -15,4% | -14,8% | -14,7% |  |  |  |
| Animation RGB 444 | -0,3% | -0,3% | -0,3% | -0,2% | -0,2% | -0,2% |  |  |  |
| SC YUV 444 | -14,2% | -13,7% | -13,6% | -14,9% | -14,0% | -14,0% |  |  |  |
| Animation YUV 444 | -0,1% | -0,3% | -0,4% | -0,1% | -0,2% | -0,2% |  |  |  |
| RangeExt | 0,0% | -0,1% | 0,1% | 0,0% | 0,0% | 0,2% |  |  |  |
| SC(444) GBR | -31,3% | -30,3% | -30,3% | -31,6% | -30,3% | -30,3% |  |  |  |
| SC(444) YUV | -32,8% | -31,9% | -32,1% | -34,5% | -33,3% | -33,5% |  |  |  |
| Enc Time[%] | 113% |  |  | 112% |  |  |  |  |  |
| Dec Time[%] | 106% |  |  | 106% |  |  |  |  |  |
| AVG | Y | U | V | Enc |  |  |  |  |  |
| scc Intra | -27,4% | -27,3% | -27,4% | 139% |  |  |  |  |  |
| scc MC | -22,3% | -21,9% | -22,0% | 114% |  |  |  |  |  |

Table 3. Intra block copy is disabled for anchor, but enabled for test. Search region is current LCU plus 4 lines of above LCU and 4 columns of left LCU (lossy test).

|  | All Intra HE Main-tier |  |  |  | All Intra HE High-tier |  |  |  | All Intra HE Super-High-tier |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V |  | Y | U | V |  | Y | U | V |  |
| Class F |  | -5,6% | -5,6% | -5,8% |  | -5,0% | -5,1% | -5,2% |  | -4,3% | -4,5% | -4,5% |  |
| Class B |  | -0,2% | -0,2% | -0,3% |  | -0,1% | -0,2% | -0,2% |  | -0,1% | -0,1% | -0,1% |  |
| SC RGB 444 |  | -16,8% | -17,0% | -17,0% |  | -16,8% | -17,1% | -17,0% |  | -16,7% | -16,9% | -16,9% |  |
| Animation RGB 444 |  | -0,6% | -0,7% | -0,6% |  | -0,5% | -0,5% | -0,5% |  | -0,3% | -0,3% | -0,3% |  |
| SC YUV 444 |  | -15,9% | -15,8% | -16,0% |  | -16,4% | -16,1% | -16,2% |  | -16,5% | -16,3% | -16,2% |  |
| Animation YUV 444 |  | -0,5% | -0,7% | -0,7% |  | -0,4% | -0,5% | -0,6% |  | -0,3% | -0,4% | -0,5% |  |
| RangeExt |  | 0,0% | -0,1% | -0,1% |  | 0,0% | -0,1% | -0,1% |  | 0,0% | 0,0% | 0,0% |  |
| SC(444) GBR |  | -33,2% | -33,2% | -33,8% |  | -33,5% | -34,0% | -34,5% |  | -34,1% | -34,1% | -34,2% |  |
| SC(444) YUV |  | -33,0% | -32,4% | -32,2% |  | -33,5% | -33,1% | -32,9% |  | -34,1% | -33,7% | -33,8% |  |
| Enc Time[%] |  | 127% |  |  |  | 126% |  |  |  | 125% |  |  |  |
| Dec Time[%] |  | 92% |  |  |  | 91% |  |  |  | 91% |  |  |  |
|  |  | Random Access HE Main-tier |  |  |  | Random Access HE High-tier |  |  |  |  |  |  |  |
|  |  | Y | U | V |  | Y | U | V |  |  |  |  |  |
| Class F |  | -4,2% | -4,8% | -4,6% |  | -3,8% | -4,2% | -4,2% |  |  |  |  |  |
| Class B |  | 0,0% | -0,1% | 0,0% |  | 0,0% | 0,0% | 0,0% |  |  |  |  |  |
| SC RGB 444 |  | -13,4% | -12,9% | -13,4% |  | -12,3% | -11,8% | -12,5% |  |  |  |  |  |
| Animation RGB 444 |  | -0,3% | -0,3% | -0,3% |  | -0,2% | -0,2% | -0,2% |  |  |  |  |  |
| SC YUV 444 |  | -12,6% | -12,9% | -13,1% |  | -12,9% | -12,8% | -13,0% |  |  |  |  |  |
| Animation YUV 444 |  | -0,2% | -0,4% | -0,3% |  | -0,2% | -0,4% | -0,3% |  |  |  |  |  |
| RangeExt |  | 0,0% | -0,1% | 0,0% |  | 0,0% | 0,0% | 0,0% |  |  |  |  |  |
| SC(444) GBR |  | -28,9% | -28,9% | -29,2% |  | -29,5% | -29,6% | -30,0% |  |  |  |  |  |
| SC(444) YUV |  | -29,1% | -28,4% | -28,6% |  | -30,1% | -29,6% | -29,7% |  |  |  |  |  |
| Enc Time[%] |  | 111% |  |  |  | 110% |  |  |  |  |  |  |  |
| Dec Time[%] |  | 98% |  |  |  | 98% |  |  |  |  |  |  |  |
|  |  | Low delay B HE Main-tier |  |  |  | Low delay B HE High-tier |  |  |  |  |  |  |  |
|  |  | Y | U | V |  | Y | U | V |  |  |  |  |  |
| Class F |  | -2,4% | -3,6% | -2,4% |  | -2,3% | -2,9% | -2,2% |  |  |  |  |  |
| Class B |  | 0,0% | 0,1% | 0,1% |  | 0,0% | -0,1% | 0,1% |  |  |  |  |  |
| SC RGB 444 |  | -9,5% | -9,2% | -9,1% |  | -9,4% | -9,0% | -9,0% |  |  |  |  |  |
| Animation RGB 444 |  | -0,3% | -0,3% | -0,2% |  | -0,2% | -0,2% | -0,1% |  |  |  |  |  |
| SC YUV 444 |  | -8,7% | -8,4% | -8,4% |  | -9,2% | -8,6% | -8,6% |  |  |  |  |  |
| Animation YUV 444 |  | 0,0% | -0,2% | -0,2% |  | 0,0% | -0,1% | -0,1% |  |  |  |  |  |
| RangeExt |  | 0,0% | -0,2% | 0,0% |  | 0,0% | -0,1% | 0,0% |  |  |  |  |  |
| SC(444) GBR |  | -23,3% | -22,5% | -22,6% |  | -23,8% | -22,8% | -22,9% |  |  |  |  |  |
| SC(444) YUV |  | -25,5% | -24,5% | -24,8% |  | -26,6% | -25,6% | -25,9% |  |  |  |  |  |
| Enc Time[%] |  | 108% |  |  |  | 108% |  |  |  |  |  |  |  |
| Dec Time[%] |  | 99% |  |  |  | 99% |  |  |  |  |  |  |  |
| AVG |  | Y | U | V |  | Enc |  |  |  |  |  |  |  |
| scc Intra |  | -19,7% | -19,7% | -19,8% |  | 126% |  |  |  |  |  |  |  |
| scc MC |  | -15,9% | -15,7% | -15,7% |  | 109% |  |  |  |  |  |  |  |

Table 4. Intra block copy is disabled for anchor, but enabled for test (lossless tests).

|  | AI Main | RA Main | LB Main | AI Main | RA Main | LB Main |
| --- | --- | --- | --- | --- | --- | --- |
| Search range | current LCU plus LCU at the left |  |  | current LCU plus 4 lines of above LCU and 4 columns of left LCU |  |  |
| Class F | 4,5% | 2,4% | 0,4% | 2,8% | 2,0% | 1,4% |
| Class B | 0,1% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| SC RGB 444 | 21,2% | 10,9% | 4,0% | 14,4% | 9,6% | 7,5% |
| Animation RGB 444 | 0,2% | 0,0% | 0,0% | 0,1% | 0,0% | 0,0% |
| SC YUV 444 | 20,7% | 11,1% | 4,2% | 13,8% | 9,5% | 6,7% |
| Animation YUV 444 | 0,1% | 0,0% | 0,0% | 0,1% | 0,0% | 0,0% |
| RangeExt | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| SC GBR 444 Optional | 37,2% | 24,1% | 7,6% | 28,6% | 25,9% | 23,9% |
| SC YUV 444 Optional | 41,5% | 27,8% | 7,9% | 31,4% | 29,2% | 28,6% |
| Enc Time[%] | 154% | 103% | 99% | 135% | 109% | 107% |
| Dec Time[%] | 100% | 99% | 100% | 93% | 100% | 100% |
| AVG SCC | 25,0% | 15,3% | 4,8% | 18,2% | 15,3% | 13,6% |

# Specification modification

Modifications are highlighted in yellow.

#### 7.4.9.9. Motion vector difference semantics

….

The variable BvIntra[ x0 ][ y0 ][ compIdx ] specifies the vector used for the intra block copying prediction mode.  The value of BvIntra[ x0 ][ y0 ] shall be in the range of 0 to 128, inclusive.

Bit stream shall not contain BvIntra [ x0 ][ y0 ][ compIdx ]values out of the range

-CTUsize +CUsize-4 ≤BvIntra [ x0 ][ y0 ][ compIdx ]≤ CTUsize –CUsize

Bit stream shall not contain BvIntra[ x0 ][ y0 ][ compIdx ] pointing to the non-availabe for copy region.

The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. The horizontal block vector component is assigned compIdx = 0 and the vertical block vector component is assigned compIdx = 1.

- If refList is equal to 0, MvdL0[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

- Otherwise (refList is equal to 1), MvdL1[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

Otherwise (refList is equal to 2), BvIntra[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

# Conclusion

Samsung proposes to introduce mandatory encoder constrain for intra block copy motion vector in order to resolve un-specified decoder behaviour and help decoder implementation. Proposed restriction of intra block copy for one LCU plus 4 lines of LCU above and 4 columns of LCU at the left preserves 70% of intra block copy gain in current design. With proposed restriction intra block copy BD-rate gain for screen content is 20% in all-intra, 16% in random access and low-delay lossy tests. In loss-less test the gain of restricted Intra BC is 18 % for all-intra and 14% for random access and low-delay.

# References

- C. Pang, J. Sole, L. Guo, M. Karczewicz, R. Joshi, “Non-RCE3: Intra Motion Compensation with 2-D MVs,” Document of Joint Collaborative Team on Video Coding, JCTVC-N0256, Jul. 2013.

# Patent rights declarations

Samsung Electronics, Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).