| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0082 |
| --- | --- |

| Title: | AHG 5 and 18: Profiles for Range Extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Sharman, N. Saunders, J. Gamei Sony BPRL Jays Close, Viables, Basingstoke RG22 4SB, UK | Tel: Email: | +44 1256 355 011 karl.sharman@eu.sony.com nick.saunders@eu.sony.com |
|  | T. Suzuki 2-10-1, Osaki, Shinagawa-ku, Tokyo, 141-8610 JAPAN | Tel: Email: | +81 50 3750 2740 teruhikos@jp.sony.com |
|  | A. Tabatabai 1730 N. First Street, MD:3NW San Jose, CA 95112 USA | Tel: Email: | +1 Ali.Tabatabai@am.sony.com |
| Source: | Sony Europe Ltd., Sony Corp., and Sony Electronics Inc. |  |  |

_____________________________

# Abstract

Discussion on profiles for RExt had been started and a set of profiles were included in DAM text. This contribution proposes the followings.

- Add all intra coding profiles

- Add profiles support up to 16 bit video

- Profiles for mono-chrome video (4:0:0)

- Consideration of tools for non-camera captured content

- Syntax order of the constraint_xxx_flag

# Introduction

The initial idea of HEVC RExt profiles was proposed in JCTVC-M0094 and m29109 at Incheon meeting. The the following profiles for RExt were included in the DAM text.

- Main 12 4:2:0

- Main 10 4:2:2, Main 12 4:2:2

- Main 10 4:4:4, Main 12 4:4:4

This contribution proposes to include additional profiles to support the applications for contents creation and production. All intra coding is used for high end production. Video with bit depth higher than 12 bit is emerging to the market. For example, the current C-MOS will be extended to 16 bit soon. It is desirable to support such emerging format in HEVC. Therefore, this contribution proposes to include profiles for all intra coding, for bit depth up to 16 bit, and for mono chrome video (4:0:0), in the RExt.

# Profiles

It is preferable from an interoperability standpoint rather than defining small number of profiles and letting implementers to choose which tools to support, to define many profiles that cover all the expected use cases and applications. Defining too many profiles would however.

This contribution proposes to define profiles as shown in Table 1.

Table 1 : Summary of the proposed profiles for RExt

The points of the above new profiles are,

- High 16 profiles support greater bit depths (up to 16 bit), and would give the opportunity for new tools to be defined (which may lead to improved coding efficiency or throughput or reduce complexity). Higher internal precisions would be included in these profiles. For High 16 profiles, extended_precision_processing_flag could be set to either 0 or 1, while extended_precision_processing_flag is set to 0 in Main profile.

- Intra only profiles are defined by limiting all pictures with the following restrictions.

  - nal_unit_type is equal to IDR_N_LP, CRA_NUT or BLA_N_LP

  - No reordering is allowed

  - No short term ref pic sets and no long term pic are allowed

  - Allow to send in-loop filter parameters, but performing in-loop filter is not mandatory

- To simplify the implementation, no new tools are to be included for intra only profiles.

- 4:0:0 profiles are defined for High 16 profiles to support monochrome video.

## Tools for non-camera captured content

Tools for non-camera captured contents are being investigated in JCT-VC. These tools are designed to compress computer graphics generated content, captions, etc, efficiently. These tools are not however useful for camera captured contents. Both coding performance and complexity of such tools are being evaluated in JCTVC-O0083. Decoder complexity for some of the coding toolse.g. intra block copy, is not low, such tools should not therefore be included in the profiles proposed in Table 1. Creating other profile(s) for non-camera captured content, should instead, be considerd. Since in current wireless transmission, natural video and computer generated contents are transmitted separately. creating separate profiles for non-camera captured content, in such environment, could be a reasonable approach.

More detail should be discussed during the Vienna meeting.

# Level definition

The maximum bitrate and CPB size should be higher in the proposed new profiles and MinCr should be lower to support higher fidelity.The scale factors are proposed as the following table. CpbBrVclFactor, CpbBrNalFactor, FormatCapabilityFactor and MinCrScaleFactor are added.

Table A-2 – Specification of CpbBrVclFactor, CpbBrNalFactor, FormatCapabilityFactor, and MinCrScaleFactor

| Profile | CpbBrVclFactor | CpbBrNalFactor | FormatCapabilityFactor | MinCrScaleFactor |
| --- | --- | --- | --- | --- |
| Main | 1000 | 1100 | 1.500 | 1.0 |
| Main 10 | 1000 | 1100 | 1.875 | 1.0 |
| Main 12 | 1000 | 1100 | 2.250 | 1.0 |
| Main 4:2:2:10 | 1250 | 1375 | 2.500 | 0.5 |
| Main 4:2:2 12 | 1250 | 1375 | 3.000 | 0.5 |
| Main 4:4:4 10 | 1500 | 1650 | 3.750 | 0.5 |
| Main 4:4:4 12 | 1500 | 1650 | 4.500 | 0.5 |
| High 4:0:0 16 | 2000 | 2200 | 4.500 | 0.5 |
| High 4:4:4 16 | 2000 | 2200 | 4.500 | 0.5 |
| Main 12 Intra | 4000 | 4400 | 2.250 | 1.0 |
| Main 4:2:2 12 Intra | 4000 | 4400 | 3.000 | 0.5 |
| Main 4:4:4 12 Intra | 4000 | 4400 | 3.750 | 0.5 |
| High 4:0:0 16 Intra | 4000 | 4400 | 4.5 | 0.5 |
| High 4:4:4 16 Intra | 4000 | 4400 | 4.5 | 0.5 |

In the case of AVC, all intra coding was supposed to be included in High 4:2:2 and High 4:4:4 profiles. The maximum bitrate of the High 4:2:2 profile was inadvertently larger than that of the High 4:4:4 profile. However, this bitrate is too high for inter-frame coding (and all intra profiles were created later).

# Syntax

To support new profiles, no new profile_idc value is used. It is signalled by constraint_xxxx_flag. The syntax and semancs changes are summarized in Annex and the attached draft specification.

# Conclusion

This contribution proposes the followings.

- Add all intra coding profiles

- Add profiles support up to 16 bit video

- Profiles for mono-chrome video (4:0:0)

- Consideration of tools for non-camera captured content

- Syntax order of the constraint_xxx_flag

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex: Syntax and semantics

The syntax and semantics changes to support new profiles are summarized in this annex.

### 7.3.3 Profile, tier and level syntax

| profile_tier_level( maxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| general_profile_space | u(2) |
| general_tier_flag | u(1) |
| general_profile_idc | u(5) |
| for( j = 0; j < 32; j++ ) |  |
| general_profile_compatibility_flag[ j ] | u(1) |
| general_progressive_source_flag | u(1) |
| general_interlaced_source_flag | u(1) |
| general_non_packed_constraint_flag | u(1) |
| general_frame_only_constraint_flag | u(1) |
| general_intra_constraint_flag | u(1) |
| general_max_10bit_constraint_flag | u(1) |
| general_max_12bit_constraint_flag | u(1) |
| general_max_16bit_constraint_flag | u(1) |
| general_reserved_zero_2bits | u(2) |
| general_max_400chroma_constraint_flag | u(1) |
| general_max_420chroma_constraint_flag | u(1) |
| general_max_422chroma_constraint_flag | u(1) |
| general_reserved_zero_35bits | u(35) |
| general_level_idc | u(8) |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) { |  |
| sub_layer_profile_present_flag[ i ] | u(1) |
| sub_layer_level_present_flag[ i ] | u(1) |
| } |  |
| if( maxNumSubLayersMinus1 > 0 ) |  |
| for( i = maxNumSubLayersMinus1; i < 8; i++ ) |  |
| reserved_zero_2bits[ i ] | u(2) |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) { |  |
| if( sub_layer_profile_present_flag[ i ] ) { |  |
| sub_layer_profile_space[ i ] | u(2) |
| sub_layer_tier_flag[ i ] | u(1) |
| sub_layer_profile_idc[ i ] | u(5) |
| for( j = 0; j < 32; j++ ) |  |
| sub_layer_profile_compatibility_flag[ i ][ j ] | u(1) |
| sub_layer_progressive_source_flag[ i ] | u(1) |
| sub_layer_interlaced_source_flag[ i ] | u(1) |
| sub_layer_non_packed_constraint_flag[ i ] | u(1) |
| sub_layer_frame_only_constraint_flag[ i ] | u(1) |
| sub_layer_intra_constraint_flag | u(1) |
| sub_layer_max_10bit_constraint_flag | u(1) |
| sub_layer_max_12bit_constraint_flag | u(1) |
| sub_layer_max_16bit_constraint_flag | u(1) |
| sub_layer_reserved_zero_2bits | u(2) |
| sub_layer_max_400chroma_constraint_flag | u(1) |
| sub_layer_max_420chroma_constraint_flag | u(1) |
| sub_layer_max_422chroma_constraint_flag | u(1) |
| sub_layer_reserved_zero_35bits[ i ] | u(35) |
| } |  |
| if( sub_layer_level_present_flag[ i ] ) |  |
| sub_layer_level_idc[ i ] | u(8) |
| } |  |
| } |  |

### 7.4.4 Profile, tier and level semantics

general_intra_constraint_flag equal to 1 specifies that all pictures have nal_unit_type equal to IDR_N_LP (or CRA_NUT or BLA_N_LP). When general_profile_idc is not equal to 4 and general_profile_compatibility_flag[ 4 ] is not equal to 1, general_intra_constraint_flag shall be equal to 0.

general_max_16bit_constraint_flag, general_max_12bit_constraint_flag, general_max_10bit_constraint_flag, general_max_422chroma_‌constraint_flag, general_max_420chroma_constraint_flag and general_max_400chroma_constraint_flag, when general_profile_idc is equal to 4 or general_profile_compatibility_flag[ 4 ] is equal to 1, have semantics specified in Annex Error: Reference source not found. When general_profile_idc is not equal to 4 and general_profile_compatibility_flag[ 4 ] is not equal to 1, general_max_16bit_constraint_flag, general_max_12bit_constraint_flag, general_max_10bit_constraint_flag, general_max_422chroma_constraint_flag, general_max_420chroma_constraint_flag and general_max_400chroma_constraint_flag shall be equal to 0 in bitstreams conforming to this version of this Specification. When general_profile_idc is not equal to 4 and general_profile_compatibility_flag[ 4 ] is not equal to 1, the value 1 for each syntax element general_max_16bit_constraint_flag, general_max_12bit_constraint_flag, general_max_10bit_constraint_flag, general_max_422chroma_constraint_flag, general_max_420chroma_constraint_flag and general_max_400chroma_constraint_flag is reserved for future use by ITU-T | ISO/IEC, and decoders shall ignore the values of these syntax elements.

general_reserved_zero_2bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for general_reserved_zero_2bits are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of general_reserved_zero_2bits.

general_reserved_zero_35bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for general_reserved_zero_35bits are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of general_reserved_zero_35bits.

sub_layer_intra_constraint_flag equal to 1 specifies that all pictures have nal_unit_type equal to IDR_N_LP (or CRA_NUT or BLA_N_LP). When sub_layer_profile_idc[i] is not equal to 4 and sub_layer_profile_compatibility_flag[ 4 ][ j ] is not equal to 1, sub_layer_intra_constraint_flag shall be equal to 0.

The semantics of the syntax elements sub_layer_profile_space[ i ], sub_layer_tier_flag[ i ], sub_layer_profile_idc[ i ], sub_layer_profile_compatibility_flag[ i ][ j ], sub_layer_progressive_source_flag[ i ], sub_layer_interlaced_source_flag[ i ], sub_layer_non_packed_constraint_flag[ i ], sub_layer_frame_only_constraint_flag[ i ], sub_layer_max_16bit_constraint_flag, sub_layer_max_12bit_constraint_flag, sub_layer_max_10bit_‌constraint_flag, sub_layer_max_422chroma_constraint_flag, sub_layer_max_420chroma_constraint_flag, sub_layer_max_400chroma_constraint_flag, sub_layer_reserved_zero_35bits[ i ], and sub_layer_level_idc[ i ] are the same as the syntax elements general_profile_space, general_tier_flag, general_profile_idc, general_profile_compatibility_flag[ j ], general_progressive_source_flag, general_interlaced_source_flag, general_non_packed_constraint_flag, general_frame_only_constraint_flag, general_max_16bit_constraint_flag, general_max_12bit_constraint_flag, general_max_10bit_constraint_flag, general_max_422chroma_constraint_flag, general_max_420chroma_constraint_flag and general_max_400chroma_constraint_flag, general_reserved_zero_35bits, and general_level_idc, respectively, but apply to the representation of the sub-layer with TemporalId equal to i.

When not present, the value of sub_layer_tier_flag[ i ] is inferred to be equal to 0.

NOTE 1 – It is possible that sub_layer_tier_flag[ i ] is not present and sub_layer_level_idc[ i ] is present. In this case, a default value of sub_layer_tier_flag[ i ] is needed for interpretation of sub_layer_level_idc[ i ].

A.3.5 Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 10, Main 4:4:4 12, High 4:0:0 16 and High 4:4:4 16 profiles

Bitstreams conforming to the Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 10, Main 4:4:4 12, High 4:0:0 16 and High 4:4:4 16 profiles shall obey the following constraints:

- In bitstreams conforming to the Main 12, Main 4:2:2 10 or Main 4:2:2 12  profile, SPSs shall have chroma_format_idc constrained as follows:

–	In bitstreams conforming to the Main 12 profile, SPSs shall have chroma_format_idc equal to 0 or 1 only.

–	In bitstreams conforming to the Main 4:2:2 10 or Main 4:2:2 12 profile, SPSs shall have chroma_format_idc equal to 0, 1 or 2 only.

- SPSs shall have bit_depth_luma_minus8 and bit_depth_chroma_minus8 constrained as follows:

–	In bitstreams conforming to the Main 4:2:2 10 or Main 4:4:4 10 profile, SPSs shall have bit_depth_luma_minus8 and bit_depth_chroma_minus8 in the range of 0 to 2, inclusive.

–	In bitstreams conforming to the Main 12, Main 4:2:2 12, or Main 4:4:4 12 profile, SPSs shall have bit_depth_luma_minus8 and bit_depth_chroma_minus8 in the range of 0 to 4, inclusive.

- In bitstreams conforming to the Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 10, Main 4:4:4 12, High 4:0:0 16 and High 4:4:4 16 profiles,

- SPSs shall have separate_colour_plane_flag, when present, equal to 0 only.

- SPSs shall have extended_precision_processing_flag, when present, equal to 0 only.

- CtbLog2SizeY shall be in the range of 4 to 6, inclusive.

- When a PPS has tiles_enabled_flag is equal to 1, it shall have entropy_coding_sync_enabled_flag equal to 0.

- When a PPS has tiles_enabled_flag is equal to 1, ColumnWidthInLumaSamples[ i ] shall be greater than or equal to 256 for all values of i in the range of 0 to num_tile_columns_minus1, inclusive, and RowHeightInLumaSamples[ j ] shall be greater than or equal to 64 for all values of j in the range of 0 to num_tile_rows_minus1, inclusive.

- The number of times read_bits( 1 ) is called in subclauses Error: Reference source not found and Error: Reference source not found when parsing coding_tree_unit( ) data for any coding tree unit shall be less than or equal to 5 * RawCtuBits / 3.

- general_level_idc shall not be equal to 255 (which indicates level 8.5).

- transform_skip_rotation_enabled_flag, transform_skip_context_enabled_flag, intra_block_copy_enabled_flag, residual_dpcm_intra_enabled_flag and residual_dpcm_inter_enabled_flag shall be equal to <TO BE DECIDED AT THE NEXT MEETING>

- In bitstreams conforming to the Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 10 and Main 4:4:4 12 profiles, extended_precision_processing_flag shall be equal to 0.

- In bitstreams conforming to the High 4:0:0 16 and High 4:4:4 16 profiles, extended_precision_processing_flag is equal to 0 or 1.

- The level constraints specified for the Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 10, Main 4:4:4 12, High 4:0:0 16 or High 4:4:4 16 profile in in subclause Error: Reference source not found, as applicable, shall be fulfilled.

Conformance of a bitstream to the Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 10, Main 4:4:4 12 , High 4:0:0 16 or High 4:4:4 16 profile is indicated by general_profile_idc being equal to 4 or general_profile_compatibility_flag[ 4 ] being equal to 1 with the following additional indications as applicable:

- Conformance of the bitstream to the Main 12 profile is indicated by general_max_12bit_constraint_flag equal to 1, general_max_10bit_constraint_flag equal to 0, general_max_422chroma_constraint_flag equal to 1, general_max_420chroma_constraint_flag equal to 1 and general_max_400chroma_constraint_flag equal to 0.

- Conformance of the bitstream to the Main 4:2:2 10 profile is indicated by general_max_12bit_constraint_flag equal to 1, general_max_10bit_constraint_flag equal to 1, general_max_422chroma_constraint_flag equal to 1, general_max_420chroma_constraint_flag equal to 1 and general_max_400chroma_constraint_flag equal to 0.

- Conformance of the bitstream to the Main 4:2:2 12 profile is indicated by general_max_12bit_constraint_flag equal to 1, general_max_10bit_constraint_flag equal to 0, general_max_422chroma_constraint_flag equal to 1, general_max_420chroma_constraint_flag equal to 0 and general_max_400chroma_constraint_flag equal to 0.

- Conformance of the bitstream to the Main 4:4:4 10 profile is indicated by general_max_12bit_constraint_flag equal to 1, general_max_10bit_constraint_flag equal to 1, general_max_422chroma_constraint_flag equal to 0, general_max_420chroma_constraint_flag equal to 0 and general_max_400chroma_constraint_flag equal to 0.

- Conformance of the bitstream to the Main 4:4:4 12 profile is indicated by general_max_12bit_constraint_flag equal to 1, general_max_10bit_constraint_flag equal to 0, general_max_422chroma_constraint_flag equal to 0, general_max_420chroma_constraint_flag equal to 0 and general_max_400chroma_constraint_flag equal to 0.

- Conformance of the bitstream to the High 4:0:0 16 profile is indicated by general_max_16bit_constraint_flag equal to 1, general_max_12bit_constraint_flag equal to 0, general_max_10bit_constraint_flag equal to 0, general_max_422chroma_constraint_flag equal to 0, general_max_420chroma_constraint_flag equal to 0 and general_max_400chroma_constraint_flag equal to 1.

- Conformance of the bitstream to the High 4:4:4 16 profile is indicated by general_max_16bit_constraint_flag equal to 1, general_max_12bit_constraint_flag equal to 0, general_max_10bit_constraint_flag equal to 0, general_max_422chroma_constraint_flag equal to 0, general_max_420chroma_constraint_flag equal to 0 and general_max_400chroma_constraint_flag equal to 0.

All other combinations of general_max_16bit_constraint_flag, general_max_12bit_constraint_flag, general_max_10bit_constraint_flag, general_max_422chroma_constraint_flag, general_max_420chroma_constraint_flag and general_max_400chroma_constraint_flag equal to 0 with general_profile_idc equal to 4 or general_profile_compatibility_flag[ 4 ] equal to 1 are reserved for future use by ITU-T | ISO/IEC. Such combinations shall not be present in bitstreams conforming to this specification. However, decoders conforming to the Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 10, Main 4:4:4 12, High 4:0:0 16 or High 4:4:4 16 profile shall allow other combinations as specified below in this subclause to occur in the bitstream.

Decoders conforming to the Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 10, Main 4:4:4 12, High 4:0:0 16 or High 4:4:4 16 profile at a specific level (identified by a specific value of general_level_idc) shall be capable of decoding all bitstreams for which all of the following conditions apply:

- One or both of the following conditions apply:

–	general_profile_idc is equal to 1 or 2, general_profile_compatibility_flag[ 1 ] is equal to 1, general_profile_compatibility_flag[ 2 ] is equal to 1.

–	general_profile_idc is equal to 4 or general_profile_compatibility_flag[ 4 ] is equal to 1, and any of the following conditions apply:

–	The decoder conforms to the Main 12 profile, general_max_12bit_constraint_flag is equal to 1, and general_max_420chroma_constraint_flag is equal to 1.

–	The decoder conforms to the Main 4:2:2 10 profile, general_max_10bit_constraint_flag is equal to 1, and general_max_422chroma_constraint_flag is equal to 1.

–	The decoder conforms to the Main 4:2:2 12 profile, general_max_12bit_constraint_flag is equal to 1 or general_max_10bit_constraint_flag is equal to 1, and general_max_422chroma_constraint_flag is equal to 1 or general_max_420chroma_constraint_flag is equal to 1.

–	The decoder conforms to the Main 4:4:4 10 profile and general_max_10bit_constraint_flag is equal to 1.

–	The decoder conforms to the Main 4:4:4 12 profile and general_max_12bit_constraint_flag is equal to 1 or general_max_12bit_constraint_flag is equal to 1.

–	The decoder conforms to the High 4:0:0 16 profile and general_max_400chroma_constraint_flag is equal to 1.

- general_level_idc is not equal to 255 and represents a level lower than or equal to the specified level.

- general_tier_flag represents a tier lower than or equal to the specified tier.

A.3.6 Main 12 Intra, Main 4:2:2 12 Intra, Main 4:4:4 12 Intra, High 4:0:0 16 Intra and High 4:4:4 16 Intra profiles

Bitstreams conforming to the Main 12 Intra, Main 4:2:2 12 Intra, Main 4:4:4 12 Intra, High 4:0:0 16 Intra and High 4:4:4 16 Intra profiles shall obey the following constraints:

- In bitstreams conforming to the Main 12 Intra, all constraints specified in subclause  for the Main 12 profile shall be obeyed.

- In bitstreams conforming to the Main 4:2:2 12 Intra, all constraints specified in subclause  for the Main 4:2:2 12 profile shall be obeyed.

- In bitstreams conforming to the Main 4:4:4 12 Intra, all constraints specified in subclause  for the Main 4:4:4 12 profile shall be obeyed.

- In bitstreams conforming to the High 4:0:0 16 Intra, all constraints specified in subclause  for the High 4:0:0 16 profile shall be obeyed.

- In bitstreams conforming to the High 4:4:4 16 Intra, all constraints specified in subclause  for the High 4:4:4 16 profile shall be obeyed.

- general_intra_constraint_flag shall be equal to 1.

- All pictures shall be IDR picture with nal_unit_type equal to IDR_N_LP.

- When vps_sub_layer_ordering_info_present_flag is equal to 1, vps_max_num_reorder_pics shall be equal to 0.

- When sps_sub_layer_ordering_info_present_flag is equal to 1, sps_max_num_reorder_pics shall be equal to 0.

- Sequence parameter sets shall have num_short_term_ref_pic_sets equal to 0.

- Sequence parameter sets shall have long_term_ref_pics_present_flag equal to 0.

- Picture timing SEI messages, whether present in the bitstream (by non-VCL NAL units) or conveyed equivalently by other means not specified in this Recommendation | International Standard, shall have pic_dpb_output_delay equal to 0.

The operation of the in-loop filter process specified in subclause Error: Reference source not found is not required for decoder conformance to the Main 12 Intra, Main 4:2:2 12 Intra, Main 4:4:4 12 Intra, High 4:0:0 16 Intra and High 4:4:4 16 Intra profiles.

NOTE – The in-loop filter process specified in subclause Error: Reference source not found or some similar post-processing filter should be performed, although this is not a requirement for decoder conformance to the Main 12 Intra, Main 4:2:2 12 Intra, Main 4:4:4 12 Intra and High 4:4:4 16 Intra profiles. The syntax elements sent by an encoder for control of the in-loop filter process specified in subclause Error: Reference source not found are considered only as advisory information for decoders conformance to the Main 12 Intra, Main 4:2:2 12 Intra, Main 4:4:4 12 Intra and High 4:4:4 16 Intra profiles. However, the application of the deblocking filter process specified in subclause Error: Reference source not found is required for decoder conformance to the Main 12, Main 4:2:2 12, Main 4:4:4 12 and High 4:4:4 16 profiles when decoding bitstreams that conform to the Main 12 Intra, Main 4:2:2 12 Intra, Main 4:4:4 12 Intra and High 4:4:4 16 Intra profiles.

A.4.2 Profile-specific level limits for the Main, Main 10, Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 10, Main 4:4:4 12, High 4:0:0 16, High 4:4:4 16, Main 12 Intra, Main 4:2:2 12 Intra, Main 4:4:4 12 Intra, High 4:0:0 16 Intra and High 4:4:4 16 Intra profiles

The following is specified for expressing the constraints in this annex:

- Let the variable fR be set equal to 1  300.

Bitstreams conforming to the Main, Main 10, Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 10, Main 4:4:4 12, High 4:0:0 16, High 4:4:4 16, Main 12 Intra, Main 4:2:2 12 Intra, Main 4:4:4 12 Intra, High 4:0:0 16 Intra and High 4:4:4 16 Intra profile at a specified tier and level shall obey the following constraints for each bitstream conformance test as specified in Annex Error: Reference source not found:

Table A-3 – Specification of CpbBrVclFactor, CpbBrNalFactor, FormatCapabilityFactor, and MinCrScaleFactor

| Profile | CpbBrVclFactor | CpbBrNalFactor | FormatCapabilityFactor | MinCrScaleFactor |
| --- | --- | --- | --- | --- |
| Main | 1000 | 1100 | 1.500 | 1.0 |
| Main 10 | 1000 | 1100 | 1.875 | 1.0 |
| Main 12 | 1000 | 1100 | 2.250 | 1.0 |
| Main 4:2:2:10 | 1250 | 1375 | 2.500 | 0.5 |
| Main 4:2:2 12 | 1250 | 1375 | 3.000 | 0.5 |
| Main 4:4:4 10 | 1500 | 1650 | 3.750 | 0.5 |
| Main 4:4:4 12 | 1500 | 1650 | 4.500 | 0.5 |
| High 4:0:0 16 | 2000 | 2200 | 4.500 | 0.5 |
| High 4:4:4 16 | 2000 | 2200 | 4.500 | 0.5 |
| Main 12 Intra | 4000 | 4400 | 2.250 | 1.0 |
| Main 4:2:2 12 Intra | 4000 | 4400 | 3.000 | 0.5 |
| Main 4:4:4 12 Intra | 4000 | 4400 | 3.750 | 0.5 |
| High 4:0:0 16 Intra | 4000 | 4400 | 4.5 | 0.5 |
| High 4:4:4 16 Intra | 4000 | 4400 | 4.5 | 0.5 |