| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0083 |
| --- | --- |

| Title: | AHG 5: analysis on coding tools for non-camera captured content |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | T. Suzuki 2-10-1, Osaki, Shinagawa-ku, Tokyo, 141-8610 JAPAN | Tel: Email: | +81 50 3750 2740 teruhikos@jp.sony.com |
|  | K. Sharman, N. Saunders, J. Gamei Sony BPRL Jays Close, Viables, Basingstoke RG22 4SB, UK | Tel: Email: | +44 1256 355 011 karl.sharman@eu.sony.com nick.saunders@eu.sony.com |
|  | A. Tabatabai 1730 N. First Street, MD:3NW San Jose, CA 95112 USA | Tel: Email: | +1-408-352-4086 Ali.Tabatabai@am.sony.com |
| Source: | Sony Europe Ltd., Sony Corp., and Sony Electronics Inc. |  |  |

_____________________________

# Abstract

Coding tools for non-camera captured content are investigated and included in the Range Extension amendment. This contribution reports the analysis of both coding efficiency and complexity. Although supporting such material is important, profiles to support specific tools should be defined carefully, studying both coding efficiency performance and complexity.

# Coding efficiency analysis

Coding efficiency of RExt3.0 and RExt 4.0 software was evaluated using the standard AHG5 test conditions.

- Reference: HM12.0_RExt3.0

- Tested: HM12.0_RExt4.1

- BD rate：Calculated using piece-wise cubic interpolation

The results are summarized in Table 1. More detail results are included in the attached Excel file.

Table 1: Comparison RExt 3.0 and RExt 4.1

It is observed that coding efficiency improvement for natural video is not so high, while encoding time has significantly increased. It is noted that encoding time is often not considered critical, given that the addition tools can be ignored. However, if a tool is too burdensome, then it is unlikely to be ever used unless it offers significant advantage. In the following sections, a brief investigation of the hardware implementation of the decoder (and encoder) for the new coding tools for non-camera captured contents is reported.

# Intra block copy

Intra block copy (Intra BC) was adopted in Vienna to improve coding efficiency of Intra coding, especially for non-camera captured content. Although decoding run time is not increased, the increase in the cost to a HW decoder implementation is significant.

In most HW decoder implementations, pixels after in-loop filtering are stored in external DRAM. The pixels for the current CTU and the neighbouring pixels (at both upper and left boundary) are stored in internal SRAM to reduce memory access bandwidth to external DRAM.

In order to support intra block copy, the following changes are necessary:

- Increase internal SRAM to store surrounding reference pixels for Intra BC.

- or increase memory bandwidth to access to external DRAM to copy pixels for Intra BC.

Those changes have significant impact on the HW implementation cost.

More detail analysis is reported in JCTVC-O0232.

The additional memory sieze is estimated as below.

SRAM:

The additional SRAM size was estimated by the following equation.

64 * 64 * Chroma * BitDepth * (1 + left CTU num) bit

Chroma is as follows,

4:2:0 -> x1.5

4:2:2 -> x2

4:4:4 -> x3

Example:

In case of 4:2:2 10 bit, SRAM increase is

Intra prediction: 1,280bit (= 64 * 1 * 2 *10, 1 column need to be stored for intra prediction)

Left 2 CTU reference: 245,760 bit (= 64 * 64 * 2 * 10 * (1 + 2)), 192 times larger than intra pred

Left 1 CTU reference: 163,840bit (= 64 * 64 * 2 * 10 * (1 + 1)), 128 times larger than intra pred

Logic increase:

The additional logic gate to r/w to SRAM is estimated and the size is around 3-40,000 gates.

# RDPCM and residue rotation

Residual DPCM and residual rotation were adopted for transform skip (and transquant-bypass). There are some design inconsistency between intra and inter coding paths related to RDPCM and residue rotation. It would be better to unify the process for both intra and inter paths. In addition, the process should be consistent between RDPCM and CDPCM (Coefficient DPCM). More detailed investigations are presented in JCTVC-O0066.

# Conclusion

This contribution provides the following information.

- Coding efficiency comparison of RExt 3.0 and 4.0 (main difference is coding tools for non-camera captured content) is shown. The improvement is not significant for natural video.

- Complexity of intra block copy

- Complexity of RDPCM and residue rotation

The additional decoder complexity to support coding tools for non-camera captured content is not ignorable, especially intra block copy. Improvements are proposed in the related contributions, JCTVC-O0066 and JCTVC-O0232. If a coding tool has no benefit to improve coding efficiency for natural video, and if its decoder complexity is not negligible, a separate profile should be created for the application of non-camera captured content.

# Reference

- JCTVC-O0232: On intra block copying in RExt

- JCTVC-O0066: AHG5: Unifying DPCM

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).