| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0085_r1 |
| --- | --- |

| Title: | AHG5/AHG8: Improvement of MV-HEVC-based RGB coding for screen contents |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Akira Minezawa Kazuyuki Miyazawa Shun-ichi Sekiguchi Tokumichi Murakami 5-1-1, Ofuna, Kamakura, Kanagawa, JAPAN | Tel: Email: | +81 467 41 2463 Minezawa.Akira@ds.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

In the last meeting, a picture-based RGB coding scheme using MV-HEVC was proposed. This scheme encodes each color plane as 4:0:0 format view video and employs inter-view prediction to exploit inter-color redundancy. This contribution proposes a modification scheme of MV-HEVC-based RGB coding. The proposed scheme introduces intra block copying instead of motion search based inter-color prediction. The inter-color prediction of the proposed scheme only permits to refer a collocated block in the reference color plane. Coding performance of the proposed scheme has been evaluated on top of HTM8.0 under AHG8 recommendation configurations. It has been confirmed that the proposed scheme achieves 18.0%-20.4%, 14.9%-17.2% and 11.4%-14.7% average BD-rate improvement relative to MV-HEVC (HTM8.0) for AI, RA and LDB configurations, respectively.

# Introduction

We proposed an efficient RGB coding using HEVC multi-view extension known as “MV-HEVC”[1] in 14th JCT-VC meeting. This scheme decomposes each color plane into a monochrome view source and applies them to MV-HEVC. It was reported that the MV-HEVC-based scheme has significant coding improvement especially for screen contents and achieves 19%-33% coding gains on average relative to RExt3.0 for screen contents[2]. In this contribution, we focus on improvement of coding efficiency for screen contents and propose an adaptive intra/inter-color prediction by introducing intra block copying technique[3].

# Proposed scheme

The base architecture of the proposed scheme is shown in Figure 1. For RGB format case, the proposed scheme encodes G plane as base view and B and R plane as enhancement view on the framework of multi-view coding. This scheme conducts motion estimation for inter-color prediction and encodes motion information. According to analysis of simulation results for screen contents, inter-color prediction is effective in case that temporal correlation is low at current picture and zero-MVs are selected at over 85% of prediction blocks selecting inter-color prediction in the case of QP=2. However, some blocks uses non-zero inter-color MV as shown in Figure 2. In this case, since there is low correlation between a block to be predicted and collocated block in a reference picture indicating 1st color plane, a block similar to the block to be predicted are looked for in surrounding area of the reference picture. Such surrounding searching is also conducted by intra block copying[3] and the intra block copying uses coded area of current picture as reference instead of 1st color component picture. Therefore, the intra block copying can be applied to not only pictures indicating enhancement view but also pictures indicating base view. From a perspective of that characteristic, the proposed scheme introduces intra block copying in all color component pictures and restricts inter-color prediction vectors to zero. Figure 3 shows the concept of non-temporal prediction of the proposed scheme.

Figure 1  The base architecture of the proposed scheme

Figure 2  An example of using non-zero MV (Sequence: CADWaveform)

Figure 3  Proposed non-temporal prediction

# Simulation results

The performance evaluation of the proposed scheme has been conducted. The proposed scheme is implemented on top of MV-HEVC mode of HTM8.0 with modification to allow 4:0:0 format video coding, which is the anchor scheme of the coding experiments. The experimental conditions are subject to AHG8 recommendation configurations based on RExt common test conditions[4]. Screen contents in the AHG8 recommendation configurations are used as test sequences in this experiment. Average PSNR for calculation of BD-rate is shown as follows:

PSNRGBRm=PSNRG+PSNRB+PSNRR3.

Table 1 shows BD-rate performance of the proposed scheme relative to the anchor. According to the result, the proposed scheme achieves 18.0%-20.4%, 14.9%-17.2% and 11.4%-14.7% average BD-rate gains for AI, RA and LDB configurations, respectively. With regard to text and graphics contents, the proposed scheme has up to 27.2% average coding gain relative to the anchor. On the other hand, “sc_robot” has up to 10.3%, 5.5% and 2.2% coding loss for AI, RA and LDB case, respectively. The degree of the degradation is higher at low bitrate.

Table 1  BD-rate performance compared with HTM8.0

To investigate the degradation, we have additionally performed the simulation of the proposed scheme with disabling the restriction of inter-color MV. The coding performance of the modified scheme relative to HTM8.0 is shown in Table 2. According the result, it has been confirmed that the degradation of coding performance for several sequences is resolved by disabling the inter-color vector restriction. This means that inter-color motion search is useful for some sequences at low bitrate although such motion search has small impact on coding efficiency at high bitrate.

Table 2  BD-rate performance of the modified scheme relative to HTM8.0

Moreover, the coding performance of the proposed scheme with inter-color MV restriction relative to HEVC Range Extensions (RExt) has been evaluated. The coding result is shown in Table 3. The anchor is HM12.0+RExt4.1 with applying the following RExt-specific tool settings.

ExtendedPrecision                   : 0

IntraBlockCopyEnabled               : 0

TransformSkipLog2MaxSize            : 2

IntraResidualDPCM                   : 0

InterResidualDPCM                   : 0

ResidualRotation                    : 0

SingleSignificanceMapContext        : 0

IntraReferenceSmoothing             : 1

This table shows that the proposed scheme has about 40%, 32% and 27% coding gain on average relative to RExt for AI, RA and LDB configurations, respectively.

Table 3  BD-rate performance of the modified scheme relative to RExt4.1

# Conclusions

This contribution proposes a RGB coding scheme based on HEVC multi-view extensions. The proposed scheme introduces intra block copying and restricts motion vector of inter-color prediction to zero. It is reportedly confirmed that the coding performance of the proposed scheme achieves 20%, 16% and 14% average BD-rate gain relative to HTM8.0 for AI, RA and LDB configurations, respectively. We recommend studying the proposed scheme in appropriate AHG to improve coding performance of HEVC for screen contents.

# References

- G. Tech, K. Wegner, Y. Chen, M. Hannuksela and J. Boyce, “MV-HEVC Draft Text 5,” Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-E1004, 5th Meeting: Vienna, AT, July, 2013.

- A. Minezawa, S. Sekiguchi and T. Murakami, “AHG5/AHG8: RGB4:4:4 video coding using HEVC multi-view extensions,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-N0116, 14th Meeting: Vienna, AT, July, 2013.

- C. Pang, J. Sole, L. Guo, M. Karczewicz and R. Joshi, “Non-RCE3: Intra Motion Compensation with 2-D MVs,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-N0256, 14th Meeting: Vienna, AT, July, 2013.

- D. Flynn, K. Sharman and C. Rosewarne, “Common test conditions and software reference configurations for HEVC range extensions,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-N1006, 14th Meeting: Vienna, AT, July, 2013.

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).